| ETSI TC SCP Meeting #68 Tokyo, Japan, 16-17 April 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCP(15)000123 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #57 Munich, Germany, 23-27 March 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCPTEC(15)000075r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 241 |  |  |  | CR | 138 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 11.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | x | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Covering EVENT_EVENT_ACCESS_TECHNOLOGY_CHANGE_MULTIPLE in the Handler availability table |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI11 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 25/03/2015 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-11 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | CR 129 in SCP(11)0281r1 added the EVENT_EVENT_ACCESS_TECHNOLOGY_CHANGE_MULTIPLE without defining the Handler availability. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Defining the Handler availability for the event. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Interoperability Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 6.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.6	System handler management

The system handlers: ProactiveHandler, ProactiveResponseHandler, EnvelopeHandler and EnvelopeResponseHandler are Temporary JCRE Entry Point Object as defined in the "Runtime Environment Specification, Java Card™ Platform, 3.0.1 Classic Edition" [Error: Reference source not found].

A system handler is available if the exception ToolkitException. HANDLER_NOT_AVAILABLE is not thrown when the corresponding getTheHandler() method is called or a method of its interface is called.

A system handler shall not be available if the corresponding getTheHandler() method is not called, directly or indirectly, from the applet's processToolkit() method. If necessary and only when explicitly stated in another specification, the ProactiveHandler and the ProactiveResponseHandler may in addition be available if the corresponding getTheHandler() method was called from within a different method than processToolkit().

The following rules define the availability and the content of the system handlers. These are generic rules and may vary with the event that triggers the Toolkit applet. These rules apply also, if the ProactiveHandler or the ProactiveResponseHandler are available in a method different from processToolkit(). Under this condition, and for all following rules concerning the ProactiveHandler and the ProactiveResponseHandler, the method name "processToolkit()" has to be replaced by the method name in which the method getTheHandler() was called. The following rules concerning the ProactiveHandler and the ProactiveResponseHandler may in addition be modified by another specification.

ProactiveHandler:

- The ProactiveHandler shall not be available if the Terminal Profile command has not yet been processed by the CAT Runtime Environment.

- When available the ProactiveHandler shall remain available until the termination of the processToolkit() method.

- If a proactive command is pending the ProactiveHandler may not be available.

- At the processToolkit() method invocation the TLV-List is cleared.

- At the call of its init method the content is cleared and then initialized.

- After a call to ProactiveHandler.send() method the content of the handler shall not be modified by the CAT Runtime Environment.

- ProactiveResponseHandler:

- The ProactiveResponseHandler shall be available as soon as the ProactiveHandler is available, its TLV list shall be empty before the first call to the ProactiveHandler.send() method. Shall remain available until the termination of the processToolkit() method.

- The ProactiveResponseHandler shall not be available if the ProactiveHandler is not available.

- The ProactiveResponseHandler TLV list is filled with the simple TLV data objects of the last TERMINAL RESPONSE APDU command. The simple TLV data objects shall be provided in the order given in the TERMINAL RESPONSE command data.

- The ProactiveResponseHandler content shall be updated after each successful call to ProactiveHandler.send() method and shall remains unchanged until the next successful call to the ProactiveHandler.send() method.

- EnvelopeHandler:

- When available (as specified in table 1) the EnvelopeHandler shall remain available and its content shall remain unchanged from the invocation to the termination of the processToolkit() method.

- The EnvelopeHandler TLV list is filled with the simple TLV data objects of the ENVELOPE APDU command. The simple TLV data objects shall be provided in the order given in the ENVELOPE command data.

- EnvelopeResponseHandler:

- The EnvelopeResponseHandler is available (as specified in table 1) for all triggered Toolkit applets, until a Toolkit applet has posted an envelope response or sent a proactive command.

- After a call to the post() method the handler is no longer available.

- After the first invocation of the ProactiveHandler.send() method the EnvelopeResponseHandler is no more available.

- At the processToolkit() method invocation the TLV-List is cleared.

Table 2 describes the minimum availability of the handlers for all the events at the invocation of the processToolkit() method of the Toolkit applet.

Table 2: Handler availability for each event

| EVENT | Reply busy allowed (see note 2) | EnvelopeHandler | Envelope Response Handler | Nb of triggered/ registered applet |
| --- | --- | --- | --- | --- |
| _MENU_SELECTION | Y | Y | N | 1/n (per Item Id) |
| _MENU_SELECTION_HELP_REQUEST | Y | Y | N | 1/n (per Item Id) |
| _CALL_CONTROL_BY_NAA | N | Y | Y | 1/1 |
| _TIMER_EXPIRATION | Y | Y | N | 1/8 (per timer) (see note 1) |
| _EVENT_DOWNLOAD |  |  |  |  |
| _MT_CALL | Y | Y | N | n/n |
| _CALL_CONNECTED | Y | Y | N | n/n |
| _CALL_DISCONNECTED | Y | Y | N | n/n |
| _LOCATION_STATUS | Y | Y | N | n/n |
| _USER_ACTIVITY | Y | Y | N | n/n |
| _IDLE_SCREEN_AVAILABLE | Y | Y | N | n/n |
| _CARD_READER_STATUS | Y | Y | N | n/n |
| _LANGUAGE_SELECTION | Y | Y | N | n/n |
| _BROWSER_TERMINATION | Y | Y | N | n/n |
| _DATA_AVAILABLE | Y | Y | N | 1/7 (per channel) (see note 1) |
| _CHANNEL_STATUS | Y | Y | N | 1/7 (per channel) (see note 1) |
| _ACCESS_TECHNOLOGY_CHANGE | Y | Y | N | n/n |
| _ACCESS_TECHNOLOGY_CHANGE_MULTIPLE | Y | Y | N | n/n |
| _DISPLAY_PARAMETER_CHANGED | Y | Y | N | n/n |
| _NETWORK_SEARCH_MODE_CHANGE | Y | Y | N | n/n |
| _BROWSING_STATUS | Y | Y | N | n/n |
| _FRAMES_INFORMATION_CHANGED | Y | Y | N | n/n |
| _HCI_CONNECTIVITY | Y | Y | N | n/n |
| _LOCAL_CONNECTION | Y | Y | N | 1/8 (per service identifier) (see note 2) |
| _CONTACTLESS_STATE_REQUEST | Y | Y | N | n/n |
| _UNRECOGNIZED_ENVELOPE | Y | Y | Y | n/n |
| _STATUS_COMMAND | N | N | N | n/n |
| _PROFILE_DOWNLOAD | N | N | N | n/n |
| _PROACTIVE_HANDLER_AVAILABLE | N | N | N | n/n |
| _FIRST_COMMAND_AFTER_ATR | N | N | N | n/n |
| _EXTERNAL_FILE_UPDATE | N | Y | N | n/n |
| _APPLICATION_DESELECT | N | Y | N | n/n |
| _REMOTE_FILE_UPDATE | N | Y | N | n/n |
| NOTE 1: One Toolkit applet can register to several timers/channels/services identifier, but a timer/channel/services identifier can only be allocated to one Toolkit applet. NOTE 2: It is recommended to use ISOException with reason code 0x9300 only for events where reply busy is allowed. |  |  |  |  |