| ETSI TC SCP Meeting #68 Tokyo, Japan, 16-17 April 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCP(15)000124 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #57 Munich, Germany, 23-27 March 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCPTEC(15)0000071r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 241 |  |  |  | CR | 139 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 11.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Support of Poll Interval Negotiation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI12 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 25/03/2015 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-12 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | Support for POLL INTERVAL NEGOTIATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Add support for a new Event for poll interval negotiation accounting for Toolkit applets subscribing to the event and existing Toolkit applets that are not aware of this new feature. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 6.2, 6.6, 6.7.1.X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.2	Definition of events

The following events can trigger a Toolkit applet.

Table 1: UICC toolkit event list

| Event Name | Reserved short value |
| --- | --- |
| Not to be used | 0 |
| EVENT_PROFILE_DOWNLOAD | 1 |
| Reserved by 3GPP | 2 |
| Reserved by 3GPP | 3 |
| Reserved by 3GPP | 4 |
| Reserved by 3GPP | 5 |
| Reserved by 3GPP | 6 |
| EVENT_MENU_SELECTION | 7 |
| EVENT_MENU_SELECTION_HELP_REQUEST | 8 |
| EVENT_CALL_CONTROL_BY_NAA | 9 |
| Reserved by 3GPP | 10 |
| EVENT_TIMER_EXPIRATION | 11 |
| EVENT_EVENT_DOWNLOAD_MT_CALL | 12 |
| EVENT_EVENT_DOWNLOAD_CALL_CONNECTED | 13 |
| EVENT_EVENT_DOWNLOAD_CALL_DISCONNECTED | 14 |
| EVENT_EVENT_DOWNLOAD_LOCATION_STATUS | 15 |
| EVENT_EVENT_DOWNLOAD_USER_ACTIVITY | 16 |
| EVENT_EVENT_DOWNLOAD_IDLE_SCREEN_AVAILABLE | 17 |
| EVENT_EVENT_DOWNLOAD_CARD_READER_STATUS | 18 |
| EVENT_STATUS_COMMAND | 19 |
| EVENT_EVENT_DOWNLOAD_LANGUAGE_SELECTION | 20 |
| EVENT_EVENT_DOWNLOAD_BROWSER_TERMINATION | 21 |
| EVENT_EVENT_DOWNLOAD_DATA_AVAILABLE | 22 |
| EVENT_EVENT_DOWNLOAD_CHANNEL_STATUS | 23 |
| Reserved by 3GPP | 24 |
| EVENT_EVENT_DOWNLOAD_ACCESS_TECHNOLOGY_CHANGE | 25 |
| EVENT_EVENT_DOWNLOAD_DISPLAY_PARAMETER_CHANGED | 26 |
| EVENT_EVENT_DOWNLOAD_LOCAL_CONNECTION | 27 |
| EVENT_EVENT_DOWNLOAD_NETWORK_SEARCH_MODE_CHANGE | 28 |
| EVENT_EVENT_DOWNLOAD_BROWSING_STATUS | 29 |
| Reserved by 3GPP | 30 |
| Reserved by 3GPP | 31 |
| EVENT_EVENT_DOWNLOAD_HCI_CONNECTIVITY | 32 |
| Reserved by 3GPP | 33 |
| EVENT_EVENT_DOWNLOAD_FRAMES_INFORMATION_CHANGED | 34 |
| EVENT_EVENT_DOWNLOAD_CONTACTLESS_STATE_REQUEST | 35 |
| EVENT_EVENT_POLL_INTERVAL_NEGOTIATION | 36 |
| RFU | 37 to 118 |
| Reserved by 3GPP | 119 |
| Reserved by 3GPP | 120 |
| Reserved by 3GPP | 121 |
| Reserved by 3GPP | 122 |
| EVENT_PROACTIVE_HANDLER_AVAILABLE | 123 |
| EVENT_EXTERNAL_FILE_UPDATE | 124 |
| EVENT_REMOTE_FILE_UPDATE | 125 |
| EVENT_APPLICATION_DESELECT | 126 |
| EVENT_FIRST_COMMAND_AFTER_ATR | 127 |
| EVENT_EVENT_DOWNLOAD_ACCESS_TECHNOLOGY_CHANGE_MULTIPLE | 128 |
| EVENT_MEMORY_FAILURE | 129 |
| RFU | 130 to 32 767 |
| EVENT_UNRECOGNIZED_ENVELOPE | -1 |
| Reserved for Proprietary Use: - range for Card manufacturer proprietary events - range for Card Issuer proprietary events | - 2 to -64 -65 to -128 |
| RFU | -129 to -32 768 |

### ….

EVENT_MEMORY_FAILURE

If the UICC is provided with memory reliability monitoring mechanism (see clause 6.7.2), the CAT Runtime Environment shall trigger all the Toolkit applets registered to this event when the card operating system has detected an irrecoverable memory failure in any location of the persistent memory.

An irrecoverable memory failure is said to occur when a loss of data is detected or when it is no longer possible to write data to memory. The event is reported once per applet, during the lifecycle of the card.

The memory reliability monitoring mechanism and its implementation specific limitations are described in clause 6.7.2.

EVENT_POLL_INTERVAL_NEGOTIATION

Upon reception of an ENVELOPE (POLL INTERVAL NEGOTIATION) APDU command as defined in TS 102 221 [Error: Reference source not found] the CAT Runtime Environment shall trigger all Toolkit applets registered to this event.

Toolkit applets can request a poll interval shorter than the poll interval suggested by the Terminal by invoking the method uicc.toolkit.ToolkitRegistry.requestPollIntervall(short duration). Requests for longer values shall be ignored by the CAT Runtime Environment.

…..

## 6.6	System handler management

….

Table 2 describes the minimum availability of the handlers for all the events at the invocation of the processToolkit() method of the Toolkit applet.

Table 2: Handler availability for each event

| EVENT | Reply busy allowed (see note 2) | EnvelopeHandler | Envelope Response Handler | Nb of triggered/ registered applet |
| --- | --- | --- | --- | --- |
| _MENU_SELECTION | Y | Y | N | 1/n (per Item Id) |
| _MENU_SELECTION_HELP_REQUEST | Y | Y | N | 1/n (per Item Id) |
| _CALL_CONTROL_BY_NAA | N | Y | Y | 1/1 |
| _TIMER_EXPIRATION | Y | Y | N | 1/8 (per timer) (see note 1) |
| _EVENT_DOWNLOAD |  |  |  |  |
| _MT_CALL | Y | Y | N | n/n |
| _CALL_CONNECTED | Y | Y | N | n/n |
| _CALL_DISCONNECTED | Y | Y | N | n/n |
| _LOCATION_STATUS | Y | Y | N | n/n |
| _USER_ACTIVITY | Y | Y | N | n/n |
| _IDLE_SCREEN_AVAILABLE | Y | Y | N | n/n |
| _CARD_READER_STATUS | Y | Y | N | n/n |
| _LANGUAGE_SELECTION | Y | Y | N | n/n |
| _BROWSER_TERMINATION | Y | Y | N | n/n |
| _DATA_AVAILABLE | Y | Y | N | 1/7 (per channel) (see note 1) |
| _CHANNEL_STATUS | Y | Y | N | 1/7 (per channel) (see note 1) |
| _ACCESS_TECHNOLOGY_CHANGE | Y | Y | N | n/n |
| _DISPLAY_PARAMETER_CHANGED | Y | Y | N | n/n |
| _NETWORK_SEARCH_MODE_CHANGE | Y | Y | N | n/n |
| _BROWSING_STATUS | Y | Y | N | n/n |
| _FRAMES_INFORMATION_CHANGED | Y | Y | N | n/n |
| _HCI_CONNECTIVITY | Y | Y | N | n/n |
| _LOCAL_CONNECTION | Y | Y | N | 1/8 (per service identifier) (see note 2) |
| _CONTACTLESS_STATE_REQUEST | Y | Y | N | n/n |
| _POLL_INTERVAL_NEGOTIATION | N | Y | N | n/n |
| _UNRECOGNIZED_ENVELOPE | Y | Y | Y | n/n |
| _STATUS_COMMAND | N | N | N | n/n |
| _PROFILE_DOWNLOAD | N | N | N | n/n |
| _PROACTIVE_HANDLER_AVAILABLE | N | N | N | n/n |
| _FIRST_COMMAND_AFTER_ATR | N | N | N | n/n |
| _EXTERNAL_FILE_UPDATE | N | Y | N | n/n |
| _APPLICATION_DESELECT | N | Y | N | n/n |
| _REMOTE_FILE_UPDATE | N | Y | N | n/n |
| NOTE 1: One Toolkit applet can register to several timers/channels/services identifier, but a timer/channel/services identifier can only be allocated to one Toolkit applet. NOTE 2: It is recommended to use ISOException with reason code 0x9300 only for events where reply busy is allowed. |  |  |  |  |

…

#### 6.7.1.3	POLL INTERVAL and POLLING OFF

At the beginning of a CAT session, the CAT Runtime Environment shall send a POLL INTERVAL system proactive command, if at least one Toolkit applet has requested a poll interval duration.

During a CAT session the CAT Runtime Environment shall send a POLL INTERVAL or POLLING OFF system proactive command whenever the system poll interval duration is changed.

#### 6.7.1.X	NEGOTIATION OF POLL INTERVAL

If at least one Toolkit applet has registered the EVENT POLL INTERVAL NEGOTIATION the Terminal can send an ENVELOPE(POLL INTERVAL NEGOTIATION) at any time.

The CAT Runtime Environment shall send a response to the EVENT POLL INTERVAL NEGOTIATION as defined in TS 102 223 [7] according to the following rules:

If none of the triggered Toolkit applets has requested a shorter poll interval duration than the one supplied by the Terminal, and if none of the Toolkit applets that is not triggered on this event has previously requested a shorter poll interval duration than the one supplied by the Terminal, the CAT Runtime Environment shall respond with the "Poll interval result" set to "Accepted" as defined in TS 102 223 [7] and the "Duration" set to the new poll interval duration.

If one or more triggered applets requested a shorter poll interval duration than the poll interval duration value supplied by the Terminal, or if at least one of the Toolkit applets that is not triggered on this event has previously requested a shorter poll interval duration than the one supplied by the Terminal, the CAT Runtime Environment shall respond with the "Poll interval result" set to "Modified" and with the lowest poll interval value duration requested by the Toolkit applets set as "Duration" as defined in TS 102 223 [7].

The "Duration" sent in the response to the ENVELOPE(POLL INTERVAL NEGOTIATION) shall be persistent and is the new value of the poll interval duration used by the CAT Runtime Environment according to the rules defined in 6.7.1.3, and as the response to an invocation of the ToolkitRegistry.getPollInterval() method.

uicc.toolkit.ToolkitRegistry

public interface ToolkitRegistry {

....

/**

* Requests a duration for the EVENT_STATUS_COMMAND event of the registering

* Toolkit applet. Due to different durations requested by other Toolkit

* applets or due to restrictions of the Terminal, the CAT Runtime Environment may

* adjust another duration.

* This method can be used at every time to request a new duration.

*

* @param  duration specifies the number of seconds requested for proactive polling.

*			The maximum value of <code>duration</code> is <code>15300</code> (255 minutes).

*			If <code>duration<code> is equal to <code>POLL_NO_DURATION</code>,

*			the calling applet deregisters from EVENT_STATUS_COMMAND, and

*			the CAT Runtime Environment may issue the POLLING OFF proactive command.

*			If <code>duration<code> is equal to <code>POLL_SYSTEM_DURATION</code>,

*			the calling applet registers to the EVENT_STATUS_COMMAND and let

*			the CAT Runtime Environment define the duration.

*			If the method is invoked during the processing of the event

*			EVENT_POLL_INTERVAL_NEGOTIATION and if the requested value is longer than the

*			one suggested by the Terminal, then the requested value shall be ignored.

* @exception  ToolkitException      with the following reason codes: <ul>

*      <li>REGISTRY_ERROR 	 if <code>duration</code> is greater than the maximum value.</ul>

* @exception  TransactionException  if the operation would cause the commit capacity to be exceeded

*/

void requestPollInterval(short duration) throws ToolkitException, TransactionException;

/**

* Returns the adjusted duration for the calling

* Toolkit Applet in seconds.

* If the returned duration is equal to <code>POLL_NO_DURATION<code>, the Toolkit

* Applet is not registered to EVENT_STATUS_COMMAND event.

* The returned duration may :

*		- be a multiple of the real adjusted poll interval time at the Terminal.

* 		- differ from the requested duration due to request of other Toolkit

*				applets or due to restrictions of the Terminal.

*			- be the poll interval duration that was determined with the POLL INTERVAL NEGOTIATION mechanism.

*		- be changed during the card session due requests from other Toolkit applets.

*		- be wrong due to direct generation of the proactive command POLL INTERVAL or POLLING OFF.

* 		- not correspond to the elapsed time due to additional STATUS commands send by the Terminal.

*

* @return    the adjusted duration for the applet in seconds.

*/

short getPollInterval();

.....

}

…

/**

* <code>ToolkitConstants</code> encapsulates constants related to the Toolkit Applets.

*

*/

public interface ToolkitConstants {

...

/**

* Event: Contactless State Request = 35

*/

final static short EVENT_EVENT_DOWNLOAD_CONTACTLESS_STATE_REQUEST=(short)35;

/**

* Event: Poll Interval Negotiation = 36

*/

final static short EVENT_EVENT_POLL_INTERVAL_NEGOTIATION=(short)36;

}

public final class TerminalProfile {

...

/**

* Event Poll Interval Negotiation supported(i.e. class ad is supported) =(short)258

*/

public static final short IDX_EVENT_POLL_INTERVAL_NEGOTIATION = (short)258;

….

}