3GPP TSG-CT6#54 	C6-100091

- San Francisco, USA, 23-26 February 2010

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.130 | CR | 0047 |  rev | 1 |  Current version: | 9.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps | X | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Addition of missing constant values |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Sun Microsystems Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | C6 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI 9 |  |  |  |  |  |  | Date:  |  | 25/02/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | Several Tag values where not introduced in the API spec |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Adding missing BER-TLV tag value and Simple-TLV tag values as constant values |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | API specification not inline with 31.111 |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | Annex A: uicc.usim.toolkit.ToolkitConstants |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

public interface ToolkitConstants extends uicc.toolkit.ToolkitConstants {

/** Event : Envelope SMS-PP Data Download (31.115 formatted)  = 2        */

public static final short EVENT_FORMATTED_SMS_PP_ENV                 = (short)2;

/** Event : Update Record EF sms APDU     (31.115 formatted)  = 3        */

public static final short EVENT_FORMATTED_SMS_PP_UPD                 = (short)3;

/** Event : Envelope SMS-PP Data Download unformatted sms  = 4          */

public static final short EVENT_UNFORMATTED_SMS_PP_ENV               = (short)4;

/** Event : Update Record EFsms APDU      unformatted sms   = 5         */

public static final short EVENT_UNFORMATTED_SMS_PP_UPD               = (short)5;

/** Event : Cell Broadcast Data Download  = 6                           */

public static final short EVENT_UNFORMATTED_SMS_CB                   = (short)6;

/** Event : MO Short Message Control by NAA = 10.

*   Note: The constant EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM was

*   erroneously used instead of the constant

*   Event EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA

*   in earlier releases of this specification. Use constant

*   EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM in applications

*  which need to be compatible with earlier releases of this specification.

*/

public static final short EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA      = (short)10;

/** Note: The constant EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM was

erroneously used for the Event EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA

in earlier releases of this specification. The constant

EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM is maintained for backwards

compatiblity with applications which were developed for earlier

releases of this specification.

@deprecated Use EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA instead.*/

public static final short EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM      = (short)10;

/** Event : Cell Broadcast Data Download Formatted = 24                 */

public static final short EVENT_FORMATTED_SMS_CB                     = (short)24;

/** Event : USSD Data Download Formatted = 121                          */

public static final byte EVENT_FORMATTED_USSD                        = (short)121;

/** Event : USSD Data Download Unformatted = 122                        */

public static final byte EVENT_UNFORMATTED_USSD                      = (short)122;

/** Event : Event Download Network Rejection = XX                 */

public static final byte EVENT_EVENT_DOWNLOAD_NETWORK_REJECTION          = (short)31;

/** @deprecatedBER-TLV: Cell Broadcast download tag = 0xD2      */

public static final byte BTAG_CELL_BROADCATS_DOWNLOAD = (byte)0xD2;

/** BER-TLV: Cell Broadcast download tag = 0xD2      */

public static final byte BTAG_CELL_BROADCAST_DOWNLOAD = (byte)0xD2;

/** BER-TLV : MO short message control tag = 0xD5    */

public static final byte BTAG_MO_SHORT_MESSAGE_CONTROL = (byte)0xD5;

/** BER-TLV : SMS-PP download tag = 0xD1 */

public static final byte BTAG_SMS_PP_DOWNLOAD = (byte)0xD1;

/** BER-TLV : USSD download tag = 0xD9 */

public static final byte BTAG_USSD_DOWNLOAD = (byte)0xD9;

/** BER-TLV : Geographical Location Reporting tag = 0xDD */

public static final byte BTAG_GEOGRAPHICAL_LOCATION_REPORTING = (byte)0xDD;

/**  Type of proactive command : SEND SS = 0x11 */

public static final byte PRO_CMD_SEND_SS = (byte)0x11;

/** Type of proactive command : SEND USSD = 0x12 */

public static final byte PRO_CMD_SEND_USSD = (byte)0x12;

/** General Result : USSD or SS transaction terminated by user = 0x14 */

public static final byte RES_CMD_PERF_USSD_TRANSAC_TERM = (byte)0x14;

/** General Result : SS return error = 0x34 */

public static final byte RES_ERROR_SS_RETURN_ERROR = (byte)0x34;

/** General Result : SMS RP-ERROR = 0x35     */

public static final byte RES_ERROR_SMS_RP_ERROR = (byte)0x35;

/** General result: USSD return error = 0x37     */

public static final byte RES_ERROR_USSD_RETURN_ERROR = (byte)0x37;

/** General result: Interaction with call control by USIM or MO short message control by USIM, permanent problem = 0x39 */

public static final byte RES_CMD_CALL_MO_CONTROL_PROBLEM = (byte)0x39;

/** Simple-TLV : BC Repeat Indicator tag = 0x2A */

public static final byte TAG_BC_INDICATOR = (byte)0x2A;

/** Simple-TLV : BCCH Channel List tag = 0x1D */

public static final byte TAG_BCCH_CHANNEL_LIST = (byte)0x1D;

/** Simple-TLV : Cause tag = 0x1A */

public static final byte TAG_CAUSE = (byte)0x1A;

/** Simple-TLV : Cell Broadcast Page tag = 0x0C; */

public static final byte TAG_CELL_BROADCAST_PAGE = (byte)0x0C;

/** Simple-TLV : SMS TPDU tag = 0x0B */

public static final byte TAG_SMS_TPDU = (byte)0x0B;

/** Simple-TLV : SS String tag */

public static final byte TAG_SS_STRING = (byte)0x09;

/** Simple-TLV : Timing Advance tag = 0x2E */

public static final byte TAG_TIMING_ADVANCE = (byte)0x2E;

/** Simple-TLV : Transaction Identifier tag = 0x1C */

public static final byte TAG_TRANSACTION_IDENTFIER = (byte)0x1C;

/** Simple-TLV : USSD String tag = 0x0A */

public static final byte TAG_USSD_STRING = (byte)0x0A;

/** Simple-TLV : PDP context Activation parameters tag = 0x52 */

public static final byte TAG_PDP_CONTEXT_ACTIVATION_PARAMETERS = (byte)0x52;

/** Simple-TLV : UTRAN Measurement Qualifier tag = 0x69 */

public static final byte TAG_UTRAN_MEASUREMENT_QUALIFIER = (byte)0x69;

/** Simple-TLV : I-WLAN Identifier tag = 4A */

public static final byte TAG_I_WLAN_IDENTIFIER = (byte)0x4A;

/** Simple-TLV : I-WLAN Access Status tag = 4B */

public static final byte TAG_I_WLAN_ACCESS_STATUS = (byte)0x4B;

/** Simple-TLV : PLMNwAcT List tag = 72 */

public static final byte TAG_PLMN_WACT_LIST = (byte)0x72;

/** Simple-TLV : Routing Area Information tag = 73 */

public static final byte TAG_ROUTING_AREA_INFORMATION = (byte)0x73;

/** Simple-TLV Update/Attach type tag = 74 */

public static final byte TAG_UPDATE_ATTACH = (byte)0x74;

/** Simple-TLV Rejection Cause Code tag = 75 */

public static final byte TAG_REJECTION_CAUSE_CODE = (byte)0x75;

/** Simple-TLV Geographical Location Parameters tag = 76 */

public static final byte TAG_GEOGRAPHICAL_LOCATION_PARAMETERS = (byte)0x76;

/** Simple-TLV GAD shapes tag = 77 */

public static final byte TAG_GAD_SHAPES = (byte)0x77;

/** Simple-TLV NMEA sentence tag = 78 */

public static final byte TAG_NMEA_SENTENCE = (byte)0x78;

/** Simple-TLV PLMN List  tag = 79 */

public static final byte TAG_PLMN_LIST = (byte)0x79;

/** Simple-TLV EPS PDN connection Activation paramaters tag  = 7C */

public static final byte TAG_EPS_PDN_CONNECTION_ACTIVATION_PARAMETERS = (byte)0x7C;

/** Simple-TLV Tracking Area Identification tag  = 7D */

public static final byte TAG_TRACKING_AREA_IDENTIFICATION = (byte)0x7D;

}