| ETSI TC SCP Meeting #46 Bonn, Germany, 27 - 29 October 2010 |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(10)0286 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #34 Sophia Antipolis, France, 06-10 Sept 2010 |  |  |  |  |  |  |  |  |  |  |  | SCPTEC(10)0418r2 |  |  |  |  |  |  |  |  |  |  |  |
|  | EP SCP CR-Form-v1.7.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 102 705 |  |  |  | CR |  | 001 |  |  |  rev | 1 |  |  |  Current version: |  |  |  |  | 9.0.0 |  |  |  |
|  | For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed change affects:  |  |  |  |  | smart card |  |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
|  | Title:  |  | Clarification about restartReaderModeProcedure method use |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 08/09/2010 |  |  |  |  |  |
|  | Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-9 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) |  |  |  |  |  |  |
|  | Reason for change:  |  |  | There is no clear restriction, associated to the Applet life cycle, for the usage of the restartReadermodeProcedure method of the API. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Summary of change:  |  |  | State that if the life cycle of an Applet is not in the state ACTIVATED, the Contactless Framework shall not allow an applet to restart a Reader Mode procedure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Consequences if  not approved: |  |  | undefined behaviour of for an Applet that can restart a Reader Mode Procedure while DEACTIVATED. Incompatibility with GP2.2 life cycle management. Interoperability issues. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clauses affected:  |  |  | 4.3, Annex A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 4.3	Reader Mode

[...]

When the state of a reader mode Applet changes to lifecycle ACTIVATED (according to "GlobalPlatform Amendment C" [8]) the framework shall ensure that the HCI gates and pipes are setup for the RF technologies that are supported by the reader mode Applet.

To be able to receive and send messages over the contactless interface in Reader mode the Applet shall activate the ReaderListener.EVENT_TARGET_DISCOVERED. An Applet shall only be able to activate this event or to use the restartReadermodeProcedure method if it is in lifecycle state ACTIVATED. To release the CLF control at the end of a transaction an Applet shall deactivate the ReaderListener.EVENT_TARGET_DISCOVERED.

When an Applet lifecycle state changes from ACTIVATED to DEACTIVATED the Contactless Framework shall enforce that the ReaderListener.EVENT_TARGET_DISCOVERED is deactivated.

The Contactless Framework shall request the reader mode control on the CLF by sending the HCI events EVT_READER_REQUESTED and EVT_END_OPERATION according to the state of the reader mode Applet. The EVT_READER_REQUESTED shall be sent by the Contactless Framework if an Applet instance activates the event ReaderListener.EVENT_TARGET_DISCOVERED and no other Applet instance has the event activated, i.e. when the Contactless Framework has sent an EVT_READER_REQUESTED due to the request from another Applet instance, which was not yet ended by an EVT_END_OPERATION. The HCI event EVT_END_OPERATION shall be sent to the CLF when an Applet instance deactivates the event ReaderListener.EVENT_TARGET_DISCOVERED. The Contactless Framework shall resend the EVT_READER_REQUESTED to the CLF if another Applet instance exists with the ReaderListener.EVENT_TARGET_DISCOVERED event activated.

The Contactless Framework shall inform an Applet instance which has activated the ReaderListener.EVENT_TARGET_DISCOVERED when a target is discovered through one of the RF technologies the Applet instance is registered to with its installation parameters as specified in TS 102 226 [7].

The Contactless Framework shall ensure that the ReaderListener.EVENT_TARGET_DISCOVERED is deactivated for all Applets when access to the interface is disabled on the UICC level (cf. "state of contactless functionality in TS 102 223[6] and setCommunicationInterface() API method of "GlobalPlatform Amendment C" [8]).

ReaderMessage.java :

/**

* This method sends first the HCI event EVT_END_OPERATION and then the

* HCI event EVT_READER_REQUESTED.

*

* This method is intended to recover from the following error states:

* <li>reception of RESP_ANY_E_TIMEOUT

* <li>reception of RESP_WR_RF_ERROR

* <li>reception of EVT_TARGET_DISCOVERED with a MULTIPLE_TARGET_STATUS

* @throws HCIException with the following reason code:  <ul>

*      <li><code>HCI_CURRENTLY_DISABLED</code> if the HCI interface was disable

*     <li><code>HCI_CONDITIONS_NOT_SATISFIED </code> if the Applet is not ACTIVATED

*      </ul>

*/

public void restartReaderModeProcedure() throws HCIException;