| ETSI TC SCP Meeting #46 Bonn, Germany, 27 - 29 October 2010 |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(10)0289 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC #34 Sophia Antipolis, France, 06. - 10. Sept 2010 |  |  |  |  |  |  |  |  |  |  | SCPTEC(10)0415r2 |  |  |  |  |  |  |  |  |  |  |  |
|  | EP SCP CR-Form-v1.7.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 102 705 |  |  |  | CR | 004 |  |  |  rev | - |  |  |  Current version: |  |  |  |  | 9.0.0 |  |  |  |
|  | For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed change affects:  |  |  |  |  | smart card |  | x | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
|  | Title:  |  | Clarification of contactless framework behaviour when HCI event EVT_FIELD_OFF is received. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 08/09/2010 |  |  |  |  |  |
|  | Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-9 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) |  |  |  |  |  |  |
|  | Reason for change:  |  |  | 1. It is not clear if the 2nd last paragraph of ch. 4.2 refers to the HCI event EVT_FIELD_OFF only or to the HCI events EVT_FIELD_OFF and EVT_CARD_DEACTIVATED. 2. The last sentence of ch. 4.2 induces an undesired side effect: Once the onCallback() method with the event EVENT_FIELD_OFF has been invoked a contactless application would not be notified on any event until it is selected again with the next card emulation mode event. This rule effectively blocks all events related to the Connectivity Service and to the Reader Service because in these cases the application would usually not be in selected state. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Summary of change:  |  |  | The last two paragraphs are reworded in order to clarify the behaviour and in order to limit the rule to card emulation mode related events. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Consequences if  not approved: |  |  | Unexpected system behaviour, bad user experience and interoperability issues. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clauses affected:  |  |  | 4.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 4.2	Card Emulation Mode

[...]

If the HCI event EVT_FIELD_OFF or EVT_CARD_DEACTIVATED defined by the HCP specified in [4] is received by the Contactless Framework and the UICC is still powered, the Applet instance shall be deselected according to "GlobalPlatform Amendment C" [8] by invocation of the deselect() method.

When the HCI event EVT_FIELD_OFF is received and if the Applet instance has activated this event the Contactless Framework shall raise an EVENT_FIELD_OFF before the invocation of the deselect() method of the Applet instance.

After the deselection of the Applet instance, it shall not be invoked by any other event defined in the interface CardEmulationListener until the Applet instance is selected again.