| ETSI TC SCP Meeting #46 Bonn, Germany, 27 - 29 October 2010 |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(10)0309 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC #35 Sophia Antipolis, France, 04 - 07 Oct 2010 |  |  |  |  |  |  |  |  |  |  | SCPTEC(10)0424r3 |  |  |  |  |  |  |  |  |  |  |  |
|  | EP SCP CR-Form-v1.7.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 102 705 |  |  |  | CR | 006 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 9.0.0 |  |  |  |
|  | For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed change affects:  |  |  |  |  | smart card |  | x | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
|  | Title:  |  | Handling of HCI data link layer reset by HCI Framework |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work item name:  |  | HCI API |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 06/10/2010 |  |  |  |  |  |
|  | Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-9 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) |  |  |  |  |  |  |
|  | Reason for change:  |  |  | TS 102 705 refers to Java Card™ Platform 3.0.1 Classic Edition, Runtime Environment Specification" [10] for the basic behaviour of the HCI Framework. In clause “3.6 Power Loss and Reset” this specification incorrectly defines: “Power loss occurs under one of the following conditions: [...] ■ When operating in contactless-only mode, a card whose contactless interface is accessed through a contactless front-end using the European TelecommunicationsStandards Institute (ETSI) defined single wire protocol (SWP) standard (ETSI TS 102 613), is reset by a data link layer (SHDLC RSET) signal.” For “Concurrent Operations Over Multiple Interfaces” clause 6.1 adds: “The following conditions are deemed as a reset of the contactless I/O interface: [...] ■ A contactless interface which is accessed using the SWP interface is logically reset. [...] If the contactless interface, using the SWP interface standard, is being logically reset, the applet instances are explicitly deselected by calling the applicable deselect method. Otherwise, the instances are implicitly deselected and the deselect method is not called.” In TS 102 622 the “reset of the underlying data link layer” is only visible as a means to recover from a link layer protocol problem which, if successful, does not impact the application layer communication at all: “If a reset of the underlying data link layer occurs, fragments of a partially received message shall be discarded and a partially sent message shall be re-sent from the beginning.” (last sentence of clause 5) If such a reset is not successful an error needs to be reported to the Application which has initiated the communication or, in the case of a command from an external reader in CardEmulation Mode the reset needs to be ignored to allow the reader to react on the problem. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Summary of change:  |  |  | Changes: - Clarification that the occurrence of a reset of the HCI data link layer (SHDLC RSET) shall be transparent to the application layer if it is possible to recover from the problem without any loss of data and that an error shall be reported to the upper layer if the reset was not successful. - Addition of events EVENT_HCI_TRANSMISSION_FAILED and EVENT_ HCI_RECEPTION_FAILED to the onCallback() method of the HCIListener interface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Consequences if  not approved: |  |  | Unexpected system behaviour because transactions will aborted in case of a successful link layer reset. Specifically, Reader Mode Applets are not informed of a HCI layer problem and can not take the required actions. Misalignment between TS 102 622 and TS 102 705 because the HCI layer shall not continue the communication after a successful “reset of the underlying layer”. (Interoperability issue if the correction is only applied to a later Release because Applets need to be written differently.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clauses affected:  |  |  | 4.1, Annex A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

--------------------------  CHANGE 1 ---------------------------------

File: TS 102 705:

## 4.1	Architecture

The present document describes an API and a Contactless Framework that enables Java Card(TM) Platform based Applets, defined in [9], [10] and [11], to send and receive messages using the HCI protocol as specified in [4] and to act as contactless Applets. The Contactless Framework shall support card emulation mode and reader mode as specified in the HCI protocol [4] specification.

The API is event driven and based on the Observer/Listener pattern. Every HCI service is encapsulated by a dedicated Service interface. These Service interfaces shall allow the registration of Listener Interfaces and the activation of events. The Listener Interfaces shall be implemented by Java objects to receive HCI messages and events in the onCallback method. The Registration of Listener Interfaces and activation of events shall be persistent.

An HCIMessage object shall encapsulates one HCP message according to the HCI protocol as specified in [4]. HCI message for the different contactless modes shall be identified by different types of interfaces. It is not guaranteed that any Applet originated HCI messages are sent before the completion of the execution of the current Applet.

Any onCallback() method of a Listener interface shall not be invoked again while another onCallback() method is still being executed. The Contactless Framework shall be able to receive one or more HCI messages while waiting for a response related to a command originated by the Applet (e.g. processing a request for parameters) especially for the EVT_FIELD_OFF case.

The HCI event EVT_FIELD_OFF shall be buffered and sent by the Contactless Framework as soon as the Contactless Framework becomes the current context.

All other HCI messages shall be delivered to the Applet instance in the same order as they were received by the Contactless Framework.

As stated in TS 102 622 [4], a reset of the underlying data link layer shall be transparent to the application layer if the data link layer recovers without any loss of data. In the case of a data loss an error shall be reported back to the Application which has initiated the data transmission. In case of a communication in CardEmulation Mode where a command from an external reader can not be forwarded to an Application due to a data link layer problem the Contactless Framework shall not take any action but wait for the reader to resolve the problem.

--------------------------  CHANGE 2 ---------------------------------

File: HCIListener.java

/******************************************************************************

*

******************************************************************************/

package uicc.hci.framework;

import javacard.framework.Shareable;

/** The base interface for all types of HCI listeners.

* <P>

* An Object shall not implement this interface but only implement one of the subinterfaces.

*

* <P>

* An Applet instance implementing the <code>*Listener</code> interface shall only receive events defined in the same <code>*Listener</code> interface.

* <P>

* Messages received through the inherited method <code>HCIListener</code> shall be the message type defined in the same package as the <code>*Listener</code> interface

* <P>

*

* @see uicc.hci.services.cardemulation.CardEmulationListener

* @see uicc.hci.services.connectivity.ConnectivityListener

* @see uicc.hci.services.readermode.ReaderListener

**/

public interface HCIListener extends Shareable {

/**

* This value is used to notify the Applet when the UICC failed to transmit a message to the CLF.

*/

public final static byte EVENT_HCI_TRANSMISSION_FAILED = (byte) 0x81;

/**

* This value is used to notify the Applet when the UICC failed to receive a message sent by the CLF.

*/

public final static byte EVENT_HCI_RECEPTION_FAILED = (byte) 0x82;

/** This method is called by the HCI framework to inform the Listener

*  Object about a specific event and pass the corresponding <code>HCIMessage</code> to

*  the Listener Object.

*

* In the case of an <code>EVENT_HCI_TRANSMISSION_FAILED</code> the message

* which could not be transmitted shall be made available in the <code>HCIMessage</code> object.

*

* If an Applet has requested the framework to send more than one message during a single

* <code>onCallback</code> method execution and a transmission to the CLF failed the framework

* shall not attempt to send any of the remaining messages.

*

* The <code>EVENT_HCI_RECEPTION_FAILED</code> shall not be raised in the case of a

* failure to receive a command from an external reader in CardEmulation Mode due to a problem

* on the data link layer.

*

*

* @param  event that triggers the invocation of the method

* @param  message the HCIMessage

*/

public void onCallback(byte event, HCIMessage message);

}