3GPP TSG CT #52	CP-110507

- Bratislava. SLOVAKIA, 1st – 3rd June 2011

| CR-Form-v9.9 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.130 | CR | 0050 |  rev | 1 |  Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps | X | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Addition of events and reservation of constant values for Java API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Gemalto, Oberthur Technologies, Sagem Orga, Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG:  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI10 |  |  |  |  |  |  | Date:  |  | 31/05/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Addition of events and reservation of constant values for Java API, to implement new features implemented in core spec TS 31.111 rel-10 (CRs C6-100645, C6-110155, C6-110167). The CR is proposed as a cat B for rel-10, since it is an alignment |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Introduction of new events - Event Download - IMS registration event - Event Download - Incoming IMS data event Reservation of constant values for Terminal Profile and new TAG constant values (javadoc) |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Features implemented in core spec TS 31.111 rel 10 can not be implemented |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.2, 6.6, Annex A (ToolkitConstants.java ; USATTerminalProfile.java) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

## 6.2	Definition of Events

The following events can trigger a Toolkit Applet in addition to the events defined in ETSI TS 102 241 [2], all short values are reserved in ETSI TS 102 241 [2]:

Table 1: (U)SAT event list

| Event Name | Reserved short value |
| --- | --- |
| EVENT_FORMATTED_SMS_PP_ENV | 2 |
| EVENT_FORMATTED_SMS_PP_UPD | 3 |
| EVENT_UNFORMATTED_SMS_PP_ENV | 4 |
| EVENT_UNFORMATTED_SMS_PP_UPD | 5 |
| EVENT_UNFORMATTED_SMS_CB | 6 |
| EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA | 10 |
| EVENT_FORMATTED_SMS_CB | 24 |
| EVENT_EVENT_DOWNLOAD_IWLAN_ACCESS_STATUS | 30 |
| EVENT_EVENT_DOWNLOAD_NETWORK_REJECTION | 31 |
| EVENT_EVENT_DOWNLOAD_CSG_CELL_SELECTION | 33 |
| EVENT_FORMATTED_USSD | 121 |
| EVENT_UNFORMATTED_USSD | 122 |
| EVENT_EVENT_DOWNLOAD_IMS_REGISTRATION | XX |
| EVENT_EVENT_DOWNLOAD_INCOMING_IMS_DATA | YY |

[…]

EVENT_EVENT_DOWNLOAD_IWLAN_ACCESS_STATUS

EVENT_EVENT_DOWNLOAD_NETWORK_REJECTION

EVENT_EVENT_DOWNLOAD_CSG_CELL_SELECTION

EVENT_EVENT_DOWNLOAD_IMS_REGISTRATION

EVENT_EVENT_DOWNLOAD_INCOMING_IMS_DATA

Upon reception of an ENVELOPE (Event Download) APDU command as defined in TS 31.111 [7] the (U)SAT Framework shall trigger all the Toolkit applets registered to the corresponding event.

## 6.6	System Handler management

[…]

Table 2: Handler availability for each event

| EVENT_ | Reply busy allowed | EnvelopeHandler / USATEnvelopeHandler | EnvelopeResponseHandler | Nb of triggered / registrered Applet |
| --- | --- | --- | --- | --- |
| _FORMATTED_SMS_PP_ENV | Y (see Note 1) | Y | Y | 1 / n (per TAR) |
| _FORMATTED_SMS_PP_UPD | N | Y | N | 1 / n (per TAR) |
| _UNFORMATTED_SMS_PP_ENV | Y | Y | Y | n / n |
| _UNFORMATTED_SMS_PP_UPD | N | Y | N | n / n |
| _FORMATTED_SMS_CB | Y | Y | N | 1/n (per TAR) |
| _UNFORMATTED_SMS_CB | Y | Y | N | n / n |
| _MO_SHORT_MESSAGE_CONTROL_BY_NAA | N | Y | Y | 1 / 1 |
| _FORMATTED_USSD | Y | Y | Y | 1 / n (per TAR) |
| UNFORMATTED_USSD | Y | Y | Y | n / n |
| EVENT_DOWNLOAD |  |  |  |  |
| _IWLAN_ACCESS_STATUS | Y | Y | N | n/n |
| _NETWORK_REJECTION | Y | Y | N | n/n |
| _IMS_REGISTRATION | Y | Y | N | n/n |
| _INCOMING_IMS_DATA | Y | Y | N | n/n |
| NOTE 1: The framework may reply busy and not trigger the toolkit applet if e.g. a PoR using SMS SUBMIT is required in the incoming message and a proactive session is ongoing. |  |  |  |  |

## Annex A - ToolkitConstants.java

package uicc.usim.toolkit;

//import uicc.toolkit.ToolkitConstants;

/**

*  <code>ToolkitConstants</code> encapsulates constants related to the USAT Toolkit applets.

*

*/

public interface ToolkitConstants extends uicc.toolkit.ToolkitConstants {

/** Event : Envelope SMS-PP Data Download (31.115 formatted)  = 2        */

public static final short EVENT_FORMATTED_SMS_PP_ENV                 = (short)2;

/** Event : Update Record EF sms APDU     (31.115 formatted)  = 3        */

public static final short EVENT_FORMATTED_SMS_PP_UPD                 = (short)3;

/** Event : Envelope SMS-PP Data Download unformatted sms  = 4          */

public static final short EVENT_UNFORMATTED_SMS_PP_ENV               = (short)4;

/** Event : Update Record EFsms APDU      unformatted sms   = 5         */

public static final short EVENT_UNFORMATTED_SMS_PP_UPD               = (short)5;

/** Event : Cell Broadcast Data Download  = 6                           */

public static final short EVENT_UNFORMATTED_SMS_CB                   = (short)6;

/** Event : MO Short Message Control by NAA = 10.

*   Note: The constant EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM was

*   erroneously used instead of the constant

*   Event EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA

*   in earlier releases of this specification. Use constant

*   EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM in applications

*  which need to be compatible with earlier releases of this specification.

*/

public static final short EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA      = (short)10;

/** Note: The constant EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM was

erroneously used for the Event EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA

in earlier releases of this specification. The constant

EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM is maintained for backwards

compatiblity with applications which were developed for earlier

releases of this specification.

@deprecated Use EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA instead.*/

public static final short EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM      = (short)10;

/** Event : Cell Broadcast Data Download Formatted = 24                 */

public static final short EVENT_FORMATTED_SMS_CB                     = (short)24;

/** Event : USSD Data Download Formatted = 121                          */

public static final byte EVENT_FORMATTED_USSD                        = (short)121;

/** Event : USSD Data Download Unformatted = 122                        */

public static final byte EVENT_UNFORMATTED_USSD                      = (short)122;

/** Event : Event Download Network Rejection = 31                 */

public static final byte EVENT_EVENT_DOWNLOAD_NETWORK_REJECTION          = (short)31;

/** Event: Event Download CSG Cell Selection event = 33 */

public static final byte EVENT_EVENT_DOWNLOAD_CSG_CELL_SELECTION_EVENT = (short)33;

/** Event: Event Download IMS Registration = XX */

public static final byte EVENT_EVENT_DOWNLOAD_IMS_REGISTRATION = (short)XX;

/** Event: Event Download Incoming IMS Data = YY */

public static final byte EVENT_EVENT_DOWNLOAD_INCOMING_IMS_DATA = (short)YY;

/** @deprecatedBER-TLV: Cell Broadcast download tag = 0xD2      */

public static final byte BTAG_CELL_BROADCATS_DOWNLOAD = (byte)0xD2;

/** BER-TLV: Cell Broadcast download tag = 0xD2      */

public static final byte BTAG_CELL_BROADCAST_DOWNLOAD = (byte)0xD2;

/** BER-TLV : MO short message control tag = 0xD5    */

public static final byte BTAG_MO_SHORT_MESSAGE_CONTROL = (byte)0xD5;

/** BER-TLV : SMS-PP download tag = 0xD1 */

public static final byte BTAG_SMS_PP_DOWNLOAD = (byte)0xD1;

/** BER-TLV : USSD download tag = 0xD9 */

public static final byte BTAG_USSD_DOWNLOAD = (byte)0xD9;

/** BER-TLV : Geographical Location Reporting tag = 0xDD */

public static final byte BTAG_GEOGRAPHICAL_LOCATION_REPORTING = (byte)0xDD;

/**  Type of proactive command : SEND SS = 0x11 */

public static final byte PRO_CMD_SEND_SS = (byte)0x11;

/** Type of proactive command : SEND USSD = 0x12 */

public static final byte PRO_CMD_SEND_USSD = (byte)0x12;

/** General Result : USSD or SS transaction terminated by user = 0x14 */

public static final byte RES_CMD_PERF_USSD_TRANSAC_TERM = (byte)0x14;

/** General Result : SS return error = 0x34 */

public static final byte RES_ERROR_SS_RETURN_ERROR = (byte)0x34;

/** General Result : SMS RP-ERROR = 0x35     */

public static final byte RES_ERROR_SMS_RP_ERROR = (byte)0x35;

/** General result: USSD return error = 0x37     */

public static final byte RES_ERROR_USSD_RETURN_ERROR = (byte)0x37;

/** General result: Interaction with call control by USIM or MO short message control by USIM, permanent problem = 0x39 */

public static final byte RES_CMD_CALL_MO_CONTROL_PROBLEM = (byte)0x39;

/** Simple-TLV : BC Repeat Indicator tag = 0x2A */

public static final byte TAG_BC_INDICATOR = (byte)0x2A;

/** Simple-TLV : BCCH Channel List tag = 0x1D */

public static final byte TAG_BCCH_CHANNEL_LIST = (byte)0x1D;

/** Simple-TLV : Cause tag = 0x1A */

public static final byte TAG_CAUSE = (byte)0x1A;

/** Simple-TLV : Cell Broadcast Page tag = 0x0C; */

public static final byte TAG_CELL_BROADCAST_PAGE = (byte)0x0C;

/** Simple-TLV : SMS TPDU tag = 0x0B */

public static final byte TAG_SMS_TPDU = (byte)0x0B;

/** Simple-TLV : SS String tag */

public static final byte TAG_SS_STRING = (byte)0x09;

/** Simple-TLV : Timing Advance tag = 0x2E */

public static final byte TAG_TIMING_ADVANCE = (byte)0x2E;

/** Simple-TLV : Transaction Identifier tag = 0x1C */

public static final byte TAG_TRANSACTION_IDENTFIER = (byte)0x1C;

/** Simple-TLV : USSD String tag = 0x0A */

public static final byte TAG_USSD_STRING = (byte)0x0A;

/** Simple-TLV : PDP context Activation parameters tag = 0x52 */

public static final byte TAG_PDP_CONTEXT_ACTIVATION_PARAMETERS = (byte)0x52;

/** Simple-TLV : UTRAN Measurement Qualifier tag = 0x69 */

public static final byte TAG_UTRAN_MEASUREMENT_QUALIFIER = (byte)0x69;

/** Simple-TLV : CSG ID list Tag = 0x7E */

public final static byte TAG_CSG_ID_LIST = (byte)0x7E;

/** Simple-TLV : CSG cell selection status =  0x55 */

public static final byte TAG_CSG_SELECTION_STATUS = (byte)55;

/** Simple-TLV : CSG ID = 0x56 */

public static final byte TAG_CSG_ID = (byte)0x56;

/** Simple-TLV : HNB name = 0x57 */

public static final byte TAG_HNB_NAME = (byte)0x57 ;

/** Simple-TLV : I-WLAN Identifier tag = 4A */

public static final byte TAG_I_WLAN_IDENTIFIER = (byte)0x4A;

/** Simple-TLV : I-WLAN Access Status tag = 4B */

public static final byte TAG_I_WLAN_ACCESS_STATUS = (byte)0x4B;

/** Simple-TLV : PLMNwAcT List tag = 72 */

public static final byte TAG_PLMN_WACT_LIST = (byte)0x72;

/** Simple-TLV : Routing Area Information tag = 73 */

public static final byte TAG_ROUTING_AREA_INFORMATION = (byte)0x73;

/** Simple-TLV Update/Attach type tag = 74 */

public static final byte TAG_UPDATE_ATTACH = (byte)0x74;

/** Simple-TLV Rejection Cause Code tag = 75 */

public static final byte TAG_REJECTION_CAUSE_CODE = (byte)0x75;

/** Simple-TLV Geographical Location Parameters tag = 76 */

public static final byte TAG_GEOGRAPHICAL_LOCATION_PARAMETERS = (byte)0x76;

/** Simple-TLV GAD shapes tag = 77 */

public static final byte TAG_GAD_SHAPES = (byte)0x77;

/** Simple-TLV NMEA sentence tag = 78 */

public static final byte TAG_NMEA_SENTENCE = (byte)0x78;

/** Simple-TLV PLMN List  tag = 79 */

public static final byte TAG_PLMN_LIST = (byte)0x79;

/** Simple-TLV EPS PDN connection Activation paramaters tag  = 7C */

public static final byte TAG_EPS_PDN_CONNECTION_ACTIVATION_PARAMETERS = (byte)0x7C;

/** Simple-TLV Tracking Area Identification tag  = 7D */

public static final byte TAG_TRACKING_AREA_IDENTIFICATION = (byte)0x7D;

/** Simple-TLV IMS Request-URI = 31 */

public static final byte TAG_IMS_REQUEST_URI = (byte) 0x31 ;

/** Simple-TLV IARI = 76 */

public static final byte TAG_IARI = (byte) 0x76 ;

/** Simple-TLV IMPU List = 77 */

public static final byte TAG_IMPU_LIST = (byte) 0x77 ;

/** Simple-TLV IMS Status Code = 78 */

public static final byte TAG_IMS_STATUS_CODE = (byte) 0x78 ;

}

## Annex A - USATTerminalProfile.java

//-----------------------------------------------------------------------------

// PACKAGE DEFINITION

//-----------------------------------------------------------------------------

package uicc.usim.toolkit;

//-----------------------------------------------------------------------------

// IMPORTS

//-----------------------------------------------------------------------------

/**

* The <code>USATTerminalProfile</code> interface contains constants for the Terminal Profile according to TS 51.014 and to TS 31.111

*

* @see uicc.toolkit.TerminalProfile

*/

public interface USATTerminalProfile {

/**

* SMS PP Data Download = 1

*/

public final static short IDX_SMS_PP_DATA_DOWNLOAD = (short) 1;

/**

* Cell Broadcast Data Download = 2

*/

public final static short IDX_CELL_BROADCAST_DATA_DOWNLOAD = (short) 2;

/**

* '9EXX' response code for SIM data download error = 4

*/

public final static short IDX_9EXX_RESPONSE = (short)4;

/**

* USSD string data object supported in Call Control = 6

*/

public static final short IDX_USSD_SUPPORT_IN_CALL_CONTROL = (short)6;

/**

* Envelope Call Control always sent to the SIM during automatic redial mode = 7

*/

public final static short IDX_CALL_CONTROLL_AUTOMATIC_REDIAL_MODE = (short) 7;

/**

* Call Control by SIM = 9

*/

public final static short IDX_CALL_CONTROL_BY_SIM = (short)9;

/**

* Cell identity included in Call Control by SIM = 10

*/

public final static short IDX_CELL_IDENTITY_IN_CALL_CONTROL_BY_SIM = (short)10;

/**

* MO short message control by SIM  = 11

*/

public final static short IDX_MO_SM_CONTROL_BY_SIM = (short)11;

/**

* Handling of the alpha identifier  = 12

*/

public static final short IDX_HANDLING_OF_ALPHA_IDENTIFIER = (short)12;

/**

* Proactive SIM: SEND SHORT MESSAGE = 25

*/

public static final short IDX_PROACTIVE_SIM_SEND_SHORT_MESSAGE = (short)25;

/**

* Proactive SIM: SEND SS  = 26

*/

public final static short IDX_PROACTIVE_SIM_SEND_SS = (short) 26;

/**

* Proactive SIM: SEND USSD = 27

*/

public final static short IDX_PROACTIVE_SIM_SEND_USSD = (short)27;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION - NMR = 31

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_NMR = (short)31;

/**

* Binary choice in GET INKEY = 59

*/

public final static short IDX_BINARY_CHOICE_IN_GET_INKEY = (short)59;

/**

* 2nd alpha identifier in SET UP CALL = 62

*/

public final static short IDX_2ND_ALPHA_IDENTIFIER_SET_UP_CALL = (short)62;

/**

* 2nd capability configuration parameter = 63

*/

public static final short IDX_2ND_CAPABILITY_PARAMETER = (short)63;

/**

* Proactive SIM: PROVIDE LOCAL INFORMATION - BCCH  = 66

*/

public final static short IDX_PROACTIVE_SIM_PROVIDE_LOCAL_INFORMATION_BCCH = (short) 66;

/**

* Proactive SIM: PROVIDE LOCAL INFORMATION (Timing Advance)  = 68

*/

public final static short IDX_PROACTIVE_SIM_TIMING_ADVANCE = (short) 68;

/**

* CALL Control on GPRS = 141

*/

public final static short IDX_CALL_CONTROL_ON_GPRS = (short) 141;

/**

* Support of UTRAN PS with extended parameters = 168

*/

public final static short IDX_EXTENDED_PARAMETERS_IN_UTRAN_PS_ = (short) 168;

/**

* Geographical Location Reporting = 181

*/

public final static short IDX_GEOGRAPHICAL_LOCATION_REPORTING = (short)181 ;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION NMR(UTRAN) *Correct Version* = 182

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_NMR_UTRAN_COR = (short) 182;

/**

* USSD Data Download and application mode *Correct Version* = 183

*/

public final static short IDX_USSD_DATA_DOWNLOAD_AND_APPLICATION_MODE_COR = (short) 183;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION NMR(UTRAN) = 173

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_NMR_UTRAN = (short) 173;

/**

* USSD Data Download and application mode= 174

*/

public final static short IDX_USSD_DATA_DOWNLOAD_AND_APPLICATION_MODE = (short) 174;

/**

* Event: I-WLAN Access Status = 195

*/

public final static short IDX_EVENT_I_WLAN_ACCESS_STATUS = (short)195;

/**

* Event: Network Rejection for GERAN/UTRAN = 196

*/

public final static short IDX_EVENT_NETWORK_REJECTION_GERAN_UTRAN = (short)196 ;

/**

* Event: Network Rejection for GERAN/UTRAN = 198

*/

public final static short IDX_EVENT_NETWORK_REJECTION_E_UTRAN = (short)198;

/**

* Event: CSG Cell Selection  = 199;

*/

public final static short IDX_EVENT_CSG_CELL_SELECTION = (short)199;

/**

* I-WLAN bearer support = 232

*/

public final static short IDX_I_WLAN_BEARER_SUPPORT = (short)232;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION (WSID of the current I-WLAN connection) = 233

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_WSID_I_WLAN = (short)233;

/**

* "Steering of Roaming" REFRESH support = 235

*/

public final static short IDX_STEERING_OF_ROAMING_REFRESH = (short)235;

/**

* Proactive UICC: Geographical Location Request = 237

*/

public final static short IDX_PROACTIVE_UICC_GEOGRAPHICAL_LOCATION_REQUEST = (short)237;

/**

* "Steering of Roaming for I-WLAN" REFRESH support = 239

*/

public final static short IDX_STEERING_OF_ROAMING_FOR_I_WLAN_REFRESH = (short)239;

/**

*  Support for CSG cell discovery = 241;

*/

public final static short IDX_CSG_CELL_DISCOVERY = (short) 241;

/**

* Communication Control for IMS = 243

*/

public final static short IDX_COMMUNICATION_CONTROL_FOR_IMS = (short) 243 ;

/**

* Support for Incoming IMS Data event = 245

*/

public final static short IDX_INCOMING_IMS_DATA_EVENT = (short) 245 ;

/**

* Support for IMS Registration event = 246

*/

public final static short IDX_IMS_REGISTRATION_EVENT = (short) 246 ;

}