3GPP TSG CT6 Meeting #49 	C6-080455

- Shanghai, P.R. China, 11-14 November 2008

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.130 | CR | 0034 |  rev | 2 |  Current version: | 7.7.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps | x | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Introduction of a geographical location discovery Java Card™ API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Gemalto, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | C6 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI8 |  |  |  |  |  |  | Date:  |  | 12/11/2008 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | There is a clear need to hide the complexity of positioning methods to Java applets inside the UICC. It shall be possible for applets to get the position from the USIM Java framework using a simple API. Therefore, in order to avoid fragmentation with proprietary APIs, it is necessary to extend the USIM Java API. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Introduction of a geographical location discovery API |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2; X (new section); Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 220: "Integrated Circuit Cards (ICC); ETSI numbering system for telecommunication; Application providers (AID)".

[2]	ETSI TS 102 241 Release 7: "UICC API for Java Card™"

[3]	3GPP TS 31.102: "Characteristics of the USIM Application".

[4]	3GPP TS 51.011 Release 4: "Specification of the Subscriber Identity Module- Mobile Equipment (SIM – ME) interface".

[5]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[6]	3GPP TS 31.101: "UICC-terminal interface; Physical and logical characteristics".

[7]	3GPP TS 31.111: "USIM Application Toolkit (USAT)".

[8]	3GPP TS 51.014 Release 4: "Specification of the SIM Application Toolkit for the Subscriber Identity Module – Mobile Equipment (SIM – ME) interface".

[9]	3GPP TS 31.115: "Secured packet structure for the (U)SIM Toolkit applications".

[10]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[11]	Sun Microsystems Java Card™ Specification: "Java Card™ 2.2.2 Application Programming Interface".

[12]	Sun Microsystems Java Card™ Specification : "Java Card™ 2.2.2 Runtime Environment (JCRE) Specification".

[13]	Sun Microsystems  Java Card™ Specification: "Java Card™ 2.2.2 Virtual Machine Specification".

Note:	SUN Java Card™ Specifications can be downloaded at http://java.sun.com/products/javacard

[14]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

# X	Geo Location API

The Geo Location API consists of the package uicc.usim.geolocation. This package defines services to allow an Applet to perform a geographical location operation, depending of the ME capabilities. When a geographical location operation is requested, the API will follow a defined way  to choose  either  "Geographical Location Request" toolkit command or "Provide Local Information" toolkit command as defined in TS 31.111 [7] to determine the location information. The result is formatted using GAD shapes as defined in TS 23.032 [14].

## Annex A: files for GeoLocation.java, GeoLocationInfo.java, GeoLocationBuilder.java, GeoLocationException.java

package uicc.usim.geolocation;

import uicc.toolkit.*;

/**

* This interface defines services allowing an application to get information on the ME current geographical location.

* Depending on card and terminal capabilities, different means may be used to compute geographical location.

*

* <p>see 3GPP TS 23.032 [14] Universal Geographical Area Description (GAD)

*/

public interface GeoLocation {

/**

* Horizontal accuracy: best effort

*/

public static final byte HORIZONTAL_ACCURACY_BEST_EFFORT = (byte)0x81 ;

/**

* Vertical coordinate: not requested

*/

public static final byte VERTICAL_COORDINATE_NOT_REQUESTED = (byte)0x80 ;

/**

* Vertical coordinate: best effort

*/

public static final byte VERTICAL_COORDINATE_REQUESTED_BEST_EFFORT = (byte)0x81 ;

/**

* Horizontal velocity requested

*/

public static final byte HORIZONTAL_VELOCITY_REQUESTED = (byte)0x01 ;

/**

* Vertical velocity requested in addition to horizontal velocity

*/

public static final byte VERTICAL_VELOCITY_REQUESTED = (byte)0x03 ;

/**

* Uncertainty of horizontal velocity requested in addition to horizontal velocity

*/

public static final byte UNCERTAINTY_OF_HORIZONTAL_VELOCITY_REQUESTED = (byte)0x05 ;

/**

* Uncertainty of vertical velocity requested in addition to horizontal and vertical velocity

*/

public static final byte UNCERTAINTY_OF_VERTICAL_VELOCITY_REQUESTED = (byte)0x11 ;

/**

* GAD shape: Ellipsoid point

*/

public static final byte GAD_SHAPES_ELLIPSOID_POINT = (byte)0x01 ;

/**

* GAD shape: Ellipsoid point with uncertainty circle

*/

public static final byte GAD_SHAPES_ELLIPSOID_POINT_WITH_UNCERTAINTY_CIRCLE = (byte)0x02 ;

/**

* GAD shape: Ellipsoid point with uncertainty ellipse

*/

public static final byte GAD_SHAPES_ELLIPSOID_POINT_WITH_UNCERTAINTY_ELLIPSE = (byte)0x04 ;

/**

* GAD shape: Ellipsoid point with altitude

*/

public static final byte GAD_SHAPES_ELLIPSOID_POINT_WITH_ALTITUDE = (byte)0x08 ;

/**

* GAD shape: Polygon

*/

public static final byte GAD_SHAPES_POLYGON = (byte)0x10 ;

/**

* GAD shape: Ellipsoid point with altitude and uncertainty ellipsoid

*/

public static final byte GAD_SHAPES_ELLIPSOID_POINT_WITH_ALTITUDE_AND_UNCERTAINTY_ELLIPSOID = (byte)0x20 ;

/**

* GAD shape: Ellipsoid arc

*/

public static final byte GAD_SHAPES_ELLIPSOID_ARC = (byte)0x40;

/**

* Set horizontal accuracy parameter for this object as defined in 3GPP TS 31.111 [7].

*/

public void setHorizontalAccuracy(byte horizontalAccuracy) ;

/**

* Set vertical coordinate parameter for this object as defined in 3GPP TS 31.111 [7].

*/

public void setVerticalCoordinate(byte verticalCoordinate) ;

/**

* Set velocity parameter for this object as defined in 3GPP TS 31.111 [7].

*/

public void setVelocity(byte velocity) ;

/**

* Set accepted GAD shapes parameter for this object as defined in 3GPP TS 31.111 [7].

*/

public void setAcceptedGADShapes(byte acceptedGADShapes) ;

/**

* Request a geographical location operation, using parameters defined in the object.

* If the ME has indicated in its Terminal Profile that it supports "class n"

* ("Geographical Location Request" see TS 31.111[7] for more details) the system will first send out

* a "Geographical Location Request" proactive command.<br>

* <ul>

*    <li>If the Terminal Responds from the ME indicates that the commands can be performed successfully the

*        actual Geo Location information is made available through a <code>GeoLocationInformation</code> object.

*    </li>

*    <li>If the Terminal Responds from the ME indicates that it can not perform this command, the system

*        will send out a Proactive command "Provide Local Information".

* <ul>

* <br>

* If the ME does not support "class n" only the proactive command "Provide Local Information" shall be sent

*

* @return a <code>GeoLocationInfo</code> object that provides the geo information received from the ME to the Applet

* @exception  NullPointerException if <code>proHandler</code> is <code>null</code>

*

* @exception  GeoLocationException with the following reason codes: <ul>

*      <li><code>INCORRECT_PARAMETERS</code>

*      <li><code>FUNCTION_DISABLED</code>

*      <li><code>UNABLE_TO_PROCESS_COMMAND</code>

*      <li><code>UNABLE_TO_ACHIEVE_ACCURACY</code>

* </ul>

*/

public GeoLocationInfo performGeoLocationRequest(ProactiveHandler proHandler) throws NullPointerException,GeoLocationException;

}

package uicc.usim.geolocation;

/**

* An object implementing this interface gives access to the Geo Location information

* received from the ME.

*

*/

public interface GeoLocationInfo{

/**

* Gives the Latitude of the geographical location

* @return int the Latitude a negative value indicates South

*/

public int getLatitude();

/**

* Gives the Longitude of the geographical location

* @return the Longitude according to TS 23.032 [14]

*/

public int getLongitude();

/**

* Gives the Altitude of the geographical location

* @return the Longitude according to TS 23.032 [14]

*/

public int getAltitude();

/**

* Gives the Velocity of the geographical location

* @return the Longitude according to TS 23.032 [14], -1 if no velocity is given in the

*         geo location information.

*/

public int getVelocity();

/**

* The byte buffer is formatted using GAD shapes as defined in TS 23.032 [14]. <p>

* @param buffer byte array where response of the operation shall be copied.

* @param offset offset of the result in <code>buffer</code>

* @param length length of data in <code>buffer</code>

* @return actual length of the data copied into the <code>buffer</code>

*/

public short getResponseBuffer(byte[] buffer, short bOffset, short length);

}

package uicc.usim.geolocation;

import uicc.toolkit.*;

/**

* The <code>GeoLocationBuilder</code> class is a <code>GeoLocation</code> object factory.

*/

public class GeoLocationBuilder {

/**

* Create an instance of the <code>GeoLocation</code> interface. Initialises the <code>GeoLocation</code>

* object with the information given in the <code>TerminalProfile</code> about the ME capabilities and performs the

* necessary registration to the <code>ToolkitRegistry</code> object of the Applet instance.

*

* @see uicc.toolkit.ToolkitRegistry;

* @see uicc.toolkit.TerminalProfile;

*

* @return <code>GeoLocation</code> object;

*/

public static GeoLocation buildGeoLocation(TerminalProfile profile, ToolkitRegistry registry) {

return null ;

}

}

package uicc.usim.geolocation;

import javacard.framework.CardRuntimeException;

/**

* Geographical Location exception and its associated constants.

*/

public class GeoLocationException extends CardRuntimeException {

// ------------------------------- Constants ------------------------------

/** This reason code (= 1) is used to indicate that the parameters are incorrect

*/

public static final short INCORRECT_PARAMETERS = (short)1;

/** This reason code (= 2) is used to indicate that the geographical location is disabled

*/

public static final short FUNCTION_DISABLED = (short)2;

/** This reason code (= 3) is used to indicate that the operation could not be processed

*/

public static final short UNABLE_TO_PROCESS_COMMAND = (short)3;

/** This reason code (= 4) is used to indicate that the requested accuracy cannot be achieved

*/

public static final short UNABLE_TO_ACHIEVE_ACCURACY = (short)4;

/**

* This reason code (= 5) is used to indicate that the information in the <code>ProactiveResponseHandler</code> or in

* the <code>EnvelopeHandler</code> is not correct.

*/

public static final short INCORRECT_INFORMATION = (short)5;

// ------------------------------- Constructors ---------------------------

/**

* Construct a GeoLocationException instance with the specified reason. To

* conserve on resources use <code>throwIt()</code> method to re-use the

* JCRE instance of this class.

*

* @param reason the reason for the exception

*/

public GeoLocationException(short reason) { super(reason); }

// ------------------------------- Public methods -------------------------

/**

* Throws the JCRE instance of the <code>GeoLocationException</code> class with

* the specified reason.

*

* @param reason the reason for the exception.

*

* @exception GeoLocationException always

*/

public static void throwIt(short reason) throws GeoLocationException {

}

}