3GPP TSG CT6 Meeting #50 	C6-090065

- Sophia-Antipolis, France, 09-12 February 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.130 | CR | 0035 |  rev | 1 |  Current version: | 8.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps | X | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Introduction of missing constant values for USIM files |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Gemalto |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | C6 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI8 |  |  |  |  |  |  | Date:  |  | 09/02/2009 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | Alignment with changes made in the USIM specification. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add constant values for files in the USIM application. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | USIM application specification and USIM API specification are not aligned. Severe interoperability issues between Java Card applets and USAT frameworks. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | Annex A Java |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

## Annex A: USIM constants.java

//-----------------------------------------------------------------------------

// PACKAGE DEFINITION

//-----------------------------------------------------------------------------

package uicc.usim.access;

//-----------------------------------------------------------------------------

// IMPORTS

//-----------------------------------------------------------------------------

/**

* The <b>USIMConstants interface</b> hold all the constants defined in 3GPP TS 31.102

*

*/

public interface USIMConstants {

// ------------------------------- Constants ------------------------------

/**

* File identifier : MF = 0x3F00

*/

public final static short FID_MF = (short) 0x3F00;

/**

* File identifier : ADF = 0x7FFF

*/

public final static short FID_ADF = (short) 0x7FFF;

/**

* EF under USIM ADF *

*/

/**

* File identifier : EF LI = 0x6F05 (under ADF)

*/

public final static short FID_EF_LI = (short) 0x6F05;

/**

* File identifier : EF IMSI = 0x6F07 (under ADF)

*/

public final static short FID_EF_IMSI = (short) 0x6F07;

/**

* File identifier : EF KEYS = 0x6F08 (under ADF)

*/

public final static short FID_EF_KEYS = (short) 0x6F08;

/**

* File identifier : EF KEYSPS = 0x6F09 (under ADF)

*/

public final static short FID_EF_KEYSPS = (short) 0x6F09;

/**

* File identifier : EF PLMNwACT = 0x6F60 (under ADF)

*/

public final static short FID_EF_PLMNWACT = (short) 0x6F60;

/**

* File identifier : EF HPPLMN = 0x6F31 (under ADF)

*/

public final static short FID_EF_HPPLMN = (short) 0x6F31;

/**

* File identifier : EF ACMMAX = 0x6F37 (under ADF)

*/

public final static short FID_EF_ACMMAX = (short) 0x6F37;

/**

* File identifier : EF UST = 0x6F38 (under ADF)

*/

public final static short FID_EF_UST = (short) 0x6F38;

/**

* File identifier : EF ACM = 0x6F39 (under ADF)

*/

public final static short FID_EF_ACM = (short) 0x6F39;

/**

* File identifier : EF GID1 = 0x6F3E (under ADF)

*/

public final static short FID_EF_GID1 = (short) 0x6F3E;

/**

* File identifier : EF GID2 = 0x6F3F (under ADF)

*/

public final static short FID_EF_GID2 = (short) 0x6F3F;

/**

* File identifier : EF SPN = 0x6F46 (under ADF)

*/

public final static short FID_EF_SPN = (short) 0x6F46;

/**

* File identifier : EF PUCT = 0x6F41 (under ADF)

*/

public final static short FID_EF_PUCT = (short) 0x6F41;

/**

* File identifier : EF CBMI = 0x6F45 (under ADF)

*/

public final static short FID_EF_CBMI = (short) 0x6F45;

/**

* File identifier : EF ACC = 0x6F78 (under ADF)

*/

public final static short FID_EF_ACC = (short) 0x6F78;

/**

* File identifier : EF FPLMN = 0x6F7B (under ADF)

*/

public final static short FID_EF_FPLMN = (short) 0x6F7B;

/**

* File identifier : EF LOCI = 0x6F7E (under ADF)

*/

public final static short FID_EF_LOCI = (short) 0x6F7E;

/**

* File identifier : EF AD = 0x6FAD (under ADF)

*/

public final static short FID_EF_AD = (short) 0x6FAD;

/**

* File identifier : EF CBMID = 0x6F48 (under ADF)

*/

public final static short FID_EF_CBMID = (short) 0x6F48;

/**

* File identifier : EF ECC = 0x6FB7 (under ADF)

*/

public final static short FID_EF_ECC = (short) 0x6FB7;

/**

* File identifier : EF CBMIR = 0x6F50 (under ADF)

*/

public final static short FID_EF_CBMIR = (short) 0x6F50;

/**

* File identifier : EF PSLOCI = 0x6F73 (under ADF)

*/

public final static short FID_EF_PSLOCI = (short) 0x6F73;

/**

* File identifier : EF FDN = 0x6F3B (under ADF)

*/

public final static short FID_EF_FDN = (short) 0x6F3B;

/**

* File identifier : EF SMS = 0x6F3C (under ADF)

*/

public final static short FID_EF_SMS = (short) 0x6F3C;

/**

* File identifier : EF MSISDN = 0x6F40 (under ADF)

*/

public final static short FID_EF_MSISDN = (short) 0x6F40;

/**

* File identifier : EF SMSP = 0x6F42 (under ADF)

*/

public final static short FID_EF_SMSP = (short) 0x6F42;

/**

* File identifier : EF SMSS = 0x6F43 (under ADF)

*/

public final static short FID_EF_SMSS = (short) 0x6F43;

/**

* File identifier : EF SDN = 0x6F49 (under ADF)

*/

public final static short FID_EF_SDN = (short) 0x6F49;

/**

* File identifier : EF EXT2 = 0x6F4B (under ADF)

*/

public final static short FID_EF_EXT2 = (short) 0x6F4B;

/**

* File identifier : EF EXT3 = 0x6F4C (under ADF)

*/

public final static short FID_EF_EXT3 = (short) 0x6F4C;

/**

* File identifier : EF SMSR = 0x6F47 (under ADF)

*/

public final static short FID_EF_SMSR = (short) 0x6F47;

/**

* File identifier : EF ICI = 0x6F80 (under ADF)

*/

public final static short FID_EF_ICI = (short) 0x6F80;

/**

* File identifier : EF OCI = 0x6F81 (under ADF)

*/

public final static short FID_EF_OCI = (short) 0x6F81;

/**

* File identifier : EF ICT = 0x6F82 (under ADF)

*/

public final static short FID_EF_ICT = (short) 0x6F82;

/**

* File identifier : EF OCT = 0x6F83 (under ADF)

*/

public final static short FID_EF_OCT = (short) 0x6F83;

/**

* File identifier : EF EXT5 = 0x6F4E (under ADF)

*/

public final static short FID_EF_EXT5 = (short) 0x6F4E;

/**

* File identifier : EF CCP2 = 0x6F4F (under ADF)

*/

public final static short FID_EF_CCP2 = (short) 0x6F4F;

/**

* File identifier : EF EMLPP = 0x6FB5 (under ADF)

*/

public final static short FID_EF_EMLPP = (short) 0x6FB5;

/**

* File identifier : EF AAEM = 0x6FB6 (under ADF)

*/

public final static short FID_EF_AAEM = (short) 0x6FB6;

/**

* File identifier : EF HIDDENKEY = 0x6FC3 (under ADF)

*/

public final static short FID_EF_HIDDENKEY = (short) 0x6FC3;

/**

* File identifier : EF BDN = 0x6F4D (under ADF)

*/

public final static short FID_EF_BDN = (short) 0x6F4D;

/**

* File identifier : EF EXT4 = 0x6F55 (under ADF)

*/

public final static short FID_EF_EXT4 = (short) 0x6F55;

/**

* File identifier : EF CMI = 0x6F58 (under ADF)

*/

public final static short FID_EF_CMI = (short) 0x6F58;

/**

* File identifier : EF EST = 0x6F56 (under ADF)

*/

public final static short FID_EF_EST = (short) 0x6F56;

/**

* File identifier : EF ACL = 0x6F57 (under ADF)

*/

public final static short FID_EF_ACL = (short) 0x6F57;

/**

* File identifier : EF DCK = 0x6F2C (under ADF)

*/

public final static short FID_EF_DCK = (short) 0x6F2C;

/**

* File identifier : EF CNL = 0x6F32 (under ADF)

*/

public final static short FID_EF_CNL = (short) 0x6F32;

/**

* File identifier : EF START-HFN = 0x6F5B (under ADF)

*/

public final static short FID_EF_START_HFN = (short) 0x6F5B;

/**

* File identifier : EF THRESHOLD = 0x6F5C (under ADF)

*/

public final static short FID_EF_THRESHOLD = (short) 0x6F5C;

/**

* File identifier : EF OPLMNWACT = 0x6F61 (under ADF)

*/

public final static short FID_EF_OPLMNWACT = (short) 0x6F61;

/**

* File identifier : EF HPLMNWACT = 0x6F62 (under ADF)

*/

public final static short FID_EF_HPLMNWACT = (short) 0x6F62;

/**

* File identifier : EF ARR = 0x6F06 (under ADF)

*/

public final static short FID_EF_ARR = (short) 0x6F06;

/**

* File identifier : EF NETPAR = 0x6FC4 (under ADF)

*/

public final static short FID_EF_NETPAR = (short) 0x6FC4;

/**

* File identifier : EF PNN = 0x6FC5 (under ADF)

*/

public final static short FID_EF_PNN = (short) 0x6FC5;

/**

* File identifier : EF OPL = 0x6FC6 (under ADF)

*/

public final static short FID_EF_OPL = (short) 0x6FC6;

/**

* File identifier : EF MBDN = 0x6FC7 (under ADF)

*/

public final static short FID_EF_MBDN = (short) 0x6FC7;

/**

* File identifier : EF EXT6 = 0x6FC8 (under ADF)

*/

public final static short FID_EF_EXT6 = (short) 0x6FC8;

/**

* File identifier : EF MBI = 0x6FC9 (under ADF)

*/

public final static short FID_EF_MBI = (short) 0x6FC9;

/**

* File identifier : EF MWIS = 0x6FCA (under ADF)

*/

public final static short FID_EF_MWIS = (short) 0x6FCA;

/**

* File identifier : EF CFIS = 0x6FCB (under ADF)

*/

public final static short FID_EF_CFIS = (short) 0x6FCB;

/**

* File identifier : EF EXT7 = 0x6FCC (under ADF)

*/

public final static short FID_EF_EXT7 = (short) 0x6FCC;

/**

* File identifier : EF SPDI = 0x6FCD (under ADF)

*/

public final static short FID_EF_SPDI = (short) 0x6FCD;

/**

* File identifier : EF MMSN = 0x6FCE (under ADF)

*/

public final static short FID_EF_MMSN = (short) 0x6FCE;

/**

* File identifier : EF EXT8 = 0x6FCF (under ADF)

*/

public final static short FID_EF_EXT8 = (short) 0x6FCF;

/**

* File identifier : EF MMSICP = 0x6FD0 (under ADF)

*/

public final static short FID_EF_MMSICP = (short) 0x6FD0;

/**

* File identifier : EF MMSUP = 0x6FD1 (under ADF)

*/

public final static short FID_EF_MMSUP = (short) 0x6FD1;

/**

* File identifier : EF MMSUCP = 0x6FD2 (under ADF)

*/

public final static short FID_EF_MMSUCP = (short) 0x6FD2;

/**

* File identifier : EF NIA= 0x6FD3 (under ADF)

*/

public final static short FID_EF_NIA = (short) 0x6FD3;

/**

* File identifier : EF VGCS= 0x6FB1 (under ADF)

*/

public final static short FID_EF_VGCS = (short) 0x6FB1;

/**

* File identifier : EF VGCSS= 0x6FB2 (under ADF)

*/

public final static short FID_EF_VGCSS = (short) 0x6FB2;

/**

* File identifier : EF VBS= 0x6FB3 (under ADF)

*/

public final static short FID_EF_VBS = (short) 0x6FB3;

/**

* File identifier : EF VBSS= 0x6FB4 (under ADF)

*/

public final static short FID_EF_VBSS = (short) 0x6FB4;

/**

* File identifier : EF VGCSCA= 0x6FD4 (under ADF)

*/

public final static short FID_EF_VGCSCA = (short) 0x6FD4;

/**

* File identifier : EF VBSCA= 0x6FD5 (under ADF)

*/

public final static short FID_EF_VBSCA = (short) 0x6FD5;

/**

* File identifier : EF GBABP= 0x6FD6 (under ADF)

*/

public final static short FID_EF_GBABP = (short) 0x6FD6;

/**

* File identifier : EF MSK= 0x6FD7 (under ADF)

*/

public final static short FID_EF_MSK = (short) 0x6FD7;

/**

* File identifier : EF MUK= 0x6FD8 (under ADF)

*/

public final static short FID_EF_MUK = (short) 0x6FD8;

/**

* DF at the USIM ADF level *

*/

/**

* File identifier : DF PHONEBOOK = 0x5F3A

*/

public final static short FID_DF_PHONEBOOK = (short) 0x5F3A;

/**

* File identifier : DF GSM ACCESS = 0x5F3B

*/

public final static short FID_DF_GSM_ACCESS = (short) 0x5F3B;

/**

* File identifier : DF MEXE = 0x5F3C

*/

public final static short FID_DF_MEXE = (short) 0x5F3C;

/**

* File identifier : DF WLAN = 0x5F40

*/

public final static short FID_DF_WLAN = (short) 0x5F40;

/**

* File identifier : DF SOLSA = 0x5F70

*/

public final static short FID_DF_SOLSA = (short) 0x5F70;

/**

* EF at the DF SOLSA level *

*/

/**

* File identifier : EF SAI= 0x4F30 (under DF SOLSA)

*/

public final static short FID_EF_SAI = (short) 0x4F30;

/**

* File identifier : EF SLL= 0x4F31 (under DF SOLSA)

*/

public final static short FID_EF_SLL = (short) 0x4F31;

/**

* EF at the DF PHONEBOOK level *

*/

/**

* File identifier : EF PBR = 0x4F30 (under DF PHONEBOOK)

*/

public final static short FID_EF_PBR = (short) 0x4F30;

/**

* File identifier : EF PSC = 0x4F22 (under DF PHONEBOOK)

*/

public final static short FID_EF_PSC = (short) 0x4F22;

/**

* File identifier : EF CC = 0x4F23 (under DF PHONEBOOK)

*/

public final static short FID_EF_CC = (short) 0x4F23;

/**

* File identifier : EF PUID = 0x4F24 (under DF PHONEBOOK)

*/

public final static short FID_EF_PUID = (short) 0x4F24;

/**

* EF at the DF GSM ACCESS level *

*/

/**

* File identifier : EF KC = 0x4F20 (under DF GSM ACCESS)

*/

public final static short FID_EF_KC = (short) 0x4F20;

/**

* File identifier : EF KCGPRS = 0x4F52 (under DF GSM ACCESS)

*/

public final static short FID_EF_KCGPRS = (short) 0x4F52;

/**

* File identifier : EF CPBCCH = 0x4F63 (under DF GSM ACCESS)

*/

public final static short FID_EF_CPBCCH = (short) 0x4F63;

/**

* File identifier : EF INVSCAN = 0x4F64 (under DF GSM ACCESS)

*/

public final static short FID_EF_INVSCAN = (short) 0x4F64;

/**

* EF at the DF MEXE level *

*/

/**

* File identifier : EF MEXEST = 0x4F40 (under DF MEXE)

*/

public final static short FID_EF_MEXEST = (short) 0x4F40;

/**

* File identifier : EF ORPK = 0x4F41 (under DF MEXE)

*/

public final static short FID_EF_ORPK = (short) 0x4F41;

/**

* File identifier : EF ARPK = 0x4F42 (under DF MEXE)

*/

public final static short FID_EF_ARPK = (short) 0x4F42;

/**

* File identifier : EF TRPRK = 0x4F43 (under DF MEXE)

*/

public final static short FID_EF_TRPRK = (short) 0x4F43;

/**

* EF at the DF WLAN level *

*/

/**

* File identifier : EF PSEUDO = 0x4F41 (under DF WLAN)

*/

public final static short FID_EF_PSEUDO = (short) 0x4F41;

/**

* File identifier : EF UPLMNWLAN = 0x4F42 (under DF WLAN)

*/

public final static short FID_EF_UPLMNWLAN = (short) 0x4F42;

/**

* File identifier : EF OPLMNWLAN = 0x4F43 (under DF WLAN)

*/

public final static short FID_EF_OPLMNWLAN = (short) 0x4F43;

/**

* File identifier : EF UWSIDL = 0x4F44 (under DF WLAN)

*/

public final static short FID_EF_UWSIDL = (short) 0x4F44;

/**

* File identifier : EF OWSIDL = 0x4F45 (under DF WLAN)

*/

public final static short FID_EF_OWSIDL = (short) 0x4F45;

/**

* File identifier : EF WRI = 0x4F46 (under DF WLAN)

*/

public final static short FID_EF_WRI = (short) 0x4F46;

/**

* EF under DF TELECOM *

*/

/**

* File identifier : EF ADN = 0x6F3A (under DF TELECOM)

*/

public final static short FID_EF_ADN = (short) 0x6F3A;

/**

* File identifier : EF EXT1 = 0x6F4A (under DF TELECOM)

*/

public final static short FID_EF_EXT1 = (short) 0x6F4A;

/**

* File identifier : EF ECCP = 0x6F4F (under DF TELECOM)

*/

public final static short FID_EF_ECCP = (short) 0x6F4F;

/**

* File identifier : EF SUME = 0x6F54 (under DF TELECOM)

*/

public final static short FID_EF_SUME = (short) 0x6F54;

/**

* File identifier : EF ICE_DN = 0x6FE0 (under DF TELECOM)

*/

public final static short FID_EF_ICE_DN = (short) 0x6FE0;

/**

* File identifier : EF ICE_FF = 0x6FE1 (under DF TELECOM)

*/

public final static short FID_EF_ICE_FF = (short) 0x6FE1;

/**

* DF at the DF TELECOM level *

*/

/**

* File identifier : DF GRAPHICS = 0x5F50

*/

public final static short FID_DF_GRAPHICS = (short) 0x5F50;

/**

* File identifier : DF MULTIMEDIA = 0x5F3B

*/

public final static short FID_DF_MULTIMEDIA = (short) 0x5F3B;

/**

* EF under DF GRAPHICS *

*/

/**

* File identifier : EF IMG = 0x4F20 (under DF GRAPHICS)

*/

public final static short FID_EF_IMG = (short) 0x4F20;

/**

* EF under DF MULTIMEDIA *

*/

/**

* File identifier : EF MML = 0x4F47 (under DF MULTIMEDIA)

*/

public final static short FID_EF_MML = (short) 0x4F47;

/**

* File identifier : EF MMDF = 0x4F48 (under DF MULTIMEDIA)

*/

public final static short FID_EF_MMDF = (short) 0x4F48;

/**

* File identifier : EF_GBANL = 0x6FDA (under ADF)

*/

public final static short FID_EF_GBANL = (short)0x6FDA;

/**

* File identifier : EF_EHPLMNPI = 0x6FDB (under ADF)

*/

public final static short FID_EF_EHPLMNPI = (short)0x6FDB;

/**

* File identifier : EF_LRPLMNSI = 0x6FDC (under ADF)

*/

public final static short FID_EF_LRPLMNSI = (short)0x6FDC;

/**

* File identifier : EF_NAFKCA = 0x6FDD (under ADF)

*/

public final static short FID_EF_NAFKCA = (short)0x6FDD;

/**

* File identifier : EF_SPNI = 0x6FDE (under ADF)

*/

public final static short FID_EF_SPNI = (short)0x6FDE;

/**

* File identifier : EF_PNNI = 0x6FDF (under ADF)

*/

public final static short FID_EF_PNNI = (short)0x6FDF;

/**

* File identifier : EF_NCP-IP = 0x6FE2 (under ADF)

*/

public final static short FID_EF_NCP-IP = (short)0x6FE2;

/**

* File identifier : EF_EPSLOCI = 0x6FE3 (under ADF)

*/

public final static short FID_EF_EPSLOCI = (short)0x6FE3;

/**

* File identifier : EF_EPSNSC = 0x6FE4 (under ADF)

*/

public final static short FID_EF_EPSNSC = (short)0x6FE4;

/**

* EF under DF HNB *

*/

/**

* File identifier : EF ACSGL = 0x4F81 (under DF HNB)

*/

public final static short FID_EF_ACSGL = (short) 0x4F81;

/**

* File identifier : EF CSGI = 0x4F82 (under DF HNB)

*/

public final static short FID_EF_CSGI = (short) 0x4F82;

/**

* File identifier : EF HNBN = 0x4F83 (under DF HNB)

*/

public final static short FID_EF_HNBN = (short) 0x4F83;

}