3GPP TSG CT6 Meeting #53 	C6-090470

- Beijing, China, 10-13 November 2009

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.130 | CR | 0045 |  rev | 1 |  Current version: | 8.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps | x | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Support of missing constants in USAT Terminal Profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Gemalto, Sagem-Orga, G&D, Oberthur Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | C6 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI8 |  |  |  |  |  |  | Date:  |  | 12/11/2009 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | Some values defined for TERMINAL PROFILE in 3GPP TS 31.111 are not defined in USIM API |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Support of missing constants |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | No constant defined, no alignment with core specification TS 31.111 |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | USATTerminalProfile.java |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

USATTerminalProfile.java

//-----------------------------------------------------------------------------

// PACKAGE DEFINITION

//-----------------------------------------------------------------------------

package uicc.usim.toolkit;

//-----------------------------------------------------------------------------

// IMPORTS

//-----------------------------------------------------------------------------

/**

* The <code>USATTerminalProfile</code> interface contains constants for the Terminal Profile according to TS 51.014 and to TS 31.111

*

* @see uicc.toolkit.TerminalProfile

*/

public interface USATTerminalProfile {

/**

* SMS PP Data Download = 1

*/

public final static short IDX_SMS_PP_DATA_DOWNLOAD = (short) 1;

/**

* Cell Broadcast Data Download = 2

*/

public final static short IDX_CELL_BROADCAST_DATA_DOWNLOAD = (short) 2;

/**

* '9EXX' response code for SIM data download error = 4

*/

public final static short IDX_9EXX_RESPONSE = (short)4;

/**

* USSD string data object supported in Call Control = 6

*/

public static final short IDX_USSD_SUPPORT_IN_CALL_CONTROL = (short)6;

/**

* Envelope Call Control always sent to the SIM during automatic redial mode = 7

*/

public final static short IDX_CALL_CONTROLL_AUTOMATIC_REDIAL_MODE = (short) 7;

/**

* Call Control by SIM = 9

*/

public final static short IDX_CALL_CONTROL_BY_SIM = (short)9;

/**

* Cell identity included in Call Control by SIM = 10

*/

public final static short IDX_CELL_IDENTITY_IN_CALL_CONTROL_BY_SIM = (short)10;

/**

* MO short message control by SIM  = 11

*/

public final static short IDX_MO_SM_CONTROL_BY_SIM = (short)11;

/**

* Handling of the alpha identifier  = 12

*/

public static final short IDX_HANDLING_OF_ALPHA_IDENTIFIER = (short)12;

/**

* Proactive SIM: SEND SHORT MESSAGE = 25

*/

public static final short IDX_PROACTIVE_SIM_SEND_SHORT_MESSAGE = (short)25;

/**

* Proactive SIM: SEND SS  = 26

*/

public final static short IDX_PROACTIVE_SIM_SEND_SS = (short) 26;

/**

* Proactive SIM: SEND USSD = 27

*/

public final static short IDX_PROACTIVE_SIM_SEND_USSD = (short)27;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION - NMR = 31

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_NMR = (short)31;

/**

* Binary choice in GET INKEY = 59

*/

public final static short IDX_BINARY_CHOICE_IN_GET_INKEY = (short)59;

/**

* 2nd alpha identifier in SET UP CALL = 62

*/

public final static short IDX_2ND_ALPHA_IDENTIFIER_SET_UP_CALL = (short)62;

/**

* 2nd capability configuration parameter = 63

*/

public static final short IDX_2ND_CAPABILITY_PARAMETER = (short)63;

/**

* Proactive SIM: PROVIDE LOCAL INFORMATION - BCCH  = 66

*/

public final static short IDX_PROACTIVE_SIM_PROVIDE_LOCAL_INFORMATION_BCCH = (short) 66;

/**

* Proactive SIM: PROVIDE LOCAL INFORMATION (Timing Advance)  = 68

*/

public final static short IDX_PROACTIVE_SIM_TIMING_ADVANCE = (short) 68;

/**

* CALL Control on GPRS = 141

*/

public final static short IDX_CALL_CONTROL_ON_GPRS = (short) 141;

/**

* Support of UTRAN PS with extended parameters = 168

*/

public final static short IDX_EXTENDED_PARAMETERS_IN_UTRAN_PS_ = (short) 168;

/**

* Geographical Location Reporting = 181

*/

public final static short IDX_GEOGRAPHICAL_LOCATION_REPORTING = (short)181 ;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION – NMR(UTRAN) = 173

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_NMR_UTRAN = (short) 173;

/**

* USSD Data Download and application mode= 174

*/

public final static short IDX_USSD_DATA_DOWNLOAD_AND_APPLICATION_MODE = (short) 174;

/**

* Event: I-WLAN Access Status = 195

*/

public final static short IDX_EVENT_I_WLAN_ACCESS_STATUS = (short)195;

/**

* Event: Network Rejection for GERAN/UTRAN = 196

*/

public final static short IDX_EVENT_NETWORK_REJECTION_GERAN_UTRAN = (short)196 ;

/**

* Event: Network Rejection for GERAN/UTRAN = 198

*/

public final static short IDX_EVENT_NETWORK_REJECTION_E_UTRAN = (short)198;

/**

* I-WLAN bearer support = 232

*/

public final static short IDX_I_WLAN_BEARER_SUPPORT = (short)232;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION (WSID of the current I-WLAN connection) = 233

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_WSID_I_WLAN = (short)233;

/**

* "Steering of Roaming" REFRESH support = 235

*/

public final static short IDX_STEERING_OF_ROAMING_REFRESH = (short)235;

/**

* Proactive UICC: Geographical Location Request = 237

*/

public final static short IDX_PROACTIVE_UICC_GEOGRAPHICAL_LOCATION_REQUEST = (short)237;

/**

* "Steering of Roaming for I-WLAN" REFRESH support = 239

*/

public final static short IDX_STEERING_OF_ROAMING_FOR_I_WLAN_REFRESH = (short)239;

}