## 4.3 ContactLess Protocol Parameter

The OPEN supports the following protocols:

- Protocol Type A: Defines the parameter values for initialization, anti-collision and protocol activation to select, for type A.  See ISO 14443-3 [1] and ISO 14443-4 [2].

- Protocol Type B: Defines the parameter values for initialization, anti-collision and protocol activation, for type B. See [2].

- Protocol Type F: Defines the parameter values for initialization, anti-collision and protocol activation, for type F. See JIS X 6319-4.

An application is able to request specific values in the protocol data for a particular protocol type. If the application has no mandatory value for a specific protocol type, it shall not provide protocol data parameters for this protocol type.

For each protocol type, Protocol Parameters consist of

- A Protocol Data setting initialization and anti-collision values for transactions with the PCD

- A Mandatory Mask defining the mandatory bits within the Protocol Data.

The OPEN is responsible for computing the Current Protocol Parameters for each supported protocol type.  The Current Protocol Parameters are the result of combining the Default Protocol Parameters and the Protocol Parameters of activated Applications.

The OPEN owns the default Protocol Parameters for all supported protocols.  There are no mandatory bits within the default Protocol Parameters.

## […]

## 4.8 Protocol Parameters for Type F

The value part of the protocol data Type F TLV when addressed to the OPEN is used to set up the PMM value with the following coding:.

| Parameter | Value description | Length |
| --- | --- | --- |
| PMm | Manufacture parameter (IC code and response time calculation parameter) | 8 |

Table 4-1: OPEN-Protocol Data for Type F value field

The following table describes the value part of the Protocol Data type for Type F protocol TLV (see Table 11-3) requested by an Application:

| Parameter | Value description | Length |
| --- | --- | --- |
| System code(s) | Application specific code that is used during the anti-collision. The PCD requests either a certain system code or a wildcard in the anti-collision request. Only PICCs with that specific system code reply to the request. If the system code doesn’t match the PICC remains silent. | 2 * N (N = 1 .. 16) |
| IDm(s) | Manufacturer assigned unique identification number (manufacture ID) | 8 * N (N = 1 .. 16) |

Table 4-2: Application-Protocol Data for Type F value field

There is a one-to-one association between a System code and an IDm. Thus, each System code parameter has to be followed by an IDm, so that the association is unambiguous.

There is no mandatory mask for Type F.

Editors Note: (To be removed once API is updated)

An additional method is required for instance in the GPCLSystem class:

short GetLastTypeFSystemcode()

Shall return the last System code used by OPEN for the anticollision.

An additional method is required for instance in the GPCLRegistryEntry class:

void SetTypeFRepsondFlag(short systemCode)

Indicates to the OPEN that this specific System Code shall be ignored for the contactless session.

[…]

## 6.3. Explicit and Implicit Selection over the Contactless Interface

This section expands the Implicit Selection capabilities of GP22 [0] with the addition of Implicit Selection by APDU pattern recognition and assigned protocol type.

A contactless session starts when the card enters the RF field and ends when the card leaves the RF field.

ISO/IEC 14443 Type A & Type B applications:

Card enters the RF field:

The first command received after the card activation according to ISO/IEC 14443-2, is used to select the application as described below.

1. If the first command is a SELECT [by Name] command:

- The OPEN locates the targeted application according to the AID.

- The located application becomes a valid candidate according to GP22 [0] and if it is ACTIVATED.

- If the applet is not a valid candidate, the OPEN continues to look for another valid candidate.

- The OPEN shall select the application. If the application refuses the selection, the OPEN restarts at step 1.a until the end of the Application list.

2. Otherwise, Implicit selection by Recognition Algorithm is attempted as follows:.

a. The OPEN parses the list of applications to look for an application that:

- Is a valid candidate according to [0],

- Is ACTIVATED,

- Supports the current contactless protocol,

- The Recognition Algorithm succeeds on the received command.

b. The OPEN shall select the application. If the application refuses the selection, the OPEN restarts at step 2.a until the end of the Application list.

3. Finally, If 1 and 2 have not identified a valid candidate, then

- The OPEN locates the default application associated with the contactless media channel

- The application becomes a valid candidate according to GP22 [0] and if it is ACTIVATED.

c. The OPEN shall select the application. If the application refuses the selection, no application is selected and the basic channel remains open.

Card leaves the RF field:

When card leaves the RF field or is deactivated according to ISO/IEC 14443-4 and the power is on, all applications selected on the contactless interface are deselected and logical channels closed.

Note: This statement on volatile data management may impact or supersede the current behavior of some runtime environments.

Applications communicating over the Type F Protocol:

The life cycle states as defined in X shall be respected.

The procedure depends on the command received according to JIS X 6319-4.

1. If the command is a REQC command:

- The OPEN evaluates the requested System Code, starting from the first parameter entry, and replies with an ATQC response including the corresponding IDm. A Respond Flag is associated to each System Code indicating whether the System Code shall be considered or not. The response is generated by the OPEN.

    - If the Respond Flag is ‘FF’, the OPEN shall not evaluate the System Code and shall not send any responses.

    - If the Respond Flag is ‘00’, the following steps shall be done. An application becomes a valid candidate for selection if it is activated, is selectable according to GP 2.2 and is configured to access the contactless interface, and

      - if the System Code matches one of the System Code parameters of an application; or

      - if one of the two bytes of the requested System Code is set to ‘FF’ (which is used as wildcard) and the other byte matches one of the System Code parameters. E.g. System Code parameter is ‘1234’ and requested System Code is either ‘12FF’ or ‘FF34’; or

      - if the requested System Code is ‘FFFF’. In that case the first parameter entry is considered for the ATQC response;

      - Else the procedure is aborted and nothing is replied.

- The OPEN selects the application matching the corresponding AID parameter to establish a Type F communication. If another application was selected previously it shall be deselected and the corresponding Respond Flag shall be reset (set to ‘00’). Note: The terms selection and deselection do not correspond with the JavaCard terms. More detailed definition is configuration dependent.

2. If the command is NOT a REQC command:

- The OPEN passes the received command to the corresponding application. This is configuration dependent.

Card leaves the RF field:

When card leaves the RF field and the power is still on, the application selected on the contactless interface is deselected and the corresponding Respond Flag is reset to the initial value. Further details are configuration dependent.

[…]

11.2.1. Contactless Protocol Parameters structure

The Contactless Protocol Parameters are TLV structured values. They may be provided as part of system-specific INSTALL parameters.

The following table identifies the possible tags for managing an application on the contactless interface:

| Tag | Length | Name | Presence |
| --- | --- | --- | --- |
| '80' | var | Assigned Protocols for implicit selection (see section 6.6) | Optional |
| '81' | 1 | Initial Activation State over the Contactless Interface (see section 8.2) | Optional |
| 'A2' | var | Contactless Protocol Parameters Profile. (See section 11.2.2) | Optional |
| '83' | var | Recognition Algorithm (see section 6.5) | Optional |
| '84' | 1-2 | Continuous Processing (see section 6.4) | Optional |
| 'A5' | var | Communication Interface Access Parameters (see section 5.2) | Optional |
| '86' | var | Protocol Data Type A (Card Emulation Mode) (see section 4.6) | Optional |
| '87' | var | Protocol Data Type B (Card Emulation Mode) (see section 4.7) | Optional |
| '88' | var | Protocol Data Type F (Card Emulation Mode) (see section 4.8) | Optional |