GlobalPlatform Card

Contactless Services

Card Specification v 2.2 - Amendment C

Version 0.87f

Public Review

7 January 2010

Document Reference: GPC_SPE_025

Table of Contents

1.	Introduction	5

1.1.	Scope	5

1.2.	Use Cases and Requirements	5

1.3.	Normative References	6

1.4.	Terminology and Definitions	7

1.5.	Abbreviations and Notations	7

1.6.	Revision History	8

2.	System Overview	9

2.1.	High Level Architecture	9

2.2.	Contactless Registry Service (CRS)	10

2.3.	High-Level Communication Flows	11

2.3.1.	Population and Update of Contactless Registry Parameters	11

2.3.2.	User Interaction	14

2.4.	Tag encoding rule	15

3.	User Interaction Management	16

3.1.	Definition and Scope	16

3.2.	Display Control Information	16

3.3.	Policy Restricted Applications	17

3.4.	Application Discretionary Data	17

3.5.	Application Family	17

3.6.	Display Required Indicator	18

3.7.	Application Groups	18

3.7.1.	Definition and Scope	18

3.7.2.	Head Application	19

3.7.3.	Member Applications	19

3.7.4.	Joining or Leaving an Application Group	19

3.7.5.	Add to the Group Authorization List	20

3.7.6.	Remove from the Group Authorization List	20

3.8.	CREL Application	21

3.8.1.	Definition and Scope	21

3.8.2.	CREL Application Registration	21

3.8.3.	Add to the CREL List	22

3.9.	CRS Application	23

3.9.1.	Definition and Scope	23

3.10.	Notification Rules	23

3.10.1.	General Rules	23

3.10.2.	CREL Notification	24

3.10.3.	CRS Notification	24

3.10.4.	Application Notifications	24

3.11.	GlobalPlatform CRS Application	24

3.11.1.	Definition and Scope	24

3.11.2.	TLV for User Interaction parameters	25

3.11.2.1.	Application AID	26

3.11.2.2.	Application Lifecycle State	26

3.11.2.3.	Update Counters	26

3.11.2.4.	Selection Priority	26

3.11.2.5.	Application Group	27

3.11.2.6.	CREL Application AID List	27

3.11.3.	GET STATUS Command.	27

3.11.3.1.	Definition and Scope	27

3.11.3.2.	Command Message	27

3.11.3.3.	Response Message	28

3.11.4.	SET STATUS Command	31

3.11.4.1.	Definition and Scope	31

3.11.4.2.	Command Message	31

3.11.4.3.	Response Message	33

3.11.5.	SELECT Command	34

3.11.5.1.	Definition and Scope	34

3.11.5.2.	Response Message	34

3.11.6.	GET DATA Command	35

3.11.6.1.	Definition and Scope	35

3.11.6.1.	Command Message	35

3.11.6.2.	Response Message	35

4.	Contactless Protocol Management	36

4.1.	Overview and Scope	36

4.2.	The OPEN requirements	36

4.3.	Contactless Protocol Parameters	36

4.4.	Current Protocol Parameter computation	37

4.4.1.	Current Protocol Parameter initialization	37

4.4.2.	Current Protocol Parameter: Computation on Activation	37

4.4.3.	Current Protocol Parameter: Full computation	39

4.5.	Conflict Detection Procedure	39

4.6.	Protocol Parameters for Type A (Card Emulation Mode)	42

4.7.	Protocol Parameters for Type B (Card Emulation Mode)	44

4.8.	Protocol Parameters for Type F	45

4.9.	Contactless Protocol Parameters Profiles	45

5.	Communication Interface Access Configuration	47

5.1.	Introduction, Overview and Rationale	47

5.2.	Communication Interface Access Parameters	47

5.3.	Security Domain Settings	48

5.4.	Application Instance Settings	48

5.5.	Rules for Extradition	48

6.	Application Selection	49

6.1.	Reset scenarios with multiple active interfaces	49

6.2.	Application Selection Priority	49

6.2.1.	GlobalPlatform Registry Order	49

6.2.2.	Volatile Priority over the Contactless interface	50

6.3.	Explicit and Implicit Selection over the Contactless Interface	50

6.4.	Continuous Processing	51

6.5.	Recognition Algorithm	51

6.6.	Assigned Protocols for Implicit Selection	52

7.	Contactless Privilege	53

7.1.	Contactless Activation Privilege	53

7.2.	Contactless Self-Activation Privilege	53

7.3.	Privilege Coding	53

8.	Application Availability on the Contactless Interface	54

8.1.	Contactless Activation State	54

8.2.	Initial Contactless Activation State	56

8.3.	Contactless Interface Availability	56

9.	Cumulative Granted Memory	57

10.	Cumulative Delete	59

10.1.	Definition and Scope	59

10.2.	Security Domain with Global Delete Privilege	59

10.3.	Security Domain hierarchy Removal	59

10.4.	Logically Deleted with References	61

11.	Security Domain APDU Commands	62

11.1.	Life Cycle State Coding	62

11.2.	INSTALL Command	62

11.2.1.	Contactless Protocol Parameters structure	62

11.2.2.	Contactless Protocol Parameters Profile structure	63

11.2.3.	User Interaction Parameters structure	64

11.2.4.	Processing state returned in the response message	64

11.3.	DELETE Command	64

11.4.	GET STATUS Command	65

11.4.1.	Filter Criterion: Tag List ('5C')	65

11.4.2.	Response Message	66

12.	Token Identifier Blacklist for Delegated Management	67

12.1.	Definition and Scope	67

12.2.	Blacklist / Rehabilitate using STORE DATA Command	67

12.3.	Add to blacklist	67

12.4.	Remove from blacklist	67

12.5.	Read blacklist	68

12.6.	Processing State returned in the response message	68

A.	CRS Java Card API	69

B.	Contactless Protocol Management: Example	73

B.1.	Current Protocol Parameters for Type A computation	73

B.1.1.	Value definition	73

B.1.2.	Protocol Data Computation: Intermediate result [A].	73

B.1.3.	Protocol Data Computation: Intermediate result [B].	74

B.1.4.	Protocol Data Computation: Intermediate result [C].	74

B.1.5.	Current Protocol Parameter of Type A: Result	74

B.2.	Protocol Parameter for Type A: Conflict Detection	74

B.2.1.	Value definition	74

# Introduction

## Scope

This document defines an extension of the GlobalPlatform Card Specification v2.2 [0] to facilitate deployment of contactless services in a Secure Element located in a mobile handset.

More specifically, this specification defines mechanisms, parameters, and interfaces to set-up and maintain the configuration of applications and control their access to system resources like communication interfaces and memory.  GlobalPlatform configuration(s) may require some or all of the capabilities specified in this document.  Applications that are configured to be usable on a contactless interface (using mechanisms defined in section 5: Communication Interface Access Configuration) are referred to as "Contactless Applications".

The term "contactless" is used throughout this document and references primarily "proximity" technologies, e.g. ISO/IEC 14443.  However, it may be possible that the mechanisms defined in this specification are useful to manage access to Applications over a variety of technologies, such as Bluetooth, WiFi, and ISO/IEC 15693.  Therefore, the term "contactless" shall not be understood as a limitation to strictly "proximity" technologies.

Certain application providers may choose to limit the accessibility of their Application(s) to specific technologies, and therefore, may define profiles specifying these industry-specific limitations.

Although the requirements motivating this Amendment originated from a mobile/contactless services perspective, some of the concepts specified in this Amendment are easily applicable beyond this core use case.

This document focuses on parameters and mechanisms required for Applications in card emulation mode.  Applications in reader or peer-to-peer mode are beyond the scope of the current version of this specification.

## Use Cases and Requirements

The following requirements shall be considered:

The end-user shall be able to:

- View and prioritize the list of services (implemented by a Standalone Application or an Application Group) based on a user-friendly service management application located in or outside of the secure element

- Activate and deactivate contactless services (so that a contactless reader is able/unable to select a specific service)

- Enable and disable communication over the contactless interface

The application provider shall be able to:

- Provide technical meta-data (e.g. parameters for the contactless protocol) to clearly define the contactless environment of the application

- Provide end-user selection meta-data (e.g. logo, family…) to help the end-user to choose the intended service for activation, deactivation, etc.

Additionally it shall be possible to:

- Grant a Security Domain a specific amount of memory resources.  All Applications and sub-Security Domains (if present) which are managed by this Security Domain shall not be allowed to consume more memory

- Delete a root Security Domain and its hierarchy in a single operation

- Prohibit the use of a specific token on a Security Domain with the Token Verification Privilege

This specification is structured according to the following topics:

- Extensions of the OPEN and the GlobalPlatform Registry

- Application Groups

- Contactless Activation states

- Management of Communication Channel Access Policy

- Application Selection

- CRS Application and CREL Application

- Conflicts between applications due to incompatible contactless parameters

- Privileges relative to Contactless Activation

- Memory resource management within a Security Domain Hierarchy

- Deletion of a Security Domain Hierarchy

- API for access to the extended GlobalPlatform registry

- API for interactions with CRS Application and CREL Application

- API extension to the SecureChannel interface enabling an Application to use the secure channel protocol when the APDU object is not available

- Token Identifier Blacklist for Delegated Management

## Normative References

| Standard / Specification | Description | Ref |
| --- | --- | --- |
| GlobalPlatform Card Specification v 2.2 | GlobalPlatform Card Specification v2.2 | [0] |
| ISO/IEC 14443-3 | Identification cards -- Contactless integrated circuit(s) cards -- Proximity cards -- Part 3: Initialization and anti-collision | [1] |
| ISO/IEC 14443-4 | Identification cards -- Contactless integrated circuit cards -- Proximity cards -- Part 4: Transmission protocol | [2] |
| ETSI TS 102 622 | Smart Cards; UICC - Contactless Front-end (CLF) Interface; Host Controller Interface (HCI) (Release 7) | [3] |
| ISO/IEC 7816-6 | Identification cards – Interindustry data elements for interchange | [4] |
| ISO/IEC 8825-1 | Information technology — ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER) | [5] |
| ISO/IEC 8859-1 | Information technology – 8-bit single-byte coded graphic character sets – Part 1: Latin alphabet No.1 | [6] |
| IETF RFC 3629 | UTF-8, a transformation format of ISO 10646 | [7] |
| ISO/IEC 18092 | Information technology – Telecommunications and information exchange between systems – Near Field Communication – Interface and Protocol (NFCIP-1) | [8] |
| ETSI TS 102 223 | Smart Cards; Card Application Toolkit (CAT) | [9] |
| Java Card™ 3.0 Runtime Environment, Classic Edition | Runtime Environment Specification – Java Card™ Platform 3.0, Classic Edition | [10] |
| ISO/IEC 10918-1 | Information technology – Digital compression and coding of continuous-tone still images: Requirements and guidelines | [11] |
| JIS X 6319-4 | Specification of implementation for integrated circuit(s) cards – Part 4: High Speed proximity cards | [12] |

Table 1-1: Normative References

## Terminology and Definitions

Table 1-2 defines the expressions used within this Specification that use an upper case first letter in each word of the expression. Expressions within this document that use a lower case first letter in each word take the common sense meaning. (Tagged data elements are also given an upper case first letter in each word of their names.)

| Term | Definition |
| --- | --- |
| Secure Element | A Secure Element (SE) is a component in a device providing the security and confidentiality required to support various business models. A SE can exist in any form factor such as UICC, Embedded SE, memory card, etc |
| Contactless Application | Applications that are explicitly or implicitly configured to be usable on a contactless interface (e.g. ISO 14443-based) are referred to as "Contactless Applications". |
| Contactless Session | A contactless session starts when the card enters the RF field and ends when the card leaves the RF field. |

Table 1-2: Terminology and Definitions

## Abbreviations and Notations

| Abbreviation | Meaning |
| --- | --- |
| AID | Application Identifier. |
| AFI | Application Family Indicator |
| APDU | Application Protocol Data Unit |
| API | Application Programming Interface |
| APSD | Security Domain of the Application Provider |
| BER-TLV | Basic Encoding Rules - Tag, Length, Value |
| CL | Contactless |
| CLF | Contactless Front-end interface |
| CRS | Contactless Registry Services |
| CREL | Contactless Registry Event Listener |
| EMV | EuroPay MasterCard Visa |
| GUI | Graphical User Interface |
| HCI | Host Controller Interface |
| IF | Interface |
| ISD | Security Domain of the Issuer |
| MIDP | Mobile Information Device Profile |
| MMI | Man-Machine Interface |
| NFC | Near Field Communication |
| OCE | Off-Card Entity |
| OPEN | GlobalPlatform Environment |
| P2P | Peer to Peer |
| PCD | Proximity Coupling Device |
| PICC | Proximity Integrated Circuit Card |
| PPSE | Proximity Payment System Environment |
| RF | Radio frequency |
| SWP | Single Wire Protocol |
| TLV | Tag, Length, Value |
| URI | Uniform Resource Identifier |

Table 1-3: Abbreviations and Notations

## Revision History

1.0	Initial Release

# System Overview

## High Level Architecture

Figure 2-1 provides a high level overview of the system architecture relevant to contactless services specified in this document.

Figure 2-1: High Level Architecture Overview

Some entities shown in the figure above are outside the scope of this document but are presented in the figure for a better understanding of the system context.  Specifically, the APIs for Reader Mode, P2P mode, and the realization of the low level contactless connection between the card and the contactless terminal, (e.g. direct antenna connection, SWP, or other), are beyond the scope of this specification.

ISD

The Issuer Security Domain (ISD) is the primary, mandatory, on-card component representative of the Card Administrator, typically the Card Issuer; cf. GP22 [0]

APSD

The Application Provider Security Domain (APSD) is an optional on-card component representative of an Application Provider.

OPEN

This entity represents the GlobalPlatform Environment (OPEN).  It provides an API to applications, command dispatch, Application selection, (optional) logical channel management, and Card Content management (see GP22 [0]).

CL Registry

The Contactless Registry adds functions to the GlobalPlatform Registry Services for managing Contactless Applications (see section 2.2).

Application

An Application is an executable component providing end-user services after having been installed and made selectable by GlobalPlatform card management functions.

Application Group

An Application Group is a concept which allows group member Applications to be represented to the end-user by a dedicated group Head Application (see section 3.3).

Head Application

The Head Application (see section 3.3) is a Contactless Application which owns the visibility parameters and the Contactless Protocol Parameters of an Application Group.

CRS (OPEN Extension)

The Contactless Registry Service (CRS) is an extension of the OPEN to manage and provide access to contactless registry parameters (see section 2.2).

CRS Application

The CRS Application is an optional component designed for the management of Contactless Applications by the end user (see section 3.9).

CREL Application

A Contactless Registry Event Listener (CREL) Application is an Application interested in being notified of the changes occurring to one or more Contactless Application (see section 3.8).

CRS GUI

This handset-side component allows the end-user to interact with the on-card CRS Application.

CL Reader

The Contactless Reader represents the card external contactless communication interface allowing on-card and off-card Contactless Applications to interact with each other.

## Contactless Registry Service (CRS)

The Contactless Registry Service (CRS) is an extension of the OPEN providing:

- The Contactless Registry; an extension of the GlobalPlatform Registry

- The CRS API; an extension of the GlobalPlatform API

- Services for:

  - Maintaining the Contactless Registry subsequent to the installation, update, or deletion of an Application

  - Maintaining the Contactless Registry subsequent to the activation, deactivation, or a change of priority of an Application

  - Providing the Contactless Registry information upon request from an authorized entity

  - Notifying Applications of changes made in the Contactless Registry

- Contactless protocol management

- Access control on Communication Interfaces

- Application selection rules on the contactless interface

- Contactless privileges

## High-Level Communication Flows

This section provides a high level overview of the main communication flows between the new system entities introduced in this specification.  The flows described below show the interaction of the entities when executing the following new functions:

- Populating contactless registry parameters during Application installation

- Populating contactless registry parameters during Application personalisation

- Removing contactless registry parameters during Application deletion

- Activation and deactivation of Contactless Applications (including conflict resolution)

### Population and Update of Contactless Registry Parameters

Figure 2-2 shows the flow of populating the contactless registry parameters during the installation of an application.  This flow also applies to the update of the contactless registry parameters using the INSTALL [for registry update] command.

Figure 2-2: Flow of populating an Application’s Contactless Registry Parameters during Installation

CRS shall send notifications about updates of the Contactless Registry to the CRS Application and registered CREL Applications.

The following steps are highlighted in the figure above:

- The Application Provider supplies its Application with Contactless Registry Parameters through its Security Domain using the INSTALL command.

- The Security Domain forwards the INSTALL command to the OPEN.

- The OPEN populates the Application-specific Parameters.

- The OPEN populates the Contactless Registry Parameters.

- The CRS API notifies the CRS Application that a change has occurred in the Registry entry of that Application.

- The OPEN notifies CREL Application(s) referenced by the Application.

Figure 2-3 shows the flow of populating or updating the contactless registry parameters of an application through the application itself.

Figure 2-3: Flow of populating an Application’s Contactless Registry Parameters through itself

The following steps are highlighted in the figure above:

- The Application Provider supplies its Application with Contactless Registry Parameters using a proprietary command.

- The Application updates its Contactless Registry Parameters using the CRS API.

- The CRS Application is notified of the changes that occurred to the Application.

- CREL Application(s) referenced by the Application, if any, are notified of the changes that occurred to the Application.

Figure 2-4 shows the flow of removing the contactless registry parameters during deletion of an application.

Figure 2-4: Flow of removing an Application’s Contactless Registry Parameters during Deletion

The following steps are highlighted in the figure above:

- An Application Provider (off-card) sends the DELETE command to the associated Security Domain of the Application which is to be deleted.

- The addressed Security Domain delegates the deletion command to the CRS (an extension of the OPEN) using proprietary interfaces.

- The CRS/OPEN invokes the "Uninstall" method of the Application to be deleted as specified in GP22 [0].

- The CRS/OPEN removes all entries of the Application which is to be deleted in the GlobalPlatform Registry.  The OPEN then deletes the data objects of the Application.

- The CRS/OPEN sends the notification about the deletion of the Application to the CRS Application.

- CREL Application(s) referenced by the Application, if any, are notified of the deletion of the Application.

### User Interaction

Figure 2-5 shows the flow of the user interaction during activation, deactivation or change of priority of an application.

Figure 2-5: Flow of User Interaction

The following steps are highlighted in the figure above:

- An Off-Card user interface provides the user with means to activate and deactivate a Contactless Application and to define the priority by which they are considered for selection through a contactless reader.

- The Off-Card entity maps the user request to one or more command APDU defined for the CRS Application and transmits them to this Application.  The CRS Application processes these commands with the help of an On-Card CRS API.

- If an Application activation is requested, the CRS Application asks the CRS to check if the request conflicts with one (or more) of the already activated Applications.

- If a conflict is detected, a corresponding response is returned to the Off-Card entity to inform the user of the conflict.

- The user can resolve the conflict by restarting at 1) with deactivating the conflicting application(s) and then repeating the original request.

- To deactivate, activate (without a conflict) or change the selection priority of an Application, the CRS updates the Registry entry of the respective Application.

- CREL Applications possibly linked to the modified Application are notified about the registry change by the CRS.

## Tag encoding rule

All TLVs defined in the Amendment C are encoded as specified in GP22 [0]. The Identifier octet is context-specific class as described in ISO/IEC 8825-1 [5].  The tag number is in the range [0,30] and is reserved for GlobalPlatform usage unless explicitly specified otherwise.

# User Interaction Management

## Definition and Scope

This section describes the roles and interaction between the entities involved in providing information that support the end-user to choose the Applications that are to be activated and deactivated and to prioritize the activated Applications on the contactless interface.  These entities are the CRS Application, Standalone Applications, Application Groups, and Contactless Registry Event Listener (CREL) Applications.

This section also defines parameters that can be used to manage the visibility of Applications as seen by off-card entities.  These User Interaction Parameters are accessed by Security Domains using the INSTALL commands as defined in section 11.1 or by the GlobalPlatform CRS Application using the GET STATUS or SET STATUS commands.  Similar functionality is also provided by the CRS API.

## Display Control Information

Display Control Information is used to manage an off-card GUI by providing information for a Logo, a URI or any related display information.  The presence of Application Visibility data is optional.

- Application Visibility data should be personalized for Applications to be visible to the end-user.  Typically, Head Applications and Standalone Applications (see section 3.7) should be personalized with this parameter.

- The Application Visibility data shall be embedded with the Display Control Template (tag '7F20') and may include one or more of the following data elements:

- Tag '5F50': Uniform Resource Locator (see ISO/IEC 7816-6 [4]).  The purpose of this URL is out of scope of this document

- Tag '6D': Application Image Template (see [4])

  - Tag '5F44' Application Image

  - Tag '67' Authentication Data (optional, otherwise the Application Image format shall be according to ISO/IEC 10918-1 [11])

    - Tag '53' Image Encoding Format

      - '01' – JPG

      - '02' – JPG-2000

      - '03' – GIF

      - '04' – PNG-8

      - '05' – PNG-24

      - '06' – BMP

      - '07' to '7F' - RFU

      - '80' to '8F'- Proprietary usage

      - '90' to 'FF' – RFU

- Tag '5F45': Display Message (Data element containing a message to display) (see [4]).  The following formats are defined for the value of the information to be displayed to the user.  A leading single byte identifier, given below, shall indicate the display message encoding format:

  - '01' - Format ASCII, which includes displayable characters (alphabetic, numerical, and special) and space in the range from '20' to '7E', coded on one byte and left-justified. (see ISO/IEC 8859-1 [6])

  - '02' - Format BCD, which includes only numerical digits, coded on a nibble (4 bits), left justified, and eventually padded on the right with an 'F' nibble if necessary (i.e. if the number of digits is odd)

  - '03' - Format HEX, which is equivalent to a transparent mode ("as is") and includes all binary values coded on one byte

  - '04' - Format UTF-8, which is able to represent any character in the UNICODE character set. (see IETF RFC 3629 [7])

## Policy Restricted Applications

The Policy Restricted Applications TLV provides one or more AIDs of Applications that should not be ACTIVATED on the contactless interface at the same time as the Application which has this TLV in its Registry.  This TLV is not used by the OPEN for contactless protocol parameter conflict resolution or for any other purpose.  This TLV may be used by the CRS Application and CREL Applications when applying their specific business logic.

Note:  If an Application has the SELF_ACTIVATION privilege, the CRS Application will not be involved in the activation of the Application, although the CRS Application will be notified of this activation and may take action based on its own business logic and privileges.

The table below defines the value part of the "Policy Restricted Applications" TLV:

| Tag | Length | Description | Presence |
| --- | --- | --- | --- |
| '4F' | 5-16 | AID of the Policy Restricted Application (e.g. PPSE) | Mandatory |
| '4F' | 5-16 | AID of the Policy Restricted Application | Optional |
| … | … | … |  |

Table 3-1: Policy Restricted Applications TLV Value Description

## Application Discretionary Data

This Application Discretionary data is accessible to the CRS Application and CREL Applications, if any, for proprietary usage.

## Application Family

The Application Family indicates the sector of industry that a Contactless Application belongs to.  This indicator may be used to group together the Applications of the same family for a similar process such as presenting only one family of Applications to the user.  A Contactless Application shall belong to a primary family only.

The Application Family is coded as the AFI defined in Table 12 of ISO/IEC 14443-3 [1].  The Application Family applies to all Applications, regardless of protocol type.

The OPEN shall not enforce that the Application Family and the AFI defined in section 4.7 are the same.

## Display Required Indicator

The Display Required Indicator TLV is an optional TLV assigned to an Application so that it can indicate whether or not it is able to perform a contactless transaction even when the display is not available.  This parameter is not evaluated by the OPEN.  If this TLV is not present, the Application is indicating that it requires a display to perform the contactless transaction.

The table below defines the value part of the "Display Required Indicator" TLV:

| Value | Description |
| --- | --- |
| '00' | Application requires a display |
| '01' | Application does not require a display |

Table 3-2: Display Required Indicator TLV Value Description

NOTE: This indicator may be used by the CRS and/or CREL Applications for their own business purposes.

## Application Groups

### Definition and Scope

An Application Group is composed of one and only one Head Application and one or more Member Applications.  The Head Application is associated with a Group Authorization List defining the possible membership of the group as a list of AIDs.  Applications referenced (by AID) in this list may request to join the group.

An Application shall not be a member of more than one Application Group.  An Application that is neither a Head Application, nor a member of an Application Group, is a Standalone Application.  At any time, a Standalone Application may register itself as a Head Application, hence defining a new Application Group, or as the member of an Application Group.

The following Contactless Activation States are used throughout this specification; ACTIVATED, DEACTIVATED, NON_ACTIVATABLE.  These states are defined in section 8.1.

Sample UI

Loyalty Program   Activated

Transport           Deactivated

Credit Issuer A  Activated

Credit Issuer B  Deactivated

Credit Issuer C   Activated

Group Transport:

TransportAID 1   Visible    Deactivated

Air Service   AID 2   Invisible  Deactivated

Rail ServiceAID 3   Invisible  Deactivated

Group Credit Issuer A:

DomesticAID 4   Visible    Activated

InternationalAID 5   Invisible  Activated

Figure 3-1: Application Group

### Head Application

- The Head Application:

  - Establishes Contactless Protocol Parameters (see section 4.3) and prioritization for the member Applications.  It is the responsibility of the Application Provider of the Head Application to personalize the Head Application with Contactless Protocol Parameter values that support all of the members applications named in its Group Authorization List.

  - When the Head Application is set to the ACTIVATED state, all the Member Applications shall be set to the ACTIVATED state unless they are in the NON_ACTIVATABLE state.

  - When the Head Application is set to the DEACTIVATED state, all the Member Applications shall be set to the DEACTIVATED state, unless the Member Application is in the NON_ACTIVATABLE state.

  - When the Head Application is set to the NON_ACTIVATABLE state, all the Member Applications shall be set to the DEACTIVATED state, unless the Member Application is in the NON_ACTIVATABLE state.

  - When the Head Application is deleted, Applications previously belonging to this Application Group become Standalone Applications.  All the former Member Applications shall be set to the DEACTIVATED state, unless the former Member Application is in the NON_ACTIVATABLE state.

### Member Applications

- Member Applications:

  - Contactless Protocol Parameters (see section 4.3) and prioritization are provided by the Head Application, even if the Member Application was personalized with its own parameter values.

  - When a member application is set to the NON_ACTIVATABLE state, only that application becomes NON_ACTIVATABLE.  The other members of the group follow the activation state of the Head Application.

  - When a Member Application is in the NON_ACTIVATABLE state, it shall remain in the NON_ACTIVATABLE state even if the Head Application is set to the ACTIVATED state.

  - When the NON_ACTIVATABLE member is set to any state other than NON_ACTIVATABLE, it follows the activation state of the Head Application.

### Joining or Leaving an Application Group

The "Head Application" TLV allows an Application to request to be member of a group by providing the AID of the Head Application of the Group or to leave an Application Group using INSTALL [for install], INSTALL [for install and make selectable] or INSTALL [for registry update].  An Application may itself request to join or leave an Application Group using the CRS API.

The table below defines the value part of the "Head Application" TLV.

| Tag | Length | Value description |
| --- | --- | --- |
| '4F' | 5-16 | AID of the group’s Head Application to join. |

Table 3-3: Head Application TLV Description

The following rules apply when joining an Application Group:

- The OPEN checks that the application does not yet belong to a group.

- If the requested Head Application is present, then the OPEN checks that the application requesting to join a group has its AID listed in corresponding Group Authorization List.

- If the requested Head Application is not present (i.e. specified Application Group does not exist), the installation does not fail.  The association will be performed at the time the Head Application is installed and if the application AID is listed in its Group Authorization List.

- The rules for the alignment of the activation state of the Member Application with the activation state of the Head Application are applied as detailed in section 3.7.3 are applied.

- The Current Protocol Parameters shall be recalculated as detailed in section 4.4 if the new Member Application is in the ACTIVATED state at the time it joins the group.

- This TLV shall not be in the same command data field as the "Add to the Group Authorization List" or "Remove from the Group Authorization List" TLVs.

- The following rules apply when leaving an Application Group:

- The Head Application TLV shall be provided with empty length.

- The OPEN shall check that the Application belongs to an Application Group.  If yes, the Application is removed from the Application Group, otherwise an error status is returned.

- The Application becomes a Standalone Application

- The Application shall be set to the DEACTIVATED state, unless the former Member Application is in the NON_ACTIVATABLE state.

- The priority of the Application shall be set to the lowest priority.

### Add to the Group Authorization List

When present in the INSTALL [for install], the INSTALL [for install and make selectable], or the INSTALL [for registry update] command, the "Add to the Group Authorization List" parameter (see Table 3-4) establishes the target application as a Head Application, by creating its Group Authorization List or adding AIDs to the existing one.  A Head Application may itself request to update its Group Authorization List using the CRS API.

The table below defines the value part of the "Add to the Group Authorization List" TLV.

| Tag | Length | Description |
| --- | --- | --- |
| '4F' | 5-16 | AID of an Application to be added to the Group Authorization List |
| '4F' | 5-16 | AID of an Application to be added to the Group Authorization List |
| … |  |  |

Table 3-4: Add to the Group Authorization List Value Description

The following rules apply to the list content:

- The OPEN checks that the Application is not a Member Application of an Application Group.

- The OPEN does not check for the uniqueness of an AID in multiple Group Authorization Lists.

- The installation does not fail if an AID in the Group Authorization List is already associated to another group or not present.

- Partial AID matching is not supported.

- This TLV shall not be in the same command data field as the "Head Application" TLV.

- If this TLV contains the AID of an Application that already exists in the Group Authorization List of the Head Application, no error is reported and no notifications are made.

### Remove from the Group Authorization List

When present in the INSTALL [for registry update] command, the "Remove from the Group Authorization List" parameter (see Table 3-5) is used to remove one or more AIDs from the Group Authorization List.  A Head Application may itself request to update its Group Authorization List using the CRS API.

If an AID that was removed from the Group Authorization List refers to a Member Application, that Application:

- is removed from the group

- becomes a Standalone Application

- shall be set to the DEACTIVATED state, unless the former Member Application is in the NON_ACTIVATABLE state.

- and its priority shall be set to the lowest priority

If the Group Authorization List becomes empty, this Head Application becomes a Standalone Application.

The table below defines the value part of the "Remove from the Group Authorization List" TLV.

| Tag | Length | Description |
| --- | --- | --- |
| '4F' | 5-16 | AID of an Application to be removed from the Group Authorization List |
| '4F' | 5-16 | AID of an Application to be removed from the Group Authorization List |
| … | … | … |

Table 3-5: Remove from Group Authorization List Value Description

This TLV shall not be in the same command data field as the "Head Application" TLV.

If this TLV contains the AID of an Application that does not exist in the Group Authorization List of the Head Application, no error is reported and no notifications are made.

## CREL Application

### Definition and Scope

The Contactless Registry Event Listener (CREL) Application is an on-card Application implementing specific business rules for a specific set of Contactless Applications.  The CREL Application is referenced by one or several Applications during their installation process.  The CREL Application shall be notified by the OPEN after any change to the Registry entry of any referencing Application.  The CREL Application shall implement and expose the CRELApplication interface.  The actions, if any, taken by the CREL Application upon receipt of the notification are out of the scope of this document.

### CREL Application Registration

During the installation process, during Registry update, or using the CRS API, any Application may indicate to the OPEN a list of CREL Applications that shall be notified of Registry events occurring to the application, by using the CREL List TLV structure (see Table 3-6).

If a referenced CREL Application is not present or does not implement nor expose the CRELApplication interface, the INSTALL command does not fail for these reasons, however, this Application will not be notified.  If the CREL Application is installed later, and it implements and exposes the CRELApplication interface, it will receive notifications.

The CREL Application shall be able to retrieve the list of all the Applications that are referencing it.  The CREL Application shall be able to retrieve the Registry entry of each Application that references it and read the content of the entry without any specific Privilege.

- The CREL Application may deactivate an Application referencing it.  The CREL Application shall not be able to make any other change to the Registry entries of referencing Applications.

### Add to the CREL List

When present in the INSTALL [for install], the INSTALL [for install and make selectable], the INSTALL [for registry update] command, or using the CRS API, the "Add to the CREL List" parameter (see Table 3-6) provides the list of AIDs of CREL Applications that shall be notified by the OPEN upon any change in the GPRegistryEntry of the target Application.  The OPEN shall be able to retrieve the list of all CREL Applications that an Application is referencing.

The table below defines the value part of the "Add to the CREL List" TLV.

| Tag | Length | Description |
| --- | --- | --- |
| '4F' | 5-16 | AID of a CREL Application that shall be added to the CREL List |
| '4F' | 5-16 | AID of a CREL Application that shall be added to the CREL List |
| … | … | … |

Table 3-6: Add to the CREL List Value Description

The following rule applies to the notification list:

- The newly added CREL Application is notified.

- If the AID of a CREL Application in the CREL List was previously added, no error is reported and no notifications are made.

    - Remove from the CREL List

When present in the INSTALL [for registry update] command or using the CRS API, the "Remove from the CREL List" parameter (see Table 3-7) provides the list of AIDs of CREL Applications that shall be removed from the notification list of that Application.

The table below defines the value part of the "Remove from the CREL List" TLV.

| Tag | Length | Description |
| --- | --- | --- |
| '4F' | 5-16 | AID of a CREL Application that shall be removed from the CREL List |
| '4F' | 5-16 | AID of a CREL Application that shall be removed from the CREL List |
| … | … | … |

Table 3-7: Remove from the CREL List Value Description

The following rule applies to the notification list:

- The removed CREL Application is notified.

- If the AID of a CREL Application does not exist in the CREL List, no error is reported and no notifications are made.

## CRS Application

### Definition and Scope

The Contactless Registry Service (CRS) Application is responsible for providing the user with means to retrieve a list of all Application Groups and Standalone Applications, and to activate, deactivate, or change the Priority or Volatile Priority of these entities on the contactless interface.

When present, the CRS Application shall be accessible by the on-card Applications to request switching their own Availability state to ACTIVATED (see section 8.1).

The following requirements apply:

    - Only one CRS Application shall exist per secure element.

    - The Contactless Activation Privilege and the Global Registry Privilege must be assigned to the CRS Application.  These privileges allow the CRS Application to successfully invoke the CRS API to

      - retrieve a list of all Applications,

      - activate or deactivate these Applications,

      - change the Priority or Volatile Priority of these Applications on the contactless interface (see section 6)

      - globally switch on or off the contactless interface (see section 8.3)

    - The CRS Application shall implement and expose the CRSApplication Interface. The CRSApplication.processCLEvent() method shall be called by the OPEN for each activation requested by an Application without the Contactless Self-Activation Privilege.

      - The reference to the CL registry entry of the targeted Application, which was passed as a parameter of CRSApplication.processCLEvent(), can be used by the CRS Application to retrieve the list of conflicting Applications.

      - The behavior of the CRSApplication.processCLEvent()method is out of scope of this specification.

A CRS Application which is installed with the reserved AID for the GlobalPlatform CRS Application shall support the APDU command set described in section 3.10.

## Notification Rules

### General Rules

When a power loss occurs, and all Applications have not all been notified of the last Registry modification, the following rules apply:

      - If no transaction was open at the time of the power loss, notifications shall be issued for all Applications.

      - If a transaction was open at the time of the power loss, previous modifications to the Registry are rolled back and the issuance of the notifications is not restarted.  However, it is strongly recommended not to open such a transaction due to the operations potentially performed by notified Applications.

The order of notification is not defined.

Applications should be aware that they may be notified again in the case of power loss.

### CREL Notification

CREL applications referenced by an Application, 'A', are notified when the Application 'A':

•	switches to the Lifecyle State INSTALLED, SELECTABLE, or LOCKED

•	is unlocked or deleted

•	switches to the Availability State ACTIVATED, NON_ACTIVATABLE, DEACTIVATED

•	has any of its Contactless Protocol Parameters (see Table 11-3) updated

•	has any of its User Interaction parameters (see Table 11-5) updated

A CREL Application shall not be notified when it is the originator of an event.

See Annex A, CRS API, where the details of the corresponding GlobalPlatform Events that are used for notifications are described.

### CRS Notification

    - The CRS Application shall be notified about all Registry changes (as defined for CREL Applications, see 3.8.5) occurring to any Application when it is not the originator for the event.  Unlike a CREL Application, the CRS Application does not need to be registered for any Application in order to be notified about a Registry change for any Application.

### Application Notifications

Applications that implement the CLApplet interface shall be notified of changes occurring to their Registry entry.  An Application shall not be notified when it is the originator of an event.

## GlobalPlatform CRS Application

### Definition and Scope

The GlobalPlatform CRS Application, if present, shall implement the APDU set as defined in this section:

- an APDU to get information on registered Contactless Applications and their respective Contactless Registry parameters

- an APDU to:

  - activate/deactivate Contactless Applications and to respond with the list of conflicting Applications' AIDs

  - establish the order of Applications in the GlobalPlatform Registry.  It shall be able to receive a list of Application AIDs (Head Applications, Member Applications, or Standalone Applications)

  - setup/discard the Volatile Priority (see section 6.2)

The following AIDs are reserved by GlobalPlatform for the purpose of installing the GlobalPlatform CRS Application:

- Executable Load File AID: 'A000000151435253'

- Executable Module AID: 'A00000015143525300'

- Application AID: 'A00000015143525300'

The standard GlobalPlatform CRS Application is defined in this specification as a possible implementation for a CRS Application.  It shall be able to process the APDU commands described in Sections 3.11.2 and 3.11.4, therefore providing the "user" with expected management capabilities as described in Section 3.9.

The GlobalPlatform CRS Application cannot be a Security Domain because of the redefinition of the GET STATUS and SET STATUS commands in section 11.

Parameters are accessed through the GET STATUS and SET STATUS commands.

### TLV for User Interaction parameters

The TLVs described below identify each User Interaction parameter accessed by the GlobalPlatform CRS Application.  These TLVs may be used with the GET STATUS command (see section 3.11.3) as search or filter criterion.  The description of each tag in Table 3-8, below, indicates which tag may be used with which command.  Filter Criteria (FC) are provided within tag list '5C' and are used to build the response field of the GET STATUS command.  Search Criteria (SC) apply to the incoming data field of the GET STATUS command.

| User Interaction Parameter | Tag | Length | GET STATUS |  | Section |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SC | FC |  |
| Application AID | '4F' | 5-16 | Yes | Yes | 3.11.2.1 |
| Application Lifecycle state | '9F70' | 2 | Yes | Yes | 3.11.2.2 |
| Display Control Template | '7F20' | var | No | Yes | 3.2 |
| Uniform Resource Locator | '5F50' | var | No | Yes | 3.2 |
| Application Image | '5F44' | var | No | Yes | 3.2 |
| Display Message | '5F45' | var | No | Yes | 3.2 |
| Application Update Counter | '80' | var | No | Yes | 3.11.2.3 |
| Priority Indicator | '81' | var | Yes | Yes | 3.11.2.4 |
| Group Head Application | 'A2' | var | Yes | Yes | 3.11.2.5 |
| Group Member’s Application | 'A3' | var | No | Yes | 3.11.2.5 |
| CREL Application AID List | 'A4' | var | No | Yes | 3.11.2.6 |
| Policy Restricted Applications | 'A5' | var | Yes | Yes | 3.3 |
| Application Discretionary Data | 'A6' | var | No | No | 3.4 |
| Application Family | '87' | var | Yes | Yes | 3.5 |
| Display Required Indicator | '88' | 1 | Yes | Yes | 3.6 |
| Continuous Processing | '8A' | var | No | Yes | 6.4 |
| Recognition Algorithm | '8B' | var | No | Yes | 6.5 |
| Assigned Protocols | '8C' | var | No | Yes | 6.6 |

Table 3-8: User Interaction parameters Tag Usage

These tags shall be used as search or filter criteria in a simple, non-constructed form.  E.g. '5F50' is a sub-tag of '7F20', however, in the search or filter criteria data field only '5F50' shall appear (see example related to Table 3-14).

### Application AID

The Application AID tag '4F' is a mandatory tag within the Contactless Registry and can be used both as a search criterion and in the tag list.  Partial AID matching is supported when this tag is used as a search criterion in the GET STATUS command (see section3.11.3.2).

### Application Lifecycle State

The Application Lifecycle State tag '9F70' provides the Application Lifecycle state on the first byte as defined in GP 2.2 [0].  The Contactless Activation State is encoded on the second byte as defined in Table 8-1.  The Application Lifecycle State tag can be used both as a search criterion and in a tag list.

When tag '9F70' is used as a search criterion

- Application-specific lifecycle state bits in the first byte shall be ignored

- RFU bits in the second byte shall be ignored

### Update Counters

The CRS maintains a counter in the range from 0 to 65535 for each registered application.  The counters initially shall be set to zero.  For each update made to the Registry entry of an Application, the corresponding counter shall be incremented by one and managed cyclically.

Additionally, the CRS maintains a global counter in the range from 0 to 65535.  This counter, which is also initially set to zero, is incremented each time an application counter is created, deleted or incremented.  This counter is also managed cyclically.

These counters can be used by on-card or off-card entities to detect that changes have occurred in the Registry and to synchronize their own state.

The GlobalPlatform CRS Application allows retrieval of these parameters through an APDU Command response.

The Global Update Counter is returned in the response of the SELECT command.  The encoding of the tag’s class is SELECT-command specific (see 3.11.5).

The Application Update Counter is retrieved using the GET STATUS command response data.  A TLV can be used in the tag list as filter criterion.

### Selection Priority

The Selection Priority indicates the order in which Contactless Applications will be considered during partial selection over the contactless interface (see 6.2).  The Selection Priority is calculated based on the absolute position of the Contactless Application's entry in the GlobalPlatform Registry, and the Volatile Priority, if assigned.  Contactless Applications assigned Volatile Priority are given the highest priority.  When computing the absolute position, both Contactless and Non-Contactless Applications are taken in to account.

The Selection Priority is coded on one byte.  The value of 0 corresponds to the highest priority.

### Application Group

In Figure 3 1: Application Group, two TLVs are used to indicate the grouping relationship as follows:

- TLV Application Group Head (AID of the Head Application); this tag can be used as a search criterion to retrieve the Application Group Membership List.

- TLV Application Group Membership List; this tag can be used in the tag list.  This is a list of AIDs of Applications that have successfully joined the group.

### CREL Application AID List

The TLV CREL Application AID List provides information related to the CREL Application(s) referenced by an Application.  The CREL Application AID List is a constructed TLV with at least one mandatory tag '4F' which provides the AID of a CREL Application.  The TLV has the following structure:

- TLV CREL Application AID List (Template – constructed tag with sub-data elements) contains the AID(s) of the CREL Application(s) as sub-data elements.

  - Tag '4F': 	AID of 1st CREL Application

  - Tag '4F': 	AID of 2nd CREL Application

  - …

When used as a search criterion, the CREL Application AID List shall contain only one AID.

### GET STATUS Command.

### Definition and Scope

The GET STATUS command is used to retrieve the CRS registered Contactless Applications display information, the Lifecycle status and other information according to the given match/search criteria.  Data on Non-Contactless Applications is not available through this command.  No authentication is required to process this command.

### Command Message

The GET STATUS command message shall be coded according to the following table:

| Code | Value | Meaning |
| --- | --- | --- |
| CLA | '80' |  |
| INS | 'F2' | GET STATUS |
| P1 | 'xx' | Reference control parameter P1 |
| P2 | 'xx' | Reference control parameter P2 |
| Lc | 'xx' | Length of data field |
| Data | 'xx...' | Search criteria |
| Le | '00' |  |

Table 3-9: GET STATUS Command Message for CRS

#### Reference Control Parameter P1

Reference control parameter P1 is used to select a subset of statuses to be included in the response message. It is coded as follows:

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - | 1 | - | - | - | - | - | - | Applications |
| X | - | X | X | X | X | X | X | RFU |

Table 3-10: GET STATUS Reference Control Parameter P1

#### Reference Control Parameter P2

The reference control parameter P2 controls the number of consecutive GET STATUS commands and indicates the format of the response message.  It shall be coded according to the following table:

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | X | X | X | X | X | X | - | RFU |
| - | - | - | - | - | - | - | 0 | Get first or all occurrence(s) |
| - | - | - | - | - | - | - | 1 | Get next occurrence(s) |

Table 3-11: GET STATUS Reference Control Parameter P2

The "Get next occurrence(s)" request shall be rejected if no prior GET STATUS [get first or all occurrence(s)] was received within the current Application Session.

#### Data Field Sent in the Command Message

The data field is structured as follows:

| Tag | Length | Value Description | Presence |
| --- | --- | --- | --- |
| '4F' | 0-16 | Application AID | Mandatory |
| 'xx' or 'xxxx' | 0-n | Other search criteria | Optional |
| ... | ... | ... | ... |
| '5C' | 1-n | Tag list | Optional |

Table 3-12: GET STATUS Command for CRS Data Field

The GET STATUS command message data field shall contain at least one TLV-coded search qualifier: the AID (tag '4F').  It shall be possible to search for all the occurrences that match the selection criteria according to the reference control parameter P1 using a search criteria of '4F' '00'.

It shall be possible to search for all occurrences with the same RID.

Other search criteria may be added. In such cases, the additional search criteria shall be TLV coded and appended after the mandatory search criterion AID (tag '4F').

The search shall be limited to Applications accessible on a contactless interface according to the Communication Interface Access Configuration Parameters defined in section 5.  When the tag '9F70' is used as a search criterion, Application-specific lifecycle state bits in the first byte shall be ignored.

The tag list (tag '5C') indicates to the CRS Application how to construct the response data for each on-card entity matching the search criteria.  Its value field contains a concatenation of tags (without delimitation) indicating the data objects to include in the response.

### Response Message

#### Data Field Returned in the Response Message

Based upon the search criteria of the GET STATUS command data field the value of the reference control parameter P1 and P2, multiple occurrences of the data structure as described in the following paragraphs may be returned.

When no tag list (tag '5C') is present in the command data field, the data structure shall be set according to the Table 3-13.  For the purpose of this table, the following definitions of Mandatory, Conditional, and Optional, apply:

- Mandatory: Shall be present in the response (if its super-tag is present in the response)

- Conditional: Shall be present in response if at least one Application matches the search

- Optional: Shall be present if the tag exists in the Contactless Registry

| Tag | Length | Value Description |  |  |  |  | Presence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| '61' | 7-n | Application Template |  |  |  |  | Conditional |
|  |  | Tag | Length | Value Description |  |  |  |
|  |  | '4F' | 5-16 | Application AID |  |  | Mandatory |
|  |  | '9F70’ | 2 | Application Lifecycle State |  |  | Mandatory |
|  |  | '7F20' | var | Display Control (Template – constructed tag with sub-data elements) |  |  | Optional |
|  |  |  |  | Tag | Length | Value Description |  |
|  |  |  |  | '5F50' | var | Uniform Resource Locator | Optional |
|  |  |  |  | '5F44' | var | Application Image (Icon or logo associated with an application) | Optional |
|  |  |  |  | '5F45' | var | Display Message (Data element containing a message to display) | Optional |
|  |  | '80' | 2 | Application Update Counter (big endian) |  |  | Mandatory |
|  |  | '81' | 1 | Priority indicator |  |  | Mandatory |
|  |  | 'A2' | var | Application Group Head (Head Application AID of the group that this application belongs to) |  |  | Optional |
|  |  |  |  | Tag | Length | Value Description |  |
|  |  |  |  | '4F' | 5-16 | Application AID | Mandatory |
|  |  | 'A3' | var | Application Group (Template – constructed tag with sub-data elements) |  |  | Optional |
|  |  |  |  | Tag | Length | Value Description |  |
|  |  |  |  | '4F' | 5-16 | Application AID | Mandatory |
|  |  |  |  | '4F' | 5-16 | Application AID | Optional |
|  |  |  |  | '4F' | 5-16 | Application AID | Optional |
|  |  | 'A4' | var | CREL Application AID List |  |  | Optional |
|  |  |  |  | Tag | Length | Value Description |  |
|  |  |  |  | '4F' | 5-16 | AID of the CREL Application | Mandatory |
|  |  |  |  | '4F' | 5-16 | AID of the CREL Application | optional |
|  |  | 'A5' | var | Policy Restricted Applications |  |  | Optional |
|  |  |  |  | Tag | Length | Value Description |  |
|  |  |  |  | '4F' | 5-16 | AID of the Policy Restricted Application (e.g. PPSE) | Mandatory |
|  |  |  |  | … | … | … | … |
|  |  | ‘A6’ | Var | Application discretionary data |  |  | Optional |
|  |  | '87' | 1 | Application Family |  |  | Optional |
|  |  | '88' | 1 | Display Required Indicator |  |  | Optional |
|  |  | '8C' | var | Assigned Protocols |  |  | Optional |
|  |  | '8A' | 1-2 | Continuous Processing |  |  | Optional |
|  |  | '8B' | var | Recognition Algorithm |  |  | Optional |

Table 3-13: On-Card Registered Contactless Applications Data (TLV)

When a partial AID is provided as input, all corresponding Applications, regardless of being a Standalone Application, Head Application, or a member of a group, shall be returned provided that the other search criteria are fulfilled.

When tag list (tag '5C') is present in the command data field, the response shall contain tag '61' and a conditional set of TLV-coded Application related data.  As an example, Table 3-14 indicates the format of this response data when the filter criterion is '4F9F707F20805F44'.  Tag '61' shall only contain tags that are listed in the tag list (tag '5C') of the command data field for the applications matching the search criteria.  The order of the data objects within the application related data (template '61') is arbitrary.  If none of the registry entries contain any of the tags listed in the tag list (tag '5C'), an error status message shall be returned.

For the purpose of Table 3-14, below, the following definitions of Conditional and Optional apply:

- Conditional: Shall be present in the response data if at least one Application matches the search

- Optional: The tag shall not be present if the content does not exist in the Contactless Registry

| Tag | Length | Value Description |  |  | Presence |
| --- | --- | --- | --- | --- | --- |
| '61' | Variable | Application related data |  |  | Conditional |
|  |  | Tag | Length | Value Description |  |
|  |  | '4F' | 5-16 | AID | Optional |
|  |  | ‘9F70' | 2 | Application Lifecycle State | Optional |
|  |  | '7F20' | var | Display Control Template (including all available sub-tags and their values) | Optional |
|  |  | '80' | 2 | Application Update Counter (big endian) | Optional |
|  |  | '5F44' | var | Application Image | Optional |

Table 3-14: Example of response data returned when filter criteria are provided

#### Processing State Returned in the Response Message

A successful execution of the command shall be indicated by status bytes '90' '00'.

The command may return the following warning condition:

| SW1 | SW2 | Meaning |
| --- | --- | --- |
| '63' | '10' | More data available |

Table 3-15: GET STATUS Warning Condition

Following status '63' '10', a subsequent GET STATUS [get next occurrence(s)] may be issued to retrieve additional data.

This command may return a general error condition as listed in section 11.1.3 of GP22 [0], or one of the following error conditions.

| SW1 | SW2 | Meaning |
| --- | --- | --- |
| '6A' | '88' | Referenced data not found |
| '6A' | '80' | Incorrect values in command data |

Table 3-16: GET STATUS Error Conditions

### SET STATUS Command

### Definition and Scope

The SET STATUS command shall be used to modify the Availability State of CRS registered Contactless Application(s), and their priority in the Application Selection process.  In addition, this command can be used to globally switch on/off the contactless interface.  Non-Contactless Application data cannot be modified through this command.  No authentication is required to process this command.

### Command Message

The SET STATUS command message is coded according to the following table.

| Code | Value | Meaning |
| --- | --- | --- |
| CLA | '80' |  |
| INS | 'F0' | SET STATUS |
| P1 | 'xx' | Status type |
| P2 | 'xx' | Status value |
| Lc | 'xx' | Length of data field |
| Data | 'xxxxx…' |  |
| Le | '00' |  |

Table 3-17: SET STATUS Command Message for CRS

#### Reference Control Parameter P1 - Status Type

The status type of the SET STATUS command message indicates the type of information that shall be updated.  The status type shall be coded according to the following table:

| b8 | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  |  | b1 |  |  | Meaning |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  | 1 |  | Availability State over the Contactless Interface |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  |  | 0 |  |  | Priority Order for Application Selection |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  |  | 0 |  |  | Communication Interface Access Configuration |
| 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  | 0 |  |  | Remaining Response Data |

Table 3-18: SET STATUS – Status Type

If the status type indicates Remaining Response Data and there are no remaining response data, then an error condition shall be returned with status bytes '6A' '86' (incorrect P1P2 parameters).

#### Reference Control Parameter P2 - Status Value

The meaning of the status value depends on the status type coded in the P1 parameter.

When the status type indicates Remaining Response Data, the status value shall be ignored.

When the status type indicates Availability State over Contactless Interface ('01'), the status value shall indicate the ACTIVATED or DEACTIVATED state and shall be coded as described in Table 8-1.  A request to transition to the NON_ACTIVATABLE state shall be rejected with status bytes '6A' '86' (incorrect P1P2 parameters).  The Applications to which this setting applies are further specified in the Command Data Field.

When the status type indicates Priority Order for Application Selection ('02'), the status value shall be coded according to the following table:

| b8 | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  |  | b2 |  | b1 |  | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  | 1 |  | Assign Highest Priority |  |
| 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  | 1 |  | Assign Lowest Priority |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 1 |  | 0 |  | Assign Volatile Priority |  |
| 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 1 |  | 0 |  | Reset Volatile Priority |  |

Table 3-19: SET STATUS – P2 values for Priority Order for Application Selection

See section 6.2 for a description of Priority and Volatile Priority.  The Applications to which this setting applies are further specified in the Command Data Field.

When the status type indicates Communication Interface Access Configuration ('04'), the status value shall be coded according to the following table:

| b8 | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  |  | b2 |  | b1 |  | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  | 0 |  | Communication interface switched ON |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 | 0 |  | 0 |  | Communication interface switched OFF |  |

Table 3-20: SET STATUS – P2 values for Communication Interface Access Configuration

In this case, the SET STATUS command is used to switch ON or OFF a communication interface globally.  The Communication Interface(s) to which the new setting shall apply is further specified in the Command Data Field.

#### Data Field Sent in the Command Message

When the P1 parameter indicates Remaining Response Data, the content of the data field shall be ignored.

When the P1 parameter indicates Availability State over Contactless Interface ('01') or Priority Order for Application Selection ('02'), the data field shall contain the AID(s) of the target Application(s) for which the modification is requested.

| Tag | Length | Value Description | Presence |
| --- | --- | --- | --- |
| '4F' | 5-16 | AID | Mandatory |
| '4F' | 5-16 | AID | Optional |
|  | ... | ... |  |

Table 3-21: SET STATUS Command Data Field for Status Types P1='01' or P1='02'

When the data field indicates several Applications, the GlobalPlatform CRS Application shall perform the requested update for one Application at a time using the CRS API, starting with the first AID occurring in the command data field.

If an AID present in the data field does not identify any CRS-registered Contactless Application, this AID shall be ignored and other requests present in the command (if any) shall be processed.  Warning information shall be returned as described in section 3.11.4.3.

When the P1 parameter indicates Availability State over Contactless Interface ('01')

- If the command is a request for activation, then all requested activations shall be performed altogether, or none.  Processing of the command for activation shall be aborted after the first conflict is detected.  In this case, all previous changes shall be reversed, and diagnostic information, including the AID of the Application in the command data field that conflicts, shall be returned as described in section 3.11.4.3.

- If activation is requested for an application currently in the NON_ACTIVATABLE state then the Application cannot be activated.  This shall not be considered as an error and other requests present in the command (if any) shall be processed.  Warning information shall be returned as described in section 3.11.4.3.

When the P1 parameter indicates Priority Order for Application Selection ('02')

- If an AID present in the data field identifies a Head Application, all Member Applications from corresponding Application Group (including the Head Application itself) shall have their priority modified accordingly, following their current order in the GlobalPlatform Registry.

This rule applies both to modifications of the GlobalPlatform registry order (P2='01' or P2='81') and to modifications of the Volatile Priority  (P2='02').

- If the P2 parameter indicates Assign Highest Priority then the last listed AID in the command data field shall receive the Highest Priority.

- If the P2 parameter indicates Assign Lowest Priority then the last listed AID in the command data field shall receive the Lowest Priority.

- If the P2 parameter indicates Assign Volatile Priority ('02'), then only a single AID shall be present in the data field, otherwise an error condition shall be returned with status bytes '6A' '80' (wrong data).  If a Volatile Priority was already set up, then it is discarded and this command sets up a new one.

- If the P2 parameter indicates Reset Volatile Priority ('82'), then the content of the data field shall be ignored and the Volatile Priority is reset (i.e. emptied).  It is not necessary to reset the Volatile Priority before assigning a new one.

When the P1 parameter indicates Communication Interface Access Configuration ('04'), then the data field shall be coded as follows:

| Tag | Length | Value Description | Presence |
| --- | --- | --- | --- |
| '80' | 1 | Communication Interface Access Configuration (see Table 5-2) | Mandatory |

Table 3-22: SET STATUS Command Data Field for Status Type P1='04'

If an attempt is made to change the availability of Contact-based communication, then an error condition shall be returned with status bytes '6A' '81' (function not supported).

### Response Message

#### Data Field Returned in the Response Message

Response data may be present and returned depending on the result of the requested operation(s).  When present, the response data field shall be formatted as follows:

| Tag | Length | Value Description |  |  |  |  | Presence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| '61' | Var | Application Template |  |  |  |  | Mandatory |
|  |  | Tag | Length | Value Description |  |  |  |
|  |  | '4F' | 5-16 | AID of the application that could not be activated due to a conflict in protocol parameters. |  |  | Conditional |
|  |  | 'A0' | Var | List of Conflicting Applications |  |  | Conditional |
|  |  |  |  | Tag | Length | Value Description |  |
|  |  |  |  | '4F' | 5-16 | AID of first or only Conflicting Application | Mandatory |
|  |  |  |  | '4F' | 5-16 | AID of next Conflicting Application | Optional |
|  |  |  |  | … | … |  |  |
|  |  | 'A1' | Var | List of applications for which the operation failed for other reasons (e.g. not found or NON_ACTIVATABLE) |  |  | Conditional |
|  |  |  |  | Tag | Length | Value Description |  |
|  |  |  |  | '4F' | 5-16 | AID of first or only non activated Application | Mandatory |
|  |  |  |  | '4F' | 5-16 | AID of next non activated Application | Optional |
|  |  |  |  | … | … |  |  |

Table 3-23: SET STATUS Response Data Field

When the P1 parameter indicates Availability State over Contactless Interface ('01')

- If the processing of the command was aborted due to a conflict in protocol parameters, then the response shall contain the AID ('4F') of the application that could not be activated due to the conflict, as well as the list ('A0') of the currently activated Applications conflicting with this Application.

- If no conflict was detected but some Applications could not be activated for other reasons (e.g. Application not found or Application was in the NON_ACTIVATABLE state), then the response shall contain the list (A1') of these Applications.

When the P1 parameter indicates Priority Order for Application Selection ('02')

- If some Applications could not have their priority changed (e.g. Application not found), then the response shall contain the list (A1') of these Applications.

If response data do not fit in a single response data field, then warning status bytes '63' '10' shall be returned.  In this case, one or more subsequent SET STATUS commands with P1='80' may be issued to retrieve remaining response data.

Following last response data,

- If template tag 'A0' was present in response data (i.e. a conflict was detected), then status bytes '69' '85' (conditions not satisfied) shall be returned. Otherwise,

- If template tag 'A1' was present in response data, then status bytes '63' '20' (warning: some Application(s) could not be operated) shall be returned.

#### Processing State Returned in the Response Message

A successful execution of the command shall be indicated by status bytes '90' '00'.

The command may return the following warning condition:

| SW1 | SW2 | Meaning |
| --- | --- | --- |
| '63' | '10' | More response data available |
| '63' | '20' | Operation failed for some Application(s) |

Table 3-24: SET STATUS Warning Conditions

This command may either return a general error condition as listed in section 11.1.3 of GP22 [0], or one of the following error conditions:

| SW1 | SW2 | Meaning |
| --- | --- | --- |
| '69' | '85' | Activation conflict detected |
| '6A' | '80' | Wrong data in data field |
| '6A' | '86' | Incorrect P1P2 parameters |

Table 3-25: SET STATUS Error Conditions

### SELECT Command

### Definition and Scope

The GlobalPlatform CRS Application SELECT command is identical to the SELECT command described in section 11.9 of GP22 [0] except for the Response Message, as described below.

### Response Message

The GlobalPlatform CRS Application SELECT response shall be coded as described in (Editor to do,

| Tag | Length | Value Description |  |  |  |  | Presence |
| --- | --- | --- | --- | --- | --- | --- | --- |
| '6F' | Var | FCI Template |  |  |  |  | Mandatory |
|  |  | Tag | Length | Value Description |  |  |  |
|  |  | '84' | Var | GlobalPlatform CRS Application AID (see section 3.11.1) |  |  |  |
|  |  | 'A5' | var | FCI Proprietary Template |  |  |  |
|  |  |  |  | Tag | Length | Value Description |  |
|  |  |  |  | '9F08' | 2 | Version number (2 bytes), Value '01' '00' |  |
|  |  |  |  | '80' | 2 | Global Update Counter (big endian) |  |

Table 3-26: GlobalPlatform CRS Application SELECT Response

### GET DATA Command

### Definition and Scope

The GlobalPlatform CRS Application GET DATA command is used to retrieve the version number and global update counter.

### Command Message

The GET DATA command message is coded according to the following table.

| Code | Value | Meaning |
| --- | --- | --- |
| CLA | '80' |  |
| INS | 'CA' | GET DATA |
| P1 | '00' | P1 value |
| P2 | 'A5' | P2 value |
| Le | '00' |  |

Table 3-27: GET DATA Command Message for CRS

### Response Message

The GlobalPlatform CRS Application GET DATA response shall be coded as described in Table 3-28.

| Tag | Length | Value Description |  |  |
| --- | --- | --- | --- | --- |
| 'A5' | var | FCI Proprietary Template |  |  |
|  |  | Tag | Length | Value Description |
|  |  | '9F08' | 2 | Version number (2 bytes), Value '01' '00' |
|  |  | '80' | 2 | Global Update Counter (big endian) |

Table 3-28: GlobalPlatform CRS Application GET DATA Response

# Contactless Protocol Management

## Overview and Scope

This section defines

- the OPEN's responsibility for Contactless Registry parameters

  - Current Protocol Parameters

  - Conflict Detection rules

- Contactless parameter structure

  - Protocol Parameters

  - Protocol Parameters Profiles

## The OPEN requirements

The OPEN owns the default parameter values. Only the Issuer Security Domain is able to update these default parameter values.  To do so, the INSTALL [for registry update] command is used without providing an AID. The default parameter values may be retrieved through the CRS API.

The OPEN is also responsible for computation of the Current Protocol Parameters used in the interaction with the Proximity Coupling Device (PCD).  These Current Protocol Parameters will be computed whenever an Application is ACTIVATED or DEACTIVATED or Protocol Parameters are updated (see Figure 4-1), so that these parameters reflect the requirements of all activated applications.  If an activation request would result in a conflict (see Figure 4-3), the OPEN shall reject this request.

The GlobalPlatform Registry is responsible for storage of the Application’s Contactless Protocol Parameters.  These parameters may be provided at application installation or using INSTALL [for registry update].  Additionally, an Application may update and retrieve its own Contactless Protocol Parameters using the CRS API.  If no specific Protocol Data are provided for an Application, the Current Protocol Parameter settings are used.

## Contactless Protocol Parameters

The OPEN supports the following protocols:

- Protocol Type A: Defines the parameter values for initialization, anti-collision and protocol activation, for Type A. See ISO 14443-3 [1] and ISO 14443-4 [2].

- Protocol Type B: Defines the parameter values for initialization, anti-collision and protocol activation, for Type B (see [1]).

- Protocol Type F as defined in ISO/IEC 18092 [8] as 212 kbps and 424 kbps passive mode:eDefines the parameter values for initialization, anti-collision and protocol activation, for type F (see .JIS X 6319-4 [12]).
EDITOR's Note: I need a JIS reference/version of the APDU-capable revision)

An Application is able to request specific values in the Protocol Data for a particular protocol type.  If the Application has no mandatory value for a specific protocol type, it shall not provide Protocol Data parameters for this protocol type.

For each protocol type, Protocol Parameters consist of:

- a Protocol Data setting initialization and anti-collision values for transactions with the PCD

- a Mandatory Mask identifying the specific fields and operation (e.g. XOR or MAX) within the Protocol Data to be considered while combining fields  the Protocol Data for conflict detection and current protocol parameter computation

The Protocol Parameters contain basic fields (byte or short values) or LV structured fields.  The length parameter of LV fields indicates the maximum length of the value part of this parameter if it is indicated in the Mandatory Mask for this parameter.

For basic fields, strict equality or maximum values may be requested if it is indicated in the Mandatory mask of a basic field.

The OPEN is responsible for computing the Current Protocol Parameters for each supported protocol type.  The Current Protocol Parameters are the result of combining the Default Protocol Parameters and the Protocol Parameters of activated Applications.

The OPEN owns the Default Protocol Parameters for all supported protocols.  There are no mandatory bits within the Default Protocol Parameters.

## Current Protocol Parameter computation

The following procedure to calculate the Current Protocol Parameters applies for each protocol type independently.  This procedure is triggered in the case of any one of the following events:

- a change to the Default Protocol Data

- a change to the Application-specific Protocol Data of an ACTIVATED Application

- a transition of an Application to the ACTIVATED or to the DEACTIVATED state

If the change or transition occurs during a contactless session, the procedure is applied and the change will take effect after the current contactless session is finished.  Section 6.3 defines the contactless session.

The Current Protocol Parameters are the result of the reconciliation of the Default Protocol Data and the Application-specific Protocol Data of all the ACTIVATED applications.

### Current Protocol Parameter initialization

The Current Protocol Parameters are initialized with the Default Protocol Parameters of the card.

- The Protocol Parameter Data is set equal to the Default Protocol Parameter data, and

- The Mandatory Mask is set to zero.

### Current Protocol Parameter: Computation on Activation

The Current Protocol Parameters shall be updated upon Application activation.  To do so, the OPEN shall combine the Protocol Data computed over the currently ACTIVATED Applications and the Protocol Data of the newly ACTIVATED Application.

Note:  If the Application Mandatory Mask contains mandatory bits in a byte index higher than the current maximum length, a conflict exists and the computation does not begin and the Current Protocol Parameters remain unchanged.  For this reason, conflicts need not to be considered in the following description.

The “Protocol Data” of the Current Protocol Parameters is computed as follows, for each protocol parameter data field, in order to reflect the Application's Mandatory Protocol Data.  The first step is to pre-format the parameter data so that the computation of the Current Protocol Parameters can be executed.

For values with numeric representation (e.g. data rates), the following rules apply:

- If an Application requests (i.e. indicated by a Mandatory Mask) a maximum value, and the Current Protocol Parameter requests a different maximum value, then the minimum of these two values is taken as the result.

- If an Application does not request a maximum value, then the current Protocol Parameter is kept.

For the LV structured parameters, the following rules apply:

- If an Application requests a maximum value, and the Current Protocol Parameter requests a different maximum value, then the minimum of these two values is taken is the result for the value of L.  For the next step of the processing of these parameters, the value field of the longer LV will be truncated to the length of the resulting L value, keeping the leftmost L bytes.

- If an Application does not request a maximum value, and the Current Protocol Parameter requests a maximum value, then the Application protocol data and the Mandatory Mask shall be aligned starting from the leftmost byte to the length of the Current Protocol Parameters.  Alignment is accomplished either by truncation at the rightmost byte or by padding with '00's at the rightmost byte.

- If neither an Application nor the Current Protocol Parameter requests a maximum value, then the protocol data and the Mandatory Masks shall be aligned to the maximum length of both.  Alignment is accomplished by padding with '00's at the rightmost byte.

For the basic fields and value fields of a LV structured parameter, the following rules apply:

- Current Protocol Parameter’s Protocol Data is updated with the Application mandatory bits set to 1: "Intermediate Result" [C] = "Current Protocol Data" OR [A], where [A] is a mask forcing bits to "1" and is calculated as [A] = "Application Protocol Data" AND "Application Mandatory Mask".

- The Intermediate Result is updated with the Application mandatory bits set to 0 [B] to obtain the new Protocol Data. "Current Protocol Parameter’s Protocol Data" = "Intermediate Result" [C] AND [B], where [B] = NOT (NOT("Application Protocol Data") AND "Application Mandatory Mask").

Figure 4-1 illustrates the computation of the Current Protocol Parameters for basic fields and the values within LVs with their Mandatory Masks set to 'FF', according to the rules given above.

Figure 4-1: Current Protocol Parameters: Protocol Data computation

The "Mandatory Mask" computation of the Current Protocol Parameters is illustrated in the figure below.

Figure 4-2: Current Mandatory Mask Computation

The “Mandatory Mask” of the Current Protocol Parameters is computed as follows:

- "Mandatory Mask" <- "Mandatory Mask" OR "Application Mandatory Mask"

### Current Protocol Parameter: Full computation

The Current Protocol Parameters shall be fully recomputed

- When an Application is de-activated or when the Default Protocol Parameters are updated, the Current Protocol Parameters shall be computed as follows:

  - The current protocol is set to the initial value, as specified in section 4.4.1.

  - The current protocol shall be recomputed for each activated Application as specified in section 4.4.2.

- When the Protocol Data of one of the activated Applications is updated,
the following process applies:

  - The Application is temporarily deactivated.

  - The current protocol is computed as for Application deactivation.

  - The Conflict Detection is launched with the updated Application Protocol Data.

    - If there is no conflict, the current protocol shall combine the new Application Protocol Data as specified in section 4.4.2.

    - If there is a conflict, the Application remains de-activated. The CRS Application may be requested to perform a conflict resolution action.

## Conflict Detection Procedure

The conflict detection algorithm shall check that one set of Protocol Parameters does not conflict with another one.

The conflict detection shall be performed:

- when an Application is activated,

- when protocol parameters of an activated Application are set or updated

- during conflict resolution to retrieve the Applications that conflict with the one to be activated

The figure below illustrates the Conflict Detection Algorithm that shall be implemented by the OPEN.  It describes conflict detection between the Protocol Parameter 1 and the Protocol Parameter 2 for a given protocol type.  Conflict detection shall be launched upon activation with the current protocol parameter or during conflict resolution with the Application involved.  In such cases, the following assignment applies:

- Protocol Parameter 1 is the Protocol Data of the Application being evaluated upon activation or during conflict resolution.

- Protocol Parameter 2 is the Current Protocol Data or Application participating in the conflict resolution (e.g. current Activated Applications for the given protocol type).

Figure 4-3: Protocol Parameter Conflict Detection

- For each field of the protocol parameter, the RESULT depends on the operation associated with the protocol data field being evaluated.

  - For LV structured fields with different lengths, a conflict exists if:

    - One of the protocol parameters requests a MAX value and the other contains mandatory bits in its mask of a byte index greater than the MAX value

    - If both protocol parameters request a different MAX value and the parameter with a higher MAX value contains mandatory bits in its mask of a byte index greater than the smaller MAX value

  - If no length conflict exists, further conflict detection will take in to account the smaller size of the two parameter sets (data and mask).

- For basic fields and lengths of LV fields and values, the RESULT is computed as follows:

  - If the operator associated with a field is MAX, then 

if the Protocol Data Parameter 1 is greater than or equal to Protocol Data Parameter 2, the RESULT is '00' to indicate no conflict, '01' otherwise.

  - Otherwise, a logical XOR between the value of the Protocol Data Parameter 1 and Protocol Data Parameter 2 produces the value of RESULT.

- The COMMON MASK is computed by performing a logical AND between the Mandatory Mask of protocol parameter 1 and the Mandatory Mask of protocol parameter 2 of the same protocol type

- The CONFLICT is identified by applying a logical AND between the DIFFERENCE and the COMMON_MASK

- If one single bit of the CONFLICT is set to 1 then protocol parameter 1 is in conflict with protocol parameter 2.  Therefore, the Application is conflicting and cannot be set to ACTIVATED at this time.

## Protocol Parameters for Type A (Card Emulation Mode)

The value part of the Protocol Data Type A TLV (see Table 11-3) when addressed to the OPEN is used to establish or update the default value.  It has the following coding:

| Tag | Len | Value description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 'A0' | Var | Protocol Parameter Data |  |  |  |
|  |  | Tag | Len | Value Description |  |
|  |  | '80' | var | Unique Identifier is LV structured |  |
|  |  |  |  | Length | Value Description |
|  |  |  |  | 0, 4, 7 or 10 | Unique Identifier values as defined in ETSI TS 102 622 [3]. There is no consistency check with UID size which is coded in ATQA See ISO 14443-3 [1]. |
|  |  | '81' | 1 | SAK: Preformatted Select AcKnowledge as defined in [1], when the UID is complete |  |
|  |  | '82' | 2 | ATQA: Coding of the ATQA as defined in [1]. |  |
|  |  | '83' | 1-15 | ATS LV structure. |  |
|  |  |  |  | Length | Value Description |
|  |  |  |  | 0-15 | ATS Historical bytes in answer to select as defined in [2]. |
|  |  | '84' | 1 | FWI, SFGI: Frame waiting time and Start-up frame guard time as defined in [2]. for Type A |  |
|  |  | '85' | 1 | CID Support: Support of the Card Identifier field as defined in [2]. '01' ATS indicates "CID Support" '00' ATS may not indicate “CID Support" |  |
|  |  | '86' | 3 | DATA_RATE_MAX: Maximum data rate supported as defined in [3] |  |

Table 4-1: OPEN-Protocol Data for Type A value field

The following table describes the coding Protocol Data value fields, for the value part of the ISO 14443 Type A TLV (see Table 11-3), requested by an Application.

| Tag | Len | Description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 'A0' | Var | Protocol Parameter Data |  |  |  |  |  |  |  |  |  |
|  |  | Tag | Len | Value Description |  |  | Operator |  |  |  |  |
|  |  | '80' | var | Unique Identifier: Value is LV structured |  |  |  |  |  |  |  |
|  |  |  |  | Len | Value Description |  |  |  |  |  |  |
|  |  |  |  | 0, 4, 7 or 10 | Unique Identifier values as defined in ETSI TS 102 622 [3]. There is no consistency check with UID size is coded in ATQA. See ISO 14443-3 [1]. |  | MAX |  | XOR |  |  |
|  |  | '81' | 1 | SAK: Preformatted Select AcKnowledge as defined in [2], when the UID is complete |  |  | XOR |  |  |  |  |
|  |  | '82' | 2 | ATQA: Coding of the ATQA as defined in [1]. |  |  | XOR |  |  |  |  |
|  |  | '83' | 1-15 | ATS LV structure. |  |  |  |  |  |  |  |
|  |  |  |  | Len | Value Description |  |  |  |  |  |  |
|  |  |  |  | 0-15 | ATS Historical bytes in answer to select as defined in [2] |  | MAX |  |  | XOR |  |
|  |  | '84' | 1 | FWI, SFGI: Frame waiting time and Start-up frame guard time as defined in [2] for Type A |  |  | MAX |  |  | MAX |  |
|  |  | '85' | 1 | CID Support: Support of the Card Identifier field as defined in [2]. '01' ATS indicates "CID supported" '00' ATS indicates "CID not supported" |  |  | XOR |  |  |  |  |
|  |  | '86' | 3 | DATA_RATE_MAX: Maximum data rate supported as defined in ETSI TS 102 622 [3]. |  |  | MA X | MAX |  |  | XOR |
| 'A1' | Var | Protocol Parameter Mandatory Mask |  |  |  |  |  |  |  |  |  |
|  |  | '80' | Var | Mandatory Mask of LV UID |  |  |  |  |  |  |  |
|  |  |  |  | Length Mask |  | Value Mask |  |  |  |  |  |
|  |  |  |  | '00': Application doesn’t care about UID length, 'FF': Application does care about UID length |  | Mask for mandatory bits within the UID value. |  |  |  |  |  |
|  |  | '81' | 1 | Mask for mandatory bits within SAK |  |  |  |  |  |  |  |
|  |  | '82' | 1 | Mask for mandatory bits within ATQA. |  |  |  |  |  |  |  |
|  |  | '83' | Var | Mandatory Mask of LV ATS |  |  |  |  |  |  |  |
|  |  |  |  | Length Mask |  | Value Mask |  |  |  |  |  |
|  |  |  |  | '00': Application doesn’t care about ATS historical bytes length, 'FF': Application requests that ATS historical bytes length does not exceed a maximum. |  | Mask for mandatory historical bit in answer to select as defined in ISO 14443-4 [2]. |  |  |  |  |  |
|  |  | '84' | 1 | Mask for FWI, SFGI '00': Application doesn’t care about the FWI and SFGI values 'F0': Application requests FWI does not exceed the specified value, but does not care about the SFGI value. 'FF': Application requests that the values for FWI and SFGI do not exceed their specified values. '0F': Application requests that the value of SFGI does not exceed the specified value and does not care about the value of FWI. |  |  |  |  |  |  |  |
|  |  | '85' | 1 | Mask for mandatory bits in CID_SUPPORT |  |  |  |  |  |  |  |
|  |  | '86' | 3 | Mask for mandatory bits within DATARATE_MAX. - Byte 1: 00, Application does not care about the Maximum bit rate in the PCD to PICC direction - FF, Application requests that the indicated Maximum bit rate in the PCD to PICC direction is not exceeded - Byte 2: 00, Application does not care about the Maximum bit rate in the PICC to PCD direction - FF, Application requests that the indicated Maximum bit rate in the PICC to PCD direction is not exceeded - Byte 3: 00, Application does not care about this parameter - FF, Application requests that value of Byte 3 of DATARATE_MAX is respected |  |  |  |  |  |  |  |

Table 4-2: Application-Protocol Data for Type A value field

## Protocol Parameters for Type B (Card Emulation Mode)

The value part of the Protocol Data Type B TLV (see Table 11-3) when addressed to the OPEN is used to establish or update the default value.  It has the following coding:

| Tag | Len | Value description |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 'A0' | Var | Protocol Parameter Data |  |  |  |
|  |  | Tag | Len | Value Description |  |
|  |  | '80' | 1 or 5 | PUPI: Value is LV structure |  |
|  |  |  |  | Length | Value Description |
|  |  |  |  | 0 or 4 | Pseudo Unique PICC Identifier. Coding is as specified in ETSI TS 102 622 [3] |
|  |  | '81' | 1 | AFI: Application Family Identifier as defined in ISO 14443-3 [1]. |  |
|  |  | '82' | 2 | ATQB: Coding of the ATQB as defined in [3] |  |
|  |  | '83' | var | Higher layer response in response to ATTRIB as defined in ETSI TS 102 622 [3]. Value is a LV structure |  |
|  |  |  |  | Length | Value Description |
|  |  |  |  | var | Application historical bytes in answer to select as defined in [3] |
|  |  | '84' | 3 | Maximum data rate supported as defined in [3] |  |

Table 4-3: OPEN-Protocol Data for Type B value field

The OPEN does not enforce the consistency of the AFI and the Application Family defined in section 3.5.

The following table describes the value part of the Protocol Data type for ISO/IEC 14443 Type B TLV (see Table 11-3) requested by an Application:

| Tag |  | Len | Value Description |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 'A0' |  | Var | Protocol Parameter Data |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Tag | Len |  | Value Description |  |  |  | Operator |  |  |  |
|  |  |  | '80' | 1 or 5 |  | PUPI: Value is LV structure |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Length | Value Description |  |  |  |  |  |  |
|  |  |  |  |  |  | 0 or 4 | Pseudo Unique PICC Identifier. Coding is as specified in ETSI TS 102 622 [3] |  |  | XOR |  | XOR |  |
|  |  |  | '81' | 1 |  | AFI: Application Family Identifier as defined in ISO/IEC 14443-3 [1]. |  |  |  | XOR |  |  |  |
|  |  |  | '82' | 4 |  | ATQB: Coding of the ATQB as defined in [3] |  |  |  | XOR |  |  |  |
|  |  |  | '83' | var |  | Higher Layer Response in response to ATTRIB as defined in ETSI TS 102 622 [3]. Value is a LV structure |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Length | Value Description |  |  |  |  |  |  |
|  |  |  |  |  |  | var | Application historical bytes in answer to select as defined in [3] |  |  | MAX |  | XOR |  |
|  |  |  | '84' | 3 |  | Maximum data rate supported as defined in ETSI TS 102 622 [3] |  |  |  | M A X | MAX |  | X O R |
| 'A1' | Var |  | Protocol Parameter Mandatory mask |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Tag | Len | Value Description |  |  |  |  |  |  |  |  |
|  |  |  | '80' | 1 or 5 | Mandatory Mask for LV PUPI |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Length Mask |  |  |  | Value Mask |  |  |  |  |
|  |  |  |  |  | '00': Application doesn’t care about PUPI Length 'FF': Application does care about PUPI length. |  |  |  | Mask for mandatory bits within the PUPI |  |  |  |  |
|  |  |  | '81' | 1 | Mask for mandatory bits within the AFI. |  |  |  |  |  |  |  |  |
|  |  |  | '82' | 4 | Mask for mandatory bits in ATQB |  |  |  |  |  |  |  |  |
|  |  |  | '83' | var | Mandatory Mask for LV Higher Layer Response |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Length Mask |  |  | Value Mask |  |  |  |  |  |
|  |  |  |  |  | '00': Application doesn’t care about Higher Layer Response Length, 'FF': Application requests Higher Layer Response Length to not exceed the specified value |  |  | Mask for mandatory bits in the Higher layer response. |  |  |  |  |  |
|  |  |  | '84' | 3 | Mask for mandatory bits within DATARATE_MAX. • Byte 1: 00, Application does not care about the Maximum bit rate in the PCD to PICC direction • FF, Application requests that the indicated Maximum bit rate in the PCD to PICC direction is not exceeded • Byte 2: 00, Application does not care about the Maximum bit rate in the PICC to PCD direction • FF, Application requests that the indicated Maximum bit rate in the PICC to PCD direction is not exceeded • Byte 3: 00, Application does not care about this parameter • FF, Application requests that value of Byte 3 of DATARATE_MAX is respected |  |  |  |  |  |  |  |  |

Table 4-4: Application-Protocol Data for Type B value field

## Protocol Parameters for Type F (Card Emulation Mode)

Protocol Dat.e part o  Prtocol Data Type F TLV (see Table 11 3) when addressed to the OPEN is used to set up the PMM value with the following coding:

| Tag | Len | Value description |  |  |
| --- | --- | --- | --- | --- |
| 'A0' | Var | Protocol Parameter Data |  |  |
|  |  | Tag | Len | Value Description |
|  |  | '80' | 8 | Manufacture parameter (IC code and response time calculation parameter) |

Table 4-5: OPEN-Protocol Data for Type F value field

The following table describes the value part of the Protocol Data type for Type F protocol TLV (see Table 11 3) requested by an Application:

| Tag | Len | Value description |  |  |
| --- | --- | --- | --- | --- |
| 'A0' | Var | Protocol Parameter Data |  |  |
|  |  | Tag | Len | Value Description |
|  |  | '80' | 2 * N (N = 1 .. 16) | Application-specific code that is used during anti-collision. The PCD requests either a certain system code or a wildcard in the anti-collision request. Only PICCs with that specific system code reply to the request. If the system code doesn't match, the PICC remains silent. |
|  |  | '81' | 8 * N (N = 1 .. 16) | Manufacturer-assigned unique identification number (manufacture ID) |

Table 4-6: Application Protocol Data for Type F value field

There is a one-to-one association between a System code and an IDm.  Therefore, each System code parameter shall be followed by an IDm so that the association is unambiguous.

There is no Mandatory Mask for the Type F protocol.

## Contactless Protocol Parameters Profiles

Contactless Protocol Parameters Profiles are introduced to facilitate initialization of the Contactless Application during instantiation or personalization of the Application.  Instead of providing the values for each Contactless Protocol Parameter for the Application, a profile is provided to identify a collection of defined parameter values.

Contactless Protocol Parameters Profiles belong to the OPEN.

For initialization and update of profiles, the INSTALL [for registry update] command shall be issued to the ISD without an AID in the command data field.  Within the command data field for each profile,

- Protocol Data shall be provided as described in Table 11-3 and,

- Contactless Protocol Parameters Profile shall be present and the data structure shall be as defined in section 11.2.2.  Note that for each protocol type, only one Protocol Parameter Profile can be set in the same INSTALL [for registry update] command.

Each Contactless Protocol Parameters Profile shall provide the Mandatory Mask for each Protocol that it supports.  The "Current Protocol Parameter" computation and conflict detection shall take in to account the Contactless Protocol Parameter Profile, if present, for each Application.

# Communication Interface Access Configuration

## Introduction, Overview and Rationale

This section describes mechanisms which allow configuration of the access of an external entity to an application on the card for each communication interface.  These mechanisms are provided in order to support various deployment scenarios where the access to a Standalone Application or to an Application Group within the same Security Domain hierarchy depends on the communication interface.  In certain scenarios, a Standalone Application or an Application Group shall be available on a particular interface by default without providing specific installation parameters to the INSTALL [for install] command.  In other scenarios it is essential that a particular Standalone Application or Application Group within the same Security Domain hierarchy are not available on a particular interface (e.g. the SIM/USIM/ISIM application on a telecommunication card should not be accessible on the contactless interface).

This section introduces three System Parameters:

Communication Interface Access Restriction 	
This parameter may be provided for a Security Domain.  It defines the set of communication interfaces that an Application may use.

Communication Interface Access Default	
This parameter may be provided for a Security Domain.  It defines the accessibility configuration of an Application which is installed under the hierarchy of the Security Domain and that does not have an Application specific “Communication Interface Access for Instance” parameter.

Communication Interface Access for Instance	
This parameter may be provided for an Application. If supplied, this parameter defines the accessibility configuration of an Application. If missing, the default Communication Interface Access configuration of the Application's associated Security Domain will be used.

The coding of the parameters, the detailed rules associated with these parameters, and the rules which shall be applied in case of the extradition of an application to another Security Domain, are explained in the following sections.  Note: Applications operating in the Java Card™ runtime environment may use APDU.getProtocol() to determine over which communication interface commands are being received.

Communication interface access is independent of the Availability State of the Application on the contactless interface.  Although independent, the communication interface access configuration supersedes the value of the Availability State of an Application.

## Communication Interface Access Parameters

The value portion of the Communication Interface Access Configuration TLV (see Table 11-3) is coded as follows:

| Length | Value Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Var | Communication Interface Access Configuration |  |  |  |  |
|  | Tag | Length | Value Description | SD | Non-SD Application |
|  | '80' | 1 | Communication Interface Access Restriction | Optional | Not Present |
|  | '81' | 1 | Communication Interface Access Default | Optional | Not Present |
|  | '82' | 1 | Communication Interface Access per Instance | Optional | Optional |

Table 5-1: Communication Interface Access Configuration Parameters

Each of the Communication Interface Access Parameters which are defined in this chapter shall be coded according to the following table:

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Communication Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - | - | - | - | - | - | - | Contact-based communication (e.g. ISO 7816) is supported. |
| - | 1 | - | - | - | - | - | - | Proximity-based communication (e.g. ISO 14443) is supported. |
| - | - | x | x | x | x | x | x | RFU |

Table 5-2: Communication Interface Access Configuration Parameter Value

## Security Domain Settings

The Security Domain installation parameter "Communication Interface Access Restriction" defines the set of communication interfaces that an Application may use.  If this parameter is not provided at the time of installation, the value shall be initialized as per the value of the associated Security Domain.  The value of this parameter may be updated using the INSTALL [for registry update] command.  The initial value of the "Communication Interface Access Restriction" parameter of the Issuer Security Domain is platform-dependent.

The Security Domain installation parameter "Communication Interface Access Default" defines the set of communication interfaces that an Application will use by default unless otherwise specified at installation.  The value of this parameter shall not be less restrictive than the "Communication Interface Access Restriction" of this Security Domain.  If this is not the case, an error status code shall be returned.  If this parameter is not provided at the time of installation, the value shall be initialized according to the value of the "Communication Interface Access Restriction" of this Security Domain.  For the Issuer Security Domain, the initial value of the "Communication Interface Access Default" parameter is platform-dependent.  The value of this parameter may be updated using the INSTALL [for registry update] command.

The capabilities of the Security Domain must be compatible with, i.e. not weaken the restrictions set up by, its associated Security Domain.  Restrictions set up by this Security Domain only apply to associated Applications.

## Application Instance Settings

The parameter "Communication Interface Access per Instance" defines the set of communication interfaces that an Application is able to use.

When initializing or updating the value of this parameter, the OPEN shall check that the value of the parameter is not less restrictive than the "Communication Interface Access Restriction" of its associated Security Domain.  If this is not the case, an error status code shall be returned.

The value of this parameter may be updated using the INSTALL [for registry update] command.  The value of this parameter for the ISD is platform-dependent.

If the "Communication Interface Access per Instance" parameter is not provided for a new application at installation time, it shall be set up according to the value of the "Communication Interface Access Default" parameter of its associated Security Domain.

## Rules for Extradition

In the case of extradition, the values of these parameters are not modified.  NOTE: Any updates following the extradition shall follow the restrictions of the newly associated Security Domain.

# Application Selection

## Reset scenarios with multiple active interfaces

If the power remains on while the contactless interface is reset, the following rules shall apply:

- All logical channels of the contactless interface are closed.  The deselect(), or equivalent, method of each Application that is selected on a logical channel is invoked (see [10]).

- The volatile data associated with the activated Applications during the interface session are cleared if the other interface does not have an Application from the same load file selected on any logical channel.

Note: This statement on volatile data management may impact or supersede the current behavior of some runtime environments.

## Application Selection Priority

The Application selection priority is based on the position of the Application's entry in the GlobalPlatform Registry.  Additionally, for selection over the Contactless interface, the Volatile Priority, as defined below, shall be considered.  The Volatile Priority may be assigned by the end user to an Application so that so that the Application shall temporarily receive the highest priority.

### GlobalPlatform Registry Order

The GlobalPlatform Registry is a persistent list that can be reordered.The OPEN provides a service to promote an Application to the top position in the GlobalPlatform Registry.  It also provides a service to demote an Application to the bottom of the GlobalPlatform Registry.  The CRS Application shall use these services to reorder the GlobalPlatform Registry list based on the user priority set up.  For instance, if the user wants to set the priority of an Application to the third position then the CRS Application shall invoke the service to put this Application on top then invoke the service to put the Application with priority 2 on top, and then again invoke the service to put the Application with priority 1 on top.  When using this service to prioritize an Application Group, the CRS Application shall only invoke the service for the Head Application.

During installation, the OPEN shall reorder the entire Application Group to the desired position in the following manner:  The first Application installed as part of the Application Group shall get the user requested priority, the second Application installed as part of the Application Group gets the immediate next priority and so on until the last Application installed as part of the Application Group.  Note that the order in which the Applications are installed plays an important role in prioritization of the Member Applications.

The CRS API provides services to Applications with the CONTACTLESS_ACTIVATION privilege to move the specified Application Group or a Standalone Application, to the top or to the bottom of the list.  When a Head Application is moved, all Member Applications are moved.  The relative order of the Head Application and its Member Applications is not affected.

For example, let's consider a Head Application H, member Applications M1 and M2, and other Applications A1, A2 and A3.  If the current order of the GlobalPlatform registry is { A1, M1, H, A2, M2, A3 }, then a request to give H the highest priority (P2='01') will reorder the GlobalPlatform registry as follows: { M1, H, M2, A1, A2, A3 }.  Similarly, a request to give H the lowest priority (P2='01') will reorder the GlobalPlatform as follows: { A1, A2, A3, M1, H, M2 }.

### Volatile Priority over the Contactless interface

The Volatile Priority may be assigned by the Application with the CONTACTLESS_ACTIVATION privilege to an Application Group or a Standalone Application that shall be considered first upon explicit or implicit selection.  If the Volatile Priority is assigned to an Application Group, the Volatile Priority is assigned to the Head Application and all Member Applications in the same order as in the persistent Registry.

The CRS API provides the capability to setup/discard the Volatile Priority.

The Volatile Priority is discarded upon card reset or power-on.

## Explicit and Implicit Selection over the Contactless Interface

This section extends the Implicit Selection capabilities of GP22 [0] with the addition of Implicit Selection by APDU pattern recognition and Assigned Protocol type on the Basic Logical Channel.

### ISO/IEC 14443 Type A and Type B Applications

Card enters the RF field:

Upon explicit or implicit selection over the contactless interface, the OPEN first looks for the Applications assigned the Volatile Priority.  If no Applications match, the OPEN parses the GlobalPlatform Registry to locate the Application.

The OPEN shall check that the contactless communication interface is switched on.

The first command received after the card activation according to ISO/IEC 14443-3 [1], is used to select the application as described below.

- If the first command is a SELECT [by Name] command:

  - The OPEN locates the targeted Application according to the selection rules defined in GP22 [0] (section 6.4).

  - The located Application becomes a valid candidate for selection if it is ACTIVATED, is selectable according to GP22 [0], and is configured to access the contactless interface.

  - If the Application is not a valid candidate, the OPEN continues to look for another valid candidate.

  - The OPEN shall select the Application. If the Application refuses the selection, the OPEN locates the next Application matching the given AID and restarts at step 1.b until the end of the Application list is reached.

- Otherwise, if the first command is not a SELECT [by Name] command, the Implicit selection by Recognition Algorithm is attempted as follows, when the command is received on the Basic Logical Channel:

  - The OPEN parses the list of applications to look for an application that:

    - Is a valid candidate according to [0],

    - Is ACTIVATED and is configured to access the contactless interface,

    - Supports the current contactless protocol,

    - The Recognition Algorithm succeeds on the received command.

  - The OPEN shall select the Application. If the Application refuses the selection, the OPEN restarts at step 2.a until the end of the Application list.

- Finally, If 1 and 2 have not identified a valid candidate, then

  - The OPEN locates the default Application associated with the contactless interface

  - The Application becomes a valid candidate according to GP22 [0], if it is ACTIVATED, and if it is configured to access the contactless interface.  If not, no Application is selected and the channel remains open.

  - The OPEN shall select the Application. If the Application refuses the selection, no Application is selected and the channel remains open.

#### Card leaves the RF field:

When card leaves the RF field or is deactivated according to ISO/IEC 14443-4 [2] and the power is on, all Applications selected on the contactless interface are deselected and logical channels closed.

### Applications communicating over the Type F Protocol

The lifecycle states as defined in GP22 [0] and section 8 shall be respected.

The procedure depends on the command received according to JIS X 6319-4 [12].

1. If the command is a REQC command:

- The OPEN evaluates the requested System Code, starting from the first parameter entry, and replies with an ATQC response including the corresponding IDm.  A Respond Flag is associated to each System Code indicating whether the System Code shall be considered or not.  The response is generated by the OPEN.

    - If the Respond Flag is 'FF', the OPEN shall not evaluate the System Code and shall not send any responses.

    - If the Respond Flag is '00', the following steps shall be done.  An Application becomes a valid candidate for selection if it is activated, is selectable according to GP 22 [0] and is configured to access the contactless interface, and

      - if the System Code matches one of the System Code parameters of an Application; or

      - if one of the two bytes of the requested System Code is set to 'FF' (which is used as wildcard) and the other byte matches one of the System Code parameters.  E.g. System Code parameter is '1234' and requested System Code is either '12FF' or 'FF34'; or

      - if the requested System Code is 'FFFF'.  In that case the first parameter entry is considered for the ATQC response;

      - Else the procedure is aborted and no response is made.

- The OPEN selects the Application matching the corresponding AID parameter to establish a Type F communication.  If another Application was selected previously it shall be deselected and the corresponding Respond Flag shall be reset (set to '00').  Note: The terms selection and deselection do not correspond with the Java Card™ terms.  More detailed definition is configuration dependent.

2. If the command is NOT a REQC command:

- The OPEN passes the received command to the corresponding application.  This is configuration dependent.

Card leaves the RF field:

When a card leaves the RF field and the power is still on, the Application selected on the contactless interface is deselected and the corresponding Respond Flag is reset to the initial value.  Further details are configuration dependent.

  - Continuous Processing

An application may be installed with a continuous processing indicator.  When such an Application is selected on the contactless interface, the counter begins on the time-out and any command received on the contact interface is postponed until the contactless session ends or the time-out defined below expires and the current contactless APDU processing is completed.  This applies to all protocols used on the contactless interface.

The following table describes the value part of the "Continuous Processing" tag (see Table 11-3) applicable to the OPEN:

| Length | Value Description |
| --- | --- |
| 2 | Continuous processing timeout. Unsigned short value: Time in ms (default: 0 ms) |

Table 6-1: OPEN - Continuous processing value description

In the context of the OPEN, the value is the continuous processing timeout value.  Note: Due to card internal inaccuracy, deviations of up to 10 ms may occur.

The following table describes the value part of the "Continuous Processing" tag (see Table 11-3) requested by an Application:

| Length | Value Description |
| --- | --- |
| 1 | '01': Continuous processing is disabled (default if TLV is missing) '02': Continuous processing when this Application is selected on the contactless interface is enabled |

Table 6-2: Application - Continuous processing value description

This TLV controls the continuous processing requirements of the Application.

## Recognition Algorithm

The recognition algorithm provides the capability to identify and select, on the first command of the contactless session (see section 6.3) received on the Basic Logical Channel, a legacy Contactless Application not supporting the SELECT by AID command.  The Recognition Algorithm does not apply for supplementary logical channels.

| Length | Value Description |
| --- | --- |
| var | Algorithm Identifier: 1 byte Algorithm Parameter: Length-1 bytes |

Table 6-3: Recognition Algorithm

The coding of the Algorithm Identifier is as follows:

'01'	String recognition

'02'	Bitmap recognition

The coding of the Algorithm Parameter is as follows:

- For String Recognition:

- Offset: 2 bytes

- Pattern: X bytes

The pattern associated with an Application is a string of X bytes to be compared against the first APDU received on the contactless interface starting from a given offset.  Recognition is successful if there is a bitwise match (XOR) of the pattern.  If the pattern exceeds the end of the APDU, the recognition is considered not successful.

- For Bitmap Recognition

- Reference Data: X bytes

- Mask : X bytes

Reference Data and Mask have the same length.  Recognition is successful if (DATAIN AND Mask) XOR Reference Data is equal to 0, where DATAIN is the first APDU received on the contactless interface.  If the length of the APDU does not match the length of the reference data/mask, the recognition is considered not successful.

## Assigned Protocols for Implicit Selection

An Application can configure its ability to be implicitly selected (as specified above) on the contactless interface, using any of the protocol types A, B, or F.  This mechanism is provided to support Applications that are not selected by the PCD with an explicit SELECT [by Name] command, and that support only a subset of these protocols.

The Assigned Protocols for Implicit Selection TLV defines the protocol types that are supported by an Application

- If this TLV is not provided or is provided and is zero length, the Application is a valid candidate for implicit selection through any of these protocol types.

- If this TLV has nonzero length then the protocol types which are listed are usable for the application.  The OPEN shall not perform implicit selection for any protocol type which is not listed.

| Length | Value Description |
| --- | --- |
| 0-3 | List of protocol types assigned for the application: '81': Type A '82': Type B '84': Type F |

Table 6-4: Assigned Protocol for Implicit Selection value field

# Contactless Privilege

## Contactless Activation Privilege

The Contactless Activation privilege identifies the CRS Application.  There shall be at most one Application in the secure element that is assigned this Privilege.  This rule shall be enforced by the OPEN.  In order to assign this privilege to another Application, the privilege shall first be removed from the current Application that is assigned this privilege or the Application with this privilege shall be deleted.

This Privilege allows:

- The Activation/Deactivation of Applications on the Contactless Interface

- The update of the Selection Priority

  - Manage the Volatile Priority

  - Reorder the GlobalPlatform Registry

- Notification by the OPEN when:

  - An application is INSTALLED, LOCKED, unlocked or deleted

  - The availability state of an Application is changed between NON_ACTIVATABLE, ACTIVATED, or DEACTIVATED.

  - One of the Application’s contactless registry parameters is updated.

## Contactless Self-Activation Privilege

The Contactless Self-Activation Privilege allows an Application to activate itself without a prior request to the CRS Application.  This Privilege can be assigned to any Application and allows an Application to transition to the ACTIVATED state itself.  Activation fails if it would result in a conflict with currently activated Applications.  Upon successful activation, the CRS Application and associated CREL Applications, if any, are notified of this activation.

An Application with the Contactless Activation Privilege can also transition an Application with the Contactless Self-Activation Privilege to the ACTIVATED or DEACTIVATED states.

## Privilege Coding

The following table updates and replaces Table 11-9, Privileges (Byte 3), of GP22 [0]:

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning | Privilege Number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - | - | - | - | - | - | - | Receipt Generation | 16 |
| - | 1 | - | - | - | - | - | - | Ciphered Load File Data Block | 17 |
| - | - | 1 | - | - | - | - | - | Contactless Activation | 18 |
| - | - | - | 1 | - | - | - | - | Contactless Self-Activation | 19 |
| - | - | - | - | X | X | X | X | RFU | - |

Table 7-1: Privileges

# Application Availability on the Contactless Interface

The ability of an Application to communicate through the contactless interface might be restricted in general, upon installation or registry update (see section 5).  The contactless interface might also be fully deactivated as described in section 3.11.4 (SET STATUS).

When no such restrictions exist, the ability of an Application to communicate through the contactless interface may still be temporarily activated or deactivated.  To reflect this, the following new Application states are defined: ACTIVATED, DEACTIVATED, and NON_ACTIVATABLE.

The value of any of the following; Contactless Activation State, Communication Interface Access Configuration and Contactless Interface Availability, does not impact the value of any of the others.

## Contactless Activation State

The Contactless Activation State exists concurrently with the existing Application Lifecycle State.  The encoding of these new States is shown in the table below:

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | - | - | - | - | - | - | 1 | ACTIVATED |
| 0 | - | - | - | - | - | - | 0 | DEACTIVATED |
| 1 | - | - | - | - | - | - | 0 | NON_ACTIVATABLE |
| - | X | X | X | X | X | X | - | RFU (Reserved for future use) |

Table 8-1: Contactless Activation State byte coding

The Contactless Activation State byte defined above is encoded independently of the Application Lifecycle State byte.  The value of the Contactless Activation State byte can be retrieved independently using the GET STATUS command on the CRS Application or the associated Security Domain.

These states shall be interpreted as follows:

- An Application currently in the ACTIVATED state is able to communicate through the contactless interface.

- Conversely, an Application currently in the DEACTIVATED state is not able to communicate through the contactless interface.

- An Application in the NON_ACTIVATABLE state is implicitly DEACTIVATED, and due to some internal reason known by the Application or its provider (e.g. a possible attempt of fraudulent use) cannot be ACTIVATED.  Any attempt to activate an Application that is currently in the NON_ACTIVATABLE state, shall fail.

Transitions between these states abide by the following rules:

- An Application is always able to transition itself into the DEACTIVATED state.

- An Application cannot transition itself into the ACTIVATED state, except if it was granted the Contactless Self-Activation Privilege.

- A unique Application may be granted the Contactless Activation privilege, allowing it to activate/deactivate any Application on the card.

- Applications registered as listeners for contactless events occurring to an Application (see section 3.8.2), are allowed to deactivate that particular Application.

- At any moment, an Application may transition itself into the NON_ACTIVATABLE state, therefore indicating to the OPEN that it is both deactivated and not in a suitable internal state to become ACTIVATED on the contactless interface.

- An Application is able to transition itself from the NON_ACTIVATABLE state to the DEACTIVATED state, hence indicating to the OPEN that it is in a suitable state to become ACTIVATED again, whenever this request would be made.

The following diagram summarizes the rules described above:

Figure 8-1: Contactless Activation States

In addition, the following rules apply, which are not illustrated in Table 8-1 above:

- When an Application becomes selectable for the first time (i.e. after installation), the OPEN may attempt to activate the Application according to the means described below in section 8.2, depending on the default policy.  This attempt shall fail if the activation of the Application conflicts with other currently activated Applications.

- An Application cannot communicate through the contactless interface if it is not in a selectable state.  This is the case, in particular, when the Application is in the LOCKED state.

- When an Application is unlocked, it becomes NON_ACTIVATABLE if it was in that state previously to the transition to the LOCKED state, otherwise it becomes DEACTIVATED.

## Initial Contactless Activation State

When an application is installed and made selectable, its Initial Contactless Activation State depends upon a parameter owned by the OPEN.  The default value of this parameter is configuration-dependent.  It can only be updated by the Issuer Security Domain using the "Initial Contactless Activation State" tag defined in Table 11-3.

This TLV may only be present in the INSTALL [for registry update] command when the command is processed by the Issuer Security Domain and no Instance AID is present, otherwise an error shall be returned.

The value part of the "Initial Contactless Activation State" TLV requested by the Issuer Security Domain is described in the table below:

| Length | Value Description |
| --- | --- |
| 1 | '00' Applications are installed in the DEACTIVATED state. '01' Applications are installed in the ACTIVATED state. |

Table 8-2: Initial Contactless Activation State value field

During the installation process, if the Application that is to be ACTIVATED by default has conflicting Protocol Data, the installation is successful; however, the Application is set to be in the DEACTIVATED state.  A warning code (see section 11.2.4) shall be returned indicating that the Application is installed but is in the DEACTIVATED state.  Otherwise, the Application is installed in the ACTIVATED state.

## Contactless Interface Availability

The contactless interface may be switched globally on or off by the Application that is granted the Contactless Activation Privilege (see sections 3.9.1 and 3.11.4).  Switching the contactless interface on or off shall not impact the Contactless Activation State or the Communication Interface Access Configuration (see section 5) of an Application.

# Cumulative Granted Memory

This amendment introduces a new install parameter that may be used to set up Cumulative Granted Memory (CGM) for a Security Domain and its sub-hierarchy. CGM amount may be setup for Volatile and Non-Volatile memory using the Cumulative Granted Volatile Memory and Cumulative Granted Non Volatile Memory tags in the Install Parameters field of the INSTALL [for install] and INSTALL [for registry update] as defined in Table 11-2.  The usage of Cumulative Granted Memory may be limited in a specific configuration.

Note: The implementation of this feature may be limited, or otherwise impacted, by the specific runtime environment.  Specifically, implementations of CGM for Volatile memory on the Java Card™ runtime environment may not be interoperable.

Cumulative Granted Memory (CGM), when assigned to a Security Domain, specifies the exact amount of memory granted to that Security Domain, to its associated Applications and its entire sub-hierarchy.  This cumulative amount shall be interpreted both as a reserved memory amount, and as a memory quota.  The sum of memory allocated to this set of applications (including the SD itself) must not exceed that cumulative amount.  On the other hand, that cumulative amount is guaranteed to be available for these Applications.  Any system data overhead shall be included in the calculation of the memory consumed by the Applications.

Cumulative Granted Memory applies to, and may be charged for any kind of memory allocation request, including card content management operations.  In the case of extradition and deletion operations, Cumulative Granted Memory shall be charged back consistently.

A Security Domain may be assigned a CGM amount when already in the scope of a CGM amount (i.e. a CGM amount was assigned to one of its ancestor Security Domains).  In this case, the newly assigned CGM amount shall be lower than, and is charged against, the currently applicable CGM amount.

When an Application is in the scope of a CGM amount (set up for its associated Security Domain, or one of its ancestor Security Domains), then the following rules shall apply:

When assigning Reserved Memory to an Application during its installation, Reserved Memory shall be charged against Cumulative Granted Memory.

Memory allocation requests performed by the Application during its lifetime

- Shall be charged in this order

  - Against Reserved Memory, if any was assigned to the Application

  - Against Cumulative Granted Memory, if Reserved Memory is exhausted, or no Reserved Memory was assigned to the Application

- Shall fail if the total amount of memory allocated by the Application would be

  - Greater than a Memory Quota, if any was assigned to the Application

  - Greater than remaining amount of Reserved Memory (if any) and remaining amount of Cumulative Granted Memory

When deleting an Application, the greater amount between Reserved Memory (if any was assigned to the Application) or the total amount of memory allocated by the Application, is credited back to the Cumulative Granted Memory.

- When extraditing an Application to another Security Domain, the following rules shall apply (before the actual extradition):

- If the target Security Domain was assigned a CGM amount, or is in the scope of a CGM amount, then the greater amount between Reserved Memory (if any was assigned to the Application) and the total amount of memory allocated by the Application, is charged against that CGM amount.

- If the origin Security Domain (associated with the Application) was assigned a CGM amount, or is in the scope of a CGM amount, then the greater amount between Reserved Memory (if any was assigned to the Application) and the total amount of memory allocated by the Application, is charged back to that CGM amount.

- When updating the CGM amount for a Security Domain the following rules shall apply:

- When decreasing CGM, the new value shall not be lower than the total amount of memory allocated or reserved by the hierarchy.

- The policy for updating the CGM amount is configuration dependent.

# Cumulative Delete

## Definition and Scope

Cumulative Delete is an option for a Security Domain with the Global Delete privilege (e.g. an Issuer) to delete a complete Security Domain hierarchy in case of technical problems or according to changing business relationships.

According to 9.5 Content Removal, of GP22 [0], a delete process is deemed to fail in case of open references (9.5.1, 9th bullet; 9.5.2, 12th bullet; 9.5.3, 9th, 10th, 14th and 15th bullet of [0]) or remaining Security Domain associations (9.5.1, 10th bullet, of [0]).

The Cumulative Delete process provides a solution to delete the root of a Security Domain hierarchy and all associated Executable Load Files or Applications.  The GlobalPlatform and the underlying operating system dependencies are taken into account by logically deleting Executable Load Files and Applications with open references, but internally keeping the Executable Load Files and Applications until the references and relations are resolved.  This allows extradited Applications to continue using the code of its Executable Load File.

A Security Domain may conditionally retrieve a list of all logically deleted Executable Load Files and Applications.

## Security Domain with Global Delete Privilege

This privilege (see 9.1.3.4 of GP22 [0]) provides the capability to remove any Executable Load File or Application from the card even if the Executable Load File or Application does not belong to this Security Domain (see section 9.5 of GP22 [0]).

This privilege also provides the capability to delete a Security Domain hierarchy together with all associated Security Domains, Executable Load Files and Applications.

## Security Domain hierarchy Removal

A Security Domain associated with itself is the root of a hierarchy (see 7.2, Security Domain Association, of GP22 [0]).  This optional feature allows the deletion of a hierarchy with all its associated objects.

When supported by the card, the following runtime behavior requirements apply during the Security Domain hierarchy removal process.

Runtime Behavior

On receipt of an Application deletion request (DELETE command), the Security Domain performing the deletion shall:

- Apply its own secure communication policy

- Apply its own security policy, e.g. check that its Lifecycle State is PERSONALIZED (only applicable to a Security Domain other than the Issuer Security Domain)

- If the Security Domain performing the deletion has the Delegated Management privilege or the Authorized Management privilege and the off-card entity at the origin of the delete request is not authenticated as its Security Domain Provider (see section 10.4 – Entity Authentication, of [0]), check that a Delete Token is present in the DELETE command

- If a Token is present in the DELETE command, request the OPEN to obtain verification of the Delete Token

- Request the OPEN to obtain a Delete Receipt

On receipt of a request to remove a Security Domain hierarchy, the OPEN shall:

- Check that the card Lifecycle State is not CARD_LOCKED or TERMINATED

- Check that the OPEN and the requesting on-card entity have no restriction for deletion

- Check that the requesting on-card entity is a Security Domain with the Global Delete privilege

- Check that the Security Domain to be deleted is the root of a hierarchy

- If the Security Domain performing the deletion has the Delegated Management privilege or the Authorized Management privilege and the off-card entity at the origin of the delete request is not authenticated as its Security Domain Provider, request the Security Domain with Token Verification privilege to verify the Delete Token

- Determine that the Security Domain and its associated Executable Load Files and Applications being deleted have entries within the GlobalPlatform Registry

- For each Application:

  - Determine that the Application is not currently selected on another logical channel.

  - If other Applications or Executable Load Files, not part of the same hierarchy, maintain references to any data within this Application, then the OPEN shall reset these references to null.

- For each Executable Load File:

  - Determine if Applications or other Executable Load Files present in the card maintain references to this Executable Load File.

    - If other Applications or other Executable Load Files that are part of the same hierarchy make references to this Executable Load File then the OPEN shall delete the referencing Applications, the referencing Executable Load Files and the Executable Load File itself.

  - If other Applications or other Executable Load Files not part of the same hierarchy make references to this Executable Load File, then the OPEN shall mark this Executable Load File as Logically Deleted with References (see 10.4).

- Release and mark as available any Mutable Persistent Memory and when supported and apply the memory resource management rules described in section 9.7 – Memory Resource Management, of [0].

- At the request of the Security Domain performing the deletion, the Security Domain with the Receipt Generation privilege is requested to generate a series of Delete Receipts; one for each Executable Load File and one for each Application.

If the OPEN determines that any of the above verification steps have failed, the OPEN shall not initiate the delete process and shall inform the Security Domain to return the appropriate response.  Once the delete processes begin, they shall all complete in the current Card Session or, in the event of an interruption, at least the updates to the GlobalPlatform Registry shall complete in a subsequent Card Session.

Only Mutable Persistent Memory is released and marked as available.  Executable Load Files contained in Immutable Persistent Memory cannot be deleted but the entry for the Executable Load File and the entries for the Executable Modules present in the Executable Load File shall be deleted from the GlobalPlatform Registry or marked in the GlobalPlatform Registry as Logically Deleted with References.  The OPEN can only guarantee that the hierarchy is logically deleted.  A best effort shall be made to release Mutable Persistent Memory.

At the request of the OPEN, a Security Domain with the Token Verification privilege shall:

- Apply the issuer’s policy to accept or reject a deletion authorization request without the presence of a Delete Token

- Verify the Delete Token

At the request of the OPEN, a Security Domain with the Receipt Generation privilege shall:

- Apply the issuer’s policy to generate or not generate a Delete Receipt

## Logically Deleted with References

Executable Load Files which are marked as Logically Deleted with References remain physically in the Mutable Persistent Memory and in the GlobalPlatform Registry.  The following rules shall apply:

- There shall not be any Security Domain associated with Executable Load Files or Applications that are marked as "Logically Deleted with References".

- The OPEN shall not allow any Card Content Management to be performed on Executable Load Files that are marked as "Logically Deleted with References"..

- The OPEN shall not allow installation of an Executable Load File which references one or more Executable Load Files marked as "Logically Deleted with References”.

- Once all the references to an Executable Load File that is marked as "Logically Deleted with References" are resolved (e.g. referencing Application was deleted), the OPEN shall remove its entry from the GlobalPlatform Registry and release the Mutable Persistent Memory.

- The OPEN shall prevent the loading or installation of any load file or application with an AID that has been marked as “Logically Deleted with References”.

The GET STATUS command (see section 11.4 GET STATUS (Logically Deleted with References) Command) can be used to retrieve a list of all Logically Deleted with References marked Executable Load Files.  The OPEN shall return this list to any Security Domain with the Global Registry privilege.

# Security Domain APDU Commands

## Life Cycle State Coding

The following state for Load file is added to the Table 11-3 of GP22 [0] as follows:

The Executable Load File Life Cycle is coded on one byte as described in the following table:

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | logically deleted with references |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | LOADED |

Table 11-1: Executable Load File Life Cycle Coding

## INSTALL Command

The SD installation command is coded according to GP22 [0].

The following TLV-structured values are added to the TLV System Specific Parameters as follows:

| Tag | Length | Value Description |  |  | Presence |
| --- | --- | --- | --- | --- | --- |
| 'EF' | var | System Specific Parameters |  |  |  |
|  |  | Tag | Length | Value Description |  |
|  |  | … | .. | other parameters as defined in GP22 [0] |  |
|  |  | 'A0' | var | Contactless Protocol Parameters | Optional |
|  |  | 'A1' | var | User Interaction Parameters | Optional |
|  |  | 'B0' | var | (Assigned to ETSI) | Optional |
|  |  | '82' | 2 or 4 | Cumulative Granted Volatile Memory (see section 9) | Optional |
|  |  | '83' | 2 or 4 | Cumulative Granted Non Volatile Memory (see section 9) | Optional |

Table 11-2: Contactless Specific Parameters

The parameters defined above may be updated using INSTALL [for registry update] as defined in GP22 [0].  The parameters above may also be used with the INSTALL [for install] and INSTALL [for install and make selectable] commands.

### Contactless Protocol Parameters structure

The Contactless Protocol Parameters are TLV structured values.  They may be provided as part of system-specific INSTALL parameters.

The following table identifies the possible tags for managing an application on the contactless interface:

| Tag | Length | Value Description | Presence |
| --- | --- | --- | --- |
| '80' | var | Assigned Protocols for implicit selection (see section 6.6) | Optional |
| '81' | 1 | Initial Contactless Activation State (see section 8.2 for specific usage of this TLV). This TLV shall only be present when addressed to the Issuer Security Domain. | Optional |
| 'A2' | var | Contactless Protocol Parameters Profile. (See section 11.2.2) | Conditional |
| '83' | var | Recognition Algorithm (see section 6.5) | Optional |
| '84' | 1-2 | Continuous Processing (see section 6.4) | Optional |
| 'A5' | var | Communication Interface Access Parameters (see section 5.2) | Optional |
| '86' | var | Protocol Data Type A (Card Emulation Mode) (see section 4.6) | Conditional |
| '87' | var | Protocol Data Type B (Card Emulation Mode) (see section 4.7) | Conditional |
| '88' | Var | Protocol Data Type F (Card Emulation Mode) (see section 4.8) | Conditional |

Table 11-3: Contactless Protocol Parameters

When configuring an application that has its own contactless parameters the following rule shall apply:

- Tag 'A2' shall not be present.

When configuring an application that references a Contactless Protocol Parameters Profile the following rules apply:

- Tag 'A2' shall be present

- Tag '86' and Tag '87' shall not be present

When configuring a Contactless Protocol Parameters Profile the following rules apply:

- Tag 'A2' shall be present

- The TLV with tag '86' shall be present if the TLV with tag 'A2' references Protocol Type A, otherwise it shall be absent. 
The TLV with tag '87' shall be present if the TLV with tag 'A2' references Protocol Type B, otherwise it shall be absent.

Note:  The Tag '81' is used only with INSTALL [for registry update].  For all other modes of the INSTALL command, as specified in GP22 [0], an error status is returned.

### Contactless Protocol Parameters Profile structure

As described in section 4.9, Contactless Protocol Parameters may be managed by referencing Contactless Protocol Parameter Profiles.

The Profile Value (value part of TLVs 'A0' and 'A1') of the generalized system-specific INSTALL parameter structure is shown in the table below:

| Tag | Length | Value Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 'A2' | Var | Contactless Protocol Parameter Profile |  |  |  |  |  |
|  |  | Tag | Length | Value Description |  |  |  |
|  |  | 'xx' | Var | Profile Value for Protocol |  |  |  |
|  |  |  |  | Tag | Length | Name |  |
|  |  |  |  | 'yy' | 2 | Contactless Protocol Profile Identifier |  |

Table 11-4: Contactless Protocol Parameters Profile

Profile Value for Protocol Type Tag ('xx'):

- Tag 'A0' is used to identify the Profile Value for Protocol Type A

- Tag 'A1' is used to identify the Profile Value for Protocol Type B

- Tags 'A2' to 'AF' are reserved for assignment by GlobalPlatform of other Protocol Type profiles.

- Tags 'B0' to 'BE' are reserved for proprietary usage.

Protocol Profile Identifier Tag ('yy'):

- Tag '80' is reserved for use by EMV to identify EMV's profile for all Protocol Types.

- Tags '81' to '8F' are reserved for assignment by GlobalPlatform to identify other organizations.

- Tags '90' to '9E' are reserved for proprietary usage.

### User Interaction Parameters structure

The User Interaction Parameters are TLV structured values. The following table defines the TLVs for encoding the parameters. They may be provided as part of system-specific INSTALL parameters.

| Tag | Length | Name | Presence |
| --- | --- | --- | --- |
| '7F20' | var | Display Control Template (see section 3.2) | Optional |
| 'A0' | var | Head Application (see section 3.7.2) | Conditional |
| 'A1' | var | Add to the Group Authorization List (see section 3.7.5) | Conditional |
| 'A2' | var | Remove from the Group Authorization List (see section 3.7.6) | Conditional |
| 'A3' | var | Add to the CREL List (see section 3.8.3) | Optional |
| 'A4' | var | Remove from the CREL List (see section 3.8.4) | Optional |
| 'A5' | Var | Policy Restricted Applications (see section 3.3) | Optional |
| 'A6' | Var | Application discretionary data (see section 3.4) | Optional |
| '87' | 1 | Application Family (see section 3.5) | Optional |
| '88' | 1 | Display Required Indicator (see section 3.6) | Optional |

Table 11-5: User Interaction Parameters

### Processing state returned in the response message

The INSTALL command may return the following warning condition:

| SW1 | SW2 | Meaning |
| --- | --- | --- |
| '62' | '00' | Applet could not be activated on the contactless interface |

Table 11-6: INSTALL Warning Condition

## DELETE Command

The DELETE command is processed as described in GP22 [0].

The following table updates and replaces Table 11-21 DELETE Reference Control Parameter P2 of GP 22 [0]:

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | - | - | - | - | - | - | - | Delete object |
| 1 | - | - | - | - | - | - | - | Delete object and related object |
| 0 | 1 | - | - | - | - | - | - | Delete a root Security Domain and all associated Applications |
| - | - | X | X | X | X | X | X | RFU |

Table 11-7: DELETE Command Reference Control Parameter P2

Note: b7 = 1 - If the AID in the Command Data doesn’t refer to a root Security Domain, the deletion process shall fail.  The entity processing this command shall have the Global Delete Privilege.

## GET STATUS Command

The following is an extension to Table 11-32: GET STATUS Reference Control Parameter P1 of GP22 [0]:

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - | - | - | - | - | - | - | Issuer Security Domain |
| - | 1 | - | - | - | - | - | - | Applications, including Security Domains |
| - | - | 1 | - | - | - | - | - | Executable Load Files |
| - | - | - | 1 | - | - | - | - | Executable Load Files and Executable Modules |
| - | - | - | - | 1 | - | - | - | Logically Deleted with References |
| - | - | - | - | - | X | X | X | RFU |

Table 11-8: GET STATUS Reference Control Parameter P1

The following values of the reference control parameter shall be supported:

- '80' – Issuer Security Domain only. In this case the search criteria is ignored and the Issuer Security Domain information is returned

- '40' – Applications and Supplementary Security Domains only

- '20' – Executable Load Files only

- '10' – Executable Load Files and their Executable Modules only

- '08' – Executable Load Files that are marked as "Logically Deleted with References".

The ability to differentiate a Security Domain from an Application is achieved through privileges.

The Executable Load Files that are marked as "Logically Deleted with References" are provided by the OPEN and are not part of the GlobalPlatform Registry entries associated with the Security Domain receiving the GET STATUS command.

### Filter Criterion: Tag List ('5C')

The tag list (tag ‘5C’) indicates to the card how to construct the response data for each on-card entity matching the search criteria. Its value field contains a concatenation of tags (without delimitation) indicating the data objects to include in the response. The possible criteria are listed in the following table:

| Tag | Name |  |
| --- | --- | --- |
| '4F' | AID |  |
| '9F70' | Application Life Cycle State on the first byte as defined in section 11.1. The Contactless Activation State is encoded on the second byte as defined in Table 8-1. |  |
| 'C5' | Privileges (byte 1 - byte 2 – byte 3) see section 11.1.2 of GP22 [0] |  |
| 'C4' | Application's Executable Load File AID |  |
| 'CE' | Executable Load File Version Number |  |
| '84' | List of Executable Module AID |  |
| 'CC' | Associated Security Domain's AID |  |
| '7F20' |  | Display Control Template (see section 3.2) |
| '5F50' |  | Uniform Resource Locator (see section 3.2) |
| '6D' |  | Application Image Template (see section 3.2) |
| '5F45' |  | Display Message (see section 3.2) |
| 'A0' |  | Application Group Head (coded as in Table 3-3) |
| 'A1' |  | Application Group (coded as in Table 3-4) |
| 'A2' |  | CREL Application AID List (coded as in Table 3-6) |
| 'A3' |  | Policy Restricted Applications (see section 3.3) |
| ‘A5’ |  | Application discretionary data (see section 3.4) |
| '86' |  | Application Family (see section 3.5) |
| '87' | Assigned Protocols for implicit selection (see section 6.6) |  |
| '88' | Initial Contactless Activation State (see section 8.2) |  |
| 'A9' | Contactless Protocol Parameters Profile. (See section 11.2.2) |  |
| '8A' | Recognition Algorithm (see section 6.5) |  |
| '8B' | Continuous Processing (see section 6.4) |  |
| 'AC' | Communication Interface Access Parameters (see section 5.2) |  |
| '8D' | Protocol Data Type A (Card Emulation Mode) (see section 4.6) |  |
| '8E' | Protocol Data Type B (Card Emulation Mode) (see section 4.7) |  |
| '8F' | Cumulative Granted Non-Volatile Memory (see section 9) |  |
| '90' | Cumulative Granted Volatile Memory (see section 9) |  |
| '91' | Cumulative Remaining Non-Volatile Memory (see section 9) |  |
| '92' | Cumulative Remaining Volatile Memory (see section 9) |  |

Table 11-9: Filter Criteria (Tag '5C')

These tags shall be used as filter criteria in a simple, non-constructed form.  E.g. '5F50' is a sub-tag of '7F20', however, in the filter criteria data field only '5F50' shall appear (see example related to Table 3-14).

### Response Message

The following GlobalPlatform Registry Data replaces the Table 11-36 of GP22 [0]: GlobalPlatform Registry Data (TLV) for cards compliant with this amendment.

| Tag | Length | Value Description | Presence |
| --- | --- | --- | --- |
| 'E3' | Variable | GlobalPlatform Registry related data | Conditional |
| '4F' | 5-16 | AID | Conditional |
| '9F70' | 2 | Application Life Cycle State on the first byte as defined in section 11.1. The Contactless Activation State is encoded on the second byte as defined in Table 8-1. | Conditional |
| 'C5' | 0,1,3 | Privileges (byte 1 - byte 2 – byte 3) see section 11.1.2 of GP22 [0] | Conditional |
| 'C4' | 1-n | Application's Executable Load File AID | Conditional |
| 'CE' | 1-n | Executable Load File Version Number | Conditional |
| '84' | 1-n | First or only Executable Module AID | Conditional |
| … | ... | … | … |
| '84' | 1-n | Last Executable Module AID | Conditional |
| 'CC’ | 0, 5-16 | Associated Security Domain's AID If the Registry is marked as logically deleted with references the length is 0 | Conditional |

Table 11-10: GlobalPlatform Registry Data (TLV)

# Token Identifier Blacklist for Delegated Management

## Definition and Scope

GP v2.2 specifications (GP22 [0]) define token formats for card content management, however a means to prevent re-use of delegated management tokens is not provided.  This section defines a mechanism to blacklist token use based upon the Token Identifier.  Token Identifiers present in the blacklist cannot be used for delegated card content management.  Each Security Domain with the Token Verification Privilege may own a blacklist.

## Blacklist / Rehabilitate using STORE DATA Command

The STORE DATA command may be used to add or remove a Token Identifier in the blacklist.  Each operation is represented by a tag.  The Token Identifier on which the operation applies is in the value body.  The encoding format of data shall be as described in §11.11.2.3 of GP22 [0].

Every SD with the Token Verification Privilege may support the following TLV coded data objects:

- Add Token Identifiers into the blacklist

- Remove Token Identifiers from the blacklist

Upon receipt of these TLV, the Blacklist shall be updated and the Token Identifier(s) present in the command shall be Added / Removed from the list.

If a Token Identifier to be removed is not blacklisted, an error condition shall be returned according to GP22 [0], and the entire operation shall fail.

If a Token Identifier to add to the blacklist is already present, no error is returned.

Any attempt to store, update or retrieve the blacklist on a SD that does not have the “Token Verification” privilege shall fail. An error condition code shall be returned according to GP22 [0].

## Add to blacklist

The content of the TLV is coded as follows:

| Tag | Length | Value Description |  |  |
| --- | --- | --- | --- | --- |
| 'A0' | Var | Add Token Identifier(s) to the blacklist |  |  |
|  |  | Tag | Length | Value Description |
|  |  | '93' | Var | Token Identifier |
|  |  | … |  |  |
|  |  | '93' | Var | Token Identifier |

Table 12-1: Add Token Identifiers to the blacklist TLV

## Remove from blacklist

The content of the TLV is coded as follows:

| Tag | Length | Value Description |  |  |
| --- | --- | --- | --- | --- |
| 'A1' | Var | Remove Token Identifier(s) from the blacklist |  |  |
|  |  | Tag | Length | Value Description |
|  |  | '93' | Var | Token Identifier |
|  |  | … |  |  |
|  |  | '93' | var | Token Identifier |

Table 12-2: Remove Token Identifier(s) from the blacklist TLV

## Read blacklist

The list of the blacklisted Token Identifiers can be retrieved from an SD with the Token Verification privilege using the GET DATA command with Tag 'A0'.  The SD shall return the list of all blacklisted Token Identifiers it has.

The content of the TLV is coded as follows:

| Tag | Length | Value Description |  |  |
| --- | --- | --- | --- | --- |
| 'A0' | Var | Token Identifier blacklist |  |  |
|  |  | Tag | Length | Value Description |
|  |  | '93' | Var | Token Identifier |
|  |  | … |  |  |
|  |  | '93' | Var | Token Identifier |

Table 12-3: Token Identifier blacklist TLV

## Processing State returned in the response message

A successful execution of the command shall be indicated by status bytes '90' '00'.

The following error conditions may occur and are indicated by the status bytes given in the table below:

| SW1 | SW2 | Meaning |
| --- | --- | --- |
| '6A' | '80' | Incorrect values in command data |
| '6A' | '88' | Referenced data not found |

Table 12-4: Error Conditions

- CRS Java Card API

- Executable Load File AID of org.globalplatform.contactless is 'A00000015102' with version 1.0

- The version of org.globalplatform shall be 1.4 or above.

  - This ensures that the secureChannelx2 interface is defined.

  - SecureChannel services may be invoked using either a global array or an array that belongs to the calling Application.

The figures in this appendix describe the expected logic flow for certain activities defined elsewhere in this specification, as viewed at the API level.

Figure A-1: APDU: Contactless Self-Activation, Conflict Detection and Conflict Resolution

Figure A-2: Contactless Self-Activation; No Conflict

Figure A-3: No Contactless Self-Activation; Conflict Detection and Conflict Resolution using CAT framework (see [9])

- Contactless Protocol Management: Example

## Current Protocol Parameters for Type A computation

The computation of the Current Protocol Parameters is detailed in section 4.4.2.

## Value definition

The current Protocol Parameters for Type A is:

| Current Protocol Parameters for Type A |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Protocol Data |  |  |  |  |  |  |
| LV UID | SAK | ATQA | LV ATS Historical bytes | FWI,SFGI | CID Support | Max data rate |
| '00' | '00' | '0000' | '03010203' | 'EE' | '00' | '000303' |
| Mandatory Mask |  |  |  |  |  |  |
| 'FF' | '81' | 'FF00' | 'FF00FF00' | 'FF' | 'FF' | 'FFFFFF' |

The Protocol Parameter for Type A of the Activated Application is:

| Activated Application: Protocol Parameters for Type A |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Protocol Data |  |  |  |  |  |  |
| LV UID | SAK | ATQA | LV ATS Historical bytes | FWI,SFGI | CID Support | Max data rate |
| '00' | '21' | '8200' | '020102' | 'EE' | '01' | '000303' |
| Mandatory Mask |  |  |  |  |  |  |
| 'FF' | '24' | 'FF00' | 'FFFF00' | 'FF' | 'FF' | 'FFFFFF' |

## Protocol Data Computation: Intermediate result [A].

The length of the ATS Historical bytes of the current protocol data is reduced from 3 to 2 since the Application is requesting that the length of the ATS historical bytes shall not exceed 2.

| Intermediate Result: Mandatory '1' bit mask [A] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Protocol Data |  |  |  |  |  |  |
| LV UID | SAK | ATQA | LV ATS Historical bytes | FWI,SFGI | CID Support | Max data rate |
| '00' | '20' | '8200' | '020002' | 'EE' | '01' | '000303' |

## Protocol Data Computation: Intermediate result [B].

| Intermediate Result: Mandatory '0' bit mask [B] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Protocol Data |  |  |  |  |  |  |
| LV UID | SAK | ATQA | LV ATS Historical bytes | FWI,SFGI | CID Support | Max data rate |
| '00' | 'FB' | '82FF' | '0200FD' | 'EE' | '01' | '000303' |

## Protocol Data Computation: Intermediate result [C].

| Intermediate Result[C] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Protocol Data |  |  |  |  |  |  |
| LV UID | SAK | ATQA | LV ATS Historical bytes | FWI,SFGI | CID Support | Max data rate |
| '00' | '20' | '8200' | '02FE00' | 'EE' | '01' | '000303' |

## Current Protocol Parameter of Type A: Result

| New Current Protocol Parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Protocol Data |  |  |  |  |  |  |
| LV UID | SAK | ATQA | LV ATS Historical bytes | FWI,SFGI | CID Support | Max data rate |
| '00' | '20' | '8200' | '020102' | 'EE' | '01' | '000303' |
| Mandatory Mask |  |  |  |  |  |  |
| 'FF' | 'A5' | 'FF00' | 'FFFFFF' | 'FF' | 'FF' | 'FFFFFF' |

## Protocol Parameter for Type A: Conflict Detection

The conflict detection algorithm is specified in section 4.5.

## Value definition

The current Protocol Parameter for Type A is:

| Current Protocol Parameter for Type A |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Protocol Data |  |  |  |  |  |  |
| LV UID | SAK | ATQA | LV ATS Historical bytes | FWI,SFGI | CID Support | Max data rate |
| '00' | '20' | '8200' | '020102' | 'EE' | '01' | '000303' |
| Mandatory Mask |  |  |  |  |  |  |
| 'FF' | 'A5' | 'FF00' | 'FF00FF' | 'FF' | 'FF' | 'FFFFFF' |

The Protocol Parameter for Type A of the Application being activated is:

| Application: Protocol Parameter for Type A |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Protocol Data |  |  |  |  |  |  |
| LV UID | SAK | ATQA | LV ATS Historical bytes | FWI,SFGI | CID Support | Max data rate |
| '00' | '00' | '8200' | '03010203' | 'EE' | '03' | '000303' |
| Mandatory Mask |  |  |  |  |  |  |
| 'FF' | '24' | '00FF' | 'FFFF00FF' | 'FF' | 'FF' | 'FFFFFF' |

In this example, the computation stops at this point.  The conflicts are highlighted in the table directly above (bold text) and described below:

- Two different SAK values are requested.

- The LV ATS Historical bytes conflict due to a requested third value in the Historical bytes, even though the requested MAX length does not conflict.

- Different CID Support values are requested

Table of Figures and Tables

Figure 2-1: High Level Architecture Overview	9

Figure 2-2: Flow of populating an Application’s Contactless Registry Parameters during Installation	12

Figure 2-3: Flow of populating an Application’s Contactless Registry Parameters through itself	13

Figure 2-4: Flow of removing an Application’s Contactless Registry Parameters during Deletion	14

Figure 2-5: Flow of User Interaction	15

Figure 3-1: Application Group	18

Figure 4-1: Current Protocol Parameters: Protocol Data computation	38

Figure 4-2: Current Mandatory Mask Computation	39

Figure 4-3: Protocol Parameter Conflict Detection	40

Figure 8-1: Contactless Activation States	55

Figure A-1: APDU: Contactless Self-Activation, Conflict Detection and Conflict Resolution	70

Figure A-2: Contactless Self-Activation; No Conflict	71

Figure A-3: No Contactless Self-Activation; Conflict Detection and Conflict Resolution using CAT framework (see [9])	72

Table 1-1: Normative References	7

Table 1-2: Terminology and Definitions	7

Table 1-3: Abbreviations and Notations	8

Table 3-1: Policy Restricted Applications TLV Value Description	17

Table 3-2: Display Required Indicator TLV Value Description	18

Table 3-3: Head Application TLV Description	19

Table 3-4: Add to the Group Authorization List Value Description	20

Table 3-5: Remove from Group Authorization List Value Description	21

Table 3-6: Add to the CREL List Value Description	22

Table 3-7: Remove from the CREL List Value Description	22

Table 3-8: User Interaction parameters Tag Usage	25

Table 3-9: GET STATUS Command Message for CRS	27

Table 3-10: GET STATUS Reference Control Parameter P1	28

Table 3-11: GET STATUS Reference Control Parameter P2	28

Table 3-12: GET STATUS Command for CRS Data Field	28

Table 3-13: On-Card Registered Contactless Applications Data (TLV)	29

Table 3-14: Example of response data returned when filter criteria are provided	30

Table 3-15: GET STATUS Warning Condition	30

Table 3-16: GET STATUS Error Conditions	30

Table 3-17: SET STATUS Command Message for CRS	31

Table 3-18: SET STATUS – Status Type	31

Table 3-19: SET STATUS – P2 values for Priority Order for Application Selection	32

Table 3-20: SET STATUS – P2 values for Communication Interface Access Configuration	32

Table 3-21: SET STATUS Command Data Field for Status Types P1='01' or P1='02'	32

Table 3-22: SET STATUS Command Data Field for Status Type P1='04'	33

Table 3-23: SET STATUS Response Data Field	33

Table 3-24: SET STATUS Warning Conditions	34

Table 3-25: SET STATUS Error Conditions	34

Table 3-26: GlobalPlatform CRS Application SELECT Response	35

Table 3-27: GET DATA Command Message for CRS	35

Table 3-28: GlobalPlatform CRS Application GET DATA Response	35

Table 4-1: OPEN-Protocol Data for Type A value field	42

Table 4-2: Application-Protocol Data for Type A value field	43

Table 4-3: OPEN-Protocol Data for Type B value field	44

Table 4-4: Application-Protocol Data for Type B value field	45

Table 5-1: Communication Interface Access Configuration Parameters	47

Table 5-2: Communication Interface Access Configuration Parameter Value	48

Table 6-1: OPEN - Continuous processing value description	51

Table 6-2: Application - Continuous processing value description	51

Table 6-3: Recognition Algorithm	51

Table 6-4: Assigned Protocol for Implicit Selection value field	52

Table 7-1: Privileges	53

Table 8-1: Contactless Activation State byte coding	54

Table 8-2: Initial Contactless Activation State value field	56

Table 11-1: Executable Load File Life Cycle Coding	62

Table 11-2: Contactless Specific Parameters	62

Table 11-3: Contactless Protocol Parameters	63

Table 11-4: Contactless Protocol Parameters Profile	63

Table 11-5: User Interaction Parameters	64

Table 11-6: INSTALL Warning Condition	64

Table 11-7: DELETE Command Reference Control Parameter P2	64

Table 11-8: GET STATUS Reference Control Parameter P1	65

Table 11-9: Filter Criteria (Tag '5C')	66

Table 11-10: GlobalPlatform Registry Data (TLV)	66

Table 12-1: Add Token Identifiers to the blacklist TLV	67

Table 12-2: Remove Token Identifier(s) from the blacklist TLV	68

Table 12-3: Token Identifier blacklist TLV	68

Table 12-4: Error Conditions	68

END OF DOCUMENT