| ETSI TC SCP Meeting #47 Sophia Antipolis, France, 12 - 14 January 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(11)0053 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #36 Sophia Antipolis, France, 13-16 Dec 2010 |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(10)0491r1 |  |  |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 009 |  |  |  rev | - |  |  |  |  Current version: |  |  |  |  | 9.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Definition of the behaviour of HCIDevice.getHCIService() method when Applet.register() method has not been invoked. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | HCI API |  |  |  |  |  |  |  |  |  |  | Date:  |  |  |  | 16/12/2010 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  | Release:  |  |  |  | Rel-9 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | Currently the behaviour of the HCIDevice.getHCIService() method is not specified when the Applet.register() method has not been invoked. As the method accesses the Java Card™ applet registry, its invocation should return a null object before the applet registration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Specify that the invocation of the method shall return a null object if invoked before Applet.register(). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Interoperability issues when applications invoke HCIDevice.getHCIService() method when Applet.register() method has not been invoked. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Annex A. (HCIDevice.java) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Annex A (normative):
Java Card™ Platform HCI API for the UICC

### HCIDevice.java

### [...]

### /**

### * This method is used to retrieve an instances that implement one of <code>HCIService</code>

### * subinterfaces defined in this specification.

### * <P>

* This method returns <code>null</code> if the <code>Applet.register()</code> has not yet

* been invoked.

*

### * @param serviceID shall be one of the constants (*)_SERVICE_ID defined

### * in this class.

### * @return reference to the permanent JCRE entry point object of the <code>HCIService</code> or <code>null</code> if not yet registered.

### * @throws HCIException with reason:

### * <ul>

### * <li>HCI_ACCESS_NOT_GRANTED if access to the indicated service is

### * not permitted for the Applet

### * <li>HCI_SERVICE_NOT_AVAILABLE if the requested service type is not available

### * </ul>

### * @exception SystemException with the following reason code: <ul>

### *      <li><code>ILLEGAL_VALUE</code> <ul><li>if the serviceID does not match with the predefined values</li></ul>

### *

### */

### public static HCIService getHCIService(short serviceID) throws HCIException, SystemException

### {

### [...]