| ETSI TC SCP Meeting #47 Sophia Antipolis, France, 12 - 14 January 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(11)0058 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #36 Sophia Antipolis, France, 13-16 Dec 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(10)0504r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 013 |  |  |  rev | - |  |  |  Current version: |  |  |  |  | 9.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | WRITE_EXCHANGE event for HCIService activation, deactivation and get status methods |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | HCI API |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 14/12/2010 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-9 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | The HCIService interface misses the ReaderListener.EVENT_WRITE_EXCHANGE_DATA_RESPONSE in activateEvent(), deactivateEvent() and getEventNotificationStatus() methods. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Add ReaderListener.EVENT_WRITE_EXCHANGE_DATA_RESPONSE as value for parameter in HCIService activateEvent(), deactivateEvent() and getEventNotificationStatus() methods. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | It is not possible to invoke an applet on the response to the Write Exchange Data HCI command. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Annex A. (HCIService.java) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Annex A (normative):
Java Card™ Platform HCI API for the UICC

### HCIService.java

### [...]

### /**

### * This method is used to activate an event.

### *

### *

### * <P>Possible values for the parameter event for the different Service objects shall be:

### * <ul>

### * <li>for CardEmulationService:

### * <ul>

### *   <li><code>CardEmulationListener.EVENT_GET_PARAMETER_RESPONSE</code>

### *   <li><code>CardEmulationListener.EVENT_SEND_DATA</code>

### *   <li><code>CardEmulationListener.EVENT_FIELD_OFF</code>

### * </ul>

### * <li>for ReaderService:

### * <ul>

### *   <li><code>ReaderListener.EVENT_GET_PARAMETER_RESPONSE</code>

### *   <li><code>ReaderListener.EVENT_WRITE_EXCHANGE_DATA_RESPONSE</code>

### *   <li><code>ReaderListener.EVENT_TARGET_DISCOVERED</code>

### * </ul>

### * <li>for ConnectivityService:

### * <ul>

### *   <li><code>ConnectivityListener.EVENT_STANDBY</code>

### * </ul>

### * </ul>

### * @param event event to be activated

### * @throws HCIException with reason code

### *     <ul>

### *      <li><code>HCI_WRONG_EVENT_TYPE</code> in case a wrong event was activated for this Service instance

### *      <li><code>HCI_CONDITIONS_NOT_SATISFIED</code> if one or more conditions to activate the event are not satisfied

### *     </ul>

### */

### public void activateEvent(byte event) throws HCIException;

### /**

### * This method is used to deactivate an event from the list of activated events.

### *

### * <P>Possible values for the parameter event for the different Service objects shall be:

### * <ul>

### * <li>for CardEmulationService:

### * <ul>

### *   <li><code>CardEmulationListener.EVENT_GET_PARAMETER_RESPONSE</code>

### *   <li><code>CardEmulationListener.EVENT_SEND_DATA</code>

### *   <li><code>CardEmulationListener.EVENT_FIELD_OFF</code>

### * </ul>

### * <li>for ReaderService:

### * <ul>

### *   <li><code>ReaderListener.EVENT_GET_PARAMETER_RESPONSE</code>

### *   <li><code>ReaderListener.EVENT_WRITE_EXCHANGE_DATA_RESPONSE</code>

### *   <li><code>ReaderListener.EVENT_TARGET_DISCOVERED</code>

### * </ul>

### * <li>for ConnectivityService:

### * <ul>

### *   <li><code>ConnectivityListener.EVENT_STANDBY</code>

### * </ul>

### * </ul>

### * @param event event to be activated

### * @throws HCIException with reason code <code>HCIException.HCI_WRONG_EVENT_TYPE</code> in case a wrong event was deactivated for this Service instance	 or in case the event wasn't activated before.

### */

### public void deactivateEvent(byte event) throws HCIException;

### /**

### * return the activation state of an event.

### *

### *

### * <P>Possible values for the parameter event for the different Service objects shall be:

### * <ul>

### * <li>for CardEmulationService:

### * <ul>

### *   <li><code>CardEmulationListener.EVENT_GET_PARAMETER_RESPONSE</code>

### *   <li><code>CardEmulationListener.EVENT_SEND_DATA</code>

### *   <li><code>CardEmulationListener.EVENT_FIELD_OFF</code>

### * </ul>

### * <li>for ReaderService:

### * <ul>

### *   <li><code>ReaderListener.EVENT_GET_PARAMETER_RESPONSE</code>

### *   <li><code>ReaderListener.EVENT_WRITE_EXCHANGE_DATA_RESPONSE</code>

### *   <li><code>ReaderListener.EVENT_TARGET_DISCOVERED</code>

### * </ul>

### * <li>for ConnectivityService:

### * <ul>

### *   <li><code>ConnectivityListener.EVENT_STANDBY</code>

### * </ul>

### * </ul>

### *

### * @param event event to request the activation state

### * @return true if the event is activated for this listener, else false

### * @throws HCIException with reason code <code>HCIException.HCI_WRONG_EVENT_TYPE</code>  in case the event was not one of the possible values.

### */

### public boolean getEventNotificationStatus(byte event) throws HCIException;

### }