3GPP TSG-CT6#54 	C6-100108

- San Francisco, USA , 23-26 February 2010

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.130 | CR | 0049 |  rev | 2 |  Current version: | 9.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps | X | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Support of CSG cell discovery and CSG selection event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Sun Microsystems Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | C6 |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI 9 |  |  |  |  |  |  | Date:  |  | 25/02/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change:  |  | Support CSG cell discovery and CSG selection event |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Introduce support for the new event and new constant values in the API |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | API specification not inline with the USAT specification |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.2 , Annex A : uicc.usim.toolkit.ToolkitConstants, uicc.usim.toolkit.USATTerminalProfile |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

## 6.2	Definition of Events

The following events can trigger a Toolkit Applet in addition to the events defined in ETSI TS 102 241 [2], all short values are reserved in ETSI TS 102 241 [2]:

Table 1: (U)SAT event list

| Event Name | Reserved short value |
| --- | --- |
| EVENT_FORMATTED_SMS_PP_ENV | 2 |
| EVENT_FORMATTED_SMS_PP_UPD | 3 |
| EVENT_UNFORMATTED_SMS_PP_ENV | 4 |
| EVENT_UNFORMATTED_SMS_PP_UPD | 5 |
| EVENT_UNFORMATTED_SMS_CB | 6 |
| EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA | 10 |
| EVENT_FORMATTED_SMS_CB | 24 |
| EVENT_EVENT_DOWNLOAD_IWLAN_ACCESS_STATUS | 30 |
| EVENT_EVENT_DOWNLOAD_NETWORK_REJECTION | 31 |
| EVENT_EVENT_DOWNLOAD_CSG_CELL_SELECTION | XX |
| EVENT_FORMATTED_USSD | 121 |
| EVENT_UNFORMATTED_USSD | 122 |

EVENT_FORMATTED_SMS_PP_ENV, EVENT_UNFORMATTED_SMS_PP_ENV,
EVENT_FORMATTED_SMS_PP_UPD, EVENT_UNFORMATTED_SMS_PP_UPD

There are two ways for a card to receive a Short Message Point to Point: via an ENVELOPE(SMS-PP Download) APDU as defined in TS 31.111 [7] and TS 51.014 [8] or an UPDATE RECORD EFSMS APDU as defined in TS 31.102 [3] and TS 51.011 [4]. The EFSMS can be either located under the DFTelecom or under any ADF as defined in TS 31.102 [3] and TS 51.011 [4].

The received Short Message may be:

-	formatted according to TS 31.115 [9] or an other protocol to identify explicitly the toolkit applet for which the message is sent;

-	unformatted (e.g. a toolkit applet specific protocol ) then the (U)SAT Framework will pass this data to all registered toolkit applets.

When the Short Message is received as Concatenated Short Messages as defined in TS 23.040 [10], it is the responsibility of the (U)SAT Framework to link single Short Messages together to re – assemble the original message before any further processing. The original Short Message shall be placed in one SMS TPDU TLV (with TP-UDL field coded on one octet) included in the USATEnvelopeHandler. The concatenation control headers used to re-assemble the short messages in the correct order shall not be present in the SMS TPDU. The TP-elements of the SMS TPDU and the Address (TS – Service-Centre-Address) shall correspond to the ones in the last received Short Message (independently of the Sequence number of Information-Element-Data).

The minimum requirement for the (U)SAT Framework is to process a concatenated short message with the following properties:

-	the Information Element Identifier is equal to the 8-bit reference number.

-	it contains uncompressed 8 bit data or uncompressed UCS2 data.

EVENT_FORMATTED_SMS_PP_ENV

Upon reception of a TS 31.115 [9] formatted Short Message Point to Point (Single or Concatenated) via  an ENVELOPE, the (U)SAT Framework shall:

-	verify the security of the Short Message as per TS 31.115 [9];

-	trigger the toolkit applet registered with the corresponding TAR;

-	take the optional Application Data posted by the triggered toolkit applet if present;

-	secure and send the response packet using SMS-DELIVER-REPORT or  SMS-SUBMIT.

- When the toolkit applet is triggered, data shall be provided deciphered.

- EVENT_UNFORMATTED_SMS_PP_ENV

- Upon reception of an unformatted Short Message Point to Point (Single or Concatenated) via an ENVELOPE, the (U)SAT Framework shall trigger all the Toolkit Applets registered to this event.

NOTE:	As a consequence of the EnvelopeResponseHandler availability rules specified in clause 6.6, only the first triggered toolkit applet is guaranteed to be able to send back a response.

- EVENT_FORMATTED_SMS_PP_UPD

Upon reception of a TS 31.115 [9] formatted Short Message Point to Point (Single or Concatenated) via an UPDATE RECORD EFSMS, the (U)SAT Framework shall:

-	update the EFSMS file with the data received, it is then up to the receiving toolkit applet to change the SMS stored in the file (i.e. the toolkit applet need to have access to the EFSMS file)

-	verify the security of the Short Message as per TS 31.115 [9];

-	convert the UPDATE RECORD EFSMS APDU into a COMPREHENSION TLV List;

-	trigger the toolkit applet registered with the corresponding TAR;

- When the toolkit applet is triggered, data shall be provided deciphered.

- The USATEnvelopeHandler provided to the applet shall:

-	return BTAG_SMS_PP_DOWNLOAD to the getTag() method call;

-	return the Comprehension TLV list length to the getLength() method call;

- The USATEnvelopeHandler provided to the applet shall contain the following COMPREHENSION TLVs:

-	Device Identities TLV

The Device Identities Comprehension TLV is used to store the information about the absolute record number in the EFSMS file and the value of the EFSMS record status byte, and is formatted as defined below:

Device identities Comprehension TLV

Device Identities tag

length = 02

Absolute Record Number

Record Status

With the absolute record number the toolkit applet can update EFSMS in absolute mode to change the received SMS (e.g. in a readable text).

For Concatenated Short Message the Absolute Record Number and the Record Status will correspond to the last UPDATE RECORD EFSMS APDU received.

-	Address TLV

The value is the TS-Service-Centre-Address (RP-OA) of the last UPDATE RECORD EFSMS APDU.

-	SMS TPDU TLV

The value is the SMS TPDU provided deciphered and reassembled, if needed

-	AID TLV

The AID comprehension TLV is present only if the EFSMS file updated is under an ADF. The value is the AID of the ADF as defined TS 31.111 [7].

The order of the TLVs given in the USATEnvelopeHandler is not specified,

NOTE:	To get each COMPREHENSION TLV, it is recommended that the applet uses the ViewHandler.findTLV() methods

EVENT_UNFORMATTED_SMS_PP_UPD

Upon reception of an unformatted Short Message Point to Point (Single or Concatenated) via UPDATE RECORD EFSMS APDU, the (U)SAT Framework shall :

-	update the EFSMS file with the data received;

-	convert the UPDATE RECORD EFSMS APDU data into a COMPREHENSION TLV List (as described for EVENT_FORMATTED_SMS_PP_UPD);

-	trigger all the Toolkit Applets registered to this event.

The content of EFSMS may have been modified by a previously triggered Toolkit Applet..

EVENT_FORMATTED_SMS_CB, EVENT_UNFORMATTED_SMS_CB

The received Cell Broadcast Message, via an ENVELOPE (CELL BROADCAST DOWNLOAD) APDU as defined in TS 31.111 [7] and TS 51.014 [8] and, can be either:

-	formatted according to TS 31.115 [9] or an other protocol to identify explicitly the toolkit applet for which the message is sent;

-	unformatted ( e.g. using a toolkit applet specific protocol ), then the (U)SAT Framework will pass this data to all registered toolkit applets.

When the Cell Broadcast Message is received as multiple pages as defined in TS 23.041 [5], it is the responsibility of the (U)SAT Framework to link single pages together to re-assemble the original message before any further processing. The original Cell Broadcast message shall be placed in one Cell Broadcast page TLV included in the USATEnvelopeHandler. The message parameters shall correspond to the ones in the last received Cell Broadcast page (independently of the Page Parameter).

EVENT_FORMATTED_SMS_CB

Upon reception of a TS 31.115 [9] formatted Cell Broadcast message, the (U)SAT Framework shall:

-	verify the security of the Cell Broadcast message as per TS 31.115 [9];

-	trigger the toolkit applet registered with the corresponding TAR;

- When the toolkit applet is triggered, data shall be provided deciphered.

EVENT_UNFORMATTED_SMS_CB

- Upon reception of an unformatted Cell Broadcast message, the (U)SAT Framework shall trigger all the Toolkit Applets registered to this event.

EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA

Upon reception of an ENVELOPE (MO SHORT MESSAGE CONTROL defined in TS 51.014 [8] and TS 31.111 [7]) APDU as defined in TS 31.101 [6] and TS 51.011 [4] the (U)SAT Framework shall trigger the Toolkit Applet registered to this event. The (U)SAT Framework shall not allow more than one Toolkit Applet to be registered to this event at a time(e.g. if a Toolkit Applet is registered to this event but not in selectable state the (U)SAT Framework shall not allow another Toolkit Applet to register to this event).

EVENT_FORMATTED_USSD, EVENT_UNFORMATTED_USSD

The received USSD String, via an ENVELOPE (USSD Data Download) APDU as defined in TS 31.111 [7], may be:

  - formatted according to TS 31.115 [9] or an other protocol to identify explicitly the toolkit applet for which the message is sent;

  - unformatted (e.g. a toolkit applet specific protocol) then the (U)SAT Framework will pass this data to all registered toolkit applets.

When the USSD Message is received as concatenated as defined in TS 31.115 [9], it is the responsibility of the (U)SAT Framework to link single USSD Messages together to re-assemble the original message before any further processing. The original USSD message shall be placed in one USSD String TLV included in the USATEnvelopeHandler. The USSD String parameters (DCS, PFI, CCF) shall correspond to the ones in the last received USSD String (independently of the CCF Sequence number).

EVENT_FORMATTED_USSD

Upon reception of a TS 31.115 [9] formatted USSD Message via an ENVELOPE, the (U)SAT Framework shall:

- verify the security of the USSD Message as per TS 31.115 [9];

- trigger the toolkit applet registered with the corresponding TAR;

- take the optional Application Data posted by the triggered toolkit applet if present;

- secure and send the response packet.

- When the toolkit applet is triggered, data shall be provided deciphered.

- EVENT_UNFORMATTED_USSD

Upon reception of an unformatted USSD String via an ENVELOPE, the (U)SAT Framework shall trigger all the Toolkit Applets registered to this event.

Note:	As a consequence of the EnvelopeResponseHandler availability rules specified in clause 6.6, only the first triggered toolkit applet is guaranteed to be able to send back a response.

EVENT_EVENT_DOWNLOAD_IWLAN_ACCESS_STATUS

EVENT_EVENT_DOWNLOAD_NETWORK_REJECTION

EVENT_EVENT_DOWNLOAD_CSG_CELL_SELECTION

Upon reception of an ENVELOPE (Event Download) APDU command as defined in TS 31.111 [7] the (U)SAT Framework shall trigger all the Toolkit applets registered to the corresponding event.

The following events defined in TS 31.111 [7] shall be raised upon reception of the corresponding APDU defined in either TS 51.011 [4] or TS 31.101 [6].

EVENT_PROFILE_DOWNLOAD

EVENT_MENU_SELECTION, EVENT_MENU_SELECTION_HELP_REQUEST

EVENT_CALL_CONTROL_BY_NAA

EVENT_TIMER_EXPIRATION

EVENT_EVENT_DOWNLOAD_MT_CALL

EVENT_EVENT_DOWNLOAD_CALL_CONNECTED

EVENT_EVENT_DOWNLOAD_CALL_DISCONNECTED

EVENT_EVENT_DOWNLOAD_LOCATION_STATUS

EVENT_EVENT_DOWNLOAD_USER_ACTIVITY

EVENT_EVENT_DOWNLOAD_IDLE_SCREEN_AVAILABLE

EVENT_EVENT_DOWNLOAD_CARD_READER_STATUS

EVENT_STATUS_COMMAND

EVENT_EVENT_DOWNLOAD_LANGUAGE_SELECTION

EVENT_EVENT_DOWNLOAD_BROWSER_TERMINATION

EVENT_EVENT_DOWNLOAD_DATA_AVAILABLE

EVENT_EVENT_DOWNLOAD_CHANNEL_STATUS

EVENT_EVENT_DOWNLOAD_ACCESS_TECHNOLOGY_CHANGE

EVENT_EVENT_DOWNLOAD_DISPLAY_PARAMETER_CHANGED

EVENT_EVENT_DOWNLOAD_LOCAL_CONNECTION

EVENT_EVENT_DOWNLOAD_NETWORK_SEARCH_MODE_CHANGE

EVENT_EVENT_DOWNLOAD_BROWSING_STATUS

EVENT_PROACTIVE_HANDLER_AVAILABLE

EVENT_EXTERNAL_FILE_UPDATE

EVENT_FIRST_COMMAND_AFTER_ATR

EVENT_UNRECOGNIZED_ENVELOPE

……..

public interface ToolkitConstants extends uicc.toolkit.ToolkitConstants {

/** Event : Envelope SMS-PP Data Download (31.115 formatted)  = 2        */

public static final short EVENT_FORMATTED_SMS_PP_ENV                 = (short)2;

/** Event : Update Record EF sms APDU     (31.115 formatted)  = 3        */

public static final short EVENT_FORMATTED_SMS_PP_UPD                 = (short)3;

/** Event : Envelope SMS-PP Data Download unformatted sms  = 4          */

public static final short EVENT_UNFORMATTED_SMS_PP_ENV               = (short)4;

/** Event : Update Record EFsms APDU      unformatted sms   = 5         */

public static final short EVENT_UNFORMATTED_SMS_PP_UPD               = (short)5;

/** Event : Cell Broadcast Data Download  = 6                           */

public static final short EVENT_UNFORMATTED_SMS_CB                   = (short)6;

/** Event : MO Short Message Control by NAA = 10.

*   Note: The constant EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM was

*   erroneously used instead of the constant

*   Event EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA

*   in earlier releases of this specification. Use constant

*   EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM in applications

*  which need to be compatible with earlier releases of this specification.

*/

public static final short EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA      = (short)10;

/** Note: The constant EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM was

erroneously used for the Event EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA

in earlier releases of this specification. The constant

EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM is maintained for backwards

compatiblity with applications which were developed for earlier

releases of this specification.

@deprecated Use EVENT_MO_SHORT_MESSAGE_CONTROL_BY_NAA instead.*/

public static final short EVENT_MO_SHORT_MESSAGE_CONTROL_BY_SIM      = (short)10;

/** Event : Cell Broadcast Data Download Formatted = 24                 */

public static final short EVENT_FORMATTED_SMS_CB                     = (short)24;

/** Event : USSD Data Download Formatted = 121                          */

public static final byte EVENT_FORMATTED_USSD                        = (short)121;

/** Event : USSD Data Download Unformatted = 122                        */

public static final byte EVENT_UNFORMATTED_USSD                      = (short)122;

/** Event : Event Download Network Rejection = 31                 */

public static final byte EVENT_EVENT_DOWNLOAD_NETWORK_REJECTION          = (short)31;

/** Event: Event Download CSG Cell Selection event = XX */

public static final byte EVENT_EVENT_DOWNLOAD_CSG_CELL_SELECTION_EVENT = (short)XX;

/** @deprecatedBER-TLV: Cell Broadcast download tag = 0xD2      */

public static final byte BTAG_CELL_BROADCATS_DOWNLOAD = (byte)0xD2;

/** BER-TLV: Cell Broadcast download tag = 0xD2      */

public static final byte BTAG_CELL_BROADCAST_DOWNLOAD = (byte)0xD2;

/** BER-TLV : MO short message control tag = 0xD5    */

public static final byte BTAG_MO_SHORT_MESSAGE_CONTROL = (byte)0xD5;

/** BER-TLV : SMS-PP download tag = 0xD1 */

public static final byte BTAG_SMS_PP_DOWNLOAD = (byte)0xD1;

/** BER-TLV : USSD download tag = 0xD9 */

public static final byte BTAG_USSD_DOWNLOAD = (byte)0xD9;

/**  Type of proactive command : SEND SS = 0x11 */

public static final byte PRO_CMD_SEND_SS = (byte)0x11;

/** Type of proactive command : SEND USSD = 0x12 */

public static final byte PRO_CMD_SEND_USSD = (byte)0x12;

/** General Result : USSD or SS transaction terminated by user = 0x14 */

public static final byte RES_CMD_PERF_USSD_TRANSAC_TERM = (byte)0x14;

/** General Result : SS return error = 0x34 */

public static final byte RES_ERROR_SS_RETURN_ERROR = (byte)0x34;

/** General Result : SMS RP-ERROR = 0x35     */

public static final byte RES_ERROR_SMS_RP_ERROR = (byte)0x35;

/** General result: USSD return error = 0x37     */

public static final byte RES_ERROR_USSD_RETURN_ERROR = (byte)0x37;

/** General result: Interaction with call control by USIM or MO short message control by USIM, permanent problem = 0x39 */

public static final byte RES_CMD_CALL_MO_CONTROL_PROBLEM = (byte)0x39;

/** Simple-TLV : BC Repeat Indicator tag = 0x2A */

public static final byte TAG_BC_INDICATOR = (byte)0x2A;

/** Simple-TLV : BCCH Channel List tag = 0x1D */

public static final byte TAG_BCCH_CHANNEL_LIST = (byte)0x1D;

/** Simple-TLV : Cause tag = 0x1A */

public static final byte TAG_CAUSE = (byte)0x1A;

/** Simple-TLV : Cell Broadcast Page tag = 0x0C; */

public static final byte TAG_CELL_BROADCAST_PAGE = (byte)0x0C;

/** Simple-TLV : SMS TPDU tag = 0x0B */

public static final byte TAG_SMS_TPDU = (byte)0x0B;

/** Simple-TLV : SS String tag */

public static final byte TAG_SS_STRING = (byte)0x09;

/** Simple-TLV : Timing Advance tag = 0x2E */

public static final byte TAG_TIMING_ADVANCE = (byte)0x2E;

/** Simple-TLV : Transaction Identifier tag = 0x1C */

public static final byte TAG_TRANSACTION_IDENTFIER = (byte)0x1C;

/** Simple-TLV : USSD String tag = 0x0A */

public static final byte TAG_USSD_STRING = (byte)0x0A;

/** Simple-TLV : PDP context Activation parameters tag = 0x52 */

public static final byte TAG_PDP_CONTEXT_ACTIVATION_PARAMETERS = (byte)0x52;

/** Simple-TLV : UTRAN Measurement Qualifier tag = 0x69 */

public static final byte TAG_UTRAN_MEASUREMENT_QUALIFIER = (byte)0x69;

/** Simple-TLV : CSG ID list Tag = 0x7E */

public final static byte TAG_CSG_ID_LIST = (byte)0x7E;

/** Simple-TLV : CSG cell selection status =  0x55 */

public static final byte TAG_CSG_SELECTION_STATUS = (byte)55;

/** Simple-TLV : CSG ID = 0x56 */

public static final byte TAG_CSG_ID = (byte)0x56;

/** Simple-TLV : HNB name = 0x57 */

public static final byte TAG_HNB_NAME = (byte)0x57 ;

}

……

public interface USATTerminalProfile {

/**

* SMS PP Data Download = 1

*/

public final static short IDX_SMS_PP_DATA_DOWNLOAD = (short) 1;

/**

* Cell Broadcast Data Download = 2

*/

public final static short IDX_CELL_BROADCAST_DATA_DOWNLOAD = (short) 2;

/**

* '9EXX' response code for SIM data download error = 4

*/

public final static short IDX_9EXX_RESPONSE = (short)4;

/**

* USSD string data object supported in Call Control = 6

*/

public static final short IDX_USSD_SUPPORT_IN_CALL_CONTROL = (short)6;

/**

* Envelope Call Control always sent to the SIM during automatic redial mode = 7

*/

public final static short IDX_CALL_CONTROLL_AUTOMATIC_REDIAL_MODE = (short) 7;

/**

* Call Control by SIM = 9

*/

public final static short IDX_CALL_CONTROL_BY_SIM = (short)9;

/**

* Cell identity included in Call Control by SIM = 10

*/

public final static short IDX_CELL_IDENTITY_IN_CALL_CONTROL_BY_SIM = (short)10;

/**

* MO short message control by SIM  = 11

*/

public final static short IDX_MO_SM_CONTROL_BY_SIM = (short)11;

/**

* Handling of the alpha identifier  = 12

*/

public static final short IDX_HANDLING_OF_ALPHA_IDENTIFIER = (short)12;

/**

* Proactive SIM: SEND SHORT MESSAGE = 25

*/

public static final short IDX_PROACTIVE_SIM_SEND_SHORT_MESSAGE = (short)25;

/**

* Proactive SIM: SEND SS  = 26

*/

public final static short IDX_PROACTIVE_SIM_SEND_SS = (short) 26;

/**

* Proactive SIM: SEND USSD = 27

*/

public final static short IDX_PROACTIVE_SIM_SEND_USSD = (short)27;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION - NMR = 31

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_NMR = (short)31;

/**

* Binary choice in GET INKEY = 59

*/

public final static short IDX_BINARY_CHOICE_IN_GET_INKEY = (short)59;

/**

* 2nd alpha identifier in SET UP CALL = 62

*/

public final static short IDX_2ND_ALPHA_IDENTIFIER_SET_UP_CALL = (short)62;

/**

* 2nd capability configuration parameter = 63

*/

public static final short IDX_2ND_CAPABILITY_PARAMETER = (short)63;

/**

* Proactive SIM: PROVIDE LOCAL INFORMATION - BCCH  = 66

*/

public final static short IDX_PROACTIVE_SIM_PROVIDE_LOCAL_INFORMATION_BCCH = (short) 66;

/**

* Proactive SIM: PROVIDE LOCAL INFORMATION (Timing Advance)  = 68

*/

public final static short IDX_PROACTIVE_SIM_TIMING_ADVANCE = (short) 68;

/**

* CALL Control on GPRS = 141

*/

public final static short IDX_CALL_CONTROL_ON_GPRS = (short) 141;

/**

* Support of UTRAN PS with extended parameters = 168

*/

public final static short IDX_EXTENDED_PARAMETERS_IN_UTRAN_PS_ = (short) 168;

/**

* Geographical Location Reporting = 181

*/

public final static short IDX_GEOGRAPHICAL_LOCATION_REPORTING = (short)181 ;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION NMR(UTRAN) *Correct Version* = 182

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_NMR_UTRAN_COR = (short) 182;

/**

* USSD Data Download and application mode *Correct Version* = 183

*/

public final static short IDX_USSD_DATA_DOWNLOAD_AND_APPLICATION_MODE_COR = (short) 183;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION NMR(UTRAN) = 173

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_NMR_UTRAN = (short) 173;

/**

* USSD Data Download and application mode= 174

*/

public final static short IDX_USSD_DATA_DOWNLOAD_AND_APPLICATION_MODE = (short) 174;

/**

* Event: I-WLAN Access Status = 195

*/

public final static short IDX_EVENT_I_WLAN_ACCESS_STATUS = (short)195;

/**

* Event: Network Rejection for GERAN/UTRAN = 196

*/

public final static short IDX_EVENT_NETWORK_REJECTION_GERAN_UTRAN = (short)196 ;

/**

* Event: Network Rejection for GERAN/UTRAN = 198

*/

public final static short IDX_EVENT_NETWORK_REJECTION_E_UTRAN = (short)198;

/**

* Event: CSG Cell Selection  = 199;

*/

public final static short IDX_EVENT_CSG_CELL_SELECTION = (short)199;

/**

* I-WLAN bearer support = 232

*/

public final static short IDX_I_WLAN_BEARER_SUPPORT = (short)232;

/**

* Proactive UICC: PROVIDE LOCAL INFORMATION (WSID of the current I-WLAN connection) = 233

*/

public final static short IDX_PROACTIVE_UICC_PROVIDE_LOCAL_INFORMATION_WSID_I_WLAN = (short)233;

/**

* "Steering of Roaming" REFRESH support = 235

*/

public final static short IDX_STEERING_OF_ROAMING_REFRESH = (short)235;

/**

* Proactive UICC: Geographical Location Request = 237

*/

public final static short IDX_PROACTIVE_UICC_GEOGRAPHICAL_LOCATION_REQUEST = (short)237;

/**

* "Steering of Roaming for I-WLAN" REFRESH support = 239

*/

public final static short IDX_STEERING_OF_ROAMING_FOR_I_WLAN_REFRESH = (short)239;

/**

*  Support for CSG cell discovery = 241;

*/

public final static short IDX_CSG_CELL_DISCOVERY = (short) 241;

}