| ETSI TC SCP Meeting #48 Sophia Antipolis, France, 02 - 04 March 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCP(11)0159 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #37 Sophia Antipolis, France, 31 Jan - 04 Feb 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCPTEC(11)0014r3 |  |  |  |  |  |  |  |  |
|  | EP SCP CR-Form-v1.7.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 102 705 |  |  |  | CR | 015 |  |  |  rev |  |  |  |  | - |  Current version: |  |  |  |  | 9.2.0 |  |  |
|  | For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed change affects:  |  |  |  |  | smart card |  | x | terminal |  |  |  | server / network entity |  |  |  |  |  |  |  |  |  |  |
|  | Title:  |  | Clarification of use of events EVENT_HCI_RECEPTION_FAILED and EVENT_HCI_TRANSMISSION_FAILED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work item name:  |  | HCI API |  |  |  |  |  |  |  |  |  |  | Date:  |  |  |  | 03/02/2011 |  |  |  |  |  |
|  | Category:  |  | F |  |  |  |  |  |  |  |  |  |  | Release:  |  |  |  | REL-9 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) |  |  |  |  |  |  |
|  | Reason for change:  |  |  | CR 006 introduced the new events EVENT_HCI_RECEPTION_FAILED and EVENT_HCI_TRANSMISSION_FAILED in the interface HCIListener. However, it is not stated if these events need to be activated and in which callback methods these events may occur. According to the current rules these events would never occur. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Summary of change:  |  |  | It is clarified that the events EVENT_HCI_RECEPTION_FAILED and EVENT_HCI_TRANSMISSION_FAILED need to be activated for each specific service. It is further clarified that these events may occur in each *Listener interface event though these events are defined in HCIListener and not in the specific *Listener interface as was previously specified. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Consequences if  not approved: |  |  | Unclear specification and interoperability problems. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clauses affected:  |  |  | Annex A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex A (normative):
Java Card™ Platform HCI API for the UICC

The source files for the UICC Application Programming Interface for Java Card™ for contactless Applets (102705_Annex_A_Java.zip and 102705_Annex_A-HTML.zip) are contained in ts_102705v090x00p0.zip, which accompanies the present document.

In HCIService.java:

package uicc.hci.framework;

…

public interface HCIService {

…

/**

* This method requests the HCI framework to call the listener method

* <code>onCallback(byte event, HCIMessage message)</code>

* to provide access to the HCIMessage object of the addressed service.

* <P>

* Calling <code>requestCallbackNotification</code> while no corresponding Listener

* to this Service is registered has no effect.

*

* <P>The request for invocation of <code>onCallback</code> with this

* <code>event</code> shall only be valid one time and transient. It is not required that

* the corresponding Listener has activated the <code>event</code> with

* <code>activateEvent</code>. A deactivation of the <code>event</code> with

* <code>deactivateEvent</code> has no effect.

* <P>The notification will go to the corresponding listener interface of this

*    Service instance.

* <P>Any incoming HCI event may trigger the <code>HCIListener</code> before this request is

* forwarded to the <code>HCIListener</code>.

*

* <P>For use in this method, parameters are restricted to the range 0xC0 to 0xFF.     *

* @param event event value to be passed to the <code>HCIListener</code>.

* @throws HCIException with reason code <code>HCI_WRONG_EVENT_TYPE</code> in case

*                    <code>event</code> is not in the allowed parameter range.

*/

public void requestCallbackNotification(byte event) throws HCIException;

/**

* This method is used to activate an event.

*

* <ul>

* <li>Possible values for the parameter <code>event</code> for the different Service objects are:

* <ul>
     * For all *Service interfaces:
     * <li>HCIListener.EVENT_HCI_TRANSMISSION_FAILED
     * <li>HCIListener. EVENT_HCI_RECEPTION_FAILED
     *

* For CardEmulationService:

* <li>CardEmulationListener.EVENT_GET_PARAMETER_RESPONSE

* <li>CardEmulationListener.EVENT_ON_SEND_DATA,

* <li>CardEmulationListener.EVENT_FIELD_OFF.

* </ul>

* <ul>

* For ReaderService:

* <li>ReaderListener.EVENT_GET_PARAMETER_RESPONSE

* <li>ReaderListener.EVENT_TARGET_DISCOVERED.

* </ul>

* <ul>

* For ConnectivityService:

* <li>ConnectivityListener.EVENT_STANDBY.

* </ul>

* </ul>

* @param event event to be activated

* @throws HCIEception with reason code

*     <ul>

*      <li><code>HCI_WRONG_EVENT_TYPE<code> in case a wrong event was activated for this Service instance

*      <li><code>HCI_CONDITIONS_NOT_SATISFIEDE<code> if one or more conditions to activate the event are not satisfied

*     </ul>

*/

public void activateEvent(byte event) throws HCIException;

/**

* This method is used to deactivate an event from the list of activated events.

*

* <ul>

* <li>Possible values for the parameter <code>event</code> for the different Service objects are:

* <ul>

* For all *Service interfaces:
     * <li>HCIListener.EVENT_HCI_TRANSMISSION_FAILED
     * <li>HCIListener. EVENT_HCI_RECEPTION_FAILED
     *

* For CardEmulationService:

* <li>CardEmulationListener.EVENT_GET_PARAMETER_RESPONSE

* <li>CardEmulationListener.EVENT_ON_SEND_DATA

* <li>CardEmulationListener.EVENT_FIELD_OFF

* </ul>

* <ul>

* For ReaderService:

* <li>ReaderListener.EVENT_GET_PARAMETER_RESPONSE

* <li>ReaderListener.EVENT_TARGET_DISCOVERED.

* </ul>

* <ul>

* For ConnectivityService:

* <li>ConnectivityListener.EVENT_STANDBY.

* </ul>

* </ul>

* @param event event to be deactivated

* @throws HCIEception with reason code <code>HCIException.HCI_WRONG_EVENT_TYPE<code> in case a wrong event was deactivated for this Service instance	 or in case the event wasn't activated before.

*/

public void deactivateEvent(byte event) throws HCIException;

/**

* return the activation state of an event.

*

* <ul>

* <li>Possible values for the parameter <code>event</code> for the different Service objects are:

* <ul>

* For all *Service interfaces:
     * <li>HCIListener.EVENT_HCI_TRANSMISSION_FAILED
     * <li>HCIListener. EVENT_HCI_RECEPTION_FAILED
     *

* For CardEmulationService:

* <li>CardEmulationListener.EVENT_GET_PARAMETER_RESPONSE

* <li>CardEmulationListener.EVENT_ON_SEND_DATA

* <li>CardEmulationListener.EVENT_FIELD_OFF

* </ul>

* <ul>

* For ReaderService:

* <li>ReaderListener.EVENT_GET_PARAMETER_RESPONSE

* <li>ReaderListener.EVENT_TARGET_DISCOVERED.

* </ul>

* <ul>

* For ConnectivityService:

* <li>ConnectivityListener.EVENT_STANDBY.

* </ul>

* </ul>

*

* @param event event to request the activation state

* @return true if the event is activated for this listener, else false

* @throws HCIEception with reason code <code>HCIException.HCI_WRONG_EVENT_TYPE<code>  in case the event was not one of the possible values.

*/

public boolean getEventNotificationStatus(byte event) throws HCIException;

}

In HCIListener.java:

package uicc.hci.framework;

import javacard.framework.Shareable;

/** The base interface for all types of HCI listeners.

* <P>

* An Object shall not implement this interface but only implement one of the subinterfaces.

*

* <P>

* An Applet instance implementing a <code>*Listener</code> interface shall only receive events defined in the same <code>*Listener</code> interface or in <code>HCIListener</code>.

* <P>

* Messages received through the inherited method <code>HCIListener.onCallback</code> shall be of the message type defined in the same package as the <code>*Listener</code> interface

* <P>

*

* @see uicc.hci.services.cardemulation.CardEmulationListener

* @see uicc.hci.services.connectivity.ConnectivityListener

* @see uicc.hci.services.readermode.ReaderListener

**/

public interface HCIListener extends Shareable {

/**

* This value is used to notify the Applet when the UICC failed to transmit a message to the CLF.

The Applet has to activate this event (for a specific service) if it wants to be notified about data transmission errors (that occur for a specific service).

*/

public final static byte EVENT_HCI_TRANSMISSION_FAILED = (byte) 0x81;

/**

* This value is used to notify the Applet when the UICC failed to receive a message sent by the CLF. 
The Applet has to activate this event (for a specific service) if it wants to be notified about data reception errors (that occur for a specific service).

*/

public final static byte EVENT_HCI_RECEPTION_FAILED = (byte) 0x82;

…

}

In CardEmulationListener.java:

package uicc.hci.services.cardemulation;

import uicc.hci.framework.HCIListener;

/**

The instance of an Applet which implements the interface <code>CardEmulationListener</code> can receive events through the <code>onCallback</code> method of this interface and can process <code>CardEmulationMessage</code> objects passed through the <code>onCallback</code> method of this interface.

The Contactless Framework shall invoke the method <code>CardEmulationListener.onCallback</code> only with events which are defined in <code>CardEmulationListener</code> or <code>HCIListener</code> and only with <code>CardEmulationMessage</code> objects.

The Applet instance must activate the events with <code>CardEmulationService.activateEvent</code> before it will receive any event notification.

*

* @see CardEmulationService

* @see CardEmulationMessage

**/

public interface CardEmulationListener extends HCIListener {

…

}

In ReaderListener.java:

package uicc.hci.services.readermode;

import uicc.hci.framework.HCIListener;

/**

The instance of an Applet which implements the interface <code>ReaderListener</code> can receive events through the <code>onCallback</code> method of this interface and can process <code>ReaderMessage</code> objects passed through the <code>onCallback</code> method of this interface.

The Contactless Framework shall invoke the method <code>ReaderListener.onCallback</code> only with events which are defined in <code>ReaderListener</code> or <code>HCIListener</code> and only with <code>ReaderMessage</code> objects.

The Applet instance must activate the events with <code>ReaderService.activateEvent</code> before it will receive any event notification.

*

* @see ReaderService

* @see ReaderMessage

**/

public interface ReaderListener extends HCIListener {

…

}

In ConnectivityListener.java:

package uicc.hci.services.connectivity;

import uicc.hci.framework.HCIListener;

/**

The instance of an Applet which implements the interface  <code>ConnectivityListener</code> can receive events through the <code>onCallback</code> method of this interface and can process <code>ConnectivityMessage</code> objects passed through the <code>onCallback</code> method of this interface.

The Contactless Framework shall invoke the method <code>ConnectivityListener.onCallback</code> only with events which are defined in <code>ConnectivityListener</code> or <code>HCIListener</code> and only with <code>ConnectivityMessage</code> objects.

The Applet instance must activate the events with <code>ConnectivityService.activateEvent</code> before it will receive any event notification.

*

* @see ConnectivityService

* @see ConnectivityMessage

**/

public interface ConnectivityListener extends HCIListener {

…

}