3GPP TSG-CT WG6 Meeting #119bis	C6-240516

# Maastricht, Netherlands; 20th – 23rd August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0768 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add testcase of reject 3G Session Reset for EFSUPI_NAI Changing procedure during mobile originated call in 5G |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R16 |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the user is on a mobile originated call in 5G, 3G Session Reset should be rejected for the EFSUPI_NAI Changing procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add testcase of reject 3G Session Reset for EFSUPI_NAI Changing procedure during mobile originated call in 5G. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for EFSUPI_NAI changing procedure are not fully covered in NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.4.7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C6-240449 |  |  |  |  |  |  |  |  |

****************************start of changes ********************************

3.4 Applicability table

| 27.22.4.6 |  | POLL INTERVAL |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | duration | R99 |  | M | E.1/22 | No |  |  |
| 27.22.4.7 |  | REFRESH |  |  |  |  |  |  |  |
| 27.22.4.7 |  | REFRESH |  |  |  |  |  |  |  |
|  | 1.1 | USIM initialization, enabling FDN mode | R99 | Rel-7 | C146 AND C177 AND C178 AND C180 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  | 1.2 | file change notification of FDN file | R99 | Rel-7 | C146 AND C177 AND C178 AND C180 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  | 1.3 | USIM initialization and file change notification of ADN | R99 | Rel-5 | C168 AND C177 AND C178 | E.1/24 AND E.1/110 AND E.1/111 | No |  |  |
|  |  |  | Rel-6 |  | C177 AND C178 | E.1/24 AND E.1/110 AND E.1/111 | No |  |  |
|  | 1.4 | USIM initialization and full file change notification, enabling FDN mode | R99 | Rel-7 | C146 AND C177 AND C178 AND C180 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  | 1.5 | UICC reset | R99 |  |  | E.1/24 | No |  |  |
|  | 1.6 | USIM Initialization after SMS-PP data download | R99 | Rel-7 | C146 AND C177 AND C178 AND C180 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  | 1.7 | USIM Application Reset | R99 | Rel-5 |  | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  |  |  | Rel-6 | Rel-7 | C146 AND C177 AND C178 AND C180 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  | 2.1 | UICC Reset for IMSI Changing procedure | R99 | Rel-7 |  | E.1/24 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | M | E.1/24 | USS or SS only |  |  |
|  | 2.2 | USIM Application Reset for IMSI Changing procedure | R99 | Rel-5 |  | E.1/24 | Yes |  |  |
|  |  |  | Rel-6 |  | M | E.1/24 | Yes |  |  |
|  | 2.3 | 3G Session Reset for IMSI Changing procedure | R99 | Rel-7 |  | E.1/24 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | M | E.1/24 | USS or SS only |  |  |
|  | 2.4 | reject 3G Session Reset for IMSI Changing procedure during CScall | R99 | Rel-5 |  | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  |  |  | Rel-6 | Rel-7 | C177 AND C178 AND C180 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  | 2.5 | reject UICC Reset for IMSI Changing procedure during CS call | R99 | Rel-7 |  | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | USS or SS only |  |  |
|  | 2.6 | UICC Reset for IMSI Changing procedure during active PDP context | R99 | Rel-7 |  | E.1/24 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C215 | E.1/24 | USS or SS only |  |  |
|  | 2.7 | 3G Session Reset for IMSI Change procedure during active PDP context | R99 | Rel-7 |  | E.1/24 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C215 | E.1/24 | USS or SS only |  |  |
|  | 3.1 | Steering of roaming, UTRAN | Rel-7 | Rel-7 | M | E.1/24 AND E.1/236 | USS only |  |  |
|  |  |  | Rel-8 |  | C184 | E.1/24 AND E.1/236 | USS only |  |  |
|  | 3.2 | Steering of roaming, InterRAT | Rel-7 | Rel-7 | C167 | E.1/24 AND E.1/236 | USS and SS |  |  |
|  |  |  | Rel-8 |  | C167 AND C184 | E.1/24 AND E.1/236 | USS and SS |  |  |
|  | 3.3 | Steering of roaming, E-UTRAN | Rel-8 | Rel-12 | C190 | E.1/24 AND E.1/135 AND E.1/236 | E-USS or NB-SS (See NOTE) |  |  |
|  |  |  | Rel-13 |  | C222 | E.1/24 AND E.1/135 AND E.1/236 | E-USS or NB-SS (See NOTE) |  |  |
|  | 3.4 | Steering of roaming, NG-RAN | Rel-16 |  | C231 | E.1/24 AND E.1/236 | NG-SS only |  |  |
|  | 4.1 | Refresh with AID, E-UTRAN or UTRAN | Rel-7 | Rel-7 | C203 | E.1/24 | E-USS only or USS |  |  |
|  |  |  | Rel-8 |  | C202 OR C203 | E.1/24 | E-USS only or USS |  |  |
|  | 5.1 | UICC Reset for IMSI Changing procedure, E-UTRAN | Rel-8 |  | C190 | E.1/24 | E-USS or NB-SS (See NOTE) |  |  |
|  | 5.2 | 3G Session Reset for IMSI Changing procedure, E-UTRAN | Rel-8 |  | C190 | E.1/24 | E-USS or NB-SS (See NOTE) |  |  |
|  | 6.1 | REFRESH, UICC Reset for IMSI Changing procedure, NG-RAN | Rel-16 |  | C231 | E.1/24 OR (E.1/24 AND E.1/256) | NG-SS only |  |  |
|  | 6.2 | REFRESH, 3G Session Reset for IMSI Changing procedure, NG-RAN | Rel-16 |  | C231 | E.1/24 OR (E.1/24 AND E.1/256) | NG-SS only |  |  |
|  | 7.1 | REFRESH, UICC Reset for SUPI_NAI Changing procedure, NG-RAN | Rel-16 |  | C231 AND C233 | E.1/24 | NG-SS only |  |  |
|  | 7.2 | REFRESH, 3G Session Reset for SUPI_NAI Changing procedure, NG-RAN | Rel-16 |  | C231 AND C233 | E.1/24 | NG-SS only |  |  |
|  | 7.X | REFRESH, reject 3G Session Reset for SUPI_NAI Changing procedure during mobile originated call, NG-RAN | Rel-16 |  | C231 AND C233 | E.1/24 | NG-SS only |  |  |

****************************next of changes ********************************

27.22.4.7.7	REFRESH (SUPI_NAI changing procedure, NG-RAN)

27.22.4.7.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.7.7.2	Conformance requirement

The ME shall support the REFRESH command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.7, clause 6.4.7.1, clause 6, clause 6.6.13, clause 5.2, clause 8.6, clause 8.7 and clause 8.18.

Additionally the ME shall support the USIM Initialization and USIM application closure procedure as defined in:

-	TS 31.102 [14] clause 5.1.1.2, clause 5.1.3 and Annex I.

27.22.4.7.7.3	Test purpose

To verify that the ME performs the Proactive Command – REFRESH in accordance with the Command Qualifier and the SUPI_NAI changing procedure. This may require the ME to perform:

-	the USIM initialization

-	a re-read of the contents and structure of the SUPI_NAI on the USIM

-	a restart of the card session

-	a successful return of the result of the execution of the command in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.7.7.4	Method of test

27.22.4.7.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and connected to the NG-SS.

The NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

The elementary files are coded as the default NG-RAN UICC with the following exceptions:

EFUST (USIM Service Table)

Logically:

User controlled PLMN selector available

Fixed dialling numbers available

The GSM Access available

The Group Identifier level 1 and level 2 not available

Service n 33 (Packed Switched Domain) shall be set to '1'

Enabled Services Table available

EPS Mobility Management Information available

Allowed CSG Lists and corresponding indications

5GS Mobility Management Information available

5G Security Parameters available

Subscription identifier privacy support available

SUCI calculation by the USIM not available

Support for SUPI of type NSI or GLI or GCI available

| Byte: | B1 |  | B2 |  | B3 |  |  | B4 |  | B5 |  |  | B6 |  | B7 |  | B8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x |  | xxxx xxxx |  | xxxx 1x00 |  |  | xxxx x1xx |  | xxxx xx11 |  |  | xxxx xxxx |  | xxxx xxxx |  | xxxx xxxx |  |  |
|  | B9 |  | B10 |  | B11 |  |  |  |  | B16 |  |  | B17 |  |  |  |  |  |  |
|  | xxxx xxxx |  | xxxx xxxx |  | xx11 xxxx |  |  | ..... |  | xxx0 111x |  |  | xxxxxx1x |  |  |  |  |  |  |

The coding of EFUST shall conform with the capabilities of the USIM used.

EFSUPI_NAI (SUPI as Network Access Identifier)

Logically:	userid18@example.com

SUPI Type: NSI

Username: userid18

Realm:  example.com

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 14 | 75 | 73 | 65 | 72 | 69 | 64 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 31 | 38 | 40 | 65 | 78 | 61 | 6D | 70 |
|  | B17 | B18 | B19 | B20 | B21 | B22 |  |  |
|  | 6C | 65 | 2E | 63 | 6F | 6D |  |  |

EFIMSI (IMSI)

This file shall not be available.

EFAD (Administrative Data)

Logically:	Type approval operations

OFM to be deactivated by the Terminal

Length of MNC in the IMSI: 0

| Coding: | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 80 | 00 | 00 | 00 |

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.7.7.4.2	Procedure

Expected Sequence 7.1 (REFRESH, UICC Reset for SUPI_NAI Changing procedure, NG-RAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. | The ME registers using SUPI_NAI "userid18@example.com" in NG-RAN |
| 2 | UICC | Update EF SUPI_NAI and EF 5GSN3GPPLOCI. | The content of EF SUPI_NAI has been changed to "userid19@example.com" and the 5G-GUTI in EF 5GSN3GPPLOCI is updated to 'FF FF FF FF FF FF FF FF FF FF FF FF FF'. |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 7.1.1 | [To inform the ME that SUPI_NAI has changed] |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: REFRESH 7.1.1 or 7.1.2 | IF terminal supports PD_ Refresh_Enforcement_Policy use PROACTIVE COMMAND: REFRESH 7.1.2, ELSE 7.1.1. |
| 6 | MENG-SS | Deregistration Request |  |
| 7 | ME UICC | ME performs UICC reset. | Both cold and warm resets are allowed |
| 8 | MENG-SS | Registration Request | The ME will register using SUPI_NAI "userid19@example.com" in NG-RAN. |
| 9 | NG-SSME | Registration Accept |  |
| 10 | MENG-SS | Registration Complete |  |

PROACTIVE COMMAND: REFRESH 7.1.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: REFRESH 7.1.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	UICC

Destination device:	ME

Refresh enforcement policy: Force immediate REFRESH even if the terminal is busy on data call

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 81 | 82 | 3A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 02 |

Expected Sequence 7.2 (REFRESH, 3G Session Reset for SUPI_NAI Changing procedure, NG-RAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. | The ME registers using SUPI_NAI "userid18@example.com" in NG-RAN |
| 2 | UICC | Update EF SUPI_NAI and EF 5GSN3GPPLOCI. | The content of EF SUPI_NAI has been changed to "userid19@example.com" and the 5G-GUTI in EF 5GSN3GPPLOCI is updated to 'FF FF FF FF FF FF FF FF FF FF FF FF FF'. |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 7.2.1 | [To inform the ME that SUPI_NAI has changed] |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: REFRESH 7.2.1 or 7.2.2 | IF terminal supports PD_ Refresh_Enforcement_Policy use PROACTIVE COMMAND: REFRESH 7.2.2, ELSE 7.2.1. |
| 6 | MENG-SS | Deregistration Request |  |
| 7 | ME UICC | STATUS[P1='02'] | If A.1/172 is supported, then the ME indicates to USIM that the termination procedure is starting, completes the 3G session termination procedure and resets the application via SELECT by DF name command with the AID. The ME performs the USIM initialization. |
| 8 | ME UICC | TERMINAL RESPONSE: REFRESH 7.2.1A or TERMINAL RESPONSE: REFRESH 7.2.1B | [normal ending] |
| 9 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 10 | MENG-SS | Registration Request | The ME will register using SUPI_NAI "userid19@example.com" in NG-RAN. |
| 11 | NG-SSME | Registration Accept |  |
| 12 | MENG-SS | Registration Complete |  |

PROACTIVE COMMAND: REFRESH 7.2.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	2

File:	EF SUPI_NAI

File:	EF 5GSN3GPPLOCI

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11 | 02 | 3F | 00 | 7F | FF | 5F | C0 | 4F | 09 | 3F | 00 |
|  | 7F | FF | 5F | C0 | 4F | 02 |  |  |  |  |  |  |

PROACTIVE COMMAND: REFRESH 7.2.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	2

File:	EF SUPI_NAI

File:	EF 5GSN3GPPLOCI

Refresh enforcement policy: Force immediate REFRESH even if the terminal is busy on data call

Coding:

| BER-TLV: | D0 | 1F | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11 | 02 | 3F | 00 | 7F | FF | 5F | C0 | 4F | 09 | 3F | 00 |
|  | 7F | FF | 5F | C0 | 4F | 02 | 3A | 01 | 02 |  |  |  |

TERMINAL RESPONSE: REFRESH 7.2.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 7.2.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 7.X (REFRESH, reject 3G Session Reset for SUPI_NAI Changing procedure during mobile originated call, NG-RAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. | The ME registers using SUPI_NAI "userid18@example.com" in NG-RAN |
| 1 | User ME | Set up an IMS voice call | Call needs to be connected |
| 2 | ME NG-SS | Establish IMS voice call | The established IMS voice call needs to be held |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 7.X.1 | [To inform the ME that SUPI_NAI has changed] |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: REFRESH 7.X.1A Or REFRESH 7.X.1B |  |
| 6 | ME UICC | TERMINAL RESPONSE: REFRESH 7.X.1A Or TERMINAL RESPONSE: REFRESH 7.X.1B | ME rejects REFRESH proactive command |
| 7 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 8 | USER ME | The MO call is terminated |  |

PROACTIVE COMMAND: REFRESH 7.X.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	2

File:	EF SUPI_NAI

File:	EF 5GSN3GPPLOCI

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11 | 02 | 3F | 00 | 7F | FF | 5F | C0 | 4F | 09 | 3F | 00 |
|  | 7F | FF | 5F | C0 | 4F | 02 |  |  |  |  |  |  |

PROACTIVE COMMAND: REFRESH 7.X.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	2

File:	EF SUPI_NAI

File:	EF 5GSN3GPPLOCI

Refresh enforcement policy: Force immediate REFRESH even if the terminal is busy on data call

Coding:

| BER-TLV: | D0 | 1F | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11 | 02 | 3F | 00 | 7F | FF | 5F | C0 | 4F | 09 | 3F | 00 |
|  | 7F | FF | 5F | C0 | 4F | 02 | 3A | 01 | 02 |  |  |  |

TERMINAL RESPONSE: REFRESH 7.X.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command
Additional information on result:	ME currently busy on call

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 02 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: REFRESH 7.X.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command
Additional information on result:	Screen is busy

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 02 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

27.22.4.7.7.5	Test requirement

The ME shall operate in the manner defined in expected sequences 7.1 to 7.X.

****************************end of changes ********************************