3GPP TSG-CT WG6 Meeting #119bis	C6-240518

# Maastricht, Netherlands; 20th – 23rd August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0770 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add testcase about SEND DATA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R16 |  |  |  |  |  | Date: |  |  | 2024-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Before the ME send the Terminal Response for SEND DATA to the UICC, the ME may receive a RECEIVE DATA command from the remote system platform, and it should suspend the RECEIVE DATA command until the SEND DATA command finished. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a testcase about SEND DATA that RECEIVE DATA should be suspended during the process of SEND DATA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for SEND DATA are not fully covered |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.4.30.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | C6-240451 |  |  |  |  |  |  |  |  |

****************************start of changes ********************************

3.4	Applicability table

NOTE:	It is possible that the applicability of some tests indicated in table B.1 does not match with the value in the Release column, due to late definition of the test sequences. Tests should be performed without considering the Release column, but only based on the conditions indicated for each release.

| 27.22.4.30 |  | SEND DATA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | immediate mode | R99 | Rel-7 | C121 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C121 AND C183 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  | 1.2 | Store mode | R99 | Rel-7 | C121 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C121 AND C183 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  | 1.3 | Store mode, Tx buffer fully used | R99 | Rel-7 | C121 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C121 AND C183 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  | 1.4 | 2 consecutive SEND DATA Store mode | R99 | Rel-7 | C121 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C121 AND C183 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  | 1.5 | immediate mode with a bad channel identifier | R99 | Rel-7 | C121 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  |  |  | Rel-8 |  | C121 AND C183 | E.1/89 AND E.1/92 | USS or SS only |  |  |
|  | 1.6 | void |  |  |  |  |  |  |  |
|  | 2.1 | Text attribute– left alignment | Rel-5 | Rel-7 | C121 AND C153 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/217 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C153 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/217 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | 2.2 | Text attribute – center alignment | Rel-5 | Rel-7 | C121 AND C154 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/218 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C154 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/218 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | 2.3 | Text attribute – right alignment | Rel-5 | Rel-7 | C121 AND C155 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/219 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C155 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/219 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | 2.4 | Text attribute – large font size | Rel-5 | Rel-7 | C121 AND C157 AND C156 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C157 AND C156 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | 2.5 | Text attribute – small font size | Rel-5 | Rel-7 | C121 AND C158 AND C156 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C158 AND C156 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | 2.6 | Text attribute – bold on | Rel-5 | Rel-7 | C121 AND C160 AND C159 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C160 AND C159 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | 2.7 | Text attribute – italic on | Rel-5 | Rel-7 | C121 AND C161 AND C159 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C161 AND C159 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | 2.8 | Text attribute – underline on | Rel-5 | Rel-7 | C121 AND C162 AND C159 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C162 AND C159 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | 2.9 | Text attribute – strikethrough on | Rel-5 | Rel-7 | C121 AND C163 AND C159 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C163 AND C159 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | 2.10 | Text attribute– foreground and background colours | Rel-5 | Rel-7 | C121 AND C164 AND C165 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | USS or SS only |  | TCEP001 |
|  |  |  | Rel-8 |  | C121 AND C164 AND C165 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | USS or SS only |  | TCEP001 |
|  | TBD | Frames | Rel-6 |  |  | E.1/89 AND E.1/92 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  |  |
|  | 3.1 | Immediate mode – E-UTRAN, Default EPS bearer | Rel-8 | Rel-12 | C182 | E.1/89 AND E.1/92 | E-USS or NB-SS (See NOTE) |  |  |
|  |  |  | Rel-13 |  | C223 | E.1/89 AND E.1/92 | E-USS or NB-SS (See NOTE) |  |  |
|  | 3.2 | Store mode – E-UTRAN, APN different from default APN | Rel-8 | Rel-12 | C182 | E.1/89 AND E.1/92 | E-USS or NB-SS (See NOTE) |  |  |
|  |  |  | Rel-13 |  | C224 | E.1/89 AND E.1/92 | E-USS or NB-SS (See NOTE) |  |  |
|  | 4.1 | NG-RAN, bearer type '03' – Default PDU Session, immediate mode | Rel-16 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 4.2 | SEND DATA, NG-RAN, bearer type '0C', Store mode | Rel-16 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 4.x | SEND DATA, NG-RAN, RECEIVE DATA suspended during the process of SEND DATA | Rel-16 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |

****************************next of changes ********************************

27.22.4.30.4	SEND DATA (NG-RAN)

27.22.4.30.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.4.2	Conformance requirements

The ME shall support the class "e" commands and NG-RAN as defined in:

-	TS 31.111[15] clause 5.2, clauses 6.4.27 and 6.6.27, clause 8.6, clause 8.7, clause 9.2, clause 8.2, clause 8.15, clause 8.52, clause 8.59, clause 8.61,

-	TS 24.501 [40], clauses 6.2.2 and 6.2.3,

-	TS 23.501 [41], clauses 5.15.5.2 and 5.15.5.3,

-	TS 23.503 [42], clause 6.6.2,

-	TS 24.526 [43], clause 4.1, 4.2.2, 5.2 and 5.15.5.3,

-	TS 23.003 [44], clause 9A.

27.22.4.30.4.3	Test purpose

To verify that the ME shall send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC after the ME receives the SEND DATA proactive command. The TERMINAL RESPONSE sent back to the UICC is the result of the ME and the network capabilities against requested parameters by the UICC.

27.22.4.30.4.4	Method of test

27.22.4.30.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and the NG-SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default NG-RAN UICC, the default NG-RAN parameters and the following URSP rules stored in the ME are used:

URSP:

Rule Precedence =1

Traffic Descriptor:

DNN=TestGp.rs

Route Selection Descriptor:

Precedence=1

Network Slice Selection, S-NSSAI: 01 01 01 02 (ST: MBB, SD: 010102)

SSC Mode Selection: SSC Mode 1

Access Type preference: 3GPP access

Rule Precedence = <lowest priority>

Traffic Descriptor: *

Route Selection Descriptor:

Precedence =1

Network Slice Selection, S-NSSAI: 01 01 01 01 (ST: MBB, SD: 010101)

SSC Mode Selection: SSC Mode 1

DNN Selection: internet

The Allowed S-NSSAI list is configured in NG-SS as '01 01 01 01', '01 01 01 02'and '01 01 01 03'

For sequence 4.2 the NG-SS shall be able to support 2 active PDU sessions at the same time.

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

Prior to test case execution the apparatus supplier shall have provided the "Preferred buffer size supported by the terminal for Open Channel command" as requested in table A.2/29.

27.22.4.30.4.4.2	Procedure

Expected sequence 4.1 (SEND DATA, NG-RAN, bearer type '03' – Default PDU Session, immediate mode)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure URSP rules with DNN "TestGp.rs" in the terminal configuration if required. Internet PDU session using DNN "internet" is configured in the terminal. | [see initial conditions] |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | An Internet PDU Session is established successfully. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 4.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 4.1.1 |  |
| 7 | ME USER | The ME may display channel opening information. |  |
| 8 | ME NG-SS | The terminal shall not send a PDU SESSION ESTABLISHMENT REQUEST to the network. |  |
| 9 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 4.1.1 | [Command performed successfully] |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 4.1.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 4.1.1 |  |
| 10 | ME NG-SS | Transfer of 8 Bytes of data to the NG-SS through channel 1 |  |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 4.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 4.1.1

Same as 27.22.4.27.8.4.2 PROACTIVE COMMAND: OPEN CHANNEL 8.1.1 in expected sequence 8.1

TERMINAL RESPONSE: OPEN CHANNEL 4.1.1

Same as 27.22.4.27.8.4.2 TERMINAL RESPONSE: OPEN CHANNEL 8.1.1 in expected sequence 8.1

PROACTIVE COMMAND: SEND DATA 4.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |  |

TERMINAL RESPONSE: SEND DATA 4.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

Expected sequence 4.2 (SEND DATA, NG-RAN, bearer type '0C', Store mode)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure URSP rules with DNN "TestGp.rs" in the terminal configuration if required. Internet PDU session using DNN "internet" is configured in the terminal. | [see initial conditions] |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | An Internet PDU Session is established successfully. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 4.2.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL4.2.1 |  |
| 7 | ME USER | The ME may display channel opening information. |  |
| 8 | ME NG-SS | PDU SESSION ESTABLISHMENT REQUEST within UL NAS TRANSPORT is sent to the network. | DNN=TestGp.rs, S-NSSAI='01 01 01 02', SSC mode=1. |
| 9 | NG-SS ME | PDU SESSION ESTABLISHMENT ACCEPT |  |
| 10 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 4.2.1 | [Command performed successfully] |
| 11 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA4.2.1 |  |
| 12 | ME UICC | FETCH |  |
| 13 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 4.2.1 | Send 500 Bytes of data (200 + 200 + 100) |
| 14 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 4.2.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 4.2.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 4.2.2 | [200 Bytes] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 4.2.2 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 4.2.3 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: SEND DATA (Immediate mode) 4.2.3 | [100 Bytes] |
| 22 | ME NG-SS | Transfer of 500 Bytes of data to the NG-SS through channel 1 |  |
| 23 | ME UICC | TERMINAL RESPONSE: SEND DATA (Immediate mode) 4.2.3 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 4.2.1

Same as 27.22.4.27.8.4.2 PROACTIVE COMMAND: OPEN CHANNEL 8.2.1 in expected sequence 8.2

TERMINAL RESPONSE: OPEN CHANNEL 4.2.1

Same as 27.22.4.27.8.4.2 TERMINAL RESPONSE: OPEN CHANNEL 8.2.1 in expected sequence 8.2

PROACTIVE COMMAND: SEND DATA 4.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	00 01 .. C7 (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | .. | C7 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DATA 4.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 4.2.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	C8 C9 .. FF 00 01 .. 8F (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | C8 | C9 | .. | FF | 00 | 01 | .. | 8F |  |

TERMINAL RESPONSE: SEND DATA 4.2.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 4.2.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Immediate mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	90 91 .. F3 (100 Bytes of data)

Coding:

| BER-TLV: | D0 | 6F | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 64 | 90 | 91 | .. | F3 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND DATA 4.2.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Immediate mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

Expected sequence 4.X (SEND DATA, NG-RAN, RECEIVE DATA suspended during the process of SEND DATA )

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure URSP rules with DNN "TestGp.rs" in the terminal configuration if required. Internet PDU session using DNN "internet" is configured in the terminal. | [see initial conditions] |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | An Internet PDU Session is established successfully. |  |
| 4 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 4.X.1 |  |
| 5 | ME  UICC | FETCH |  |
| 6 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 4.X.1 |  |
| 7 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 4.X.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 4.X.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 4.X.1 |  |
| 11 | ME USER | The ME may display channel opening information. |  |
| 12 | ME NG-SS | The terminal shall not send a PDU SESSION ESTABLISHMENT REQUEST to the network. |  |
| 13 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 4.X.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 4.X.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 4.X.1 |  |
| 17 | ME NG-SS | Transfer of 8 Bytes of data to the NG-SS through channel 1 | [To retrieve ME's port number] |
| 18 | NG-SS ME | Transfer of 200 Bytes of data to the ME through channel 1 using the ME's port number, which was retrieved in step 17 |  |
| 19 | ME | The ME receives a RECEIVE DATA command, but it shall not send the command to the UICC | [ RECEIVE DATA related command should be suspended until the TERMINAL RESPONSE for SEND DATA command finished] |
| 20 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 4.X.1 | [Command performed successfully] |
| 21 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Data available 4.X.1 | (200 Bytes of data in the ME buffer) |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 4.X.1 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 4.X.1 | 200 Bytes |
| 25 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 4.X.1 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 4.X.1

Same as 27.22.4.29.1.4.2 PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 in expected sequence 1.1

TERMINAL RESPONSE: SET UP EVENT LIST 4.X.1

Same as 27.22.4.29.1.4.2 TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 in expected sequence 1.1

PROACTIVE COMMAND: OPEN CHANNEL 4.X.1

Same as 27.22.4.27.6.4.2 PROACTIVE COMMAND: OPEN CHANNEL 6.5.1 in sequence 6.5

TERMINAL RESPONSE: OPEN CHANNEL 4.X.1

Same as 27.22.4.27.6.4.2 TERMINAL RESPONSE: SET UP EVENT LIST 6.5.1 in sequence 6.5

PROACTIVE COMMAND: SEND DATA 4.X.1

Same as 27.22.4.29.1.4.2 PROACTIVE COMMAND: SEND DATA 1.3.1 in expected sequence 1.3

TERMINAL RESPONSE: SEND DATA 4.X.1

Same as 27.22.4.29.1.4.2 TERMINAL RESPONSE: SEND DATA 1.3.1 in expected sequence 1.3

ENVELOPE: EVENT DOWNLOAD - Data available 4.X.1

Same as 27.22.4.29.1.4.2 ENVELOPE: EVENT DOWNLOAD - Data available 1.7.1 in expected sequence 1.7

PROACTIVE COMMAND: RECEIVE DATA 4.X.1

Same as 27.22.4.29.1.4.2 PROACTIVE COMMAND: RECEIVE DATA 1.7.1 in expected sequence 1.7

TERMINAL RESPONSE: RECEIVE DATA 4.X.1

Same as 27.22.4.29.1.4.2 TERMINAL RESPONSE: RECEIVE DATA 1.7.1 in expected sequence 1.7

27.22.4.30.4.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.1 to 4.X.

****************************end of changes ********************************