3GPP TSG-CT WG6 Meeting #112-e	C6-220466

# E-Meeting, 23rd – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0652 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add a Expected sequence-Trigger LAUNCH BROWSER during a call, as caller party |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2022-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | LAUNCH BROSWER can be used in many ways to provide services for users, for example, LAUNCH BROSWER may be used during a call to to provide helpful information for customer service, or triggered by CALL CONTROL or MT CALL event to provide more diverse USAT services.To support the above scenarios, some test cases about LAUNCH BROSWER are need for MEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new test case of Trigger LAUNCH BROWSER during a call, as caller party. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current test will not include test procedure for this aspect of LAUNCH BROWSER. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.4.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************************* Start of change ****************************************

3.4 Applicability table

| Item | Description | Tested feature defined in Release | Test sequence(s) | R99 ME | Rel-4 ME | Rel-5 ME | … | Rel-15-ME | Rel-16 ME | Terminal Profile | Network Dependency | Support | Additional test case execution recommendation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … | … | … | … | … |  | … | … | … |
| 30 | LAUNCH BROWSER27.22.4.26 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | … | … | … | … | … | … | … | … | … |  | … | … | … |
|  | Trigger LAUNCH BROWSER during a call, as caller party | Rel-16 | 8.X |  |  |  |  |  | C231 | E.1/71 AND E.1/98 AND E.1/110 AND E.1/111 | NG-RAN |  |  |

*******************************Next of change ****************************************

27.22.4.26.8	LAUNCH BROWSER (NG-RAN bearer)

27.22.4.26.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.8.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15 and clause 8.31.

27.22.4.26.8.3	Test purpose

To verify that when the ME is in idle state, it launches properly the browser session required in LAUNCH BROWSER, and returns a successful result in the TERMINAL RESPONSE command.

27.22.4.26.8.4	Method of test

27.22.4.26.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and the NG-SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default NG-RAN UICC, the default NG-RAN parameters and the following URSP rules stored in the ME are used:

URSP:

Rule Precedence =1

Traffic Descriptor:

- DNN=TestGp.rs

Route Selection Descriptor:

- Precedence=1

- Network Slice Selection, S-NSSAI: 01 01 01 02 (ST: MBB, SD: 010102)

- SSC Mode Selection: SSC Mode 2

- Access Type preference: 3GPP access

Rule Precedence = <lowest priority>

Traffic Descriptor: *

Route Selection Descriptor:

- Precedence =1

- Network Slice Selection, S-NSSAI: 01 01 01 01 (ST: MBB, SD: 010101)

- SSC Mode Selection: SSC Mode 3

- DNN Selection: internet

The Allowed S-NSSAI list is configured in NG-SS as '01 01 01 01', '01 01 01 02' and '01 01 01 03'.

The browser's cache shall have been cleared before execution of the test sequence.

27.22.4.26.8.4.2	Procedure

Expected Sequence 8.1 (LAUNCH BROWSER, only NG-RAN bearer specified and gateway proxy identity)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode, NG-RAN supported by the ME and activated, the terminal might need to be configured with an entry linking the Gateway/Proxy Identity in the proactive command with the corresponding connectivity parameters in the mobile. The browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 8.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 8.1.1 | [connect to the defined URL, "launch browser, if not already launched, 1 bearer specified, gateway/proxy id specified] |
| 4 | ME USER | ME may display a default message |  |
| 5 | USER ME | The user may confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 8.1.1 | [Command performed successfully] |
| 7 | ME NG-SS | The ME attempts to connect the URL specified in LAUNCH BROWSER command using the requested bearer and proxy identity | [The USS shall handle the request of additional URLs as defined in the initial conditions clause] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the browser session is properly established with the required bearer. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 8.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Bearer	NG-RAN

Gateway/Proxy id	DCSunpacked, 8 bits data

Text string	abc.def.ghi.jkl (different from the default IP address)

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 32 | 01 | 03 | 0D | 10 |
|  | 04 | 61 | 62 | 63 | 2E | 64 | 65 | 66 | 2E | 67 | 68 | 69 |
|  | 2E | 6A | 6B | 6C |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 8.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 8.X(LAUNCH BROWSER, Trigger LAUNCH BROWSER during a call, as caller party )

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode, NG-RAN supported by the ME and activated, the terminal might need to be configured with an entry linking the Gateway/Proxy Identity in the proactive command with the corresponding connectivity parameters in the mobile. The browser's cache shall have been cleared.] |
| 1 | User  ME | Set up a call to "+01234567890123456789" | Call need to be connected and held |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: SMS-PP DOWNLOAD 8.X.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 8.X.1 |  |
|  | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 8.X.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 8.X.1 | [connect to the defined URL, "launch browser, if not already launched, 1 bearer specified, gateway/proxy id specified] |
| 7 | ME USER | ME may display a default message |  |
| 8 | USER ME | The user may confirm the launch browser. | [option: user confirmation] |
| 9 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 8.X.1 | [Command performed successfully] |
| 10 | ME NG-SS | The ME attempts to connect the URL specified in LAUNCH BROWSER command using the requested bearer and proxy identity | [The NG-SS shall handle the request of additional URLs as defined in the initial conditions clause] |
| 11 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 12 | USER ME | The user verifies that the browser session is properly established with the required bearer. |  |

ENVELOPE: SMS-PP DOWNLOAD  8.X.1

Logically:

Device identities

Source device:	Network

Destination device:	UICC

Address

TON: International number

NPI:	 "ISDN / telephone numbering plan"

Dialling number string:  "+01234567890123456789"

SMS TPDU

TP-MTI:	SMS-DELIVER

TP-MMS:	 No more messages waiting for the MS in this SC

TP-RP:  TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI: TP-UD field contains only the short message

TP-SRI:	A status report will not be returned to the SME

TP-OA

TON:  International number

NPI:	 "ISDN / telephone numbering plan"

Address value	"1234"

TP-PID : (U)SIM Data download

TP-DCS:

Coding Group: Data coding/message class

Message Code: 8-bit data

Message Class: Class 2 (U)SIM Specific Message

TP-SCTS: 01/01/98 00:00:00 +0

TP-UDL:08

TP-UD: 3132333435363738

| BER-TLV: | D1 | 23 | 82 | 02 | 83 | 81 | 06 | 04 | 02 | 91 | 00 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 22 | 0B | 17 | 04 | 04 | 91 | 21 | 43 | 7F | F6 | 89 | 10 |
|  | 10 | 00 | 00 | 00 | 00 | 08 | 31 | 32 | 33 | 34 | 35 | 36 |
|  | 37 | 38 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 8.X.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Bearer	NG-RAN

Gateway/Proxy id	DCSunpacked, 8 bits data

Text string	abc.def.ghi.jkl (different from the default IP address)

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 32 | 01 | 03 | 0D | 10 |
|  | 04 | 61 | 62 | 63 | 2E | 64 | 65 | 66 | 2E | 67 | 68 | 69 |
|  | 2E | 6A | 6B | 6C |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 8.X.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.1 to 8.X.

*******************************End of change ****************************************