3GPP TSG-CT WG6 Meeting #114	C6-230075

# Athens, Greece, 28th Feb – 03rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0982 | rev |  | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of file identifier for EF5G_PROSE_UIR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | IDEMIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Within the work on 5G_ProSe new EFs have been introduced. the EF description for EF5G_PROSE_UIR has an incorrect identifier as well as an incorrect short file identifier. In other parts of the specification the correct file identifer is used. Also the SFI in Annex H is correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of file identifier and short file identifier in clause 4.4.13.7 . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong file identifier is confusing and may lead to incompatibility and wrong implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.13.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.4.13.7	EF5G_PROSE_UIR (5G ProSe configuration data for usage information reporting)

If service n°139 is "available" in the USIM Service Table and service n°5 is "available" in EF5G_PROSE_ST, this file shall be present. This EF contains 5G ProSe policy for usage information reporting. The format of the 5G ProSe policy for usage information reporting are specified in 3GPP TS 24.555 [115].

| Identifier: '4F06' |  | Structure: Transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '06' |  |  |  |  |  |  |
| File size: X bytes bytes, (X ≥ 32) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | 5G ProSe configuration data for usage information reporting TLV objects |  |  | M |  | X bytes |

The 5G ProSe configuration data for usage information reporting data object parameters tags:

| Description | Tag Value |
| --- | --- |
| 5G ProSe configuration data for UE-to-network relay UE Tag | 'A0' |
| Validity timer Tag | '85' |
| Collection period Tag | '94' |
| Reporting window Tag | '95' |
| Reporting indicators Tag | '96' |
| 5G DDNMF CTF address for uploading the usage information reports Tag | '97' |

The 5G ProSe configuration data for usage information reporting contents:

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| 5G ProSe configuration data for usage information reporting Tag | 'A0' | M | 1 |
| Length | Note 1 | M | Note 2 |
| Validity timer Tag | '85' | M | 1 |
| Length | 5 | M | Note 2 |
| Validity timer information | -- | M | 5 |
| Collection period Tag | '94' | M | 1 |
| Length | 3 | M | Note 2 |
| Collection period information | -- | M | 3 |
| Reporting window Tag | '95' | M | 1 |
| Length | 3 | M | Note 2 |
| Reporting window information | -- | M | 3 |
| Reporting indicators Tag | '96' | M | 1 |
| Length | 2 | M | Note 2 |
| Reporting indicators information | -- | M | 2 |
| 5G DDNMF CTF address for uploading the usage information reports Tag | '97' | M | 1 |
| Length | X1 | M | Note 2 |
| 5G DDNMF CTF address for uploading the usage information reports information | -- | M | X1 |
| Note 1: This is the total size of the constructed TLV object. Note 2: The length is coded according to ISO/IEC 8825-1 [35]. |  |  |  |

-	Validity timer Tag '85'

Contents:

The Validity timer information contains the timer for controlling the validity of 5G ProSe configuration data for usage information reporting.

Coding:

The Validity timer information is encoded as shown in figure 5.7.2.1 and table 5.7.2.1 of 3GPP TS 24.555 [115].

-	Collection period Tag '94'

Contents:

The Collection period information contains the time interval, in unit of minutes, at which the UE shall generate the usage information reports.

Coding:

The Collection period information is encoded as shown in figures 5.7.2.1 and tables 5.7.2.1 of 3GPP TS 24.555 [115].

-	Reporting window Tag '95'

Contents:

The Reporting window information contains the time window, in units of minutes, during which the UE shall upload the usage information report.

Coding:

The Reporting window information is encoded as shown in figures 5.7.2.1 and tables 5.7.2.1 of 3GPP TS 24.555 [115].

-	Reporting indicators Tag '96'

Contents:

The Reporting indicators information contains the status of several reporting indicators.

Coding:

The reporting indicator is set as follows:
bit = 1: Set indication active
bit = 0: Set indication inactive.

Byte 1:

|  |  | b8 |  | b7 |  | B6 |  | b5 |  | b4 |  | b3 |  |  |  | b2 |  | b1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Data received reporting indicator, as defined in tables 5.7.2.1 of 3GPP TS 24.555 [115]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Data transmitted reporting indicator, as defined in tables 5.7.2.1 of 3GPP TS 24.555 [115]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Time stamps of the first transmission/reception reporting indicator, as defined in tables 5.7.2.1 of 3GPP TS 24.555 [115]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Time stamps in and out of NG-RAN coverage reporting indicator, as defined in tables 5.7.2.1 of 3GPP TS 24.555 [115]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Group parameters reporting indicator, as defined in tables 5.7.2.1 of 3GPP TS 24.555 [115]. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | UE locations reporting indicator, as defined in tables 5.7.2.1 of 3GPP TS 24.555 [115]. |

Byte 2:

|  |  | B8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  |  | b1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Address type, as defined in tables 5.7.2.1 of 3GPP TS 24.555 [115]. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | QoS flow reporting indicator, as defined in tables 5.7.2.1 of 3GPP TS 24.555 [115]. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Radio parameters reporting indicator, as defined in tables 5.7.2.1 of 3GPP TS 24.555 [115]. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU |  |

-	5G DDNMF CTF address for uploading the usage information reports Tag '97'

Contents:

The 5G DDNMF CTF  address for uploading the usage information reports information contains a list of ProSe identifier to ProSe application server address mapping rules.

Coding:

The 5G DDNMF CTF address for uploading the usage information reports information is encoded as shown in figures 5.7.2.1 and tables 5.7.2.1 of 3GPP TS 24.555 [115].