3GPP TSG-CT WG6 Meeting #123	C6-250452

# Göteborg, Sweden; 26th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.213 | CR | 0036 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for the getCipherAlgorithm method in GBAUSignatureClass of the GBA_U API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile,THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEST_GBA_U_APIs |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During CT#100 was approved the new WID GBA_U Based APIs (UID 1000003) (CP-231107). During CT6#119 meeting was agreed the CR0102r2 on 3GPP TS 31.130 (TDoc: C6-240313) defining the new GBA_U API, uicc.usim.gba_u package. New test cases shall be added to cover the GBA_API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new Test cases for the getCipherAlgorithm method in GBAUSignatureClass of the GBA_U API |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case covering the GBA_U API (uicc.usim.gba_u package) from 3GPP TS 31.130. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X (new) and sub-clauses |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | was C6-250229 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.4.X	Class GBAUSignature

5.4.X.2	Method 	getCipherAlgorithm()

Test Area Reference: Api_3_Gsi_Gcal

5.4.X.2.1	Conformance requirements

The method with following header shall be compliant to its definition in the API.

public abstract byte getCipherAlgorithm()

5.4.X.2.1.1	Normal execution

-	CRRN1: The method shall return the cipher algorithm code defined in the javacard.security.Signature class.

5.4.X.2.1.2	Parameter Errors

No requirements.

5.4.X.2.1.3	Context errors

No requirements.

5.4.X.2.2	Test area files

Test Source:	Test_Api_3_Gsi_Gcal.java

Test Applet:	Api_3_Gsi_Gcal.java

Cap File:	Api_3_Gsi_Gcal.cap

5.4.X.2.3	Test coverage

| CRR number | Test case number |
| --- | --- |
| N1 | 1,2 |

5.4.X.2.4	Test procedure

| Id | Description | API Expectation | APDU Expectation |
| --- | --- | --- | --- |
|  | ADF USIM exist and its AID is defined in record#1 of EFDIR in MF, is value is “ADF_AID“). ADF USIM content the EFAC_GBAUAPI file as defined in annex B.2.25 Internal applet buffer adfAID1 is set to ADF_AID from EFDIR. Internal applet buffer adfAID2 is set to ADF_AID2 (ADF_ADI2 is not equal to ADF_AID). Internal applet buffer nafID1 is set to allowed NAF_ID from EFAC_GBAUAPI file as defined in annex B.2.25. Internal applet buffer nafID2 is set to not allowed NAF_ID (NAF_ID is not equal to nafID1). | N/A |  |
| 1 | - Set GBAUSignature1 object to returned getInstance(). Parameters are set to: - messageDigestAlgorithm= javacard.security.MessageDigest.ALG_SM3 - cipherAlgorithm = javacard.security.Signature.SIG_CIPHER_HMAC - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NULL - externalAccess = false - Call GBAUSignature1.getAlgorithm() | GBAUSignature1 is a GBAUSignature instance. The cipherAlgorithm code is SIG_CIPHER_HMAC |  |
| 2 | - Set GBAUSignature1 object to returned getInstance(). Parameters are set to: - messageDigestAlgorithm= javacard.security.MessageDigest.ALG_NULL - cipherAlgorithm = javacard.security.Signature.SIG_CIPHER_AES_MAC128 - paddingAlgorithm = javacardx.crypto.Cipher.PAD_NOPAD - externalAccess = false - Call GBAUSignature1.getAlgorithm() | GBAUSignature1 is a GBAUSignature instance. The cipherAlgorithm code is SIG_CIPHER_AES_MAC128 |  |

* * * End of Changes * * * *