3GPP TSG-CT6 Meeting #91	C6-180697

# West Palm Beach, Florida, United States, 27th November – 30th November 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.102 | CR | 0829 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolution of Editor's Note for SUCI Calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Giesecke+Devrient MS GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2018-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | New CRs in 3GPP TS 23.003 and 3GPP TS 24.501 clarify the format of the SUCI. There are some Editor’s Note that were relying on the progress on the above mentioned specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolve the Editor’s Notes wherever it applies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Format of the SUCI is not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.11.8, 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 24.501 CR 0615 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

### 4.4.11.8	EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

If "SUCI calculation is to be performed by the ME" (i.e. service n°124 is "available" in EFUST and service n°125 is not "available" in EFUST), this file shall be present.

If "SUCI calculation is to be performed by the USIM" (i.e. service n°124 is "available" in EFUST and service n°125 is "available" in EFUST), this file shall not be available to the ME.

If service n°124 is not "available" in EFUST, this file shall not be available to the ME. This EF contains information needed by the ME for the support of subscription identifier privacy as defined in 3GPP TS 33.501[105].

| Identifier: '4F07' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '07' |  |  |  |  |  |  |
| File size: X bytes (X ≥ 2) |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to Z | Protection Scheme Identifier List data object |  |  | M |  | Z bytes |
| Z+1 to Y+Z | Home Network Public Key List data object |  |  | C |  | Y bytes |
| X-W+1 to X | Routing Information TLV data object |  |  | C |  | W bytes |

-	Protection Scheme Identifier List data object.

Contents:

This data object shall always be present. If Protection Scheme Identifier List data object length is not zero, this data object contains a list of the Protection Scheme Identifier and the corresponding Key Index. The first Protection Scheme Identifier entry has the highest priority and the last Protection Scheme Identifier entry has the lowest priority. The Key Index value indicates the position of the Home Network Public Key in the Home Network Public Key List, that is applicable to the Protection Scheme.

Coding:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| Protection Scheme Identifier List data object tag | 'A0' | M | 1 |
| Protection Scheme Identifier List data object length | L1 | M | Note 1 |
| Protection Scheme Identifier 1 (Highest priority) | -- | O | 1 |
| Key Index 1 | -- | C (Note 2) | 1 |
| Protection Scheme Identifier 2 | -- | O | 1 |
| Key Index 2 | -- | C (Note 2) | 1 |
| … | … | … | … |
| Protection Scheme Identifier N (Lowest priority) | -- | O | 1 |
| Key Index N | -- | C (Note 2) | 1 |
| Note 1: The length is coded according to ISO/IEC 8825-1 [35] Note 2: This field is present only if the corresponding Protection Scheme Identifier field is present |  |  |  |

The Protection Scheme Identifier represents a protection scheme as described in 3GPP TS 33.501 [105] and it is coded in one byte as follows:

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Protection Scheme identifier coded as described in 3GPP TS 24.501 [104] |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU, bit = 0 |

The Key Index is coded in one byte such that its value indicates the position of the Home Network Public Key in the Home Network Public Key List data object, that is applicable to the Protection Scheme. A Key Index with a value of "1" refers to the first Network Public Key entry in the Home Network Public Key List, and so on. A Key Index with a value of "0" indicates that there is no Home Network Public Key associated with that Protection Scheme (e.g., in the case of null-scheme).

-	Home Network Public Key List data object.

Contents:

This data object contains a list of the Home Network Public Key and the corresponding Home Network Public Key Identifier that shall be used by the ME to calculate the SUCI.

This data object may not be present if none of the protection scheme profiles identified by the Protection Scheme Identifiers included in the Protection Scheme Identifier List data object use the Home Network Public Key (e.g. null-scheme). If this data object is present, it shall contain at least one Home Network Public Key and the corresponding Home Network Public Key Identifier.

Coding:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| Home Network Public Key List data object | 'A1' | C | 1 |
| Home Network Public Key List data object length | L1 | C | Note 1 |
| Home Network Public Key 1 Identifier tag | '80' | C | 1 |
| Home Network Public Key 1 Identifier length | L2 | C | Note 1 |
| Home Network Public Key 1 Identifier | -- | C | L2 |
| Home Network Public Key 1 tag | '81' | C | 1 |
| Home Network Public Key 1 length | L3 | C | Note 1 |
| Home Network Public Key 1 | -- | C | L3 |
| … | … | … | … |
| Home Network Public Key N Identifier tag | '80' | O | 1 |
| Home Network Public Key N Identifier length | L4 | O | Note 1 |
| Home Network Public Key N Identifier | -- | O | L4 |
| Home Network Public Key N tag | '81' | O | 1 |
| Home Network Public Key N length | L5 | O | Note 1 |
| Home Network Public Key N | -- | O | L5 |
| Note 1: The length is coded according to ISO/IEC 8825-1 [35] |  |  |  |

The Home Network Public Key Identifier may have any value in the range from 0 to 255 as described in 3GPP TS 23.003 [25] and it is coded in one byte as described in 3GPP TS 24.501 [104].

The length of the Home Network Public Key depends on the Protection Scheme used and it is coded in hexadecimal digits.

-	Routing Information TLV data object.

Contents:

This data object contains Routing Indicator that allows together with the MCC and MNC to route network signalling with SUCI to AUSF and UDM instances capable to serve the subscriber, as specified in 3GPP TS 23.003 [25]. This data object may not be present in the case of null-scheme. If this data object is present, it shall have a valid Routing Indicator.

Coding:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| Routing Information TLV data object tag | 'A2' | C | 1 |
| Routing Information TLV data object length | 2 | C | Note 1 |
| Routing Information TLV data object value | -- | C | 2 |
| Note 1: The length is coded according to ISO/IEC 8825-1 [35] |  |  |  |

The Routing Indicator is coded in 2 bytes as described in 3GPP TS 24.501 [104].

***** Next change *****

## 7.5	GET IDENTITY

### 7.5.1	Command description

The function can be used in the following contexts:

-	a SUCI context, to retrieve the SUCI when "SUCI calculation is to be performed by the USIM".

The function is related to a particular USIM and shall not be executable unless the USIM application has been selected and activated, and the current directory is the USIM ADF or any subdirectory under this ADF and a successful PIN verification procedure has been performed (see clause 5).

If GET IDENTITY command is not supported by the UICC, then the status word '6D00' (Instruction code not supported or invalid) shall be returned.

#### 7.5.1.1	SUCI context

SUCI context shall be supported if "SUCI calculation is to be performed by the USIM" (i.e. Service n°124 and Service n°125 are "available").

The command returns the SUCI which is a privacy preserving identifier containing the concealed SUPI. The function is used in 5GS in the specific cases described in 3GPP TS 33.501 [105] prior to mutual authentication between the UE and the SN.

The SUCI returned is calculated as described in 3GPP TS 33.501 [105].

For the execution of the command, the following information shall be available in the USIM:

- Home network identifier (i.e. MCC and MNC) (see Note).

- Routing indicator (see Note).

- Home network public key (see Note).

- Home network public key identifier (see Note).

- Protection scheme identifier (see Note).

- SUPI.

NOTE:	Provision and storage of the information in the USIM when "SUCI calculation is to be performed by the USIM" (i.e. Service n°124 and Service n°125 are "available") is out of the scope of the specification.

Editor’s Note: It is FFS to specify if SUPI Type shall be available in the USIM

The SUCI is designed for one-time use, however, the freshness and randomness of SUCI returned upon each call of the command depends on the protection scheme configured. There is the special case where the protection scheme used is null-scheme, in such case SUCI contains the non concealed SUPI.

If the home network public key is not provisioned in the USIM, the SUCI shall be calculated using the null-scheme irrespective of the protection scheme stored in the USIM.

The returned SUCI consists of the concatenation of the following information as described in 3GPP TS 23.003 [25]:

-	SUPI Type

-	Home network identifier (i.e. MCC and MNC when SUPI Type is an IMSI).

-	Routing indicator.

-	Protection scheme identifier.

-	Home network public key identifier.

-

Scheme output, resulting from the protection scheme profile, identified by the protection scheme identifier. The protection scheme profile shall be one of those defined in Annex C of 3GPP TS 33.501 [105] or one of those specified by the Home network.

If SUCI context is supported and:

-	Service n°124 is not "available" or:

-	"SUCI calculation is to be performed by the ME” (i.e. Service n°124 is "available", and Service n°125 is not "available")

the status word '6985' (Conditions of use not satisfied) shall be returned

Editor’s Note: It is FFS to specify the behavior in case other parameters (e.g. Home network public key identifier, some necessary data is not provisioned) are not correctly configured

### 7.5.2	Command parameters and data

| Code | Value |
| --- | --- |
| CLA | As specified in TS 31.101 [11] |
| INS | '78' |
| P1 | '00' |
| P2 | Identity context, see Table X below' |
| Lc | Length of the subsequent data field or not present, see below |
| Data | See below |
| Le | '00', or maximum length of data expected in response |

Parameter P2 specifies the identity context as follows:

# Table 8: Coding of the reference control P2

| b8 | b7 | b6 | b5 | b4 | b3 | b2 | b1 | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - | X | X | X | X | X | X | X | Identity Context (See below) |
| - | 0 | 0 | 0 | 0 | 0 | 0 | 1 | SUCI |

All other codings are RFU.

#### 7.5.2.1	SUCI context

Command parameters/data: None

Response parameters/data:

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 to Le | SUCI TLV data object | Le |

Subscription Concealed Identifier TLV data object:

| Description | Value | M/O/C | Length (bytes) |
| --- | --- | --- | --- |
| SUCI TLV data object tag | 'A1' | M | 1 |
| Length | X | M | Note |
| SUCI value | -- | M | X |
| NOTE: The length is coded according to ISO/IEC 8825-1 [35] |  |  |  |

-  SUCI

It contains the SUCI as defined in 3GPP TS 33.501 [105].

When SUPI Type is IMSI, the SUCI is coded as the SUCI part of 5GS mobile identity information element defined in 3GPP TS 24.501 [104]. The correspondence between the SUCI value and the octets of the 5GS mobile identity information element is provided below:

Byte 1 corresponds to "octet 4":

|  |  | b8 |  | b7 |  | b6 |  | b5 |  | b4 |  | b3 |  | b2 |  | b1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | RFU (see 3GPP TS 24.501 [104]) |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |

From byte 2 to 4, the Home Network Identifier (i.e. MCC and MNC) is coded and corresponds from "octet 5" to "octet 7".

Byte 5 and 6 code the Routing Indicator which correspond to "octet 8" and "octet 9".

Byte 7 codes the Protection Scheme Identifier which corresponds to "octet 10".

Byte 8 codes the Home Network Public Key Identifier which corresponds to "octet 11".

Byte 9 corresponds to "octet 12". From Byte 9 onwards, the Scheme Output is coded and the length depends on the Protection Scheme used.

Editor’s Note: SUCI with SUPI format NAI coding is to be completed in 3GPP TS 24.501 [104].