NB M2M - Cell Selection and Reselection

Introduction

Cell selection and reselection is the basic function to allow devices successfully camping on a suitable cell for subsequent data transmission, and it is agreed that only UE autonomous cell (re)selection is required for Cellular IoT. This contribution proposes specific cell selection and reselection mechanism for NB M2M solution.

This paper is an update of GP-140848 presented at GERAN#64. Major updates have been highlighted in red colour.

Cell selection

Initial cell selection

The cell selection starts when UE is power on, or cannot find any suitable cells by cell reselection procedures.

Firstly the UE shall scan all carrier frequencies in the NB-M2M bands to find a suitable cell. Unlike legacy GRPS, the frequency reuse-factor is 1 in NB M2M solution at this stage. The UE needs do cell search procedure [1] in one carrier frequency to find out all the cells in the frequency and then measures SSRP (Synchronization Signal Received Power) [2] on PBSCH of each cell to find the strongest cell in the frequency.

Secondly the UE reads the system information of the strongest cell in the frequency. If the cell is a suitable cell (i.e. the S criteria can be satisfied, the PLMN or Tracking Area/Routing Area allows the UE to access, and the cell is not barred), the UE will try to camp on that cell. If UE can camp on that cell successfully, UE will stop search the next frequency to speed up the cell selection period. Otherwise, the UE needs to search for the strongest cell on the next frequency in order of frequency occurrence, and read the necessary system information of the cell.

Stored information cell selection

In case that the UE has already configured or recorded information of cells (e.g. frequency and/or Physical Cell ID), the UE is not necessary to start from initial cell selection and can make cell selection by these stored information.

When Physical Cell ID is stored, if UE finds a suitable cell among these stored cells the UE can camp on that cell to speed up the cell selection procedure. Else the UE does cell search and cell measurement for each of stored frequencies. If no suitable cell is found according to the stored information, the initial cell selection procedure has to be started.

Stored information cell selection is mainly used in the following cases:

- Connected mode: there is still unsuccessful UL/DL data after max retransmission has reached;

- Connected mode: DCI decoding failure on the worst coverage class for several times;

- RACH access failure on the worst coverage class for several times.

It is worth to be mentioned that in the later two cases, UE will firstly trigger coverage class adaptation according to the principle described in [3] if UE is not on the worst coverage class.

Criteria for cell selection

The criteria for cell selection (criteria S) are defined as below:

Srxlev>0                                (1)

Where:

Srxlev=Qrxlevmeas-Qrxlevmin                      (2)

The parameters in formula 1, 2 are defined in Table 1.

Table 1. Cell Selection parameters

| Parameter | Description |
| --- | --- |
| Srxlev | Cell selection RX level value |
| Qrxlevmeas | Measured cell RX level value (SSRP) |
| Qrxlevmin | Minimum required RX level in the cell which is broadcast in system information [4]. This is to differ priorities of different cells |

Cell reselection

After camping on a cell, the UE might search for another better cell triggered by some specific events. If a better cell is found according to the criteria, that cell is reselected for better services.

If some abnormal cases happen when UE in idle mode, cell reselection will be used.

Neighbour cell list for Cell Reselection

To perform cell reselection, the UE needs to get neighbour cell information by reading the system information, which is defined in [4]. Based on this information, the UE will take measurements for the neighbour cells.

Measurements for Cell Reselection

The measurements will include both serving and neighbour cells.

Measurement on serving cell

For the measurements of the serving cell, it is worth mentioning different UE behaviours for different situation.

- PSM applied

If PSM is used, the measurements for serving cell will be implemented when the UE wakes up. This is mainly because the sleeping period would last very long and during this period the UE might already move to different cells, therefore the measurements are necessary in this case.

- Normal DRX/paging applied

In this case the measurements would be done for each paging cycle as legacy mechanism.

Measurement on neighbour cells

The neighbour cell measurements also are not necessary to be periodic to save power consumption from UE side. Instead, the UE can initiate the neighbour cell measurements when any one of the following conditions is fulfilled:

- the serving cell fulfils Srxlev < SSearch;

- DCI decoding failure on the worst coverage class for several times;

- System information decoding failure for several times.

After measurements of neighbour cells, the UE performs ranking of all cells which are satisfy S criteria according to the R criteria specified in Sec 3.3. UE reads the necessary system information of the cell which is ranked as the best cell. If the cell is suitable for camping on and the following reselection conditions are met, the UE shall reselect to this cell:

- the new cell is better ranked than the serving cell during a time interval Treselection (during this period UE needs measure serving cell and neighbor cells);

- the UE has maintained camping on the current serving cell for a certain period;

If the best cell is not a suitable cell, the UE can reselect the second best cell.

Table 2. Parameters for measurement on neighbour cells

| SSearch | This specifies the Srxlev threshold (in dB) for neighbour cell measurements broadcast in system information [4]. |
| --- | --- |
| Treselection | This specifies the cell reselection timer value broadcast in system information [4]. |

Criteria for cell reselection

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs=Qmeas,s+QHyst                          (3)

Rn=Qmeas,n+Qoffset                         (4)

The parameters in formula 3, 4 and other relevant cell reselection parameters are defined in Table 3. It should be noted that the parameters QHyst, Qoffset, SSearch, Treselection are obtained from the system information [2].

Table 3. Parameters for criteria

| Parameter | Description |
| --- | --- |
| Qmeas,s | SSRP measurement quantity for serving cell used in cell reselections. |
| Qmeas,n | SSRP measurement quantity for neighbouring cells used in cell reselections. |
| QHyst | This specifies the hysteresis value for ranking criteria in cell reselection, broadcast in system information [4]. |
| Qoffset | A cell or frequency specific offset to be applied when evaluating candidates for cell re-selection, broadcast in system information [4]. |

Overview the flow of Cell Selection and Reselection

The general procedure of cell selection and reselection can be shown in Figure 1.

Figure 1. Process of cell selection and reselection

Conclusion

In this contribution, the cell selection and reselection for NB M2M has been introduced. For NB M2M, the measurement for cell reselection are triggered by specific events and it is believed to be benefit for power saving. It is proposed to adopt the principle and criteria for cell selection and reselection above for NB M2M in the TR.

References

  - GP-140864, “NB M2M - Cell Search Mechanism”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN#64.

  - GPC150035, “NB M2M - Overview of Measurement design”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN Adhoc#1 on FS_IoT_LC.

  - GPC150037, “NB M2M - Overview of Coverage Class”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN Adhoc#1 on FS_IoT_LC.

  - GPC150046, “NB M2M - System Information Broadcasting for Cellular IoT”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN Adhoc#1 on FS_IoT_LC.