# EC-GSM – Exception Report Latency

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things  [1] (WI code: FS_IoT_LC)  was approved.

One of the main objectives is to increase the coverage compared to existing GPRS services.

In this contribution, calculations of the exception report latency according to the agreed framework [2] at normal and 20 dB extended coverage are presented.

# Calculations

## Latency calculation assumptions

Calculations have been performed according to the agreed framework for latency analysis where:

- The latency is measured from when a device first decides to send a report (UL application layer payload) and includes the time to synchronize to the network, the time to perform an access attempt and the time for a BSS to successfully detect reception of the UL application layer payload at the base station [2] i.e. this last point in time occurs at the point where the BSS determines that all UL RLC data blocks have been received.

- The worst case uplink payload being  (without header compression)

  - 95 bytes = 20 bytes (application) +  65 bytes (Header above SNDCP) + 4 bytes (SNDCP) + 6 bytes (LLC)

- The latency should be reported at three different coverage conditions: GPRS reference MCL+0dB, GPRS reference MCL+10 dB, and maximum achievable coverage of MCL+20dB.

Here, only latency evaluations for GPRS reference coverage and 20 dB extended coverage are presented. Furthermore, assuming that data will be transferred using MCS-1 (22 bytes payload) the total number of RLC data blocks to transfer is 5.

Latency is evaluated according to the formula agreed at Cellular IoT telco#8:

Latency for DATA transmission = (1) T Synchronization+ (2) T Transmission +(3) T Receiving +(4)T Wait

- T Synchronization: The time for UE to synchronize to the network. This value depends on the coverage condition.

- T Transmission: The transmitting time for any signaling and data.

- T Receiving: The receiving time for any signaling and data. It should be highlighted that the valid scheduling information receiving such as USF belonging to the UE which is not included in an obvious signaling should be taken into account.

- T Wait: The time between any transmission and receiving, and also the time between two consecutive transmission or receiving. The waiting time is relevant with the scheduling mechanism for a specific solution.

## Latency evaluations

### General

In the context of this paper the network synchronization time it is assumed be the time it takes for a device so synchronize to the same cell (i.e. confirm suitability) as it was previously camped on.  It is more over assumed to include reading both the FCCH and SCH channel.

Furthermore, as discussed in [2], GSM Evolution will use the concept of fixed uplink allocation (FUA) of radio blocks on UL PDTCH resources in the interest of avoiding USF based uplink transmissions for devices operating in extended coverage.

### GPRS reference case

For a device in normal GPRS reference coverage the evaluation of average network synchronization is if fairly straight forward as the FCCH and SCH channels are mapped in pairs occurring every 10th TDMA frame on TS0.  This implies that the average acquisition time of the FCCH and SCH channels are 5.5 TDMA frames or approximately 26 ms (i.e. 5.5 TDMA frames).

The sequence of signaling events for a device at GPRS reference coverage using FUAis shown in Figure 1, where, in order not to underestimate the total latency, it has been assumed that one re-transmission of an RLC data block is needed.

### Figure 1, Illustration of signaling events for transmission of data using fixed allocation.

Furthermore, it is assumed that the time an MS needs to wait for before sending a channel Request  tMS-channel req. is equal to 1 TDMA frame and that the average response time in the BSS before transmitting an Immediate Assignment message tBSS-Imm. Assign. is 80 ms. The reaction times in the BSS and MS during packet transfer, tMS and tBSS, are assumed to be equal to one data transmission TTI or 20 ms.

Using the above assumptions the total latency for transmitting data can be evaluated. The latency is calculated in Table 1 .

Table 1. Latency calculation for GPRS reference case with fixed allocation

|  |  | Number of occurrences | Time per occurrence (ms) | Total time (ms) |
| --- | --- | --- | --- | --- |
| 1 | Network synchronization | 1 | 26 | 26 |
| 2 | tMS-Channel Req. | 1 | 4.651 | 4.651 |
| 3 | Channel request | 1 | 0.577 | 0.577 |
| 4 | tBSS-Imm. Assign. | 1 | 80 | 80 |
| 5 | Immediate Assignment | 1 | 20 | 20 |
| 6 | tMS | 1 | 20 | 20 |
| 7 | Transmission of data blocks | 5 | 20 | 100 |
| 8 | tBSS | 1 | 20 | 20 |
| 9 | PUAN | 1 | 20 | 20 |
| 10 | tMS | 1 | 20 | 20 |
| 11 | Re-transmission of 1 data block | 1 | 20 | 20 |
|  | Total time |  |  | 331 ms |

### GPRS reference case + 20 dB

For a device at 20 dB extended coverage the time average time to to read the FCCH [5]  is 530 ms and the time to read the SCH [6] is 58 TDMA frames (i.e. 268 ms) implying a total average network synchronization time of ca 0.8 seconds.

The sequence of signaling events at 20 dB extended coverage assuming a total of 4 HARQ transmissions for successful transfer of data[7], where each of the uplink RLC Data blocks are transmitted with 16 blind repetitions, are illustrated in Figure 2. In order to calculate the latency the average number of transmitted radio blocks (excluding blind transmissions) needs to be evaluated. Assume first that there are N blocks to transmit and that the residual BLER after the 1st, 2nd and 3rd transmission are  denoted rBLER 1_Tranmisson, rBLER 2_Tranmisson and rBLER 3_Tranmisson then the average total number of transmitted radio blocks NTOT is

NTOT = 	N+N*rBLER 1_Tranmisson+N*rBLER 2_Tranmisson+N*rBLER 3_Tranmisson=

N*(1+ rBLER 1_Tranmisson+rBLER 2_Tranmisson+ rBLER 3_Tranmisson)

From  performance simulations it is found that the term

1+ BLER 1_Tranmisson+* BLER 2_Tranmisson+ BLER 3_Tranmisson+BLER 4_Tranmisson is equal to 1.6*N.

For the latency evaluation of an alarm report the total number of radio block to transmit is N=5 block resulting in a total average of 8 radio block to transmit (excluding blind repetitions).

Figure 2, Illustration of the sequence of signaling events for transmission of data with a total of 4 HARQ transmissions.

Furthermore, the time an MS needs to wait for sending a channel Request,  tMS-channel req.,is equal to 9 TDMA frames [8] (from the last EC-SCH frame to first possible EC-RACH occurrence for a coverage class 6 device. Moreover, the scheduling delay time in the BSS before transmitting an Immediate Assignment message tBSS-Imm. Assign. is on average one 51 multi frame (235 ms). The wait times in the BSS and MS during packet transfer, tMS and tBSS, are assumed to be mainly due to scheduling delay and are assumed to be equal to one data transmission TTI or 80 ms.

The time it takes to complete the transmission of message on each logical channel is taken from “EC-GSM – Mapping of logical channels onto physical channels” paper [8].  For a coverage class 6 device the EC-RACH channel is mapped over 2 multi frame implying an average transmission time of 51+16 or 67 TDMA frames. Similarly, for a coverage class 6 device the EC-AGCH is mapped over 4 multi frames implying a total of 940 ms for the 32 repetitions of the EC-AGCH block. For the EC-PDTCH/U the 16 transmissions are mapped onto 4 TS and 4 TTI (each TTI being 20 ms) implying a total TTI of 80 ms.

Using the above assumptions the total latency for transmitting data at 20 dB extended coverage can be evaluated – the latency is calculated in Table 2

Table 2. Latency calculation for GPRS reference case with fixed allocation

|  |  | Number of occurrences | Time (ms) | Total time (ms) |
| --- | --- | --- | --- | --- |
| 1 | Network synchronization | 1 | 800 | 800 |
| 2 | Wait channel Request (9 TDMA frames) | 1 | 42 | 42 |
| 3 | Channel request (67 TDMA frames) | 1 | 309 | 309 |
| 4 | tBSS-Imm. Assign. | 1 | 235 | 235 |
| 5 | Immediate Assignment | 1 | 940 | 940 |
| 6 | tMS | 4 | 80 | 320 |
| 7 | Transmission of 8 data blocks | 8 | 80 | 640 |
| 8 | tBSS | 3 | 80 | 240 |
| 9 | Transmission of 3 PUAN | 3 | 80 | 240 |
|  | Total time |  |  | 3766 |

# Conclusions

Based on the results provided in this paper it can be seen that the EC-GSM concept can meet the 10 second latency target of delivering an exception report with quite some margin. In fact the results show that approximately 3.8 seconds is required for delivering an exception report at 20 dB extended coverage. The corresponding value at GPRS reference coverage is approximately 331 ms.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GPC150009, 3GPP TR 45.820 V0.3.0 (2015-03), Source Vodafone, GERAN Ad Hoc#1 on FS_IoT_LC

GP-140363, “Short and long sync up procedure”, 	 source Ericsson, TSG GERAN#62

GP-140624, “PDCH UL Resource Management”, source Ericsson, TSG GERAN#63

GPC150066, “EC-GSM, FCCH overview”, Source Ericsson LM, Cellular IoT adhoc#1

GPC150064, “C-GSM, EC-SCH design, performance and mapping”, Source Ericsson LM, Cellular IoT adhoc#1

GPC150056, “EC-GSM – Coverage improvement target”, source Ericsson, Cellular IoT adhoc#1

GPC150055, “EC-GSM – Mapping of logical channels onto physical channels”, source Ericsson, Cellular IoT adhoc#1