C-UNB technology for Cellular IoT

-

Down Link Physical Layer Design

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. Various radio access technology proposals are candidates for this WI. One of the proposals in the Cooperative Ultra Narrow Band (C-UNB), for which key features are:

ultra-narrow band transmission,

random access,

cooperative reception.

Overall description of the C-UNB radio access technology can be found in [2]. The present document details the design of the C-UNB physical layer in downlink (DL). It is an extract of document [3] with more inputs on radio matters.

# DL Physical design

## Bit rate

The downlink bit rate is set to 600bps. This value is low enough to have UNB transmissions. It is higher that the UL transmission rate to get a reduced transmission time of DL radio packets, hence reducing the reception time in devices.

## Modulation

The proposed modulation for C-UNB downlink transmission is D-BPSK. It is chosen because of its robustness against noise and interferers. Differential modulation is implemented with a  phase shift for each bit 0 transmitted. Bit 1 keeps the phase constant.

## Transmission power

The sensitivity of receivers in devices is slightly lower than receivers in base stations, because the reception bandwidth can't be as narrow as what we have in base stations. The bit rate is higher. Given the maximum value of +164dB for the DL MCL, it is proposed to set the maximum transmission power on each ad'hoc micro-channel to +34dBm. Table 1 gives the MCL calculation in DL.

### Micro-channel power management

In practice, actual transmission power in downlink is adjusted according to the strength of the received signal upsteam. This very simple power management can be implemented because every DL transmission is triggered by a UL packet.

If we consider the regular distribution of devices in a cell, the mean distance to the base station is 2/3 of the max distance. This mean value improves the link budget by about 5 dB.

### Maximum PSD in base station

The power spectral density (PSD) of a GSM/GPRS base station is an important limiting factor for C-UNB, because the total available power is split among all the DL micro-channels, which are transmitted simultaneously. The worst case is where all micro-channel transmissions are aimed at devices close to the cell border: each micro channel is set to its maximum transmit power, i.e. +34dBm. Given the PSD of +43dBm in GSM/GPRS base stations, the total number of micro-channels, which can be transmitted simultaneously, is 8.

If we consider the mean distance of devices, the mean transmit power is +29dBm. Hence, the mean number of DL micro-channels is about 25.

The values (i.e. 8 max and 25 mean) are reasonable for carrying the traffic in downstream.

## Center frequency in DL

The center frequency of each DL micro-channel is chosen according to the actual frequencies used by the device during uplink transmission. The precise center frequency of a given micro-channel is not known a priori. It is calculated for each downlink message (see figure 1).

This feature helps reducing the complexity and cost of devices, because no automatic frequency control is needed during reception in devices: the AFC is performed by the base station itself.

## Pulse shaping and spectrum mask

Amplitude of the transmitted signal is reduced during phase transition in order to keep the signal spectrum within tight boundaries, i.e. 1200 Hz. Pulse shaping is illustrated in figure 2.

Figure 3 gives the spectrum mask

## Error correction code

Each MAC-PDU is protected by a Error Correction Code (ECC) of 32 bits. Given the max length of a MAC-PDU of 384 bits, this ECC adds a 1dB gain to the MCL.

Figure 1: Relationship between frequencies of corresponding UL and DL micro-channels

Figure 2: Pulse shaping in DL

Figure 3: Spectrum mask of DL in C-UNB

# Downlink MCL

Table 1 gives the maximum coupling loss (MCL) of the downlink transmission in C-UNB. Calculation is based on methodology given in subclause 5.1 of TR 36.888 [4].

Table 1: Maximum Coupling Loss in C-UNB downlink

| Logical channel name | C-UNB downlink |
| --- | --- |
| Data rate (bps) | 600 |
| Transmitter |  |
| (1) Tx power (dBm) | +34 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 6 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 1000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -138 |
| (7) Required SINR (dB) | 9 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -129 |
| (9) Rx processing gain | 1 |
| (10) MCL = (1) (8) + (9) (dB) | +164 |

# Conclusion

This contribution details the DL physical layer design of the C-UNB technology and shows how this radio access technology can meet requirements from Cellular IoT with very limited complexity in the device modems.

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia

[2]	GPC150052, "Cooperative ultra narrow band technology for Cellular IoT", SIGFOX, Ad'hoc meeting, 02-05 Feb'15, Sophia Antipolis

[3]	GP-150057, "C-UNB technology for Cellular IoT - Physical layer", SIGFOX, GERAN#65, 09-12 March'15, Shanghai

[4]	TR 36.888, "Study on provision of low cost Machine-Type Communication (MTC) User Equipment (UEs) based on LTE (v12.0.0)"