# EC-GSM, Link modeling methodology for EC-PDTCH capacity evaluations

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

One objective is to “Scale to support a massive number of MTC Mobile Stations”, this is evaluated by system level simulations in the study.

One part of the system level evaluations is to use an accurate model of the link level performance.

This paper describes the methodology used for the EC-PDTCH evaluations for EC-GSM.

# Traditional modeling

Traditionally in system level simulations, the link level performance is modeled by one or several mapping tables. In GSM, a common way of mapping performance from instantaneous C/I values to Block Error Probabilities (BLEP) is to use a two-stage mapping.

Figure 1. Traditional mapping methodology for GSM

The first stage maps instantaneous SINR per burst to raw BER. This will consider impairments of different kinds, demodulator performance etc. At least one mapping per modulation is required. For more advanced receivers (that use for example interference suppression) more than one mapping table is required. Four instantaneous C/I values are collected for PS services in GERAN, representing the four bursts of a radio block.

The second stage typically maps the mean and standard deviation of the raw BER values of the different bursts (four bursts in case of a radio block) to a Block Error Rate Probability (BLEP). This is to reflect the impact of the channel coding of the MCS. Typically one mapping is required per MCS.

# Methodology

## Principle

The traditional methodology is followed for the PDTCH capacity evaluations with details as given below:

- The receiver in the UL is a 2-branch Maximum Ratio Combining (MRC) type receiver.

- Only MCS-1-4 is modeled (the low complexity implementation option in EC-GSM)

- Repetitions of MCS-1 is supported for extended coverage

- Both interference and sensitivity limited performance are modeled

## Mapping tables

### First stage mapping (SINR  BER)

The mapping tables used for the first stage mapping are based on single antenna performance. Impairment models, e.g. frequency offset, according to the ongoing study are used in the generation of the results. Since only one modulation type and one demodulator is considered there is no multitude of mapping tables for this reason.

No separate mapping is used for repeated bursts (see how SINR is derived in this case in Section 3.3).

The mapping is done by linear interpolation of a tabulated SINR to BER values from link level simulations.

Two different mappings are used; one to represent interference limited scenarios, and one for sensitivity limited scenarios.

### Second stage mapping (BER  BLEP)

The second stage mapping is generated per used MCS. That is, one mapping is generated for MCS-1, MCS-2, MCS-3 and MCS-4 respectively.

This mapping is only dependent on the input bit error rates (BER), and hence the BER from both the sensitivity and interference limited first stage mapping is using the same second stage mapping.

To capture the impact on the error correction capabilities by the different code rates of the MCSs both the average BER and the standard deviation of the BER over the four bursts are collected. A high standard deviation indicates more diversity, and is typically favorable for MCSs with low enough code rate, while the opposite is true for MCSs with code rate close to 1.

### Mapping choice

With these mappings Figure 1 can be expanded to what is shown in Figure 2.

In the first stage mapping the mapping table is chosen based on sensitivity or interference per burst and instantaneous SINR value. In the second stage mapping, the mapping table is chosen based on the MCS used by the radio link.

Figure 2. L2S methodology and mapping selection

## SINR handling

### MRC

Since the first stage mapping tables are based on single antenna performance, a conversion from single antenna SINR to experienced SINR by the MRC receiver is needed.

This is modeled by eq. 1.

SINRMRC=S1I1+N+S2I2+N   (1)

### Blind repetition

In EC-GSM blind repetitions are performed when in extended coverage.  At the receiver side, the blind repetitions can be accumulated on IQ level or on soft bit level. How the receiver handles the multiple repetitions being received is implementation dependent.  To model this in a straightforward way the following approach is taken.

First, assume that the wanted signals are added coherently. This is the case for the EC-GSM simulations that have so far been provided within the study. The propagation channel is stationary/close to stationary during the IQ accumulation, so that coherent accumulation can be performed. This implies that the amplitudes of the signals are added, but the interfering signal/noise are added in terms of their powers, here the interference/noise is represented by n.  Assume further that a weight can be put to the received signals when combined and that noise is limiting the performance. This is shown in eq. 2.

SINRcomb=k1S1+k2S22k1n12+k2n22=S1+γS22n12+γ2n22   (2)

The combined SINR is maximized when the derivative of eq 2 is 0.

d(SINRcomb)dγ= 0   (3)

This gives the result in eq. 4.

γ=n1n22s1s2    (4)

Insertion into eq 2 yields eq 5.

SINRmax=S1N1+S2N2    (5)

Hence, the maximization of SINR occurs when the linear SINRs are summarized.

It can be noted that for EC-GSM and coherent IQ accumulation, s1 and s2 would be identical, and hence it is the ratio of interfering levels that is of importance for the signal combinations.

Equation 5 used to model the accumulation of IQ samples and/or soft bits when using blind repetitions in the system level simulator.

The model in Figure 1 has been modified to describe this aspect in Figure 3.

Figure 3. SINR handling for MRC and blind repetition.

# Verification

## Simulation assumptions

### General

Link level assumptions are listed in Table 1.

Table 1. Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Propagation condition | TU1.2nFH |
| MCS | MCS-1 MCS-2 MCS-3 MCS-4 |
| Impairments | Typical Tx/Rx |
| # transmissions | 1 |
| Frames | 20,000 |
| Interference/noise | AWGN CCI DTS-2, see [2] CCI+AWGN DTS-2+AWGN |

### Interference scenarios

Both strictly noise limited and interference limited scenarios are investigated. Interference limited scenarios are represented by single co-channel interference, and a mix of co-channel and adjacent-channel interference (DTS-2).

The DTS-2 scenario was defined when performance requirements were introduced for DARP Phase I, and was derived based on system level simulations. Later, this interference scenario has been shown to give similar profiles for example for VAMOS networks in earlier studies, see [3]. This interference scenario is considered to be representable also for EC-GSM.

Table 2. DTS-2 interferer scenario

| Interferer type | Relative interferer power [dB] |
| --- | --- |
| CCI | 0 |
| CCI | -10 |
| ACI | +3 |
| AWGN | -17 |

Also, variations from CCI and DTS-2 have been investigated to investigate the performance when having a mix of thermal noise component and interference. This is achieved by varying the interference to noise ratio (I/N0).

## Performance verification

### Sensitivity

In Figure 4 the link level simulation (LLS) results are compared to the Link-to-system mapping approach described in Section 3. Both Raw BER and MCS BLER are shown. As can be seen, the agreement is good with a difference of less than 0.5 dB.

Figure 4. Link Level Simulations (LLS) in sensitivity compared to Link-to-system mapping (L2S). Raw BER (top), MCS BLER (bottom).

### CCI

In Figure 5 the performance of the single co-channel interferer case is verified. There is perfect agreement in Raw BER performance. For MCS BLER, a good agreement can be observed for most cases. For the worst case, only less than 0.5 dB in difference is seen for MCS-1, MCS-2 and MCS-3. For MCS-4, the difference is less than 1 dB.

Figure 5. Link Level Simulations (LLS) in CCI compared to Link-to-system mapping (L2S). Raw BER (top), MCS BLER (bottom).

### Mix of interference and sensitivity

#### CCI+AWGN

To investigate how the mapping perform in a mix of thermal receiver noise and interference, a single co-channel interference scenario was generated with a noise level added (I/N0) relative to the interference at either -10, 0 or +10 dB.

Figure 6. Link Level Simulations (LLS) in CCI+AWGN at different I/N0 levels compared to Link-to-system mapping (L2S).

The agreement in performance is seen have a difference of at most 0.6 dB for the worst case.

#### DTS-2+AWGN

Considering that adding multiple interferers will destroy the signal structure to more appear like noise at the receiver, the exercise in Section 4.2.3.1 was repeated for the multi-interferer scenario DTS-2 where both multiple co-channel interferers, and a mix of co-channel interference and adjacent-channel interference is modeled. The results are shown in Figure 7.

Figure 7. Link Level Simulations (LLS) in DTS-2+AWGN at different I/N0 levels compared to Link-to-system mapping (L2S).

The agreement in performance is seen to be within 0.7 dB.

# Conclusion

This document has described and verified the link performance methodology used for EC-GSM PDTCH capacity simulations.

Both sensitivity and interference limited scenarios have been verified, as well as scenarios where a mix of single or multi interferer and noise is observed.

Good agreement in performance of less than 0.5 dB is seen in the majority of cases. In one of the cases the difference is almost 1 dB (MCS-4, CCI), but in this case the L2S is underestimating the performance.

Verification for repetition based transmissions is left FFS.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

3GPP TS 45.005, “Radio transmission and reception”

GP-101843, “System level evaluation of wider pulse shape for VAMOS (update of GP-101543)”, source Telefon AB LM Ericsson, ST-Ericsson SA. GERAN#48