# EC-GSM, EC-PDTCH system capacity evaluation 
(update of GP-150436)

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

One objective of the study is to “Scale to support a massive number of MTC Mobile Stations”. This includes providing sufficient capacity on the traffic channels.

In this contribution the system capacity of EC-GSM is evaluated on system level for the EC-PDTCH channel.

This document is an update of [6]. The updates include:

- Downlink CIoT traffic has been included

  - According to the Network Command traffic model

  - Application ACK as response to MAR periodic reports

- Latency results include time for network synchronization and random access

- EC-PACCH signaling has been included in both uplink and downlink

- Impact of IP header compression is evaluated

- Increased CS load

# EC-GSM basics

## Coverage classes

There are in total six coverage classes defined. These are also used by the system level simulations. The determination of coverage class is based on the received signal strength. Further, this is assumed to be known by the mobile, i.e. in sensitivity limited scenario the estimation of the coverage class (or in this case signal strength) would be ideal. How sensitive the results are to estimation errors in coverage class is not covered in these simulations.

## Fixed allocation

The EC-GSM concept makes use of the concept of fixed allocation. In short, this implies that the device requests a limited number of resources in the random access attempt, and the network schedules resources by the use of EC-AGCH and/or EC-PACCH.

The fixed allocation in the performed system simulations is done in accordance with the EC-GSM concept. This means:

- Users that are in normal coverage will only be allocated one TS per TTI.

- Users in extended coverage performing two blind repetitions will be allocated two TS per TTI.

- Users in extended coverage performing four or more blind repetitions will be allocated four TS per TTI.

# Traffic generation

Traffic is generated according to the MAR periodic reporting and Network Command traffic models [3]. The split between these is 80 % MAR periodic and 20 % Network command.

## MAR periodic reporting

MAR periodic reports are sent in UL. The inter-arrival times and message size distribution described in [3] has been used in the simulations. 50 % of the MAR periodic reports result in a DL Application ACK with application payload size of 0 bytes sent immediately after the base station successfully receives the UL packet (i.e., no delay assumed in CN, application, etc).

## Network Command

The Network Commands are sent in DL. They have the same inter-arrival times as the MAR periodic reporting [3] and an application payload size of 20 bytes. 50 % of the Network Commands result in an UL Application Response. The application payload size distribution for the response is the same as for the MAR periodic reporting.

# Simulations

## Simulation assumptions

The system level simulation assumptions in [3] have been followed. Other specific assumptions are shown in Table 1.

### System parameters

Table 1. Simulation assumptions, in addition to [3]

| Parameter | Value |
| --- | --- |
| General |  |
| Number of seeds | 1 |
| Simulation time | 100 s |
| System size | 192 cells |
| Direction | UL and DL |
| Frequency band | 900 MHz |
| Frequency re-use on BCCH layer | 12 |
| BTS antenna diversity | MRC |
| BTS output power | 43 dBm |
| CIoT parameters |  |
| EC-PDTCH timeslots per cell | 4 shared with legacy PDTCH(note 1) |
| CIoT arrival rate per cell and second | 1, 5, 6.8(note 2), 9 |
| Fixed DL/UL allocation | On |
| MCSs | MCS-4, MCS-1 |
| Minimum delay between subsequent transmissions on EC-PDTCH and EC-PACCH (scaled per CC) | 1 radio block |
| Chase combining | On |
| Incremental Redundancy | Off |
| Power control | Off |
| IP header compression | Off/On |
| Device output power | 23, or 33 dBm (100%) |
| BPL model(note 3) | 100 % CIoT devices subject to BPL |
| BPL scenario and inter-site correlation | Scenario 1, correlation 0.50 Scenario 2, correlation 0.75 |
| Device timeout | 20 seconds |
| CS parameters |  |
| CS timeslots per cell | 4 dedicated(note1, note4) |
| CS load | 4 E per cell |
| CS output power | 43 dBm in DL, max 33 dBm in UL |
| CS power control | Off in DL, on in UL |
| DTX | On |
| BPL model | CS devices are not subject to BPL |
| NOTE 1: The allocation of the CS and IoT timeslots in the TDMA frame are randomly offset per cell to model an unsynchronized network. NOTE 2: Derived from traffic models in [3]. 6.8 reports/commands per cell and second corresponds to the targeted number of devices per sector in the study. NOTE 3: According to model in [3]. NOTE 4: CS are put on timeslots 0 – 1 to model BCCH and RACH. |  |

### Coverage classes

Table 2 gives the details of the coverage classes.

Table 2. MCS and number of repetitions per coverage class

| Coverage Class | Number of blind transmissions for PDTCH and PACCH | MCS for PDTCH |
| --- | --- | --- |
| CC1 | 1 | MCS-4 |
| CC2 | 1 | MCS-1 |
| CC3 | 2 | MCS-1 |
| CC4 | 4 | MCS-1 |
| CC5 | 8 | MCS-1 |
| CC6 | 16 | MCS-1 |

### Control signaling

Packet uplink ACK/NACK (PUAN) is sent on EC-PACCH/D to (negatively) acknowledge data sent in the UL and assign fixed allocations to the MS. In the simulations its performance is pessimistically modeled with MCS-1 using the same number of blind transmissions as EC-PDTCH/D (but without HARQ retransmissions). If a PUAN with a fixed UL allocation assignment is unsuccessfully received the corresponding fixed UL allocation will be silent, i.e. the allocated MS will not transmit anything, but the radio block resources are consumed.

Packet downlink ACK/NACK (PDAN) is sent on EC-PACCH/U to (negatively) acknowledge data sent in the DL. It is transmitted using the correct amount of radio resources but it is always successfully decoded.

### Circuit switched users

The CS users are used as interferers on the remaining 4 TS on the used UL and DL carrier. In UL, power control and DTX are used, according to common assumptions, see [5]. The maximum UL output power for CS users is 33 dBm in all scenarios. For DL, power control is not used since the BCCH layer is modelled.

No BPL is applied to the CS users.

### Simulated scenarios

Table 3 summarizes the simulated scenarios and clarifies the legends in the figures presented in section 4.2.

Table 3: Simulated scenarios

| Legend text | CIoT output power [dBm] | BPL scenario | BPL inter-site correlation coefficient | IP Header Compr. |
| --- | --- | --- | --- | --- |
| 33 dBm BPL1 CS | 33 | 1 | 0.5 | Off |
| 23 dBm BPL1 CS | 23 | 1 | 0.5 | Off |
| 33 dBm BPL4 CS | 33 | 2 | 0.75 | Off |
| 23 dBm BPL4 CS | 23 | 2 | 0.75 | Off |
| 33 dBm BPL1 CS IPc | 33 | 1 | 0.5 | On |
| 23 dBm BPL1 CS IPc | 23 | 1 | 0.5 | On |
| 33 dBm BPL4 CS IPc | 33 | 2 | 0.75 | On |
| 23 dBm BPL4 CS IPc | 23 | 2 | 0.75 | On |

### Failed attempts

A timeout function is implemented in the simulator that cancels a transfer if it last for more than 20 seconds. In this case, the attempt to transfer the report is considered to have failed.

## Results

The results presented are:

- Latency of MAR periodic reports

  - The latency includes time to synchronize to the network, time for random access and time to transfer the message.

  - Network synchronization and random access are evaluated in separate simulations [7][8]. Delays from these are added in post-processing based on latency CDFs per coverage class for network synchronization time and random access time, respectively.

  - The results are presented as CDFs of the delay at the target traffic load (6.8 users per cell and second), see Figure 1.

  - Failed attempts are not included in the statistics (following the agreed methodology).

- Resource utilization

  - This represents the average amount of EC-PDTCH UL and DL resources required per cell in the system, plotted against the traffic load, see Figure 5 and Figure 6.

- Failed attempts

  - This represents the percentage of the attempts that were not successful, i.e. did not manage to get the report through during 20 seconds.

  - The percentage of failed attempts is plotted against the traffic load, see Figure 7.

### Latency of MAR periodic reports

In Figure 1, the CDF over the delay for the traffic load 6.8 users per cell and second is shown. It can be seen that:

- 95 % of the users with 33 dBm output power and BPL scenario 1 with correlation coefficient 0.5 (denoted BPL1 in the legend) have a delay less than 1.0 seconds without IP header compression and 0.9 seconds with IP header compression.

- 95 % of the users with 33 dBm output power and for the most aggressive BPL scenario (scenario 2 with correlation coefficient 0.75, denoted BPL4 in the legend) have a delay less than 1.7 seconds without IP header compression and 1.2 seconds with IP header compression.

- For users with output power 23 dBm 95 % of the users have a delay less than 1.3 seconds in BPL scenario 1 without IP header compression and 1.1 seconds with IP header compression.

- 95 % of the users with 23 dBm output power and the most aggressive BPL scenario have a delay less than 6.7 seconds without IP header compression and 4.0 seconds with IP header compression.

Figure 1: CDF of delay for UL reports with 6.8 users per cell-second.

In Figure 2 the 95th percentile is plotted against the traffic load.

### Resource utilization

Figure 5 and Figure 6 show the resource utilization for the CIoT users against the CIoT traffic load, for UL and DL respectively. At the traffic load of 6.8 users per cell and second, without IP header compression, the following can be observed:

- With 33 dBm output power and BPL scenario1, EC-GSM only utilizes approximately 0.8 time slots in UL and 0.7 time slots in DL on average.

- With 33 dBm output power and BPL scenario 2 the corresponding figures are 1.1 (UL) and 0.8 (DL) time slots.

- With 23 dBm output power and BPL scenario 1, simulations show average of 1.4 (UL) and 0.8 (DL) time slots

- For 23 dBm output power and BPL scenario 2, the corresponding figures are 2.5 (UL) and 1.0 (DL).

IP header compression reduces the resource utilization in all cases.

Figure 5: Timeslot utilization in UL per cell.

Figure 6: Timeslot utilization in DL per cell.

### Failed attempts

The percentage of failed attempts (i.e., the report did not get delivered within 20 seconds) is shown in Figure 7. At the traffic load 6.8 users per cell and second, less than 0.01 % does not get their report through for 33 dBm output power. For 23 dBm output power and BPL scenario 1, approximately 0.03 % does not get the report through within 20 seconds. For BPL scenario 2 and only 23 dBm output power, approximately 1.6 % of the users do not get the report through within 20 seconds. The total number of users passing through the system for this simulation is approximately 120 000.

Figure 7: Timed out users.

### Uplink capacity

In the TR [3] capacity is defined as “spectral efficiency in number of reports/200 kHz/hour. This definition is made with a standalone CIoT system in mind. Since EC-GSM is embedded in a legacy GSM network, in which only a fraction of the traffic is CIoT traffic, it does not make sense to measure the system capacity using that definition. Measuring only the delivered CIoT traffic in relation to the total amount of radio resources used by the system would be very misleading since most of the radio resources would be used by other services. Instead, the spectral efficiency of EC-GSM is presented by other means, such as the resource utilization in average number of utilized timeslots in section 4.2.3 and the fraction of failed attempts in section 4.2.4.

### Summary

The simulation results without/with IP header compression are summarized in Table 4 and Table 5, respectively.

Table 4: Summary of results without IP header compression

| Scenario without IP header compression | 95th percentile delay [s] | Average Resource Utilization UL [TS] | Average Resource Utilization DL [TS] | Failed Attempts |
| --- | --- | --- | --- | --- |
| 33 dBm, BPL scenario 1 (corr. 0.5) | 1.0 | 0.8 | 0.7 | <0.01 % |
| 33 dBm, BPL scenario 2 (corr. 0.75) | 1.7 | 1.1 | 0.8 | <0.01 % |
| 23 dBm, BPL scenario 1 (corr. 0.5) | 1.3 | 1.4 | 0.8 | 0.03 % |
| 23 dBm, BPL scenario 2 (corr. 0.75) | 6.7 | 2.5 | 1.0 | 1.6 % |

Table 5: Summary of results with IP header compression

| Scenario with IP header compression | 95th percentile delay [s] | Average Resource Utilization UL [TS] | Average Resource Utilization DL [TS] | Failed Attempts |
| --- | --- | --- | --- | --- |
| 33 dBm, BPL scenario 1 (corr. 0.5) | 0.9 | 0.6 | 0.45 | <0.01 % |
| 33 dBm, BPL scenario 2 (corr. 0.75) | 1.2 | 0.8 | 0.5 | <0.01 % |
| 23 dBm, BPL scenario 1 (corr. 0.5) | 1.1 | 1 | 0.6 | <0.01 % |
| 23 dBm, BPL scenario 2 (corr. 0.75) | 4.0 | 1.8 | 0.8 | 0.5 % |

# Conclusions

In this contribution the system performance of EC-GSM data traffic channels has been investigated in a mixed traffic scenario (CIoT and legacy CS). Performance at the target CIoT traffic load of 6.8 users per cell and second is summarized in Table 4 and Table 5 above.

E.g., with an MS output power of CIoT devices of 33 dBm in BPL scenario 1 with correlation 0.5, without IP header compression, and with interfering CS users with a traffic load of 4 Erlang per cell:

- The 95th percentile latency of MAR periodic reports is 1 second

- Less than 0.01 % of the reports time out, with a timeout limit of 20 seconds

- Only 0.8 EC-PDTCH UL and 0.7 EC-PDTCH DL timeslots per cell are needed on average to support the CIoT traffic

# References

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

- GPC150280, “EC-GSM, EC-PDTCH UL system capacity evaluation”, source Ericsson LM. GERAN Ad Hoc #2 on FS_IoT_LC.

- TR 45.820 V1.3.1, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (Release 13)”

- GPC150055, “EC-GSM, Mapping of logical channels onto physical channels”, source Ericsson LM. GERAN Ad Hoc #1 on FS_IoT_LC.

- 3GPP TR 45.914, “Circuit switched voice capacity evolution for GERAN“

- GP-150436, “EC-GSM, EC-PDTCH UL system capacity evaluation”, source Ericsson LM. GERAN#66

- GPC150491, “EC-GSM, Random access system capacity evaluation (update of GP-150433)”, source Ericsson LM. GERAN Ad Hoc #3 on FS_IoT_LC.

- GPC150437, “EC-GSM, Network synchronization with alternative EC-SCH design (update of GPC150198)”, source Ericsson LM. GERAN Ad Hoc #3 on FS_IoT_LC.

L2S for EC-PACCH has not been implemented.