3GPP TSG GERAN1&2 Adhoc#1 on CIoT_EC_GSM & eDRX_GSM	GPC150040

# Stockholm, Sweden, 12 – 13 October, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 43.022 | CR | 0035 | rev |  | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2015-10-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Enhanced procedures for service acquisition, system access, idle mode and packet transfer mode need to be introduced in support of EC-EGPRS operation for devices that make use of a power saving state (e.g. extended DRX). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhanced procedures for service acquisition, system access, idle mode and packet transfer mode are introduced in support of EC-EGPRS operation for devices that make use of a power saving state (e.g. extended DRX). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EC-EGPRS operation will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1.2, 3.2.1, 3.2.2, 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.013 0135 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR 1025,1026 TS 44.060 CR 1609,1610,1611,1612 TS 45.001 CR 0079,0080 TS 45.002 CR 0183,0184,0185,0186 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.005 CR 0578 TS 45.010 CR 0065 TS 45.008 CR 0626,0627 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.010 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The CR contains changes that are relevant to both Work item code eDRX_GSM and work item code CIoT_EC_GSM. Background text, or conflicting text from the corresponding eDRX_GSM CR is included without highlighting, and text specific to CIoT_EC_GSM is included with grey highlighted text. CR conflicts need to be resolved before agreeing the CRs |  |  |  |  |  |  |  |  |

*** First Change ***

## 1.2	Definitions and abbreviations

For the purposes of the present document, the terms and definitions given in 3GPP TS 21.905 and the following apply.

Selected PLMN	This is the PLMN that has been selected according to subclause 3.1, either manually or automatically.

Available PLMN	This is a PLMN where the MS has found a cell that satisfies conditions (ii) and (iv) of subclause 3.2.1.

Home PLMN (HPLMN)	See 3GPP TS 23.122.

Registered PLMN (RPLMN)	This is the PLMN on which certain LR outcomes have occurred (see 3GPP TS 23.122).

Equivalent PLMN	A PLMN contained in the stored list of equivalent PLMNs. These PLMNs are considered equivalent to the selected PLMN regarding PLMN selection, cell selection, cell re-selection and handover (see 3GPP TS 23.122).

Registration	This is the process of camping on a cell of the PLMN and doing any necessary LRs.

Camped on a cell	The MS (ME if there is no SIM) has completed the cell selection/reselection process and has chosen a cell from which it plans to receive all available services. Note that the services may be limited, and that the PLMN may not be aware of the existence of the MS (ME) within the chosen cell.

Current serving cell	This is the cell on which the MS is camped.

Suitable Cell	This is a cell on which an MS may camp. It must satisfy criteria  defined in subclause 3.2.1. For an MS in group receive mode, the suitable cell is determined by the criteria defined in subclause 5.2.3.

Acceptable Cell	This is a cell that the MS may camp on to make emergency calls and to receive warning notifications. It must satisfy criteria defined in sub clause 3.2.2.

Group call	A communication in which several MSs can receive, but at most one may be allowed to transmit on a radio channel. Examples of group calls are those established for the voice group call service (VGCS, see 3GPP TS 43.068).

Broadcast call	A communication in which several MSs can receive, but only the originator of the call is allowed to transmit on the radio channel. Examples of the broadcast call are those established for the voice broadcast service (VBS, see 3GPP TS 43.069).

Group receive mode	State of the MS when it is engaged in a voice group or voice broadcast call as a listener.

GPRS MS	An MS capable of GPRS services is a GPRS MS.

CTS MS	An MS capable of CTS services is a CTS MS.

Location Registration (LR)	An MS which is IMSI attached to non-GPRS services only performs location registration by the Location Updating procedure. A GPRS MS which is IMSI attached to GPRS services or to GPRS and non-GPRS services performs location registration by the Routing Area Update procedure only when in a network of network operation mode I. Both procedures are performed independently by the GPRS MS when it is IMSI attached to GPRS and non-GPRS services in a network of network operation mode II or III (see 3GPP TS 23.060).

Localised Service Area (LSA)	A localised service area consists of a cell or a number of cells. The cells constituting a LSA may not necessarily provide contiguous coverage.

EC-EGPRS:	An optional feature used by a MS in normal or extended coverage to reduce its power consumption through the use of relaxed mobility requirements, one phase system access and a power saving feature. An EC-EGPRS capable MS in a cell that supports EC-EGPRS that has negotiated the use of eDRX and/or PSM (see 3GPP TS 23.060) is said to have enabled EC-EGPRS operation.

Extended DRX (eDRX): 	A power saving feature where an extended DRX paging cycle ranging from a few seconds to about 52 minutes is used for determining MS reachability (see 3GPP TS 45.002).

Power Efficient Operation (PEO): An optional feature used by a MS to reduce its power consumption through the use of relaxed mobility requirements, one phase system access and a power saving feature. A PEO capable MS in a cell that supports PEO that has negotiated a power saving feature (i.e. eDRX or PSM - see 3GPP TS 23.060) is said to have enabled PEO operation.

Power Saving Mode (PSM): 	A power saving feature wherein a MS originated packet data transmission is used for determining MS reachability (see 3GPP TS 24.008, 3GPP TS 23.060, and 3GPP TS 23.682).

Power Saving Mode (PSM):	 Mode allowing the MS to reduce its power consumption, as defined in TS 24.008 [32], TS 23.060 [27], TS 23.682 [40].

SoLSA exclusive access	Cells on which normal camping is allowed only for MS with Localised Service Area (LSA) subscription.

Registration Area	A registration area is an area in which mobile stations may roam without a need to perform location registration. The registration area corresponds to location area (LA) for performing location updating procedure and it corresponds to routing area for performing the routing area update procedure.

The PLMN to which a cell belongs (PLMN identity) is given in the system information transmitted on the BCCH (MCC + MNC part of LAI).

***Next Change ***

### 3.2.1	Normal camping

For normal service, the MS has to camp on a suitable cell, tune to that cell's control channel(s), and possibly register within the PLMN so that the MS can:

a)	Receive system information from the PLMN , e.g., the cell options ;

b)	Receive paging messages from the PLMN, e.g., when there is an incoming call for the MS;

c)	Initiate call setup for outgoing calls or other actions from the MS (where possible, see subclauses 3.5.3 and 3.5.4).

The choice of such a suitable cell for the purpose of receiving normal service is referred to as "normal camping". There are various requirements that a cell must satisfy before an MS can perform normal camping on it:

i)	It should be a cell of the selected PLMN or, if the selected PLMN is equal to the last registered PLMN, an equivalent PLMN;

ii)	It should not be "barred" (see subclause 3.5.1);

iii)	It should not be in an LA which is in the list of "forbidden LAs for roaming";

iv)	The radio path loss between MS and BTS must be below a threshold set by the PLMN operator. This is estimated as shown in subclause 3.6;

v)	It should not be a SoLSA exclusive cell to which MS does not subscribe.  This requirement is only valid for MSs supporting SoLSA.

Initially, the MS looks for a cell which satisfies these 5 constraints ("suitable cell") by checking cells in descending order of received signal strength. If a suitable cell is found, the MS camps on it and performs any registration necessary. Cells can have two levels of priority, suitable cells which are of low priority are only camped on if there are no other suitable cells of normal priority. (This is called "cell selection").

When camped on a cell the MS regularly looks to see if there is a better cell in terms of a cell re-selection criterion, and if there is, the better cell is selected. Also if one of the other criteria changes, (e.g., the current serving cell becomes barred), or there is a downlink signalling failure (see subclause 3.6), a new cell is selected. (This is called "cell reselection"). A MS that has enabled PEO operation or EC-EGPRS operation shall use an implementation specific method for determining how often it verifies the suitability of its serving cell and the suitability of neighbour cells for re-selection (see 3GPP TS 45.008 and 3GPP TS 44.018).

In order to speed up these processes, a list of the RF channels containing BCCH or CPBCCH carriers of the same PLMN is broadcast in the system information messages, see subclause 4.8. Also, the MS does not need to search all possible RF channels to find a suitable cell. If, after searching the number of RF channels, given for each frequency band below, with the strongest received signal level, a BCCH or CPBCCH carrier has been found but no suitable cell of the selected PLMN has been found, the MS can stop the attempt to find a suitable cell of the selected PLMN.

The number of channels to be searched are 15 for T-GSM 380, 15 for T-GSM 410, 15 for GSM 450, 15 for GSM 480, 25 for GSM 710, 25 for GSM 750, 25 for T-GSM 810, 30 for GSM 850 Band, 30 for GSM 900, 15 for T-GSM 900 and 40 for DCS 1800 and PCS 1900.

***Next Change ***

### 3.2.2	"Camp on any cell"

If the MS cannot find a suitable cell, it attempts to camp on any acceptable cell and enter a "limited service state", as defined in subclause 3.7. An MS attempting to camp on any acceptable cell shall not scan for CPBCCH. A PEO capable MS may choose to only camp on cells that support PEO and an EC-EGPRS capable MS may choose to only camp on cells that support EC-EGPRS.

There are various requirements that a cell must satisfy before being considered an acceptable cell to be camped on in  "limited service state":

i)	It should not be "barred" (see subclause 3.5.1);

ii)	The radio path loss between MS and BTS must be below a threshold set by the PLMN operator. This is estimated as shown in subclause 3.6.

*** Last Change ***

## 4.5	Cell selection process

Whenever a PLMN is selected, the MS attempts to find a suitable cell of that PLMN to camp on. Two methods of searching for a suitable cell are possible:

a)	Normal cell selection ("Normal cell selection" state) - Here the MS has no prior knowledge of which RF channels are BCCH or CPBCCH carriers. It searches at least the number, given in subclause 3.2.1, of the strongest RF channels in descending order of received signal level to see which are BCCH or CPBCCH carriers. (If no BCCH or CPBCCH carriers have yet been found, searching will continue until at least one BCCH or CPBCCH carrier is found.). The first BCCH or CPBCCH carrier found which is from a suitable cell and on which there is a normal priority indication is taken and that cell is camped on. If at least the number, given in subclause 3.2.1, of the strongest RF channels have been tried and the only suitable cells found have low priority indication the MS shall camp on the strongest of these cells.

b)	Stored list cell selection (optional) ("Stored list cell selection" state)- Here the MS has a list of the BCCH and CPBCCH carriers used by the PLMN. (This list may be derived by the MS from information gathered during previous selections of the PLMN). The BCCH and CPBCCH carriers in the list are searched in descending order of received signal level, and the first BCCH or CPBCCH carrier found which is from a suitable cell and on which there is a normal priority indication is taken, and that cell is camped on. If an MS decodes system information from a cell of the selected PLMN but is unable to camp on that cell, the BA of that cell shall be examined and any BCCH and CPBCCH carriers in the BA which are not in the list of BCCH or CPBCCHcarriers to be searched shall be added to the list. If all these BCCH and CPBCCH carriers have been tried and the only suitable cells found have low priority indication, the MS shall camp on the strongest of these cells. When required to search for CPBCCH, the stored list cell selection is mandatory for CPBCCH carrier from the HPLMN.

If no suitable cell is found using method b), method a) is then tried.

An MS with GSM voice capability shall not search for CPBCCH carriers. An MS supporting GSM circuit-switched data without supporting GSM voice may search for CPBCCH carriers.

NOTE:	Since CPBCCH carriers are not transmitting continuously, another measurement algorithm than the one used for BCCH carriers must be followed when measuring signal strength of CPBCCH carriers (see 3GPP TS 45.008).

While camped on a cell of the registered PLMN ("camped normally"), the MS may need to select a different cell ("normal cell reselection" state). The following events trigger a cell reselection:

i)	The path loss criterion parameter C1 (see subclause 3.6) indicates that the path loss to the cell has become too high. An exception is the case of a MS that has enabled PEO operation or EC-EGPRS operation in which case the mobile station uses the pathloss criterion C3 to determine if the path loss to the cell has become too high (see 3GPP TS 45.008);

ii)	There is a downlink signalling failure (subclause 3.6); An exception is the case of a MS that has enabled PEO operation in or EC-EGPRS operation which case the mobile station uses the PCH_DSC counter for detecting (EC-)PCH based signalling failures (see 3GPP TS 45.008) and the BCCH_DSC counter for detecting (EC-)BCCH based signalling failures (see 3GPP TS 44.018).

iii)	The cell camped on (current serving cell) has become barred;

iv)	There is a better cell (in terms of the path loss criterion C2) in the same registration area, or a much better cell in another registration area of an equivalent PLMN (using the CRH parameter, subclause 3.4). An exception is the case of a MS that has enabled PEO operation or EC-EGPRS operation in which case the mobile station only evaluates C2 for the serving and C1 and C2 for the non-serving cells when it determines the serving cell is no longer suitable (see 3GPP TS 45.008).

v)	A random access attempt is still unsuccessful either after "Max retrans" repetitions; "Max retrans" being a parameter broadcast on control channel, or in case the time given for the packet access procedure expires before the mobile station receives any response from the network;

vi)	Upper layers have determined that the network has failed an authentication check (see subclause 3.5.5).

A GPRS MS in Ready state applies the READY_STATE CELL RESELECTION HYSTERESIS together with the path loss criterion when reselecting the cell within the registration area. The GPRS MS in Ready state shall inform the network about cell reselection within the registration area by the cell update procedure. The network may control cell reselection of a GPRS MS in Ready state (see 3GPP TS 43.064).

Instead of the parameter C2, a GPRS MS applies the corresponding GPRS parameter if provided. If this GPRS parameter is provided the cell shall also broadcast a list BA(GPRS) indicating BCCH or CPBCCH carriers to be monitored by GPRS MSs for cell re-selection purpose.

An MS supporting SoLSA with SoLSA subscription shall use the SoLSA cell re-selection parameters.

The MS will then reselect a new cell in order to fulfil the process goal (see subclause 4.1).

Before camping on the cell after re-selection, the MS shall attempt to decode the full set of system information. The MS shall check that the parameters affecting cell re-selection are unchanged. If a change is detected the MS shall check if the cell re-selection criterion is still valid using the changed parameters. If the cell selection criteria are still valid, the MS shall camp on the cell. If they are not still valid, the MS shall repeat this process for the cell with the next highest value of C2 or corresponding GPRS parameter or SoLSA parameters, respectively.

Once the MS has re-tuned to the chosen cell, it shall monitor its paging subgroup (if known) for that cell. If the MS and the cell support group or broadcast calls, it shall also monitor the notification channel. A GPRS MS shall monitor the relevant GPRS control channel(s). If the MS receives a page before having decoded the relevant system information for the new cell, the MS shall store the page and respond, if permitted, once the relevant system information has been decoded. If not permitted, no page response shall be made.

When the MS leaves "connected mode" (e.g., after a call), and returns to idle mode, a cell must be chosen ("choose cell" state) as soon as possible (e.g., to allow a new call to be made). The chosen cell will not necessarily be the same cell as the MS was camped on when the call started e.g., if the MS moved a significant distance while the call was in progress. Two cases are distinguished in 3GPP TS 45.008, for which the algorithm is slightly different:

a)	Normal case - The call is terminated, either because the user (or network) has terminated the call, or because a radio link failure has occurred and call re-establishment is not being attempted.

b)	Call re-establishment - A radio link failure has occurred and a call re-establishment is being attempted. (The MS returns to idle mode to start the call re-establishment attempt and needs to select a new cell on which to make the attempt as soon as possible in order to minimize the interruption to the call).

If no suitable cell is found by either the stored list cell selection, normal cell reselection or choose cell tasks (all of which use a limited search), the normal cell selection task (which searches all RF channels for a suitable cell) is entered.

If no suitable cell is found by the normal cell selection task, or if there is no valid SIM in the MS, the MS attempts to find an "acceptable cell" on which it may camp so that emergency calls may be made and warning notifications may be received ("any cell selection" state). The MS ignores the PLMN information when selecting an acceptable cell, and no attempt at location registration is made. This task is also entered if an LR reject is received with one of the following cause values:

-	"IMSI unknown in HLR" (only for non-GPRS MS as this impacts only the non-GPRS update state, MSs capable of GPRS and non-GPRS services take a cell selection state according to the outcome of the routing area update);

-	"Illegal MS";

-	"illegal ME";

-	"PLMN not allowed" (However, this will trigger a new PLMN selection if the MS is in automatic mode);

-	"GPRS not allowed" (only for MS capable of GPRS only, where this will trigger a new PLMN selection if the MS is in automatic mode; MSs capable of GPRS and non-GPRS services take a cell selection state according to the outcome of location updating as "GPRS not allowed" has only impact on the GPRS update status).

NOTE:	Some cause values will lead the mobile station to consider the SIM as invalid, see 3GPP TS 24.008.

If an acceptable cell is found, the MS camps on it ("Camped on any cell" state). If one of the cell reselection trigger events (i) to (v) above occurs, the MS attempts to find a new acceptable cell to camp on ("any cell reselection" state) but using a zero dB value of CRH. If no acceptable cell can be found, the "any cell selection" task is re-entered.

If while camped on an acceptable cell:

i)	an emergency call origination is made, the MS enters a connected mode and at the end of the call, chooses an acceptable cell ("choose any cell" state) so that it is ready for the next emergency call origination;

ii)	a warning notification is received, the MS continues to perform cell reselection to an acceptable cell.

In all cases, if a new PLMN is selected and there is a valid SIM in the MS, the MS searches for a suitable cell of that PLMN. However, if the MS has recently searched the strongest RF channels while a previous PLMN was selected, it may already have information about other PLMNs.

The user may request a search of RF channels to determine which PLMNs are available. This search shall be done in such a way as to minimize interruptions to the MS's monitoring of its paging subchannel.