3GPP TSG-GERAN WG3New Meeting #56 	 GP-121234

# Prague, Czech Republic, 19 - 23 November 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 1977 | rev | - | Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | GCF Priority g1 - Corrections to Multi-layer test case 13.4.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anite |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G3new |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-UEConTest_SIG |  |  |  |  |  |  | Date: |  | 2012-11-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | 1. PDP context activation is already performed during preamble procedure on the EUTRA cell. Hence, PDP context activation is not required on the GERAN cell. 2. As per sections 9.4.14 and 10.5.5.12a in 24.008, elements "E-UTRA FDD support" and "E-UTRA TDD support" are part of "MS Radio Access capability" IE. But, mentioned in Table "13.4.2.5.3.3-3" as part of "Mobile station classmark 3" IE.…’ |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Removed PDP context activation procedure on the GERAN cell, from the preamble. 2. Removed "Mobile station classmark 3" from Table "13.4.2.5.3.3-3". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |

#### 13.4.2.5	Inter-system mobility/Service based redirection from GSM/GPRS to E-UTRA

13.4.2.5.1	Test Purpose (TP)

(1)

with { UE in GPRS Registered state }

ensure that {

when { UE is requested to initiate a service based redirection to E-UTRA }

then { UE performs service based redirection to E-UTRA cell }

}

13.4.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 44.060 section 7.4.2

[TS 44.060, section 7.4.2]

The network may initiate the cell change order procedure by sending an IMMEDIATE ASSIGNMENT message for single block assignment in a CCCH block monitored by the mobile station. No TBF shall be established. The single block assignment procedure is specified in 3GPP TS 44.018.

The network shall then send the PACKET CELL CHANGE ORDER message in the assigned downlink block to the mobile station. The PACKET CELL CHANGE ORDER message contains:

-	the characteristics of the new cell that are necessary to identify it (i.e. BSIC + BCCH frequency);

-	the NC measurement parameters valid for the mobile station in the new cell (NETWORK_CONTROL_ORDER and optionally: NC_NON_DRX_PERIOD, NC_REPORTING_PERIOD_I and NC_REPORTING_PERIOD_T).

For a multi-RAT mobile station supporting UTRAN, the PACKET CELL CHANGE ORDER message may contain information on a UTRAN target cell; in this case, the establishment of channel(s) and subsequent measurement reporting are defined in 3GPP TS 25.331.

For a multi-RAT mobile station supporting “CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN”, the PACKET CELL CHANGE ORDER message may contain information on an E-UTRAN target cell; in this case, the establishment of channel(s) and subsequent measurement reporting are defined in 3GPP TS 36.331.

Upon receipt of the PACKET CELL CHANGE ORDER message, the mobile station shall stop all relevant RLC/MAC timers except for timers related to measurement reporting and start timer T3174. The mobile station shall then switch to the specified new cell and obey the relevant RLC/MAC procedures on this new cell. If a valid RRBP field was received in the PACKET CELL CHANGE ORDER message then the MS shall send a PACKET CONTROL ACKNOWLEDMENT message in the reserved uplink radio block specified by the RRBP field before switching to the new cell. If the timers related to measurement reporting expire while the reselection procedure has not yet been completed, these timers shall be restarted so that the mobile station resumes the measurement reporting procedures once camped on the new cell. A UTRAN capable mobile station ordered to a UTRAN cell shall obey the PACKET CELL CHANGE ORDER message irrespective of whether or not the target cell is known (see 3GPP TS 25.133 and 3GPP TS 25.123); an E-UTRAN capable mobile station ordered to an E-UTRAN cell shall obey the PACKET CELL CHANGE ORDER message irrespective of whether the target cell is known or not known (see 3GPP TS 36.133).

13.4.2.5.3	Test description

13.4.2.5.3.1	Pre-test conditions

System Simulator:

-	Cell 24 is serving GERAN Cell

-	Cell 1 is suitable E-UTRAN Cell

UE:

None.

Preamble:

- The UE is in state Generic RB Established, UE test mode activated (state 4) according to [18] using the UE TEST LOOP MODE B and then moved to GPRS packet idle state , with power levels as in Table 13.4.2.5.3.2-1 T0, on Cell 24.

13.4.2.5.3.2	Test procedure sequence

Table 13.4.2.5.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 13.4.2.5.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Cell24 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | off | -60 | Camping on Cell 24 is guaranteed |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -60 | Same as before | The power level is such that SrxlevCell 1 > 0 |
| Note: Srxlev is calculated in the UE |  |  |  |  |  |

Table 13.4.2.5.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 levels according to the row "T1" in table 13.4.2.5.3.2-1. |  |  |  |  |
| 2 | SS sends IMMEDIATE ASSIGNMENT | <-- | IMMEDIATE ASSIGNMENT | - | - |
| 3 | The SS sends PACKET CELL CHANGE ORDER for cell 1 as the target cell on cell 24 | <-- | PACKET CELL CHANGE ORDER | - | - |
| 4 | Steps 1 to 7 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7A-1 are performed on Cell 1. Note: The UE performs a TAU procedure and a default EPS Bearer is setup. | - | - | . | - |
| 5 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context. | <-- | IP packet | - | - |
| 6 | Check: Does the UE loop back the IP packet on the DRB associated with the default EPS bearer context? | --> | IP packet | 1 | P |

13.4.2.5.3.3	Specific message contents

Table 13.4.2.5.3.3-2: PACKET CELL CHANGE ORDER (step 5, Table 13.4.2.5.3.2-2)

| Information element | Value/remark |
| --- | --- |
| < PAGE_MODE : bit (2) > | 00 (Normal Paging) |
| 0 ¦ 10 | 0 |
| < GLOBAL_TFI : Global TFI IE > | <5 bit Uplink TFI> |
| 0 ¦ 1 | 1 |
| Message Escape < IMMEDIATE_REL > 0¦1<UTRAN FDD Target cell IE> 0¦1<UTRAN TDD Target cell IE> Additions in Rel-5 0 ¦ 1 < G-RNTI extension Additions in Rel-8 0¦1<E-UTRAN Target cell IE> EARFCN 0 ¦ 1 < Measurement Bandwidth Physical Layer Cell identity 0 ¦ 1 < Individual Priorities | 00 1 (Immediate abort of operation in the old cell is required) 0 (not present ) 0 (not present ) 1 0 (not present ) 1 1 EARFCN of the cell 1 0 (not present ) PCID of the cell 1 0 (not present ) |

Table 13.4.2.5.3.3-3: Message ROUTING AREA UPDATE REQUEST (Preamble)

| Derivation Path: Table 9.4.1/3GPP TS 24.008 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MS Radio Access capability |  |  |  |  |
| GERAN to E-UTRA support in GERAN packet transfer mode |  | ‘10’B or ‘11’B | CCN towards E-UTRAN, E-UTRAN Neighbour Cell measurement reporting and Network controlled cell reselection to E-UTRAN supported |  |
|  |  | - | - | - |
| E-UTRA FDD support |  | ‘0’B or ‘1’B |  | C1 |
| E-UTRA TDD support |  | ‘0’B or ‘1’B |  | C1 |
| C1 | At least one of these fields shall be set to ‘1’B |  |  |  |  |