Proposed Text for the TR on Clean-slate Channel Mapping of MAC Layer

Introduction

A new study item named Cellular IoT was approved at GERAN#62 (see [1]). MAC layer is an important sub-layer in the Clean-slate solution for Cellular IoT. And channel mapping is the basic function for the whole MAC layer design which has been proposed in [2]. This document contains a text proposal for the Cellular IoT TR on the channel mapping function of MAC layer of the Clean-slate solution which is the result of consolidating [2].

Proposed text for the TR

First Change

- 5.4	Concept#2 (Clean slate)

- 5.4.4	Link layer aspects

- 5.4.4.1 	Channel mapping

- 5.4.4.1.1 Channel mapping for S1-based architecture

As the S1 interface is connection oriented, the PDCP and RRC layer have to be maintained to support S1 interface without modifications on MME side. In this case, the channel mapping would have three layers as shown in Figure 5.4.4.1-1.

The uplink mapping is shown as red lines, and the downlink mapping is shown as blue lines. Most channels would have mapping on each layer with the only exception on RACH channel, as random access procedure is terminated in MAC layer.

s

Figure 5.4.4.1-1. Channel mapping for S1-based architecture

- transport channels

The transport channels are listed in Table 5.4.4.1-1. Each transport channel type is defined by the transport manner on radio interface.

Table 5.4.4.1-1. Transport channels

| Transport channel name | Acronym | Downlink | Uplink | Description |
| --- | --- | --- | --- | --- |
| Broadcast Channel | BCH | X |  | Basic system information is carried |
| Extend Broadcast Channel | E-BCH | X |  | Other system information are carried |
| Downlink Shared Channel | DL-SCH | X |  | Downlink data and control message are carried |
| Paging Channel | PCH | X |  | Paging messages are carried |
| Uplink Shared Channel | UL-SCH |  | X | Uplink data and control message are carried |
| Random Access Channel | RACH |  | X | Random access message is carried |

- logical channels

The MAC layer provides data transfer services on logical channels. Each logical channel type is defined by what type of information is transferred.

The logical channels are listed in Table 5.4.4.1-2.

Table 5.4.4.1-2. Logical channels

| Logical channel name | Acronym | Control channel | Traffic channel | Description |
| --- | --- | --- | --- | --- |
| Broadcast Control Channel | BCCH | X |  | System information is carried |
| Paging Control Channel | PCCH | X |  | Paging message is carried |
| Common Control Channel | CCCH | X |  | RRC signalling on SRB0 is carried (TS 36.331 for reference) |
| Dedicated Control Channel | DCCH | X |  | RRC signalling on SRB1 and SRB2 are carried (TS 36.331 for reference) |
| Dedicated Traffic Channel | DTCH |  | X | User data is carried |

- 5.4.4.1.2 Channel mapping for Gb-based architecture

As the Gb-based interface is connectionless, it is natural that functions would terminate at MAC layer. Therefore there is no need to have additional logical channels. The specific channel mapping is shown in Figure 5.4.4.1-2.

Figure 5.4.4.1-2. Channel mapping for Gb-based architecture

The definitions of the channels are the same as in Section 5.4.4.1.1.

End of Changes

References

  - GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, VODAFONE Group Plc., GERAN#62.

  - GP-140850, “NB M2M - Basic Introduction of Channel Mapping for MAC Layer”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN#64.