Event-triggered Neighbour Cell Measurement

Background

As described in [1], in current specification, measurements for cell re-selection in idle mode include the measurement and BCCH data reading of both serving cell and neighbour cells. It could be up 45% power consumption used for neighbour cells measurement when the MS is in idle mode for one day.

Such power consumption for the legacy MS is necessary to assure the service when the users are moving. The signal level of serving cell should be always the best. However, for the MTC devices, which are characterized by low mobility or stationary, the neighbour cells measurement can be simplified. An optimized measurement for neighbour cells is proposed in this paper.

Legacy measurement for neighbour cells

In the draft TR, the procedures of packet idle mode include measurements for cell selection and cell re-selection and PCH reading as reference case assumptions. The procedures of measurements for cell re-selection are as follows.

- measurements and data reading of serving cell

  - monitor of received signal level of serving cell (accompanied with PCH reading)

  - monitor BCCH data of the serving cell (once per 30 seconds)

- measurements and data reading of neighbour cells

  - monitor all BCCH carriers as indicated by the BA list (accompanied with PCH reading)

  - identify BSIC for 6 strongest neighbour cells (once switch on or from non-service area into the service area)

  - reconfirm BSIC for 6 strongest neighbour cells (once every 30 seconds)

  - identify new 6 strongest cells BSIC and BCCH data (once every 1h)

  - read BCCH data for 6 strongest neighbour cells (once every 5 minutes)

Monitoring the signal levels of serving cell and neighbour cells is accompanied with PCH reading.

Figure 1. Legacy periodic Measurement

Optimized measurement for neighbour cells

To simplify the neighbour cells measurement, it’s proposed that measurements and data reading of neighbour cells are triggered by several events as shown in Figure 2. Consequently, the MS should only perform the periodic measurements and data reading of serving cell in measurements for cell re-selection.

Event B

Event A

Figure 2. Optimized periodic Measurement

Event A: happens when the MS is reading PCH. It could be PCH decoding error or the low signalling of serving cell.

Event B: happens when the MS is monitoring the BCCH data of serving cell. It could be decoding error or synchronization error.

When Event A/B happens, the neighbour cells measurement is triggered as follows and shown in Figure 3.

- Step 1: MS starts to monitor all BCCH carriers as indicated by the BA list, and get the 6 strongest neighbour cells. If it is the first time Event A/B happened, the MS identifies BSIC for 6 strongest neighbour cells. Else, the MS reconfirms BSIC for 6 strongest neighbour cells.

- Step 2: If MS recognizes that a new BCCH carrier has become one of the 6 strongest, MS should identify BSIC and decode BCCH data for the new BCCH carrier.

- Step 3: If there is no new BCCH carrier of the 6 strongest, the MS tries to read the BCCH data of the 6 strongest neighbour cells. If such events have happened before and the BCCH data of part or all of the 6 strongest neighbour cells have already been stored which are still in the valid period, this step can be omitted.

After the above procedures, the MS can perform the reselection algorithm as legacy mechanism. When the MS decides to reselect to a new cell, the neighbour cell measurement is completed and the MS acts as shown in Figure 2 in the new cell.

Figure 3. Event-triggered neighbour cells measurement

Evaluation and discussion

1 hour frequency report is selected as a sample. The proposal mechanism only impacts on the idle mode, and the evaluation only include the power consumption in idle mode.

The power consumption is calculated as shown in Table 1 base on the parameters in the latest draft TR [2], and operating current and voltage in [3].

And the following parameters are utilized in evaluation for legacy and optimized measurement mechanism:

- DRX cycle is 15s (The longest paging cycle in the specification).

- The time of RSSI with 32 I,Q samples is 1.75ms (which is implementation dependent)

- Two triggered events happen in 1 day, and 1 hour measurements for neighbour cells for each trigger are needed for a suitable cell re-selection.

Table 1. Power consumption in packet idle mode

| Procedure | sub-procedure | operation | occurrence | Energy consumption for each sub-procedure (uJ) | Legacy: occasion number in 1 day | Legacy: energy consumption in 1 day (uJ) | Optimized: occasion number in 1 day | Optimized: energy consumption in 1 day (uJ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measurements for cell selection | RF signal receiving | Monitor BCCH carrier | switch on or from non-service area into the service area | 14322.00 | 1.00 | 14322.00 | 1.00 | 14322.00 |
|  | Identify BCCH carriers in the list | BSIC Identification |  | 1751.77 | 1.00 | 1751.77 | 1.00 | 1751.77 |
|  | decode the full set of BCCH data of the serving cell | Rx - 4 radio block |  | 1127.23 | 1.00 | 1127.23 | 1.00 | 1127.23 |
| Measurements for cell re-selection | Monitor of received signal level of serving cell | Monitor BCCH carrier | DRX cycle | 115.50 | 5735.77 | 662481.68 | 5735.77 | 662481.68 |
|  | Monitor BCCH data of the serving cell | Rx - One radio block | once every 30s | 281.81 | 2880.00 | 811603.58 | 2880.00 | 811603.58 |
|  | Monitor all BCCH carriers as indicated by the BA list | Monitor BCCH carrier | DRX cycle | 1848.00 | 5735.77 | 10599706.84 | 477.98 | 883308.90 |
|  | identify BSIC of 6 strongest | BSIC Identification | once | 1051.06 | 1.00 | 1051.06 | 1.00 | 1051.06 |
|  | reconfirm BSIC of 6 strongest | BSIC Reconfirmation | once every 30s | 342.74 | 2880.00 | 987085.44 | 2.00 | 685.48 |
|  | identify BSIC and decode BCCH data ( only SI3) for the new BCCH carrier which become one of the 6 strongest | BSIC Identification and Rx - One radio block | once every 1h | 456.98 | 24.00 | 10967.62 | 2.00 | 913.97 |
|  | decode BCCH data of 6 strongest ( only SI3) | Rx - One radio block | once every 5min | 1690.84 | 288.00 | 486962.15 | 2.00 | 3381.68 |
| PCH reading | read PCH | Rx - One radio block in serving cell | DRX cycle | 276.03 | 5735.77 | 1583255.50 | 5735.77 | 1583255.50 |

Table 2. Power consumption comparison

| power consumption (uJ) (legacy, in Figure 1) | power consumption (uJ) (proposal, in Figure 2) | power saving ratio |
| --- | --- | --- |
| 15,160,318 | 3,963,883 | 73.8% |

The comparison between power consumption with existing specification and optimized procedure are shown in Table 1. The power saving ratio is up to 73.8%.

Furthermore, such an optimization is only applied in the measurement for neighbour cells but not the serving cell. There is no impact on the current MO and MT service.

Conclusion

An Event-Triggered neighbour cells measurement for idle mode is proposed to applied in MTC device in idle mode for power saving. Such an optimization has no impact on the current MO and MT service. Furthermore, significant benefit can be found compared with legacy mechanism.

References

  - GP-140173, “Optimized procedure in idle mode for MTC devices”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN#61.

  - GP-140663, “[DRAFT] 3GPP TR 43.869 V0.2.0”, China Mobile, GERAN#63.

  - GP-140612, “MS energy consumption model”, Ericsson, GERAN#63.