pCR for Event-triggered Neighbour Cell Measurement

Introduction

An event-triggered neighbour cell measurement mechanism for cell reselection has been proposed in [1] to reduce power consumption of packet idle mode. This document contains a text proposal for the optimized technology which is the result of consolidating [1].

This is the revision of GP-150104, the major update is marked in yellow.

Proposed text for the TR

First Change

6	Common Assumptions

## 6.4	Reference Case Assumptions

### 6.4.1 		Packet Idle Mode

#### 6.4.1.2 		Energy consumptions as per procedure

The calculation for total reference case energy consumption is based on different MS operations. The energy consumption for every procedure listed in Table 6 can be calculated as follows based on section 6.2.

Table 6 Energy consumptions for MS in idle mode

| Procedure | sub-procedure | operation | occurrence | Energy consumption for each sub-procedure (uJ) | Energy consumption for procedure (uJ) |
| --- | --- | --- | --- | --- | --- |
| Measurements for cell selection | RF signal receiving | Monitor BCCH carrier | switch on or from non-service area into the service area | 8184 Note | 11063.00 |
|  | Identify BCCH carriers in the list | BSIC Identification |  | 1751.77 |  |
|  | decode the full set of BCCH data of the serving cell | Rx - 4 radio block |  | 1127.23 |  |
| Measurements for cell re-selection | Monitor of received signal level of serving cell | Monitor BCCH carrier | DRX cycle | 66 Note | 4945.43 |
|  | Monitor BCCH data of the serving cell | Rx - One radio block | once every 30s | 281.81 |  |
|  | Monitor all BCCH carriers as indicated by the BA list | Monitor BCCH carrier | DRX cycle | 1056 Note |  |
|  | identify BSIC of 6 strongest | BSIC Identification | once | 1051.06 |  |
|  | reconfirm BSIC of 6 strongest | BSIC Reconfirmation | once every 30s | 342.74 |  |
|  | identify BSIC and decode BCCH data ( only SI3) for the new BCCH carrier which become one of the 6 strongest | BSIC Identification and Rx - One radio block | once every 1h | 456.98 |  |
|  | decode BCCH data of 6 strongest ( only SI3) | Rx - One radio block | once every 5min | 1690.84 |  |
| PCH reading | read PCH | Rx - One radio block in serving cell | DRX cycle | 278.51 | 278.51 |

Note: The time of RSSI with 32 I,Q samples is assumed 1ms.

Second Change

7	Candidate Technologies

7.3	Event-triggered neighbour cell measurement

7.3.1	Mechanism

To simplify the neighbour cells measurement in idle mode, measurements and data reading of neighbour cells are not periodic but triggered by several events as shown in Figure 7.3-1. The periodic measurements and data reading of serving cell in measurements for cell re-selection are still performed as legacy mechanism.

Event B

Event A

Figure 7.3-1: Optimized measurement

Event A: happens when the UE is reading PCH. The event could be PCH decoding error (e.g. similar with the legacy Downlink Signaling Failure, the detail is FFS) or the low signalling of serving cell.

Event B: happens when the UE is monitoring the BCCH data of serving cell. The event could be decoding error or synchronization error for several times.

When Event A/B happens, the neighbour cells measurement is triggered as follows and shown in Figure 7.3-2.

-	Step 1: UE starts to monitor all BCCH carriers as indicated by the BA list (e.g. the frequency of monitoring could be the same as in cell selection), and get the 6 strongest neighbour cells. If it is the first time Event A/B happened, the UE identifies BSIC for 6 strongest neighbour cells. Else, the UE reconfirms BSIC for 6 strongest neighbour cells.

-	Step 2: If UE recognizes that a new BCCH carrier has become one of the 6 strongest, the UE should identify BSIC and decode BCCH data for the new BCCH carrier.

-	Step 3: If there is no new BCCH carrier of the 6 strongest, the UE tries to read the BCCH data of the 6 strongest neighbour cells. If such events have happened before and the BCCH data of part or all of the 6 strongest neighbour cells have already been stored and valid, this step can be omitted.

After the above procedures, the MS can perform the reselection algorithm as legacy mechanism.

Figure 7.3-2: Event-triggered neighbour cells measurement

TS 45.008 and TS 44.018 might need to be updated.

7.3.2	Evaluation

5 min frequency MT report is selected as a sample, and the evaluation period is 1 day. The power consumption is calculated base on the parameters of network triggered traffic model.

According to the table of energy consumptions for MS in idle mode in Section 6.4.1, and the following parameters are utilized in evaluation for legacy and optimized measurement mechanism:

-	DRX cycle is 15s.

-	Two triggered events happen in 1 day, and 1 hour measurements for neighbour cells for each trigger are needed for a suitable cell re-selection.

Table 7.3-1: Power consumption in packet idle mode per day

| Procedure | sub-procedure | operation | occurrence | Energy consumption for each sub-procedure (uJ) | Legacy: occasion number in 1 day | Legacy: energy consumption in 1 day (uJ) | Optimized: occasion number in 1 day | Optimized: energy consumption in 1 day (uJ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measurements for cell selection | RF signal receiving | Monitor BCCH carrier | switch on or from non-service area into the service area | 8184.00 | 1.00 | 8184.00 | 1.00 | 8184.00 |
|  | Identify BCCH carriers in the list | BSIC Identification |  | 1751.77 | 1.00 | 1751.77 | 1.00 | 1751.77 |
|  | decode the full set of BCCH data of the serving cell | Rx - 4 radio block |  | 1127.23 | 1.00 | 1127.23 | 1.00 | 1127.23 |
| Measurements for cell re-selection | Monitor of received signal level of serving cell | Monitor BCCH carrier | DRX cycle | 66.00 | 5760.00 | 380160.00 | 5760.00 | 380160.00 |
|  | Monitor BCCH data of the serving cell | Rx - One radio block | once every 30s | 281.81 | 2880.00 | 811603.58 | 2880.00 | 811603.58 |
|  | Monitor all BCCH carriers as indicated by the BA list | Monitor BCCH carrier | DRX cycle | 1056.00 | 5760.00 | 6082560.00 | 480.00 | 506880.00 |
|  | identify BSIC of 6 strongest Note1 | BSIC Identification | once | 1051.06 | 1.00 | 1051.06 | 1.00 | 1051.06 |
|  | reconfirm BSIC of 6 strongest | BSIC Reconfirmation | once every 30s | 342.74 | 2880.00 | 987085.44 | 240.00 | 82257.12 |
|  | identify BSIC and decode BCCH data ( only SI3) for the new BCCH carrier which become one of the 6 strongest | BSIC Identification and Rx - One radio block | once every 1h | 456.98 | 24.00 | 10967.62 | 2.00 | 913.97 |
|  | decode BCCH data of 6 strongest ( only SI3) | Rx - One radio block | once every 5min | 1690.84 | 288.00 | 486962.15 | 24.00 | 40580.18 |
| PCH reading | read PCH | Rx - One radio block in serving cell | DRX cycle | 278.51 | 5760.00 | 1604199.17 | 5760.00 | 1604199.17 |

The power consumption comparison in packet idle mode between existing specification and optimized procedure are shown in Table 7.3-2. The power saving ratio is up to 59.5%.

Table 7.3-2: Power consumption comparison in packet idle mode per day

| legacy power consumption (uJ) | optimized power consumption (uJ) | power saving ratio |
| --- | --- | --- |
| 11,652,789.83 | 4,715,845.89 | 59.5% |

End of Changes

References

  - GP-150103, “Event-triggered Neighbour Cell Measurement”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN#65.

  - GP-150104, “pCR for Event-triggered Neighbour Cell Measurement”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN#65.