pCR for text inclusion in TR 45.820 :

C-UNB general description

revision of GP-150369

Introduction

This document provides text for section 7.3 of the Technical Report 45.820 revision 1.2.1, on the C-UNB candidate solution.

The proposed text is an introduction, which is intended to provide the reader with an overview of the key design principles for the C-UNB candidate solution. Next sections, dealing with technical description of the C-UNB solution and concept evaluation, are expected for next meetings.

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band

### General

Cellular IoT devices are a new category of MSs, which require extended coverage and improved battery lifetime compared to what is possible in 2G networks. Cooperative Ultra Narrow Band (C-UNB) is a clean slate approach that answers these requests.

In C-UNB, the extended coverage is achieved by means of ultra narrow band communications, which are possible because of the low volume of data transmitted by IoT devices. In base stations, signal processing increases the receiver sensitivity and hence increases the MCL, without using large transmission power in IoT devices

In order to improve the battery life time, C-UNB radio access technology (RAT) don't require IoT devices to be synchronized or attached to base stations before being allowed to send radio packets. On the contrary, the network access is based on random transmissions by devices. This kind of medium access is equivalent to ALOHA, which is know for its simplicity. Its drawback is the packet collisions that hamper the overall efficiency when the network load becomes high. To overcome this issue, the C-UNB RAT implements two mitigations techniques that are based on frequency diversity and space diversity.

Ultra narrow band transmission occupies a very small portion of spectrum in a 200kHz block, leaving space for other transmissions. Furthermore, there is no channelization of the 200kHz block: the center frequency of each transmission is selected randomly in a 200kHz block. The random selection of carrier frequency is a kind of "frequency diversity" that helps reducing the radio packet collisions. This feature can be implemented with limited complexity in IoT devices, because it does note require precise crystal oscillators.

The second feature, used to cope with collision on the air interface, is the "space diversity" given by the multiple receptions of radio packets in UL. All C-UNB base stations listen for the same 200kHz block continuously. As there are overlaps in base station coverage, the radio access network may receive multiple copies of the same radio packet. If an UL radio packet experiences collision or interference during reception in a given base station, the probability to have bad reception in all surrounding base stations is limited. De-duplication of received packets is performed in a dedicated C-UNB server, located in the core network.

The C-UNB RAT can be deployed in a Gb architecture, provided BTSs implement software defined radios. A C-UNB software package is loaded in the BTSs. The first part of this software manages the C-UNB air interface. The second part re-routes C-UNB radio packets to a C-UNB server. This server terminates the protocol stack of Gb-based MSs and forward user data to the IoT devices.

End of Changes