# EC-GSM, Software update/reconfiguration, system evaluation

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

According to the performance evaluation methodology in [2], the impact on system capacity when running software update/reconfiguration traffic should be evaluated. In this contribution system level simulations of software update/reconfiguration for EC-GSM are presented. The setup follows the agreed framework in subclause 5.2 and Annex E in [2].

# EC-GSM basics

## Coverage classes

There are in total six coverage classes defined. These are also used by the system level simulations. The determination of coverage class is based on the received signal strength. Estimation errors of the received signal strength are modeled, see further section 3.1.4.

## Timeslot allocation

Allowed utilization of the downlink TS per coverage class are:

- Users that are in normal coverage will only be allocated one TS per TTI.

- Users in extended coverage performing two blind repetitions will be allocated two TS per TTI.

- Users in extended coverage performing four or more blind repetitions will be allocated four TS per TTI.

# Simulations

## Simulation assumptions

### Traffic model

Traffic is generated according to the software update/reconfiguration traffic model in [2].

### System parameters

The system level simulation assumptions are according to [2]. Specific assumptions are shown in Table 1.

Table 1: Simulation assumptions.

| Parameter | Value |
| --- | --- |
| General |  |
| Simulation length | > 62000 finished software update/reconfigurations per scenario |
| System size | 192 cells |
| Direction | UL and DL |
| Frequency band | 900 MHz |
| Layer | BCCH |
| Frequency re-use | 12 |
| Number of TRX per cell | 1 (BCCH) |
| BTS antenna diversity | MRC |
| BTS output power | 43 dBm |
| Signal level/CC estimation error | N(0,4) (see section 3.1.4) |
| CIoT parameters |  |
| EC-PDTCH timeslots per cell | 4 |
| CIoT arrival rate per cell and second | 0.0034(note 1) |
| Fixed UL allocation | On |
| MCSs | MCS-4, MCS-1 |
| Chase combining | On |
| Incremental Redundancy | Off |
| Power control | Off |
| IP header compression | Off/On |
| Device output power | 23/33 dBm |
| BPL model(note 2) | 100 % CIoT devices subject to BPL |
| BPL scenario and inter-site correlation | Scenario 1, correlation 0.50 Scenario 1, correlation 0.75 Scenario 2, correlation 0.50 Scenario 2, correlation 0.75 |
| Device timeout | 40 seconds |
| NOTE 1: This corresponds to 52547 devices in each cell, accessing on average every 180 days. For simulation technical reasons, a 500 times higher arrival rate was actually simulated, and the resource utilization downscaled by the same factor in post-processing. NOTE 2: According to model in [2]. |  |

### Coverage classes

Table 2 gives the details of the coverage classes.

Table 2. MCS and number of blind transmissions per coverage class

| Coverage Class | Number of blind transmissions for PDTCH and PACCH | MCS for PDTCH |
| --- | --- | --- |
| CC1 | 1 | MCS-4 |
| CC2 | 1 | MCS-1 |
| CC3 | 2 | MCS-1 |
| CC4 | 4 | MCS-1 |
| CC5 | 8 | MCS-1 |
| CC6 | 16 | MCS-1 |

### Cell selection and coverage class estimation

Based on the findings in [3] the error in the signal strength estimation can be modeled by a normal distribution with standard deviation of 4 dB. This is modeled by applying an independent estimation error, according to N(0,4 dB), to each base station. This implies that some users will not select the optimum serving cell, and also not the most appropriate coverage class (the one that minimizes resource utilization). That is, some cells will appear stronger or weaker than they actually are. The device always selects what is believed to be the strongest cell. Effectively this increases the interference levels in the network, as well as the resource utilization.

### Control signaling

Packet downlink ACK/NACK (PDAN) is sent on EC-PACCH/U to (negatively) acknowledge data sent in the DL. In the simulations its performance is pessimistically modeled with MCS-1 using the same number of blind transmissions as EC-PDTCH/U (but without HARQ retransmissions).

## Results

TS utilization represents the average amount of EC-PDTCH/EC-PACCH UL and DL resources (timeslots) required per cell in the system.

Failed attempts represent the percentage of the attempts that were not successful, i.e. did not manage to get the report through within 40 seconds. Considering the number of reports simulated (62000, see Table 1) a lowest representable percentage of failed attempts of 0.1% has been assumed (representing around 60 error events).

The results are summarized in Table 3 and Table 4 without and with IP header compression, respectively:

Table 3: TS utilization and failed attempts without IP header compression.

| CIoT MS output power [dBm] | BPL Scenario | BPL inter-site correlation coefficient | TS utilization UL | TS utilization DL | Failed attempts [%] |
| --- | --- | --- | --- | --- | --- |
| 23 | 1 | 0.50 | 0.0002 | 0.0012 | ≤ 0.1% |
| 23 | 2 | 0.50 | 0.0003 | 0.0013 | ≤ 0.1% |
| 23 | 1 | 0.75 | 0.0002 | 0.0012 | ≤ 0.1% |
| 23 | 2 | 0.75 | 0.0003 | 0.0013 | ≤ 0.1% |
| 33 | 1 | 0.50 | 0.0002 | 0.0012 | ≤ 0.1% |
| 33 | 2 | 0.50 | 0.0002 | 0.0013 | ≤ 0.1% |
| 33 | 1 | 0.75 | 0.0002 | 0.0012 | ≤ 0.1% |
| 33 | 2 | 0.75 | 0.0003 | 0.0013 | ≤ 0.1% |

Table 4: TS utilization and failed attempts with IP header compression.

| CIoT MS output power [dBm] | BPL Scenario | BPL inter-site correlation coefficient | TS utilization UL | TS utilization DL | Failed attempts [%] |
| --- | --- | --- | --- | --- | --- |
| 23 | 1 | 0.50 | 0.0002 | 0.0012 | ≤ 0.1% |
| 23 | 2 | 0.50 | 0.0003 | 0.0012 | ≤ 0.1% |
| 23 | 1 | 0.75 | 0.0002 | 0.0011 | ≤ 0.1% |
| 23 | 2 | 0.75 | 0.0003 | 0.0012 | ≤ 0.1% |
| 33 | 1 | 0.50 | 0.0002 | 0.0012 | ≤ 0.1% |
| 33 | 2 | 0.50 | 0.0002 | 0.0013 | ≤ 0.1% |
| 33 | 1 | 0.75 | 0.0002 | 0.0011 | ≤ 0.1% |
| 33 | 2 | 0.75 | 0.0003 | 0.0012 | ≤ 0.1% |

# Conclusions

The impact on system capacity when running software upgrade/reconfiguration traffic has been evaluated following the agreed evaluation framework and traffic model. The resource utilization is found to be 0.0011-0.0013 downlink timeslots and 0.0002-0.0003 uplink timeslots.

Expressed as a proportion of the total available resource per cell (i.e., four EC-PDTCH timeslots), this corresponds to approximately 0.03 % of the downlink resources and less than 0.01 % of the uplink resources.

# References

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

- TR 45.820 V1.4.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (Release 13)”

- GP-150431, “EC-GSM, DL signal level and coverage class estimation”, source Ericsson LM. GERAN#66.

This applies only to the UL since a BCCH carrier is modelled in these simulations.

L2S for EC-PACCH has not been implemented.