Narrowband LTE – Exception report latency evaluation

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC) was approved [1].

In [3], a design, i.e., narrow band LTE (NB LTE) based on re-using the current LTE system but using a 200 KHz bandwidth is prosed. In this contribution, followed methodology given in [2], the evaluation results of exception report delivery latency is presented.

# Procedures of exception report

In Figure 1, the different steps of delivering an exception report are depicted. The time used in each step varies, depending on the coverage class an MS belongs to.  Note that a Gb based architecture is assumed for the latency evaluation, since that is what has been agreed to be assumed in this GERAN study item.

Figure 1 Illustration of each step in delivering exceptional report

## Synchronization time

The time used for a device to synchronize to the network is given in [4], and summarized in Table 1. The 90% confidence value is used in the latency evaluation.

Table 1 Time to synchronize to the network

|  | Coupling loss [dB] |  |  |
| --- | --- | --- | --- |
|  | 144 | 154 | 164 |
| Tsync(ms) | 495 | 540 | 1005 |

## Time to obtain primary system information

The average reception times for PSS/SSS and M-PBCH at each coverage level are presented in [7]. For MIB reception time it is assumed that 1/1/2 code blocks are needed for reception at 144/154/164 dB coupling loss. Each code block is transmitted in the first M-subframe of each 60 ms M-frame [7]. In the worst case, assuming an MS get synchronized to the network in the beginning of an M-frame, it needs to wait for 54 ms (9 M-subframes) to begin to read the first MIB. The time used to obtain MIB is given in Table 2 for different coupling loss values.

Table 2 Time to obtain Master Information Block

|  | Coupling loss [dB] |  |  |
| --- | --- | --- | --- |
|  | 144 | 154 | 164 |
| TMIB(ms) | 54 + 6 = 60 | 54 + 6 = 60 | 54 + 60 + 6 = 120 |

## Time to send PRACH

The time used to send PRACH varies according to the coverage class an MS belongs to. In order to simplify the calculation, it is assumed that the MS sends PRACH in the next available PRACH slot. The PRACH design of NB LTE is given in [5][8], and the time used to send PRACH is summarized in Table 3, which takes into account 30 ms waiting time after reading the MIB to the next available PRACH slot.  This assumes that a PRACH slot occurs in the middle of each frame. Shorter waiting time can be expected with other PARCH configurations.

Table 3 Time to send PRACH

|  | Coupling loss [dB] |  |  |
| --- | --- | --- | --- |
|  | 144 | 154 | 164 |
| TPRACH(ms) | 30 + 4 = 34 | 30 + 12 + 48 + 12= 102 | 30 + (12 + 48)x5 + 12 = 342 |

## Time to get UL assignment

In NB-LTE, the same random access procedure as LTE is followed. The MS initiates the procedure by sending its RA preamble (Msg1). Upon successful reception the eNB replies with a random access response (RAR, Msg2) carried on the M-PDSCH. The RAR comprises an UL grant which the MS uses to perform the subsequent UL transmission on M-PUSCH (Msg3). If the UL grant provided in the RAR is sufficient to accommodate the data available in the MS’ UL buffer (here an exception report) the procedure ends upon successful reception of the message. If, on the other hand, the UL resource assignment is not enough to accommodate the exception report, the MS will include a buffer status report in the Msg3 based on which the eNB knows how much buffer is left in the MS’ queue. Based on this BSR the eNB will issue one or more subsequent grants.

Upon receiving a RA preamble the eNB will aim to send a RAR. In the undesirable and unlikely case that the eNB had just scheduled all downlink resources to a MS’ M-PDSCH it may not have resources to transmit M-PDCCH. In accordance with the repetition levels chosen in [9], we assume a repetition level of at most 36 M-subframes, i.e., 216 ms and add this time to our latency evaluation. This is the worst case assumption, as preferably, the eNB should avoid such situations by reserving some resources for M-PDCCH.

Once started, the transmission of the M-EPDCCH takes between one M-subframe and several M-subframes depending on the coverage class that an MS belongs to [6]. Due to a forward scheduling mechanism an uplink grant received on M-PDCCH in M-subframe n becomes valid in M-subframe n+1. In Table 4, the time to receive UL grant is summarized for different MS coverage classes. Note, in the evaluation in [6], 3 ms is needed for PDCCH for MSs in basic coverage (144 dB coupling loss). However, due to the forward scheduling design, a 6 ms M-PDCCH transmission time, which includes 3 ms waiting time, is assumed here for MSs in the basic coverage.

Table 4 Time to get UL assignment

|  | Coupling loss [dB] |  |  |
| --- | --- | --- | --- |
|  | 144 | 154 | 164 |
| TULAssigment(ms) | 216 + 6= 222 | 216 + 12= 228 | 216 + 96 = 312 |

## Sending UL report

Once the MS receives the UL assignment, it sends the exception report on the scheduled UL resources. For exception report, according to [2], 20 bytes application report, 65 bytes upper layer protocol header, 15 bytes SNDCP/LLC/MAC/CRC overhead is assumed. Therefore, in total 100 bytes are assumed. The UL data rate of NB LTE is evaluated in [9]. Table 5 summarizes the time to send the UL payload in different coupling loss values. In addition to the UL report sending time given in [9], one M-subframe response time at the MS is assumed. As explained above, two subsequent transmissions may be needed if the eNB does not grant sufficiently many resources in the first UL grant. For this case, we assume in the summary below that two UL messages are transmitted that both experience the delay as shown in Table 5. This is obviously a worst case assumption as a first, smaller UL transmission would have taken less time.

Table 5 Time to send exception report

|  | Coupling loss [dB] |  |  |
| --- | --- | --- | --- |
|  | 144 | 154 | 164 |
| TULdata(ms) | 24 + 6 = 30 | 384 + 6 = 390 | 1920 + 6 = 1926 |

## Time to receive acknowledgement

If the transmission is errors or lost, they network will schedule the MS to do a re-transmission. In the case of NACK, the network sends an acknowledgement to the MS on the next available M-PDCCH assuming the MS remains in the same coverage class as sending the UL assignment. The same M-PDCCH size is used the initial UL assignment. Therefore, it takes the same amount of time as sending UL assignment as to send the acknowledgement.

## Total time to send exception report

The time required to successfully deliver an exception report with 90% confidence is given in Table 6 and Table 7 as the initial message BLER for UL report is less than 10%. Two cases, as outlined in above, are presented.

Table 6 Exception report delivery time with 90% confidence, case 1

| Coupling loss (dB) Activity | 144 | 154 | 164 |
| --- | --- | --- | --- |
| Tsync(ms) | 495 | 540 | 1005 |
| TMIB(ms) | 60 | 60 | 120 |
| TPRACH(ms) | 34 | 102 | 342 |
| TULAssigment(ms) | 222 | 228 | 312 |
| TULdata(ms) | 30 | 390 | 1926 |
| Total time (ms) | 841 | 1320 | 3705 |

Table 7 Exception report delivery time with 90% confidence, case 2

| Coupling loss (dB) Activity | 144 | 154 | 164 |
| --- | --- | --- | --- |
| Tsync(ms) | 495 | 540 | 1005 |
| TMIB(ms) | 60 | 60 | 120 |
| TPRACH(ms) | 34 | 102 | 342 |
| TULAssigment(ms) | 222 | 228 | 312 |
| TULdata(ms) | 30 | 390 | 1926 |
| TULAssigment(ms) | 222 | 228 | 312 |
| TULdata(ms) | 30 | 390 | 1926 |
| Total time (ms) | 1093 | 1938 | 5943 |

In the case of the exception report needs to be re-transmitted, i.e., a negative acknowledgment is received, then the network needs to send an UL assignment for the MS to re-transmit the exceptional report. This corresponds to the 99% confidence of the successful delivery of the exceptional report, as a less than 10% BLER is assumed for each transmission. Table 8 and Table 9 summarize the transmission time of the two cases. Note, for case 2, only the one re-transmission is needed, as if the UL in the first transmission is wrong, the entire message can be re-transmitted in the second UL transmission.

Table 8 Exception report delivery time with 99% confidence, case 1

| Coupling loss (dB) Activity | 144 | 154 | 164 |
| --- | --- | --- | --- |
| Tsync(ms) | 495 | 540 | 1005 |
| TMIB(ms) | 60 | 60 | 120 |
| TPRACH(ms) | 34 | 102 | 342 |
| TULAssigment(ms) | 222 | 228 | 312 |
| TULdata(ms) | 30 | 390 | 1926 |
| TULAssigment(ms) | 222 | 228 | 312 |
| TULdata(ms) | 30 | 390 | 1926 |
| Total time (ms) | 1093 | 1938 | 5943 |

Table 9 Exception report delivery time with 99% confidence, case 2

| Coupling loss (dB) Activity | 144 | 154 | 164 |
| --- | --- | --- | --- |
| Tsync(ms) | 495 | 540 | 1005 |
| TMIB(ms) | 60 | 60 | 120 |
| TPRACH(ms) | 34 | 102 | 342 |
| TULAssigment(ms) | 222 | 228 | 312 |
| TULdata(ms) | 30 | 390 | 1926 |
| TULAssigment(ms) | 222 | 228 | 312 |
| TULdata(ms) | 30 | 390 | 1926 |
| TULAssigment(ms) | 222 | 228 | 312 |
| TULdata(ms) | 30 | 390 | 1926 |
| Total time (ms) | 1345 | 2556 | 8181 |

# Conclusions

This contribution presents latency evaluation of delivering the exceptional report of the NB-LTE system. It can be seen that the NB-LTE can deliver the exception report to the network within 10 seconds.

# References

  - GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, VODAFONE Group Plc., GERAN#62.

  - 3GPP TR 45 820, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things” v1.4.0.

  - GP-150779	“Narrowband LTE – Concept description” source: Ericsson LM, Nokia Networks, AT&T, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, GERAN#67.

  - GP-150785	“Narrowband LTE – Cell search performance evaluation” source:	Ericsson LM, Nokia Networks, GERAN#67.

  - GP-150788	“Narrowband LTE – Random access performance evaluation” source:	Ericsson LM, Nokia Networks, GERAN#67.

  - GP-150789	“Narrowband LTE – (E)PDCCH coverage performance evaluation” source:	Ericsson LM, Nokia Networks, GERAN#67.

  - GP-150780 “Narrowband LTE – Broadcast channel design and performance” source:	Ericsson LM, Nokia Networks, GERAN #67.

  - GP-150781 “Narrowband LTE – Random access design” source: Ericsson LM, Nokia Networks, GERAN#67.

  - GP-150782 “Narrowband LTE – PDSCH and PUSCH design and performance” source: Ericsson LM, Nokia Networks, GERAN#67