C-UNB for Cellular IoT

-

Battery Life Evaluation

revision of GP-150714

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. The Cooperative Ultra Narrow Band (C-UNB) is a clean slate solution, whose general concepts are described in [2], and radio frames and link layer are defined in [3], [4] and [5].

This document evaluates the battery life of C-UNB MSs.

It is a revision of GP-150714, where changes are in red.

# Evaluation methodology

## Common asumptions

The battery life evaluation for C-UNB MS is based on methodology detailed in [7] and two assumptions, specific to the C-UNB solution:

MAR are transmitted with two blind repetitions at physical layer, which gives a BLER of 10%

ack-nack packet in DL are transmitted right after the loop-back delay (i.e. with no extra delay due to overload condition in DL).

In most cases, the second assumption is valid because the large overlap of base stations allows the C-UNB server to select the overlapping base station, which is available for DL transmission.

## MAR segmentation

In [7], the battery life evaluation assumes two lengths for the MAR: 50 and 200 bytes at SNDCP level. In C-UNB, such data packets are segmented and encapsulated as stated in table 1. MAC-PDU size and total number of transmission give the inputs for power consumption.

Table 1: MAR mapping in C-UNB PHY-PDU  (all sizes in bytes)

| MAR size | Number of segments | Segment size | LLC-PDU size | MAC-PDU size | Total # of PHY-PDU | PHY-PDU duration |
| --- | --- | --- | --- | --- | --- | --- |
| 50 | 2 | 25 | 26 | 42,5 | 4 | 1 360ms |
| 200 | 7 | 29 | 30 | 46,5 | 14 | 1 488ms |

## Energy consumption profile

Table 2 gives electrical parameters for energy consumption analysis. Figures for Tx and Rx power are those from off-the-shelves sub-gigaHertz chipsets or what is expected in the next generation of chipsets where architecture and power consumption will be optimized for ultra narrow band transmission. For example, a power consumption of 450mW at +24dBm is expected to be achievable the following way: 100mW for transceiver and 350mW for Power Amplifier (PA) with 71% efficiency, which is expected to be available with UNB optimization.

Table 2: Key electrical parameters for energy consumption analysis

| (1) Battery capacity (Wh) | (2) Battery power during Tx (mW) | (3) Battery power for Rx (mW) | (4) Battery power in Processing (mW) | (5) Battery power in Power Save (mW) |
| --- | --- | --- | --- | --- |
| 5 | 450 @ +24dBm 100 @ +14dBm 30 @ +4dBm | 40 | 10 | 0,015 |

Table 3: Key timing parameters for energy consumption analysis (in ms)

| (6) trigger event | (7) Processing time from PS to Tx mode | (8) Beacon reception window | (9) Processing time between two Tx (ms) | (10) Transmit time for one PHY-PDU | (11) ack/nack reception window |
| --- | --- | --- | --- | --- | --- |
| 0 | 50 | 70 to 200 | 25 | see table 1 | 875 |

In table 3, the duration of the reception window for beacon checking takes into account the real time clock drift between two consecutive transmissions of periodic MAR.

Figure 1: Energy consumption model for C-UNB devices

# Results

Table 4 gives the battery life of C-UNB devices with a combination of two packet sizes, two reporting intervals and three coverage classes.

Table 4: Battery life (in years)

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia.

[2]	GPC150364, "Cooperative Ultra Narrow Band - General Description", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[3] 	GPC150309, "C-UNB technology for Cellular IoT - Downlink Physical Layer Design", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[4] 	GPC150310, "C-UNB technology for Cellular IoT - Uplink Physical Layer Design", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[5] 	GPC150311, "C-UNB Link Layer Aspects", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[6]	GP-150059, "C-UNB technology for Cellular IoT - Performance evaluation", SIGFOX, GERAN#65 meeting, Shanghai, 09-12 March 2015, PRC

[7]	TR 45.820, "3GPP/GERAN; Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things", v1.3.1, June 2015