pCR for text inclusion in TR 45.820 :

C-UNB Link Layer Aspects

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.4 of the Technical Report 45.820 revision 1.4.0.

The proposed text describes the link layer aspects of the C-UNB solution.

This pCR is a revision of pCR GP-150912 for inclusion of changes included in the corresponding revised contribution.

Proposed text for the TR

Start of Changes

## Cooperative Ultra Narrow Band

### General description

### Downlink physical layer design

### Uplink physical layer design

### Link layer aspects

#### Downlink link layer formats and structures

##### Format of downlink MAC-PDUs

In C-UNB, downlink MAC-PDUs are optimized considering the fact that they are transmitted in reponse to up link transmission. Figure 7.4.4.1 illustrates the MAC-PDU format in C-UNB downlink. The various fields are detailed in the following subclauses.

Figure 7.4.4.1: Format of a MAC-PDU frame in C-UNB downlink transmission

###### Header

Downlink header is 56 bits. It consists in:

a preamble for frame detection and bit rate synchronization,

a frame type,

a payload length,

acknowledgement bits.

###### Payload

The payload of a MAC-PDU is 0-34 byte length. It carries application data and, when necessary, the link layer header (LL-header) for segmentation/re-assembly.

###### Authentication

The authentication field is a hash, computed with an adaptation of the common C-MAC algorithm. It uses the 128-bit secrete key of the device plus:

the device identifier received in corresponding uplink packet,

the sequence number in the corresponding uplink packet,

the payload field.

This field is used to sign a downlink packet and also to identify the destination device implicitly (see figure 4): the probability to have many devices listening on a given frequency at a given time is very limited; hence a 16-bit hash is enough to authenticate the destination.

When a DL packet is trigged by a segmented UL packet, its authentication field uses the last sequence number transmitted in UL.

When a DL application packet is segmented at link layer, all its segments use the same sequence number, i.e. the sequence number corresponding to the UL packet that triggered the downlink transmission.

###### FCS

The frame check sequence in C-UNB downlink consists in a CRC-8. It checks the MAC-PDU after error correction, before further decoding.

###### ECC

This field is a 32-bit Error-Correction Code. It uses BCH code applied to the four first fields: header (except preamble), Payload, Authentication and FCS. Given the length of DL MAC-PDU, the ECC is expected to add 1 dB of processing gain in the DL MCL.

##### Segmentation & reassembly in DL

The present document defines the maximum application packet to 2 000 bytes, with addition of a protocol header of 65 bytes (extended) or 29 bytes (compressed). Given the small size of C-UNB radio packets, it is proposed to have a segmentation/re-assembly mechanism with 32-bytes of application data and 2 bytes for segment numbering (see figure 7.4.4.2 and 7.4.4.3).

Figure 7.4.4.2: segmentation in C-UNB downlink

Figure 7.4.4.3: LL header format in downlink

This implementation gives a maximum size of the application data of 4 096 bytes, which is enough for the data packets expected in CIoT.

##### Downlink nominal throughput

The present document defines a target throughput of at least 160 bps at SNDCP interface. As C-UNB does not implement the legacy SNDCP and LLC layers, the nominal throughput is evaluated with formula (1). In this formula, "time over the air" includes radio packet duration, gap between repetitions and four repetitions of each radio packet. Nominal throughput at LL layer (i.e. equivalent to the legacy SNDCP layer) is given in table 7.4.4.1.

Table 7.4.4.1: Nominal throughput at SNDCP-equivalent layer in DL

| Type of DL packet | Nominal throughput |
| --- | --- |
| single data packet | 104 bps |
| segmented data packet | 99 bps |

##### Format of the beacon channel

The beacon channel is broadcasted by base stations without being a response to a uplink packet. Therefore the format of beacon channel packets is different from ad'hoc micro-channel packets. Figure 4.4.4.4 illustrates the beacon channel format.

Figure 7.4.4.4: Format of beacon channel packets

The various fields of the beacon channel packets are:

a header, which consists in a preamble and a specific frame type (i.e. set to "beacon channel"),

a C-UNB network ID (15 bits)

a C-UNB cell ID (15 bits)

a Sys Info Group type (6 bits), which defines the payload content

the total number of system information groups (4 bits) that are currently on the beacon channel

a payload for carrying the various system information elements

a Group Sequence number (5 bits) that helps tracking the changes of system information groups,

a FCS, which is 8 bits CRC

a ECC, which is the same BCH field as in ad'hoc micro-channel packets

The payload contains system information elements that are grouped together. 32 different system information groups can be defined, which seems to be sufficient to carry all required system information elements.

For example, a first system information group can be defined for the C-UNB radio parameters such as:

loop-back delay (in 0.1s increment): 8 bits

preferred regular check delay for DL message (i.e. poll delay): 4 bit index for a list of values in minutes (1,2,5,10,20,30,60,120,240,360,480,720,1440,2880,)

Another system information group can control the network access with the following system information elements:

barred access

access control categories

When considering network sharing, various system information elements can be defined to broadcast other C-UNB network ID.

The various system information groups have the same priority and are broadcast with a round-robin scheduling.

The precise description of system information groups and their coding is FFS.

#### Uplink link layer formats and structures

##### Format of uplink MAC-PDU

Figure 7.4.4.5 illustrates the MAC-PDU format in C-UNB uplink. The various fields are detailed in the following subclauses.

Figure 7.4.4.5: Format of a MAC-PDU frame in C-UNB uplink transmission

###### Header

Uplink header is 40 bits. It consists in:

a preamble for frame detection and bit rate synchronization,

a frame type,

a frame length,

acknowledgement flags,

a repetition counter (value 1, 2 or 3) tagging the actual repetition number of MAC-PDUs.

Note: the number of repetition of a given MAC-PDU depends on the expected success rate. Two repetitions are for normal MAR, whereas three repetitions are for exception reports.

###### Sequence counter

The uplink sequence counter is 12 bits. It is incremented for each new link layer packet (i.e. LL-PDU) sent by a device. The sequence counter is used for authentication purpose.

###### Identifier

This field is 40 bits long. It contains the device unique identifier set during personalization of the device. Each identifier is unique in a C-UNB system.

###### Payload

The payload is 0-32 bytes. Payload length is defined by a 5-bit length field in the MAC-PDU header. Empty payload corresponds to Ack packet; they have a specific frame type.

###### Authentication

The authentication field is a 16 bits hash, used to authenticate the MAC-PDUs received by the network. The 16 bits of the authentication field are generated using a proprietary implementation of the well-known C-MAC algorithm. It uses a 128 secrete key. Authentication check is performed in the core network and not in the base stations to avoid secrete key transmission on the back-haul network. Figure 7.4.4.6 illustrates the authentication process in uplink and the corresponding process in downlink.

Figure 7.4.4.6: Authentication of UL and DL packets

###### FCS

The frame check sequence in C-UNB uplink consists in a CRC-8. In reception, it is used to check MAC-PDU consistency before transmission to the core network.

###### ECC

The last field of an uplink MAC-PDU is an Error-Correcting Code of 16 bits. It used Reed-Solomon algorithm and is evaluated over the whole MAC-PDU except the preamble.

##### Segmentation & reassembly in UL

The CIoT requirements define Mobile Autonomous Reports with 20-200 byte length. Given the fact that only small radio packets are affordable for the C-UNB radio access technology, it is proposed to have a segmentation/re-assembly mechanism based on 31 blocks of data with a LL header of 8 bits: 4 bits for segment numbering and 4 bits for total number of segments (see figure 7.4.4.7). These values set the maximum block size of application data to 496 bytes.

Figure 7.4.4.7: LL-header in C-UNB uplink

##### Uplink nominal throughput

The nominal throughput at SNDCP-equivalent layer in uplink is given in table 2. Evaluation is made for two types of data packet (un-segmented and segmented) and for various repetitions rates in uplink.

Table 7.4.4.2: Nominal throughput at SNDCP-equivalent layer in UL (in bps)

| Type of data packet | 1 PHY-PDU (no repetition) | 2 PHY-PDUs (1 repetition) | 3 PHY-PDUs (2 repetitions) |
| --- | --- | --- | --- |
| un-segmented data packet | 163 | 81 | 54 |
| segmented data packet | 160 | 80 | 53 |

#### Link Layer procedures

A new link layer and MAC layer has been designed for the C-UNB solution. Packet formats are optimized for ultra narrow band transmission and procedures are simplified or omitted to reduce the amount of transmission and reception in the mobile stations. For example, there are no attachment or cell selection procedures as in legacy GSM/GPRS network, because all base stations listen for the full CIoT 200kHz continuously. The random access procedure is also simplified because C-UNB mobile stations transmit without waiting for any king of grant from the network.

The following subclauses describe the ack/nack procedures that are implemented in C-UNB link layer.

Beacon channel listening and transmit power adaptation are radio resource management procedure. They are described in radio resource management subclauses.

##### Uplink acknowledgement

Upon reception of an application PDU, the C-UNB link layer generates the corresponding LL-PDUs and MAC-PDUs, sets the Ack Request Flag and, if ack is reqested, it enters reception state after the loop-back delay.

Figure 7.4.4.8 depicts the ack procedure in case of a single packet, whereas figure 7.4.4.9 is for segmented APP-PDU acknowledgement.

Figure 7.4.4.8: Ack procedure for single UL packet

Figure 7.4.4.9: Ack procedure for segmented UL packets

Acknowledgement of a segmented UL APP-PDU is implemented with all Ack flags transmitted in one DL packet (see figure 8), tagged with a "multiple ack" frame type.

Figure 7.4.4.10: DL packet content for multiple ack

Note: this acknowledgement procedure assumes that the C-UNB server can decode at least one packet properly. If it is not the case, the C-UNB server is not able to evaluate the frequency for the DL transmission and hence, it can't transmit a control message with NACK flags. The MS uses a timer to detect that case and to trigger or not a new transmission of its UL packets.

##### Downlink acknowledgement

The mechanism for DL packet acknowledge is very similar to what is implemented in UL.

APP-PDU in DL are transmitted in one or several DL LL-PDUs, are received by a mobile station and then acknowledged with a UL packet transmitted by the mobile station. Figure 7.4.4.11 illustrates the DL ack procedure in case of a segmented DL APP-PDU. The LL-PDU that generates the DL transmission is not necessarily related to the content of the DL APP-PDU. It can be any UL packet, i.e. user plane or regular check (pull message).

Figure 7.4.4.11: Ack procedure for a segmented DL APP-PDU

End of Changes