# Narrowband LTE – Cell search performance evaluation

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see  [1].

The study is open to both a non-legacy based design, and/or a backward compatible evolution of GSM/EDGE.

A non-legacy based solution called Narrowband LTE (NB LTE) has been proposed in [2] in order to maintain synergies with LTE. A cell search mechanism for NB LTE has been proposed in [3]. In this contribution, we provide performance evaluations of the NB LTE cell search procedure for different scenarios of interest.

# Background

Cell Search in NB LTE uses two sequences, the PSS (Primary Synchronization Sequence) and the SSS (Secondary Synchronization Sequence), repeated every 15 ms within an M-frame as shown in Figure 1. A detailed description of the design is provided in [3]. 3 PSS and 168 SSS are used giving rise to 504 unique cell identities. Each cell identity belongs to a specific cell identity group determined by the SSS and an identifier within the cell identity group determined by the PSS.

Figure 1 : PSS and SSS in NB LTE.

The structure of the PSS enables a correlation based detector to estimate the subframe timing, the carrier frequency offset and the cell identity within a cell identity group. After the timing has been determined and the frequency offset corrected, the SSS is then used to obtain the M-frame timing and the cell identity group. While the correlation based detector for the PSS operates in the time domain, the one for the SSS does so in the frequency domain.

# Simulations

## Simulation assumptions

The simulation assumptions are taken from [4] and tabulated in Table 2. Three different scenarios are investigated, corresponding to the presence of 0, 1 or 2 interferers. All the interferers are assumed to be transmitting at the same power level. The results are provided for these scenarios at three different MCLs corresponding to 164 dB, 154 dB and 144 dB.

Table 2. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 900 MHz |
| Channel Model | TU 1 Hz [4] |
| Subcarrier Spacing | 15 kHz |
| SNR | -4.6 dB, 5.4 dB and 15.4 dB1 |
| Sampling Frequency (Fs) | 1.92 MHz |
| Cyclic Prefix | 10 samples for the 1st and 7th OFDM symbol within a subframe, 9 samples for the rest |
| Timing offset | Uniformly distributed between [0,15) ms in steps of size 1/Fs |
| Antenna Configuration | 1 Tx, 1 Rx [4] |
| Frequency Offset | Randomly generated as one of the values {-18 kHz, 18 kHz} |
| Number of realizations | 1000 |
| Max. M-frames used for detection | 40 |
| NOTE1: -4.6 dB corresponds to an MCL of 164 dB. |  |

## Initial Cell Search: Timing and Frequency Offset Estimation

In the initial cell search no time reference is present and the frequency offset is assumed to be 18 kHz. The device will search for all possible cells at the chosen frequency. In order to limit the search time, a successful detection corresponds to the case where the correlation peak from the PSS exceeds the average correlation by a certain threshold. The detection probabilities are provided in Table 3 for different MCLs and varying number of interferers. Irrespective of the number of interferers, or the investigated coupling loss, the detection probability is 100%.

Table 3 : Detection probability for initial cell search

| Number of Interferers | Detection Probability at different coupling loss |  |  |
| --- | --- | --- | --- |
|  | 164 dB | 154 dB | 144 dB |
| 0 | 100 % | 100 % | 100 % |
| 1 | 100 % | 100 % | 100 % |
| 2 | 100 % | 100 % | 100 % |

The CDF of the synchronization times for timing offset estimation are provided in Figure 2, Figure 3 and Figure 4 for 164 dB, 154 dB and 144 dB MCL respectively. Note that the synchronization time is given in terms of multiples of 15 ms intervals, which is the repetition interval of the PSS. We observe that at higher coupling loss (lower SNR), the performance of synchronization is worst in the presence of no interferers in the system, and is best when equally strong signals from 3 BSs are simultaneously present. The reason is because in the presence of three cells, the mobile station has an improved chance of getting a good signal from one of the three transmitting cells as opposed to only one cell, resulting in a faster synchronization. On moving to lower MCL (higher SNR), this effect is somewhat reduced because the gains due to macro diversity are offset due to the presence of interfering signals from other cells.

Figure 2 : CDF of the time required for signal detection at 164 dB MCL.

Figure 3 : CDF of the time required for signal detection at 154 dB MCL.

Figure 4 : CDF of the time required for signal detection at 144 dB MCL.

The timing offset is estimated at the time the signal is detected and requires accumulation of the correlation over multiple 15 ms frames (A 15 ms frame consists of 15 subframes). The CDFs of the residual timing estimation error are provided in Figure 8, Figure 9 and Figure 10 for the three MCLs of 164 dB, 154 dB and 144 dB respectively. We observe that the timing estimation error is within the length of the cyclic prefix for all the realizations.

Figure 8: CDF of the timing estimation error after signal detection at 164 dB MCL.

Figure 9 : CDF of the timing estimation error after signal detection at 154 dB MCL.

Figure 10 : CDF of the timing estimation error after signal detection at 144 dB MCL.

After the timing offset has been found, an accumulation over multiple 15 ms frames is again required for accurate frequency offset estimation. The CDFs of the time required for frequency offset estimation are provided in Figure 5, Figure 6 and Figure 7  for MCLs of 164 dB, 154 dB and 144 dB respectively.

Figure 5 : CDF of the time required for frequency offset estimation at 164 dB MCL.

Figure 6 : CDF of the time required for frequency offset estimation at 154 dB MCL.

Figure 7 : CDF of the time required for frequency offset estimation at 144 dB MCL.

The CDFs for the residual frequency offset estimation error are provided in Figure 11, Figure 12 and Figure 13 for the different MCLs. We observe that the frequency offset estimation error is within +/- 50 Hz for 99 % of the realizations.

Figure 11 : CDF of the frequency offset estimation error at 164 dB MCL.

Figure 12 : CDF of the frequency offset estimation error at 154 dB MCL.

Figure 13 : CDF of the frequency offset estimation error at 144 dB MCL.

## Initial Cell Search: Cell ID and Frame Timing Detection

The cell ID detection as well as the M-frame timing is performed using the SSS after the subframe timing and frequency offsets are known and compensated for. The corresponding CDFs of the synchronization time for cell ID detection are provided in Figure 14, Figure 15 and Figure 16 for the three different MCLs of 164 dB, 154 dB and 144 dB respectively. Note that a successful detection of the cell ID also results in the correct M-frame timing, because of the inherent design of the SSS.

Figure 14: CDF of the time required for cell ID detection at 164 dB MCL.

Figure 15 : CDF of the time required for cell ID detection at 154 dB MCL.

Figure 16 : CDF of the time required for cell ID detection at 144 dB MCL.

## Initial Cell Search : Total Synchronization Time

A comparison of the total synchronization time for initial cell search for a desired percentage of the mobile stations is provided in Table 4, Table 5 and Table 6 for the three different MCLs. “Network Synchronization time” refers to the total time required for successful signal detection (including detection of frame timing), correction of frequency offset and detection of cell ID. In obtaining the total synchronization time, we assume that the frequency offset estimation procedure is started during the last 15 ms frame used for signal detection, and the cell ID detection procedure is started in the last 15 ms frame the frequency offset is correctly estimated.

Table 4: Comparison of network synchronization time (in ms) for initial cell search at 164 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 135 | 90 | 75 |
| 90 % | 570 | 390 | 390 |
| 99 % | 1245 | 1470 | 1110 |
| Average | 236.57 | 167.48 | 153.92 |

Table 5 : Comparison of network synchronization time (in ms) for initial cell search at 154 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 30 | 30 | 30 |
| 90 % | 120 | 105 | 120 |
| 99 % | 465 | 795 | 1035 |
| Average | 53.23 | 61.08 | 71.46 |

Table 6 : Comparison of network synchronization time (in ms) for initial cell search at 144 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 15 | 15 | 30 |
| 90 % | 30 | 75 | 90 |
| 99 % | 75 | 405 | 630 |
| Average | 19.28 | 41.49 | 57.84 |

## Non-Initial Cell Search: Timing and Frequency Offset Estimation

During non-initial cell search, the mobile station needs to reconfirm to a particular cell that it was previously connected to. The frequency offset is set to 2 ppm instead of 20 ppm for the non-initial cell search, in agreement with the current GERAN assumption [4]. The detection probabilities are provided in Table 7 for different MCLs and varying number of interferers.

Table 7 : Comparison of the detection probability for non-initial cell search

| Number of Interferers | Detection Probability |  |  |
| --- | --- | --- | --- |
|  | 164 dB MCL | 154 dB MCL | 144 dB MCL |
| 0 | 100 % | 99.8 % | 98.9 % |
| 1 | 100 % | 99.8 % | 99.2 % |
| 2 | 100 % | 99.8 % | 99.2 % |

The CDF of the synchronization times for timing offset estimation are provided in Figure 17, Figure 18 and Figure 19 for 164 dB, 154 dB and 144 dB MCL respectively. As before, the synchronization time is given in terms of multiples 15 ms intervals, which is the repetition interval of the PSS.

In these and subsequent comparisons, the realizations for which there was no detection are excluded. For these excluded realizations, the signal was not detected even after 40 M-frames (see Table 7  for the percentage of detection). We observe that the performance of synchronization is worst in the presence of multiple interferers in the system. The reason is because in the presence of three cells, more interference is present from the neighboring two transmitting cells as opposed to only one cell, resulting in a slower synchronization.

Figure 17 : CDF of the time required for signal detection at 164 dB MCL.

Figure 18 : CDF of the time required for signal detection at 154 dB MCL.

Figure 19 : CDF of the time required for signal detection at 144 dB MCL.

The CDFs of the residual timing estimation error are provided in Figure 23, Figure 24 and Figure 25  for the different MCLs.

Figure 23: CDF of the timing estimation error after signal detection at 164 dB MCL.

Figure 24 : CDF of the timing estimation error after signal detection at 154 dB MCL.

Figure 25 : CDF of the timing estimation error after signal detection at 144 dB MCL.

After the timing offset has been found, the frequency offset is estimated in a similar fashion as in the initial cell search procedure. The CDFs of the time required for frequency offset estimation are provided in Figure 20, Figure 21 and Figure 22  at MCLs of 164 dB, 154 dB and 144 dB respectively.

Figure 20 : CDF of the time required for frequency offset estimation at 164 dB MCL.

Figure 21 : CDF of the time required for frequency offset estimation at 154 dB MCL.

Figure 22 : CDF of the time required for frequency offset estimation at 144 dB MCL.

The CDFs of the residual frequency offset estimation error are provided in Figure 26, Figure 27 and Figure 28 for the different MCLs.

Figure 26 : CDF of the frequency offset estimation error at 164 dB MCL.

Figure 27 : CDF of the frequency offset estimation error at 154 dB MCL.

Figure 28 : CDF of the frequency offset estimation error at 144 dB MCL.

## Non-Initial Cell Search: Cell ID and Frame Timing Detection

The cell ID detection as well as the M-frame timing estimation is performed using the SSS after the subframe timing and frequency offsets are known and compensated for. The corresponding CDFs of the synchronization time for cell ID detection are provided in Figure 29, Figure 30 and Figure 31 respectively for the three different MCLs considered.

Figure 29: CDF of the time required for cell ID detection at 164 dB MCL.

Figure 30 : CDF of the time required for cell ID detection at 154 dB MCL.

Figure 31 : CDF of the time required for cell ID detection at 144 dB MCL.

## Non-Initial Cell Search : Total Synchronization Time

A comparison of the total synchronization time for non-initial cell search for a desired percentage of the mobile stations is provided in Table 8, Table 9 and Table 10 for the three different MCLs.

Table 8: Comparison of network synchronization time (in ms) for initial cell search at 164 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 150 | 165 | 210 |
| 90 % | 630 | 690 | 1005 |
| 99 % | 1410 | 1815 | 2165 |
| Average | 255.99 | 292.42 | 392.67 |

Table 9 : Comparison of network synchronization time (in ms) for initial cell search at 154 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 30 | 60 | 90 |
| 90 % | 120 | 420 | 540 |
| 99 % | 495 | 1335 | 1665 |
| Average | 59.41 | 157.19 | 215.19 |

Table 10 : Comparison of network synchronization time (in ms) for initial cell search at 144 dB MCL

| Synchronization Time | Number of Interferers |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 50 % | 15 | 45 | 75 |
| 90 % | 30 | 315 | 495 |
| 99 % | 135 | 1275 | 1725 |
| Average | 22.37 | 123.67 | 195.74 |

# Conclusions

Performance evaluations based on the cell search design in NB LTE has been shown to provide good synchronization performance for initial and non-initial cell search at different MCLs, both in the presence as well as absence of interferers.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GP-150779, “Narrowband LTE – Concept description”, source Ericsson LM; Nokia Networks; AT&T; Alcatel-Lucent; Alcatel-Lucent Shanghai Bell; Intel; Sierra Wireless; Sequans, GERAN#67.

GP-150783, “Narrowband LTE – Cell search design”, source Ericsson LM; Nokia Networks, GERAN#67.

GP-140718, “Text proposal to capture agreements on simulation assumptions”, source Vodafone Group Plc.