Source: 	Qualcomm Incorporated

Title:    	NB-CIoT – Software Update/Reconfiguration Simulation Results

Agenda:	7.1.5.3.5.5, 7.2.5.3.4.5

Document for:	Discussion

# Introduction

A converged solution [1] built upon NB-M2M and NB-OFDMA candidates [2] was proposed at GERAN#66. The downlink physical layer design of this converged solution, namely, the Narrowband Cellular IoT candidate solution (NB-CIoT), is expanded in [3].

This paper presents the simulation results for Software update/reconfiguration traffic for NB-CIoT system. Evaluation methodology and traffic model are taken from [2] Section 5.2.3 and Annex E.2.4 respectively.  Details are as follows.

# Simulation model

## Drop and System parameters

Key deployment parameters are summarized in Table 1, whereas system configuration is identical to [3].

Table 1: Simulation parameters

| Parameter | Value |
| --- | --- |
| Simulation area | 19 cell-sites with 3 sectors per site, in 2-tier Hexagonal grid with wrap around |
| ISD | 1732 meters |
| System bandwidth | 1 carrier (200 KHz) |
| OFDM subcarrier spacing | 3.75 KHz; 48 total subcarriers in 180KHz |
| Frequency Reuse | 3; Sets {0,…,14}{16,…,22,24,…31}{33,…,47} |
| BS maximum transmit power per subcarrier | 43dBm – 10*log10(48) |
| UE distribution | Uniform density, 65K UEs/sector |
| Propagation loss | 120.9 + 37.6log10(dKM); 900 MHz |
| Shadow Fading | Lognormal with σ=8dB; correlation ½ across cell-sites and 1 across sectors of each cell-site |
| Penetration loss | Build Penetration Loss Scenarios 1& 2, inter-site correlation coeff. of 0.5 and 0.75, i.i.d across UEs |
| Antenna configuration | DL: 1x1 |
| Antenna pattern | 2D, 65deg H-plane (TR45.914) – 3 sectored |
| Antenna gains | BS: 18dB; UE: -4dB |
| BS cable loss | 3dB |
| Cell association | BS with strongest RSRP |
| MCL for association | 164dB |
| UE Max. TX Power | 23 dBm |
| UE Noise Figure | 5dB |
| BS Noise Figure | 3dB |

## Traffic model

The traffic in the simulation comprises software update/reconfiguration. The uniform intensity in number of sessions per day per sector is,

λSU=1180×NMS,

where NMS=65,000 is the total UE population per sector.  The session sizes are drawn i.i.d from Pareto distribution with shape parameter 1.5, truncated in [200,2000] bytes. The total header size including CoAP/DTLS/UDP/IP is taken to be 65 bytes. In case of header compression, the total header size is assumed to be 29 bytes.

## Performance metric

A DL resource unit is define as one subcarrier for one slot. Then resource utilization by SU traffic is simply the fraction of available tone-slots that are utilized for SU traffic. This includes both PDCCH and PDSCH resources but does not include resources allocated to broadcast and synchronization channel.  More specifically, DL resource utilization at a cell-site, ρDL, is defined as,

ρDL=s=1Nst=1TsimXSU(s,t)NsTsim,

where Ns≡48 is the total number of subcarriers per cell-site, Tsim is the simulation duration in number of slots, and XSU(s,t) is an indicator function that BS transmitted on subcarrier s in slot t to support SU traffic.

UL resource utilization is defined in identical fashion by replacing a subcarrier with a channel.

# Simulation results

The DL resource utilization by SU traffic is only 0.13% for the case with no header compression and BPL Scenario 2 with inter-site correlation coefficient of 0.5.  This is worst case for utilization; the utilization is <0.13% for the remaining cases. The UL resource utilization is likewise infinitesimal.

# Summary

Resource utilization for the software update/reconfiguration traffic model are presented. The results demonstrate that NB-CIoT solution can support software update/reconfiguration traffic within 0.13% of available resources. A corresponding pCR to add evaluation data to TR is provide in [7].

References

- GP-150524, “Proposed convergence of NB M2M and NB-OFDMA clean-slate solutions,” Qualcomm Incorporated, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., Neul Limited, u-blox. GERAN #66, May 2015.

- 3GPP TR 45.820, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things, Release 13, version 1.3.0.

- GPC150407, “NB-CIOT – Downlink physical layer design,” Qualcomm Incorporated, Neul Ltd, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., July 2015.

- GPC150416, “NB-CIoT – Downlink Interference and Link-to-System Mapping,” Qualcomm Incorporated, July 2015.

- GPC150476, “pCR 45.820 NB M2M - Evaluations of Software Update and Reconfiguration,” Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd., 3GPP TSG GERAN #66, May 2015.

- GPC150477, "NB M2M - System Level Simulation for Capacity and Latency Evaluation (update of GPC150261)," Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd., 3GPP TSG GERAN #66, May 2015.

- GP-150809, “pCR 45.820 NB-CIoT – Software update/reconfiguration simulation results”, Qualcomm Incorporated, 3GPP TSG GERAN #67, August 2015.