3GPP TSG GERAN WG3#69	GP-160005

Electronic Meeting, Jan 11th – Feb 8th, 2016

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 51.010-1 | CR | 5078 | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for test case 33.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G3new |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  |  | Date: |  | 2016-02-04 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | As per TS 24.008, section 4.3.4.1 “If a RR connection exists, the MM sublayer will release locally any ongoing MM connections before the IMSI DETACH INDICATION message is sent.” . As per section 5.1 ,MS may send DISCONNECT message before IMSI DETACH to move from U10 (Active state) to U0 (Null state). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modification of steps 1.1 and 1.2 of section 33.6.5 of spec TS 51.010 so that UE may send DISCONNECT message followed by IMSI DETACH INDICATION |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant MS may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 33.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 33.6	Subscription identity management

33.6.1	Definition

Subscription identity management is the ability of the MS to prevent the establishment of MO (except MO emergency calls) and MT calls without a valid subscription.

33.6.2	Conformance requirement

An MS can only be operated, if a valid IMSI is present.

33.6.3	Test purpose

1)	To verify that during an established call: either

1.1)	on removal of the SIM from an MS, the MS will perform an IMSI detach; or

1.2)	after removing the power source from the MS, removing the SIM, and restoring the power source to the MS, the MS may perform an IMSI detach.

2)	To verify that an MS without SIM card will not establish a MO call which is not an emergency call.

3)	To verify that an MS without SIM card will not accept an incoming call.

33.6.4	Method of test

33.6.4.1	Initial conditions

Void.

33.6.4.2	Procedure

a)	A call is set up.

b)	(Reserved).

c)	Either:

(i)	the SIM is removed; or

(ii)	where this is not possible, the power source is removed from the MS, the SIM is removed and the power source is restored to the MS.

The SS observes whether or not the MS performs IMSI detach.

d)	An attempt to establish a MO call is made (not an emergency call).

e)	An attempt to establish a MT call is made.

33.6.5	Test requirements

1)	Either:

1.1)	in step c(i), the MS may send disconnect and shall perform an IMSI detach; or

1.2)	in step c(ii), the MS sends nothing when power source is removed.

the MS may perform an IMSI detach when power is restored.

2)	In step d) the MS shall not attempt to set up a new call via the Um interface.

3)	In step e), the MS shall not respond to the attempt to set up a new call via the Um interface.