3GPP TSG GERAN WG4#1	Tdoc G400100

Sophia Antiplois, France, 12-15 September  2000

Source: TSG GERAN WG4 EDGE

| 3GPP TSG GERAN WG4#1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | G400100 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia Antipolis, France 12-15 September 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 11.10 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | For approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | For information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | X | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | TSG GERAN WG4 EDGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 11 Sept 2000 |  |  |  |  |  |
| Subject: |  |  | Changes to Annex 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | EDGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | Introduction of 8PSK test equipment measurement uncertainties. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | Annex 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

Annex 5:	Test equipment

# A5.1	Introduction

## A5.1.1	General

The test equipment is either an equipment or assembly of equipments which enables the tests described in this specification to be conducted.

This annex describes requirements for the test equipment which cannot be derived from and which are assumed in, the conformance test descriptions described in this specification.

Specifically stimulus setting and measurement uncertainties are defined.

## A5.1.2	Test equipment terms

The term "System Simulator" (SS) is used to describe the complete suite of test equipment required to perform the tests in this specification when interacting with the following MS interfaces:

-	Antenna (Connector or radiated);

-	Acoustic;

-	Data Port(s);

-	Power supply;

-	DAI.

NOTE:	To perform a sub-set of tests, the SS may be simplified accordingly.

The term "SIM simulator" is used to describe the test equipment required to interact with the SIM/ME interface.

A "test SIM" has the physical characteristics of a standard SIM card, (see GSM 11.11) with specific parameters defined in annex 3.

## A5.1.3	Confidence level

All uncertainty values stated in this annex are quoted for a Confidence Level of 95 %.

# A5.2	Standard test signals

The Cx signals represent the wanted signals and the Ix signals represent the unwanted signals.

Signal C0	Unmodulated continuous carrier;

Signal C1	A standard GSM signal with GMSK modulation derived by applying a data reversals signal to the input of a channel coder. The channel coder will depend on the test and the cipher mode shall be selectable by the test method. When using this signal in the non hopping mode, the unused seven time slots shall also contain dummy bursts, with power levels variable with respect to the used timeslot, see also 2.3.1.3.

Signal I0	Unmodulated continuous carrier;

Signal I1	A GMSK modulated carrier following the structure of the GSM signals, but with all modulating bits (including the midamble period) derived directly from a random or pseudo random data stream.

Signal I2	A standard GMSK modulated signal with valid midamble, different from C1. The data bits (including bits 58 and 59) shall be derived from a random or pseudo random data stream.

# A5.3	SS functional requirements

## A5.3.1	Level setting range

It is assumed that the SS is capable of setting stimulus levels, at the MS interface, to those required in the test specification extended by the measurement uncertainty defined in this annex.

NOTE:	This ensures that the SS is able adequately to stimulate the MS performance at and just beyond the limit requirement under all conditions.

## A5.3.2	Level Measurement / operation range

It is assumed that the SS is capable of performing measurements, within the uncertainty defined in this annex, over a level range, at the MS interface, as required in the test specification extended by the SS measurement uncertainty defined in this annex and extended by a further 3dB on the MS conformity requirement.

NOTE:	This ensures that the SS is able adequately to measure the MS performance at and just beyond the limit requirement under all conditions.

## A5.3.3	MS power supply interface

Test DC power supply for MS:

Voltage setting uncertainty		< 1 %.

Ripple				< 10 mV RMS, 50 mV peak to peak.

Test AC power supply for MS:

Voltage setting uncertainty:		< 1 %.

## A5.3.4	MS antenna interface

The SS is assumed to offer a nominal 50 ohm impedance to the MS.

GSM/DCS bands	< 4 GHz	< 10 GHz	< 12,75 GHz

VSWR		<= 1,3	<= 2,0	<= 3,0	<= 3,5

### A5.3.4.1	Uplink receiver error

The SS receiver should be capable of performing the tests as specified in GSM 11.10 without the addition of bit errors in excess of 1 in 10E7 due to the receiver performance when operated with a MS which meets the transmitter requirements of GSM 05.05. This requirement shall apply for GMSK and 8PSK modulation.

NOTE:	This requirement is based on a minimum BER measurement of 1 in 10E5.

### A5.3.4.2	Power and Power versus time measurements

Measurement uncertainty of  transmitter output  power for GMSK and 8PSK signals:	+/- 1 dB

In the case of 8PSK, provision is made for power measurement by averaging over multiple bursts or by using an estimation method, see GSM 05.05, section 4. The estimation method may be based on measurements of one or more bursts, or part of a burst.

If 8PSK power is measured by averaging over multiple bursts, allowance must be made for variations in burst power as a function of the data. This allowance must be included within the +/-1dB measurement limit. The allowance is related to the number of bursts taken in the average and shall be defined as follows:

Allowance for burst power variation  = 2/SQRT(N)

Where 	 = the standard deviation of burst power variation for random data (0.2dB).

(two standard deviations yield a 95% confidence interval)

N = number of averages

EXAMPLE:  An average is calculated from 4 bursts. The allowance for burst power variation is 0.2dB. The accuracy for the power meter should then be better than +/-0.8dB.

If 8PSK power is measured using an estimation method, it shall be demonstrated, using the method described below, that the accuracy of the estimation technique is also +/-1dB.

A test signal is established consisting of properly formatted bursts with midambles and random data in the payload. The long-term average power of this signal is determined by measuring the power over 200 bursts and taking the average (Pavg). The measurement uncertainty of the equipment used to determine the long-term average shall be noted (P).

The same test signal is then measured using the estimation technique. The difference between the estimated value of long-term average power and the measured long-term average power is noted (Pest). The following inequality shall hold:

|P| + |(Pavg - Pest)| ≤  1dB

For GMSK, measurement uncertainty of power level (relative to peak transmitter carrier power):

Power level				Measurement uncertainty

+ 6 dB to - 7 dB				+/-0,25 dB

- 7 dB to -20 dB				+/-1,0 dB

-20 dB to -32 dB				+/-2,0 dB

-32 dB to -45 dB				+/-5,0 dB

-45 dB to -71 dB				+/-1,0 dB

<-71 dB					+/-2,0 dB

For 8PSK, measurement uncertainty of  power level (relative to output power):

Power level				Measurement uncertainty

+6 dB to –7dB				+/-0,25 dB

-7dB to  -16dB				+/-1,0 dB

-16dB to –32dB				+/-2,0  dB

-32 dB to -45 dB				+/-5,0 dB

-45 dB to -71 dB				+/-1,0 dB

<-71 dB					+/-2,0 dB

NOTE:	Due to the method of measurement (downconversion to I/Q baseband / filtering / A/D conversion / postprocessing) several uncertainties occur. The sources are:

a)	absolute level uncertainty;

b)	filter ripple,	
	I/Q gain imbalance,	
	I/Q imperfect quadrature;	

c)	A/D conversion (resolution),	
	I/Q offset.

Items under b) and c) affect the individual samples and can be observed as a "ripple" in the horizontal part of the power time mask.

Items under b) are uncertainties which are proportional to the signal measured.

Items under c) are constant amounts of uncertainty, independent of the signal measured.

The item a) moves the entire power time template up or down.

The uncertainties b) and c) are added to the measured signal as an uncorrelated interferer.

The above mentioned absolute measurement uncertainty refers to a). The table covers uncertainties b) and c).

Uncertainty of time measurement

The relative timing uncertainty of the transition point:

-	symbol 13 to 14 in the midamble (normal burst);

-	end of the sync sequence (access burst);

is +/- 1/8 symbol.

Timing uncertainty of the measurement samples in the vertical part of the power time mask are displayed as marked fields in the figure A5.3-1

Figure A5.3-1 Time Measurement Uncertainty for the Power Time Mask

NOTE:	With a real method of measurement one has to reckon on systematic measurement uncertainties in the vertical part of the power time template (figures 13-2 & 13-3). The reason for this is that the measurement is conducted through a filter which has to fulfil different requirements simultaneously, requirements in the frequency domain and in the time domain as well. The time behaviour of the filter causes the above mentioned measurement uncertainty. It occurs clearly when measuring the falling edge of the power burst. The measurement uncertainty, which in principle delays the actual performance, depends on the filter characteristics and on the signal shape. At favourable signal shapes the uncertainty is negligible, however, at unfavourable signal shapes it consumes the marked area in figure A5.3-1 (falling edge).

The underlying filter is:

type			inverse Chebycheff

passband			<= +/- 200 kHz

stopband (40 dB stop att.)	>= +/- 541,67 kHz.

To avoid aliasing with this filter the RF output spectrum must meet the requirements of section 13.4.

If the lowest limit line in the power time template is replaced by a -54 dBm line, measuring lower carrier powers, the area of measurement uncertainty is reduced equivalently.

The marked area in figure A5.3-1 describes the systematic measurement uncertainty of the test equipment and does not widen the design requirements.

Uncertainties associated with 13.3.5 requirement b) (power control levels, adjacent steps):

Repeatability			+/- 0,3 dB

Linearity				+/- 0,03 dB/dB

Combined uncertainty is:		+/- (0,3 + 0,03 dB/dB) dB

E.g. where the indicated value of the step size is 2,0 dB, the uncertainty is:

+/- (0,3 + 0,06) dB = +/- 0,36 dB.

### A5.3.4.3	Wideband selective power measurement

Power is to be measured selectively for spurious emissions without frequency hopping (ref.: section 12).

Uncertainty conducted		100 kHz to 1GHz		+/- 1,5 dB

1 GHz to 12,75 GHz	+/- 3,0 dB

Uncertainty radiated		30 MHz to 4 GHz		+/- 6 dB

NOTE:	The uncertainties include the effect of a worst case reflection from the MS of 0,7 for out of band signals.

It is acceptable to use a band stop filter in spurious emission measurements of the transceiver in order to fulfil the above requirements.

### A5.3.4.4	Inband selective power measurements

Power is to be measured selectively for output RF spectrum.

The measurement is performed on a single frequency while the MS is frequency hopping (ref.: section 13.3)

Uncertainty			< +/- 1,6 dB

NOTE:	The video signal of the spectrum analyser is "gated" such that the spectrum generated by at least 40 of the bits 87 to 132 of the burst is the only spectrum measured. This gating may be analogue or numerical, dependent upon the design of the spectrum analyser.

### A5.3.4.5	 Modulation accuracy and frequency error measurements

GMSK modulation

Ref.: Sections 13.1 & 13.2 for definitions and methods of measurement.

Phase measurement uncertainty	+/- 1 degree RMS

+/- 4 degrees for individual

phase measurement samples

The phase measurement uncertainties above apply during the useful bits.

Frequency measurement uncertainty:	+/- 5 Hz.

8PSK modulation

Ref.: Sections 13.17.1  for definitions and methods of measurement.

EVM measurement uncertainty		[< -1, +0,75] % RMS

< +/-4% for individual EVM measurement samples

Note 1:      The assymmetric RMS EVM specification results from the RMS EVM minimisation method used for parameter estimation (see GSM 05.05, Annex G). This method of measurement for RMS EVM always produces a result which is lower than the actual value of RMS EVM.

Note 2:	If the test equipment demodulates the transmitted signal to derive the reference signal for the EVM measurement, the symbol error rate of the demodulation process must be less than  4.4*10E-4 for 95% confidence that no detection errors occur in a burst.

Origin Offset	 uncertainty			<+/-1.5 dB for origin offset ≥ -35dBc

Frequency measurement uncertainty	<+/-20 Hz

### A5.3.4.6	RF delay measurements relative to nominal times

Range			-140 to +140 symbol periods

Resolution			1/4 symbolperiod

Uncertainty			+/- 1/8 symbol period

### A5.3.4.7	The wanted signal or traffic channel of serving cell

The Wanted signal is used in most of the specified RF measurements. The traffic channel of the serving cell is used in most of the signalling tests.

FREQUENCY:

GMSK

Uncertainty:		< +/- 5*10E-9

8PSK

Uncertainty:		<+/- 20*10E-9

MODULATION (see GSM 05.04):

GMSK

Phase uncertainty:		< +/- 1 degree RMS and

< +/- 4 degrees peak(as defined in GSM 05.05)

8PSK

EVM uncertainty			<4 % RMS

Origin offset suppression     	<-35dBc

LEVEL:

Uncertainty:		< +/- 1 dB in section 13, 14 except:-

< +/- 3 dB for test 14.2 radiated

< +/- 1,2 dB for test 14.6

< +/- 2,5 dB for all other tests.

Settling time:	< 10 us

DYNAMIC LEVEL SETTING:

The SS shall be able to switch from any power level to any other power level within the range of 30 dB on a timeslot per timeslot basis. This dynamic switching requirement only applicable for a single channel for a limited number of tests.

SPURIOUS:

inchannel:		Covered by phase error

outchannel:

Noise Power, 1 Hz bandwidth:

<-100 dBc for >100kHz carrier offset,

<-110 dBc for >300kHz carrier offset,

<-121 dBc for >1500kHz carrier offset.

Non harmonics:	< -55 dBc for > 100kHz carrier offset,

< -68 dBc for >1500kHz carrier offset

FREQUENCY HOPPING:

The signal shall be capable of hopping according to the criteria of GSM 05.02. The timing of the frequency change shall be such that frequency transitions do not occur during the active timeslot of the MS.

### A5.3.4.8	The first interfering signal or traffic channel of the first adjacent cell

The First interfering signal is used in measurements of co-channel rejection, adjacent channel rejection and intermodulation rejection. The Traffic channel of the first adjacent cell is used in handover tests.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

PHASE:

Uncertainty:		< +/- 1 degree RMS and

< +/- 4 degrees peak(as defined in GSM 05.05)

LEVEL:

Uncertainty			< +/- 1 dB relative to the wanted signal for

test 13.2 and 14.5

< +/- 0,3 dB relative to the wanted signal for

test 14.4

< +/- 1 dB for test 14.6

< +/- 2,5 dB for all other tests

MODULATION:		GMSK (as specified in GSM 05.04)

The total relative single sideband power (noise + harmonics) in the frequency range 1,5 to 1,7 MHz offset from the nominal carrier frequency shall be less than -72 dBc.

SPURIOUS:

Inchannel:		Covered by phase error

Outchannel:

Noise Power, 1 Hz bandwidth:

<-100 dBc for > 100kHz carrier offset

<-110 dBc for > 300kHz carrier offset

<-127 dBc for >1500kHz carrier offset

non harmonics	< -55 dBc for > 100kHz carrier offset

< -68 dBc for >1500kHz carrier offset

FREQUENCY HOPPING:

The signal shall be capable of hopping according to the criteria of GSM 05.02. The timing of the frequency change shall be such that frequency transitions do not occur during the active timeslot of the MS.

### A5.3.4.9	The second interfering signal

The second interfering signal is used in the measurements of intermodulation rejection and blocking.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

LEVEL:

Uncertainty:		< +/- 1 dB for test 14.6

< +/- 1,5 dB relative to the wanted signal

for all other tests.

MODULATION:		Unmodulated

SPURIOUS:

Inchannel:		No requirements

Outchannel:

Noise Power, 1 Hz bandwidth:

<-135 dBc for > 500kHz carrier offset,

<-140 dBc for > 700kHz carrier offset,

<-150 dBc for >1500kHz carrier offset.

Non harmonics	< -79 dBc for > 500kHz carrier offset,

< -84 dBc for > 700kHz carrier offset,

< -94 dBc for >1500kHz carrier offset

Harmonically related spurii	<-40 dBc

### A5.3.4.10	BCCH carriers of serving and adjacent cells

The BCCH of the serving cell is used for synchronizing the MS and to send network information to the MS under test. The BCCH signals of the adjacent cells are used in the handover tests. The MS measures the RF-levels of the BCCHs of adjacent cells.

FREQUENCY:

Uncertainty:		< +/- 5*10E-9

PHASE:

Uncertainty:		< +/- 1 degree RMS and

< +/- 4 degrees peak(as defined in GSM 05.05)

LEVEL:

Uncertainty:		< 1 dB for test 13.2 and 20

< 2,5 dB for all other tests

< 0,6 dB relative to each other and to TCH

for test 21 over the range 65 dBmicroVoltemf

to 3 dBmicroVoltemf

< 1,2 dB relative to each other and to TCH for test 26.3

MODULATION:		GMSK (as specified in GSM 05.04)

SPURIOUS:

Inchannel:		Covered by phase error

Outchannel:

Noise Power, 1Hz bandwidth:

<-100 dBc for > 100kHz carrier offset

<-125 dBc for >1500kHz carrier offset

Non harmonics	< -55 dBc for > 100kHz carrier offset

< -72 dBc for >1500kHz carrier offset

### A5.3.4.11	The wide frequency range signal

The wide frequency range signal is used in the measurements of spurious response.

FREQUENCY

Uncertainty:		< +/- 5*10E-9

LEVEL

Uncertainty:		< +/- 1,5 dB relative to the wanted signal for test 14.7

< +/- 1 dB error of substituted "wanted signal"

MODULATION:		unmodulated

SPURIOUS in the MS receiving range

non harmonics:	< -94 dBc

Harmonically related spurii:	< -40 dBc

Noise:		< -4 dBuVemf equivalent at the MS receiver input

when measured in a 200 kHz bandwidth.

### A5.3.4.12	The multipath fading function

The multipath fading function simulates the fading effects of a broadband radio channel in mobile radio communication

The propagation conditions are specified in GSM 05.05, annex 3.

The multipath fading function shall be performed only within a 5 MHz bandwidth during one test case.

## A5.3.5	MS audio interface and DAI

### A5.3.5.1	General uncertainties

Unless otherwise specified, the following uncertainties apply to the audio interface:--

Signal level measurement uncertainty:		+/- 0,2 dB

Sound pressure measurement uncertainty:	+/- 0,6 dB

Frequency Measurement uncertainty:		+/- 0,1 %

Stimulus frequency setting uncertainty:

Frequency settings are taken from ISO 3, R10 series or R40 series or from table 2 of Rec. CCITT P.79. A departure from the nominal frequencies of +/- 5 % below 240 Hz and +/-2 % at 240 Hz and above is accepted.

In the case of 4 kHz the departure is restricted to -2 %.

### A5.3.5.2	Analogue single test tone

Total distortion:				< 0,5 %

### A5.3.5.3	Delay measurement between Um and DAI

The delay measurement between the Um interface of the MS and its DAI in both directions is described in section 32.5.

Uncertainty:				< +/- 0,1 ms

# A5.4	SIM simulator functional requirements

## A5.4.1	General

The SIM simulator shall implement the functions of a SIM as described in GSM 02.17 and 11.11.

The Test Algorithm for authentication incorporated in the SIM Simulator shall operate as described in annex 3

## A5.4.2	Contacts C1, C2, C6, C7

### A5.4.2.1	Default measurement / setting uncertainties

Unless stated otherwise below, the following uncertainties apply:

Voltage measurement uncertainty:	< +/- 50 mV

Voltage setting uncertainty:		< +/- 20 mV

Time measurement uncertainty:		< +/- 100 ns

### A5.4.2.2	Contact C1

Continuous Spikes

Voltage measurement uncertainty:	< +/- 100 mV

Current Load Amplitude			0 mA - 20 mA

Adjustable Step Size		1 mA

Uncertainty			< +/- 1 mA

Additional Current Offset		0 mA - 5 mA

Adjustable Step Size		1 mA

Uncertainty			< +/- 1 mA

Pulse Width				100 ns - 500 ns

Adjustable Step Size		50 ns

Uncertainty			< +/- 25 ns

Rise and Fall Time			<= 50 ns

Pause Width				100 ns - 500 ns

Adjustable Step Size		50 ns

Uncertainty			< +/- 25 ns

Random Spikes

Voltage measurement uncertainty:	< +/- 100 mV

Current Load Amplitude			50 mA - 200 mA

Adjustable Step Size			1 mA

Uncertainty				< +/- 1 mA

Additional Current Offset		0 mA - 5 mA

Adjustable Step Size		1 mA

Uncertainty			< +/- 0,1 mA

Pulse Width				100 ns - 500 ns

Adjustable Step Size		50 ns

Uncertainty			< +/- 25 ns

Rise and Fall Time			<= 50 ns

Pause Width				0,1 ms - 500 ms, randomly varied

Adjustable Step Size		0,1 ms

Uncertainty			<+/- 0,1 ms

### A5.4.2.3	Contact C7

The Elementary Time Unit (etu) used in the sections below refer to the nominal bit duration on the I/O line, as defined in ISO 7816-3.

Rise & fall Time setting uncertainty:	< +/- 100 ns

Jitter measurement uncertainty:		< +/- 5*10E-3 etu

Jitter setting uncertainty:			< +/- 5*10E-3 etu

## A5.4.3	Contact C3

Frequency measurement uncertainty:		< +/- 0,5 %

Voltage Measurement uncertainty:		< +/- 50 mV

Rise & fall time measurement uncertainty:	< +/- 5 ns

Duty cycle measurement uncertainty:		< +/- 2,5 %

## A5.4.4	Definition of timing

It shall be possible to define all timings relative to the clock. The SIM simulator shall be able to calculate and to use the absolute values automatically, even if the ME changes the frequency during the communication.

# History

| Document history |  |  |
| --- | --- | --- |
| V5.1.0 | May 1997 | TC SMG approval as TS 1997-02-14 |
| V5.2.0 | August 1997 | TC SMG approval as TS 1997-06-13 |
| V5.3.0 | November 1997 | TC SMG approval as TS 1997-10-17 |
| V5.4.0 | February 1998 | TC SMG approval as TS 1997-12-19 |
| V5.5.0 | May 1998 | One-step Approval Procedure OAP 9841: 1998-05-20 to 1998-10-16 |
| V5.6.0 | August 1998 | One-step Approval Procedure OAP 9850: 1998-08-14 to 1998-12-11 |
| V5.5.1 | October 1998 | Publication |
| V5.7.0 | December 1998 | One-step Approval Procedure OAP 9914: 1998-12-04 to 1999-04-02 |