3GPP TSG-GERAN WG4 Meeting #6 	TDoc G4-010638

Sophia Antipolis, France, 8 - 10 October 2001

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51.010-1 |  |  |  | CR | 425 |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.5.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | x |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Wrong contents of CRTL_ACK in PCA of step 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Setcom |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | GPRS |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 2001-08-28 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-4 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The CRTL_ACK in Specific Message Contents of PACKET CONTROL ACK access bursts in step 4 must have the value 11 instead of 01. See 04.40 sectiom 11.2.2: If the PACKET CONTROL ACKNOWLEDGEMENT message is being transmitted in response to a valid RRBP field received as part of an RLC/MAC block with Payload Type not equal to '10', the CTRL_ACK field shall be set to the value '11'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Change CTRL_ACK from 01 to 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | A good MS will fail the test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 42.5.2.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | This CR applies to R97 and later releases |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 42.5.2.2	Downlink Transfer/ Polling/ Packet Polling Request/ Access Burst format

42.5.2.2.1	Definition and applicability

This test case applies to all MSs supporting GPRS.

42.5.2.2.2	Conformance Requirement

The network may send to the mobile station a PACKET POLLING REQUEST message. If the MS has received a PACKET DOWNLINK ASSIGNMENT message with no starting time or with a starting time that has already elapsed, the PACKET POLLING REQUEST message shall be sent on PACCH. Otherwise the PACKET POLLING message shall be sent on PAGCH. The mobile station shall be addressed by its TLLI or TFI.

On receipt of a PACKET POLLING REQUEST message, the mobile station shall respond to the network with the PACKET CONTROL ACKNOWLEDGEMENT message in the reserved uplink radio block specified by the RRBP field. The reserved block is considered as a one block PACCH allocation.

References

GSM 04.60, subclause 7.2.1.3

42.5.2.2.3	Test purpose

Verify that an MS responds to a PACKET POLLING REQUEST message (requesting access burst format) with a PACKET CONTROL ACKNOWLEDGEMENT message in access burst format.

42.5.2.2.4	Method of test

Initial Conditions

System Simulator:

1 cell, GPRS supported, PCCCH present

Mobile Station:

The MS is in the state "idle, GMM-registered" with a P-TMSI allocated, SPLIT PG CYCLE negotiated, and PDP context 3 activated.

Related PICS/PIXIT Statement(s)

Support GPRS service

Test Procedure

1.	MS receives a PACKET DOWNLINK ASSIGNMENT message on its assigned PCCCH, containing no starting time.

2.	SS transmits downlink RLC data blocks.

3.	SS transmits a PACKET POLLING REQUEST message, requesting access burst format.

4.	MS responds by sending a PACKET CONTROL ACKNOWLEDGE message in access burst format.

Maximum Duration of Test

5 min.

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PACKET DOWNLINK ASSIGNMENT | Triggers the MS to switch to assigned PDCH. |
| 2 | SS -> MS | RLC DATA BLOCKS | Starting 6 blocks after the previous message, on assigned PDCH, addressed to MS. |
| 3 | SS -> MS | PACKET POLLING REQUEST | Requesting access burst format |
| 4 | MS -> SS | PACKET CONTROL ACKNOWLEDGEMENT | MS acknowledges using 11-bit access burst format. |

Specific Message Contents

PACKET DOWNLINK ASSIGNMENT message in step 1:

| TIMESLOT_ALLOCATION | <one timeslot assigned> |
| --- | --- |
| TBF STARTING TIME | <IE not present> |

PACKET POLLING REQUEST in step 3:

| TYPE_OF_ACK | 0 – MS response sent as 4 access bursts |
| --- | --- |
| Global TFI (downlink) | Addressing MS |

PACKET CONTROL ACKNOWLEDGMENT access bursts in step 4:

| MESSAGE_TYPE | 1111 1100 1 |
| --- | --- |
| CTRL_ACK | 11 |