3GPP TSG-GERAN WG4 Meeting #6 	TDoc G4-010640

Sophia Antipolis, France, 8 - 10 October 2001

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51.010-1 |  |  |  | CR | 427 |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.5.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | x |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | No starting time in 42.5.4.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Setcom |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | GPRS |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 2001-08-28 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-4 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The comment in step 3 of the Expected Sequence is wrong: no starting time is used in this test. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Change comment “MS acknowledges on PACCH the RLC data block sent at the starting time.” To “MS acknowledges on PACCH the RLC data block sent on PDCH0.” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Misleading comment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 42.5.4.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | This CR applies to R97 and later releases |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 42.5.4.2	Downlink Transfer/ T3190 Expiry / Resource reallocation / With TBF starting time

42.5.4.2.1	Definition and applicability

This test case applies to all MSs supporting GPRS.

42.5.4.2.2	Conformance Requirement

On receipt of a PACKET DOWNLINK ASSIGNMENT or PACKET TIMESLOT RECONFIGURE message, and after the TBF starting time, if present, the mobile station shall switch to the assigned PDCHs.Upon switching to the new PDCHs the mobile station shall restart timer T3190.

When the mobile station receives an RLC/MAC block addressed to itself on any of the new assigned resources it shall restart timer T3190. If timer T3190 expires, the mobile station shall perform an abnormal release with return to CCCH or PCCCH (see subclause 8.7.1).

References

GSM 04.60, subclause 8.1.2.4

42.5.4.2.3	Test purpose

Verify that an MS switches to the newly assigned PDCH at the starting time given, and returns to packet idle mode when T3190 expires.

42.5.4.2.4	Method of test

Initial Conditions

System Simulator:

1 cell, GPRS supported, PCCCH present

Mobile Station:

The MS is in the state "idle, GMM-registered" with a P-TMSI allocated, SPLIT PG CYCLE negotiated, and PDP context 3 activated.

Related PICS/PIXIT Statement(s)

Support GPRS service

Test Procedure

1.	MS receives a PACKET DOWNLINK ASSIGNMENT message on its assigned PCCCH, containing no starting time.

2.	SS transmits a downlink RLC data block.

3.	MS responds by sending a PACKET DOWNLINK ACK/NACK.

4.	SS transmits another PACKET DOWNLINK ASSIGNMENT message on PACCH, containing a starting time.

5.	(T3190 * 1.2) seconds later, SS transmits a downlink RLC data block on the newly assigned resources.

6.	MS ignores the downlink block (because it has already returned to packet idle mode because T3190 expired).

Maximum Duration of Test

5 min.

Expected Sequence

| Step | Direction | Message | Comments |
| --- | --- | --- | --- |
| 1 | SS -> MS | PACKET DOWNLINK ASSIGNMENT | Triggers the MS to switch to PDCH0. (no starting time) |
| 2 | SS -> MS | RLC DATA BLOCK | 6 blocks after the previous message, with valid RRBP field, on assigned PDCH0, addressed to MS. |
| 3 | MS -> SS | PACKET DOWNLINK ACK/NACK | MS acknowledges on PACCH the RLC data block senton PDCH0. |
| 4 | SS -> MS | RLC DATA BLOCKS | SS continues to transmit RLC data blocks according to allocation from step 1. |
| 5 | SS -> MS | PACKET DOWNLINK ASSIGNMENT | Triggers the MS to switch to PDCH1 at the given starting time. |
| 6 | SS -> MS | RLC DATA BLOCK | (T3190 * 1.2)seconds after the starting time in the previous message, with valid RRBP field, on PDCH1, addressed to MS. |
| 7 | SS |  | Verify the MS does not transmit on the PACCH of PDCH1. |

Specific Message Contents

PACKET DOWNLINK ASSIGNMENT message in step 1:

| TIMESLOT_ALLOCATION | <one timeslot assigned – PDCH0> |
| --- | --- |
| TBF STARTING TIME | <IE not present> |

PACKET DOWNLINK ASSIGNMENT message in step 5:

| TIMESLOT_ALLOCATION | <one timeslot assigned – PDCH1> |
| --- | --- |
| TBF STARTING TIME | 1 - Relative frame number encoding: |
| - Starting Frame Number Description IE | 0000000101101 – k=45: first frame of block is N+199 or N+200 |

DOWNLINK RLC DATA BLOCKS (various steps):

| RRBP | 00 – Response shall be sent by MS in N+13 frames. |
| --- | --- |
| S/P | 1 – RRBP field is valid |