# EC-GSM, RLC procedures

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things  [1] (WI code: FS_IoT_LC)  was approved.

One of the main objectives is to reduce device complexity compared to existing GPRS services.

In this contribution, the general RLC procedures applicable to GSM are reviewed in an effort to reduce scope/complexity for EC-GSM devices.

# RLC procedures

## General

In general RLC procedures defined in GSM, see clause 9 of [2], applies to for EC-GSM devices.

Deviations from clause 9 is listed below.

## RLC mode

The RLC ARQ functions support three modes of operation: RLC acknowledged mode, RLC unacknowledged mode and RLC non-persistent mode where:

- RLC acknowledged mode operation uses retransmission of RLC data blocks to achieve high reliability.

- RLC unacknowledged mode operation does not utilize retransmission of RLC data blocks.

- RLC non-persistent mode operation uses non-exhaustive retransmission of RLC data blocks.

With the main aim of the study to extend coverage for devices in poor coverage both the RLC unacknowledged mode and RLC non-persistent mode are unsuitable since these modes are more suitable when a certain packet loss is acceptable. Hence, for the use cases envisioned for EC-GSM devices only RLC acknowledged mode is seen needed.

Only RLC Acknowledged mode is supported.

## Multiple TBF operation

A mobile station that supports multiple TBF procedures may operate multiple uplink and downlink TBFs concurrently. Since this implies additional complexity over only supporting single TBF operation it is therefore proposed that a Cellular IoT device only supports single TBF operation.

Only single TBF operation is supported.

## Enhanced Multiplexing for Single TBF (EMST)

Enhanced Multiplexing for Single TBF (EMST) refers to the multiplexing of upper layer data flows using one or more RLC entities on a single TBF where each RLC entity operates in a different RLC mode. EMST is applicable only to TBFs in EGPRS TBF mode. EMST is not applicable to Multiple TBF operation.

It is expected that in order to reduce complexity a Cellular IoT device will only support a single upper layer data flow per RLC entity. It is therefore proposed that EMST is not supported.

Enhanced Multiplexing for Single TBF (EMST) is not supported.

## Enhanced Multiplexing for Single RLC Entity (EMSR):

Enhanced Multiplexing for Single RLC Entity (EMSR refers to the multiplexing of upper layer data flows associated with multiple PFCs with at least two of them having unique TFIs within the context of a single RLC entity for single TBF operation. EMSR may only be used on a TBF operating in EGPRS TBF mode. EMSR is not applicable to Multiple TBF operation or when Exclusive Allocation is used.

It is expected that in order to reduce complexity a Cellular IoT device will only support a single data flow per RLC entity. It is therefore proposed that EMSR is not supported.

Enhanced Multiplexing for Single RLC Entity (EMSR) is not supported.

## Extended uplink TBF mode

In the extended uplink TBF mode, the uplink TBF may be maintained during temporary inactive periods, where the mobile station has no RLC information to send. The network determines the release of the uplink TBF.

Since scheduling on the uplink will be based on Fixed Uplink Allocation extended uplink TBF mode will not be supported.

Extended uplink TBF mode is not supported

## Fast Ack/Nack Reporting procedure

The Fast Ack/Nack reporting procedure (FANR) refers to the possibility to include, in a radio block for data transfer sent in one direction, piggy-backed ack/nack information relative to a TBF with FANR activated in the other direction[3].

This feature was introduced to reduce latency for applications like VoIP.  Given the general use cases for Cellular IoT devices and that this feature adds complexity it is proposed not to support Fast Ack/Nack reporting.

Fast Ack/Nack reporting procedure not supported.

## A maximum Sequence Number Space (Window size)

The size of the send/receive buffers drives complexity so from this perspective it is of interest to reduce the size of the sequence number space and hence window size to a minimum. Currently in GPRS and EGPRS the supported sequence number space is 128 and 2048, corresponding to window sizes of 64 and 1048, respectively. The window size in turn sets the limit for the size of the RLC buffers, see [2].

- A mobile station supporting GPRS shall buffer up to 64 RLC data blocks.

- A mobile station supporting EGPRS shall buffer up to 1024 RLC data blocks.

In the interest to reduce complexity while at the same time maintain a sufficiently large window size to avoid protocol stalling it is proposed that in the context of Cellular IoT it is to reduce the RLC buffer size to 16 and thus window size and sequence number space to 16 and 32, respectively. This window size is seen as still being sufficient for the case of sending devices software updates since these are to be modeled as having a cutoff of 2000 octets (see Traffic characteristics for Software download/reconfiguration model in [5]).

A cellular IoT device shall support a RLC WS and RLC buffer size of 16.

## Compression of Ack/Nack bitmaps

Compression is used to compress the ACK/NACK bitmaps. Given that compression drives complexity and that the maximum Window size is 16 it is proposed that Cellular IoT devices do not need to support compression.

## Compression of Ack/Nack bitmaps are not supported

# RLC/MAC header design

## Header format

The RLC/MAC header format is the same as for EGPRS MCSs (i.e. same payload size, channel coding definitions and burst mapping). Due to reduced RLC functionality the content of the header is re-defined.

In Figure 1 and Figure 2 the format of the RLC/MAC header is shown for MCS-1-4 (Header Type 3) for the DL and UL respectively. In case an IoT device supports the full EGPRS MCSs set, the same design principles applies to Header Type 1 (MCS-7-9) and Header Type 2 (MCS-5-6).

| 8 | 7 |  | 6 | 5 | 4 | 3 | 2 |  | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TFI | CPS |  |  |  |  | USF |  |  |  | 1 |
| P |  |  | CC |  | TFI |  |  |  |  | 2 |
| SPB |  | BSN |  |  |  |  |  | Spare |  | 3 |
|  | Spare |  |  |  |  |  |  |  |  | 4 |

Figure 1. EGPRS downlink RLC data block header for MCS-1, MCS-2, MCS-3 and MCS-4.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | Octet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TFI |  | Countdown Value |  |  |  | SI | R | 1 |
| BSN |  |  |  |  | TFI |  |  | 2 |
| CPS |  | Spare |  |  |  |  |  | 3 |
|  | Spare |  |  | SPB |  | CPS |  | 4 |

Figure 2. EGPRS uplink RLC data block header for MCS-1, MCS-2, MCS-3 and MCS-4.

### Temporary Flow Identity (TFI)

The TFI field identifies the TBF and is formatted in the same position as in current EGPRS RLC/MAC Headers to provide backwards compatibility.

The TFI field is 5 bits long and hence can simultaneously address up to 32 devices multiplexed on the same resources. Due to the amount of devices expected in future CIoT scenarios, it can be considered to expand the TFI field beyond 5 bits, which in that case will be catered for by any number of spare bits indicated in Figure 2. Alternatively the TFI can be expanded by bit-wise modulo two addition of the extended TFI bits to the header parity bits, as currently supported by DLMC configurations.

### Polling (P)

In EGPRS, the RRBP field (2 bits) and the ES/P field (2 bits) is used when polling the MS.The current ES/P field indicates whether the MS addressed by the DL block is polled, and the type of bitmap (First Partial Bitmap, or Next Partial Bitmap) expected, as well as a request to include a measurement report in the response.The current RRBP indicates the resources reserved for the UL response in case the ES/P is valid.

For CIoT devices it is assumed:

- A few response times from the CIoT device could be useful to support

- Due to the small RLC WS of 16, the full bitmap, as well as a full measurement report, can always be included.

Hence a polling field (P) of two bits should be able to cater for this purpose.

### Block Sequence Number (BSN)

Due to the reduction of the EGPRS WS from at most 1024 to 16 the BSN field is consequently reduced from 11 bits to 5 bits.

### Coverage Class (CC)

A 2-bit field is included to signal the coverage class used in the block.

Due to the repetition of multiple instances of the same DL block, and due to the fact that a lower coverage class is always a partial mapping of a higher coverage class, see [4], the CC indication can be used by the lower coverage class CIoT devices monitoring the DL physical channel, i.e. in case CC indicates a higher coverage class, then the lower coverage class devices, need not monitor the rest of the repetition period.

For example, a device in CC1 monitors the DL, decodes the RLC/MAC header on TS0, with an indication that CC5 is used. It need then not to process the DL blocks on TS1,2,3 since a CC5 block will always be mapped onto TS0,1,2,3 in the same TTI, and can stop monitoring the second TTI period, see [4].

### Coding and Puncturing Scheme (CPS), Uplink State Flag (USF), Split Block (SPB), Stalling Indicator (SI), Retry (R)

The CPS, USF, SPB, SI and R fields are defined as in current GSM operation, see subclause 10.4 in [2].

### Countdown Value (CV)

The Countdown Value in current GSM specification indicates the current number of remaining RLC data blocks for the uplink TBF. Similarly for CIoT devices, a functionality is foreseen to cater for an open ended TBF functionality. I.e. for the device to indicate that the initial RLC block count in the access request shall be extended.

### Spare bits

In the RLC/MAC DL header, eight spare bits are defined for future use. The bits could for example cater for an extended TFI identifier space, as mentioned in Section 3.1.1.

In the RLC/MAC UL header, nine spare bits are defined for future use. Also for the UL, an extended TFI identifier space could be catered for.

# Conclusions

Based on the review of general RLC procedures for GSM devices that following is proposed to be applicable to EC-GSM devices:

- Only RLC Acknowledged mode is supported

- Only single TBF operation is supported

- Enhanced Multiplexing for Single TBF (EMST) is not supported

- Enhanced Multiplexing for Single RLC Entity (EMSR) is not supported.

- Extended uplink TBF mode is not supported

- Fast Ack/Nack reporting procedure not supported

- A cellular IoT device shall support a RLC WS and RLC buffer size of 16

- Compression of Ack/Nack bitmaps are not supported

A design of the RLC/MAC header has also been provided.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

3GPP TS 44.060: “GSM/EDGE Radio Access Network; Radio Link Control / Medium Access Control (RLC/MAC) protocol (v12.3.0)”

3GPP TS 43.064: “GSM/EDGE Radio Access Network;General Packet Radio Service (GPRS);Overall description of the GPRS radio interface; Stage 2 (v12.1.0)

GP-150055, “EC-GSM, Mapping of logical channels onto physical channels”, source Ericsson LM. GERAN Ad Hoc#1 on FS_IoT_LC.

GPC150009, 3GPP TR 45.820 V0.3.0 (2015-03), Source Vodafone, GERAN Ad Hoc#1 on FS_IoT_LC.