# EC-GSM, Sensitivity to frequency offset

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

There have been extensive discussions on how to model the frequency offset in the study on cellular IoT. A model was agreed at GERAN#64, captured in [2], where both a candidate specific offset, and a common, time dependent, frequency drift is to be applied.

In [3] the candidate specific offset for the EC-GSM candidate was presented, which is the resulting offset after fine frequency synchronization of the EC-SCH.

Comments were received at the first Ad Hoc on FS_IoT_LC that the fine frequency synchronization presented was very accurate, and if the candidate is relying on such fine offset to work at extreme coverage scenarios, it might be sensitive to frequency offset deviations that might appear in real operation (not modeled by the above mentioned model).

This paper is investigating the sensitivity to frequency offset in EC-GSM.

The paper is a re-submission of a paper at GERAN#65 with slight updates based on the discussion on minimum frequency offset to assume, common to different candidate technologies.

# Previous results

## EC-GSM specific frequency offset

In [3] the resulting frequency offset distribution after fine frequency synchronization of the EC-SCH was shown. The performance was simulated at the maximum coupling loss of 164 dB (20 dB coverage extension compared to legacy GPRS), and are re-produced in Figure 1.

Figure 1. Frequency estimation error after reading EC-SCH at 164 dB MCL (20 dB coverage improvement).

To take into account different impairments added in the real product, such as coming from TXCO precision, a minimum root mean square error of 10 Hz is assumed instead of the earlier proposed 3 Hz.

As can be seen, the normal distribution (red curve) to the estimated offset (blue curve) is not overestimating the performance seen from the simulations. The frequency estimator used in these simulations makes use of the known repetition of the EC-SCH channel allowing a simple correlator-based frequency offset estimator to be used.

## Coverage improvement target

In [4] the coverage improvement target for EC-GSM was evaluated and it was concluded that the 20 dB coverage improvement target of the study can be reached with the EC-GSM concept.

The simulations were using the candidate specific assumption on frequency offset as shown in Figure 1, the agreed drift model included in [2], applied to EC-GSM as described in [5].

The logical channels covered by these simulations were EC-BCCH, EC-CCCH/DL (EC-AGCH, EC-PCH), EC-CCCH/UL (EC-RACH). EC-PDTCH and EC-PACCH.

# Sensitivity to frequency offset in EC-GSM

At the Ad Hoc meeting #1 on FS_IoT_LC it was commented that the candidate specific performance of the frequency offset estimator was seen to be extremely accurate, and that this type of accuracy might not always be achieved in the real products where other aspects could add to the accuracy achieved. Hence, it was asked to investigate the sensitivity of the EC-GSM proposal to frequency error.

In the simulations produced below the same simulation assumptions as used in [4] is used with the modification to the assumed candidate specific frequency offset.

Table 1. Simulation assumptions on candidate specific frequency offset.

| Parameter | Value |
| --- | --- |
| MCS | EC-BCCH EC-CCCH/DL (EC-AGCH, EC-PCH) EC-PDTCH/UL and /DL EC-PACCH/UL and /DL |
| Candidate specific frequency offset1 | N(0,3) – Simulated offset N(0,10) – Proposed candidate specific offset N(0,20) N(0,30) N(0,40) N(845) |
| NOTE1: The offsets applied follow a normal distribution with mean ‘m’ and standard deviation (‘std’) – N(m,std) |  |

Also the EC-RACH (EC-CCCH/UL) is no longer included in the set of logical channels based on the agreement at GERAN Ad Hoc#1 that “The MCL methodology agreed in the TR in table 4.3-1 does not apply for logical channels relating to network synchronization and random access.”. Hence, the performance of the EC-RACH will be treated in other input to the GERAN discussions.

It could be noted that some of the different frequency offsets investigated, see Table 1 and Figure 2, should be seen rather pessimistic.

Figure 2. Different distributions of frequency offset simulated. Current max offset allowed in GSM in 900MHz, 90 Hz, shown with dashed black lines.

For example, for N(0,40) 2.4% of the realizations violate the current requirement on frequency offset accuracy of 0.1 ppm in the GSM specifications, see [6]. N(8,45) represents the frequency offset after coarse frequency synchronization, see [7], after FCCH reading but before EC-SCH reading, where the fine synchronization takes place.

The results are presented in a radar chart to make a more compact presentation of the performance. The MCL methodology in [2] has been followed, and is further elaborated in [4]. The results are presented in Figure 2 and are presented as the Maximum Coupling Loss (MCL) achieved for each logical channel. It can be noted that the highest MCL aimed at by the study is 164 dB.

Figure 3. EC-GSM - sensitivity to frequency offset

For PDTCH, the evaluation is done according to the description in [8] where a UL and DL throughput of 396.7 bps and 356.2 bps for the UL and DL respectively is achieved at the MCL of 164 dB for 90 % of the reports.

As can be seen, EC-GSM is not sensitive to the offset assumed. The only visible degradation is seen for EC-PACCH/DL and EC-CCCH/DL in the case of N(0,40) and N(8,45), which is, as stated above, as seen as rather pessimistic assumptions. Furthermore, the degradation seen in these conditions is still smaller than 1 dB.

# Conclusions

In this contribution the sensitivity of EC-GSM to the candidate specific frequency offset has been investigated. It is concluded that even with very pessimistic assumptions on offset (for example using the distribution after coarse frequency offset after FCCH), the degradation seen is limited to less than 1 dB. For more realistic distribution no degradation is visible.

# References

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

- 3GPP TR 45.820 v0.4.0

- GPC150064, “EC-GSM, EC-SCH design, performance and mapping”, source Ericsson LM. GERAN Ad Hoc#1 on FS_IoT_LC.

- GPC150056, “EC-GSM, Coverage improvement target”, source Ericsson LM. GERAN Ad Hoc#1 on FS_IoT_LC.

- GP-140885, “Frequency offset model for cellular IoT”, source Ericsson LM. GERAN#64.

- 3GPP TS 45.010, “Radio subsystem synchronization”

- GPC150089, “EC-GSM, FCCH overview”, source Ericson LM. GERAN Ad Hoc#1 on FS_IoT_LC.

- GPC150206, “EC-GSM, Modeling of HARQ transmissions for coverage improvement target”, source Ericsson LM. GERAN Ad Hoc #2 on FS_IoT_LC.