# EC-GSM, EC-PDTCH UL system capacity evaluation

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

One objective of the study is to “Scale to support a massive number of MTC Mobile Stations”. This includes providing sufficient capacity on the traffic channels.

In this contribution the system capacity of EC-GSM is evaluated on system level for the EC-PDTCH UL channel.

# Uplink EC-PDTCH

## Coverage classes

There are in total six coverage classes defined. These are also used by the system level simulations. It has still not been investigated if a lower number of coverage classes would be suitable on the EC-PDTCH.

The determination of coverage class is only based on the received signal strength. Further, this is assumed to be known by the mobile, i.e. in sensitivity limited scenario the estimation of the coverage class (or in this case signal strength) would be ideal. How sensitive the results are to estimation errors in coverage class is not covered in these simulations.

## Fixed allocation

The EC-GSM concept makes use of the concept of fixed UL allocation. In short, this implies that the device requests a limited number of resources in the random access attempt, and the network schedules resources by the use of EC-AGCH and/or EC-PACCH.

The fixed allocation in the performed system simulations is done in accordance with the EC-GSM concept. This means:

- Users that are in normal coverage will only be allocated one TS per TTI.

- Users in extended coverage performing two blind repetitions will be allocated two TS per TTI.

- Users in extended coverage performing four or more blind repetitions will be allocated four TS per TTI.

Furthermore, a user is not allowed to try to send its report for more than 20 seconds. If the report has not been transferred in 20 seconds the user will be kicked out from the simulation and logged as a “timed out user”. This maximum delay is currently arbitrarily chosen for these set of simulations.

# Simulations

## Simulation assumptions

The system level simulation assumptions in [2] have been followed. Other specific assumptions are shown in Table 1.

Table 1. Simulation assumptions, in addition to [2]

| Parameter | Value |
| --- | --- |
| Number of seeds | 1 |
| Simulation time | 100 s |
| System size | 192 cells |
| Direction | UL |
| Frequency band | 900 MHz |
| Frequency re-use on BCCH layer | 12 |
| EC-PDCH carriers per cell | 1 dedicated |
| Arrival rate per sector and second | 1, 5, 6.8, 9 |
| Fixed UL allocation | On |
| MCSs | MCS-4, MCS-1 |
| BTS antenna diversity | MRC |
| Minimum delay between subsequent transmissions (scaled per CC) | 3 radio blocks |
| Chase combining | Off |
| Incremental Redundancy | Off |
| Power control | Off |
| IP header compression | Off |
| Device output power | 33 dBm (100%) |
| BPL scenario | 1 |
| BPL inter-site correlation coefficient | 0.5 |
| Device time out | 20 seconds |

Table 2. MCS and number of repetitions per coverage class

| Coverage Class | Number of repetitions | MCS |
| --- | --- | --- |
| CC1 | 1 | MCS-4 |
| CC2 | 1 | MCS-1 |
| CC3 | 2 | MCS-1 |
| CC4 | 4 | MCS-1 |
| CC5 | 8 | MCS-1 |
| CC6 | 16 | MCS-1 |

The delay is defined from successful decoding of the channel request to successful reception of the whole report at the BTS. Note that this includes queuing effects in the scheduler.

## Results

The results presented are:

- Delay

  - A CDF for 6.8 users per cell and second, see Figure 1

  - The 95 percentile is plotted against the traffic load, see Figure 2.

- Resource utilization

  - This represents the average EC-PDTCH UL resources required per cell in the system, plotted against the traffic load, see Figure 3

- Failed attempts

  - This represents the percentage of the attempts that were not successful, i.e. did not manage to get the report through during 20 seconds

  - The percentage of failed attempts is plotted against the traffic load, see Figure 4

### Delay CDF

In Figure 1, the CDF over the delay for the traffic load 6.8 users per cell and second is shown. 95 % of the users have a delay less than 1.5 seconds.

Figure 1: CDF of delay for UL reports with 6.8 users per cell-second.

In Figure 2 the 95th percentile is plotted against the traffic load.

Figure 2: 95-percentile of delay for UL reports.

### Resource utilization

Figure 3 shows the resource utilization against the traffic load. At the traffic load 6.8 users per cell and second EC-GSM only utilizes approximately 1.3 time slots in average for EC-PDCH UL.

Figure 3: Timeslot utilization per cell and second.

### Failed attempts

The percentage of failed attempts (i.e., the report did not get delivered within 20 seconds) is shown in Figure 4. At the traffic load 6.8 users per cell and second less than 0.01 % do not get their report through. The total number of users passing through the system for this simulation is approximately 120 000.

Figure 4: Timed out users per cell and second.

# Further investigations

Further aspects that could to be evaluated for EC-PDTCH are:

- More aggressive building penetration loss model and inter-site correlation coefficient

- Non-optimal coverage class estimation

- Overlaid CDMA

- IRC

- Chase combining/incremental redundancy

- The use of MCS-2 and MCS-3

# Conclusions

The contribution has shown that:

- 95-percentile for delay is less than 1.5 seconds for the targeted cell load of 6.8 users per cell and second.

- Less than 0.1 % of the reports has timed out with 6.8 users per cell and second, with a timeout limit of 20 seconds.

- Only 1.3 EC-PDTCH UL timeslots per cell are needed in average to support the cIoT Traffic scenario with 6.8 users per cell and second.

In summary, the results look very promising regarding the EC-PDTCH capacity for EC-GSM.

# References

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

- GP-150317, TR 45.820 Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (Release 13), v1.0.0

- GPC150055, “EC-GSM, Mapping of logical channels onto physical channels”, source Ericsson LM. GERAN Ad Hoc # 1 on FS_IoT_LC.

Derived from traffic model in [2]. Note that 6.8 users per cell and second corresponds to the targeted number of devices per sector in the study.