C-UNB technology for Cellular IoT

-

Uplink Physical Layer Design

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. Various radio access technology proposals are candidates for this WI. One of the proposals in the Cooperative Ultra Narrow Band (C-UNB), for which key features are:

ultra-narrow band transmission,

random access,

cooperative reception.

An overall description of the C-UNB radio access technology can be found in [2]. The present document details the design of the C-UNB physical layer in uplink (UL). It is a revision of [3], where changes are marked in red.

# UL Technical design

## Bit rate

The uplink bit rate is set to 250bps. This value is compliant with the ultra narrow band feature of C-UNB; it allows meeting the 160bps requirement at SNDCP interface [1].

## Modulation

The proposed modulation for C-UNB downlink transmission is D-BPSK. It is chosen because of its robustness against noise and interferers. Differential modulation is implemented with a  phase shift for each bit 0 transmitted. Bit 1 keeps the phase constant.

## Transmission power

The transmission power is a key parameter for IoT devices because it sets most part or the power consumption of devices. Considering the very good selectivity and dynamic of C-UNB receivers in base stations, the Tx power in uplink is constant and set at +24dBm.

## Uplink frequency

In Cellular IoT, the objective is to re-use the legacy 200kHz blocks for IoT traffic. In usual cellular system, the center frequency of a channel is defined precisely and thus it requires costly TCXO or complex frequency correction features.

In the case of C-UNB technology, the legacy frequency block is used to carry uplink and downlink "ad'hoc micro-channels" that are ultra narrow band. The center frequency of an ad'hoc micro-channel is not precisely defined, provided it is within the 200kHz block allocated to CIoT. On the contrary, a device sets pseudo-randomly the center frequency for each UL PHY-PDU. The precise frequency detection and correction is done by the Software Defined Radio base station in uplink.

## Pulse shaping and spectrum mask

Amplitude of the transmitted signal is reduced during phase transition in order to keep the signal spectrum within tight boundaries, i.e. 500Hz. Pulse shaping filter is given in figure 1.

The obtained spectrum and spectrum mask are illustrated in figure 2.

Figure 1: Pulse shaping filter in UL

Figure 2: Actual spectrum and spectrum mask for UL

## Error correction code (ECC)

A 16 bit Reed-Solomon ECC protects each MAC-PDU. Given the max length of a MAC-PDU of 388 bits, this ECC adds a 1dB gain to the MCL with a limited complexity in the device. The most complex part of the Reed-Solomon code (i.e. the decoding part) is performed in the base station.

# Uplink MCL

Table 1 gives the maximum coupling loss (MCL) of the uplink transmission in C-UNB. Calculation is based on methodology given in subclause 5.1 of TR 36.888 [4]. Figures in table 1 are quite understandable, except for the SINR one that is explained here after.

Subclause 5.1 of TR 36.888 [4] states that SINR is evaluated with a TU1 path loss model. TUx path loss models have been proposed for modeling the influence of multipath propagation onto GSM radio signals that have a bandwidth of 200kHz and a modulation rate of 270kbps. In the case of ultra narrow band signals, the usual four effects of TU propagation models are mitigated the following ways:

the Doppler effect that shifts the center frequency of a message (about 25Hz at 30km/h): this frequency shift is totally managed by the base station because there is no channelization in C-UNB radio interface. Center frequencies are randomly selected by MSs, therefore a Doppler shift is not seen, in itself, by the base stations

the Doppler effect that creates a frequency drift within a received message : this is the case when the relative speed of a device is not constant. Such drifts are detected and corrected by the signal processing in C-UNB base stations

fast fading, which is critical in GSM, is not an issue for C-UNB, because of the 4ms symbol duration in C-UNB transmission

slow fading, which is due to interference between waves with a long term coherency, is usually overcome with multi-antenna reception. As C-UNB implements cooperative reception and large overlap of base station coverage, at least three different base stations receive each radio packet. Slow fading is, therefore, less critical than in GSM/GPRS system.

When putting all this together, we propose to set the SINR value to 8dB.

Table 1: Maximum Coupling Loss in C-UNB uplink

| Logical channel name | C-UNB uplink |
| --- | --- |
| Data rate (bps) | 250 |
| Transmitter |  |
| (1) Tx power (dBm) | +24 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 3 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 250 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -147 |
| (7) Required SINR (dB) | 8 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -139 |
| (9) Rx processing gain | 1 |
| (10) MCL = (1) (8) + (9) (dB) | +164 |

# Conclusion

This contribution details the design of the C-UNB uplink and shows how this radio access technology can meet requirements from Cellular IoT with very limited complexity in the device modems.

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia

[2]	GPC150364, "Cooperative ultra narrow band for Cellular IoT - General Description", SIGFOX, GERAN Ad'hoc meeting#3, 29 June-02 July 2015, Kista, Sweden

[3]	GPC150144, "C-UNB technology for Cellular IoT - Uplink Physical layer design", Sigfox, GERAN ad'hoc meeting, 20-23 April 2015, Sophia Antipolis, France

[4]	TR 36.888, "Study on provision of low cost Machine-Type Communication (MTC) User Equipment (UEs) based on LTE (v12.0.0)"