# NewToN –Working Assumptions

# Introduction

The work item on New Training Sequences for GERAN (work item code NewToN) was agreed in [1] at GERAN#60.

The following document contains the status of the working assumptions for the work.

The paper has been updated since the GERAN#61 closing plenary with an Annex describing the WoW to derive a final TSC.

Also, it has been noted that some updates to the agreed working assumptions are needed for clarification. The justification of the proposed modification is highlighted in yellow, but is to be removed from the working assumption if agreed.

# Proposed Working Assumptions

## Performance evaluation framework

| # | WA | Status |  | Related discussion paper(s) |
| --- | --- | --- | --- | --- |
|  |  | WG1 | WG2 |  |
| 1 | The final performance evaluation shall only be based on simulations using a commonly agreed framework | Agreed | - | GP-140192 |
| 2 | If a TSC set is proposed by a contributing company, performance evaluation is required for the proposed TSC set, and all other TSC sets proposed by other companies. | Agreed | - | GP-140192 |
| 3 | No more than one complete TSC set shall be proposed by each contributing company | Agreed | - | GP-140192 |
| 4 | Each company evaluating performance shall evaluate the performance using at least one receiver implementation expected in real network operation (which BTS and/or MS receiver architectures to use are not commonly agreed but up to each company performing the evaluation). Only one representative set of performance figures shall be derived from the receiver(s) simulated. Note: A chosen receiver implementation shall be used to evaluate all proposed TSC sets. | Agreed | - | GP-140192 |
| 5 | Each company evaluating performance shall evaluate the performance in at least one of: CS+EGPRS, or, CS+EGPRS+EGPRS2-A. Note: If only CS+EGPRS services are evaluated, the interfering modulation need not include rotated 16QAM(UL/DL) and 32QAM(DL) with a TSC included. | Agreed | - | GP-140192 |
| 6 | If the final performance figure (considering all evaluations from all companies) of the best TSC set (a complete TSC design from one company) is less than (<) 0.1 dB better than the second best TSC set, a TSC set is randomly chosen (by blind draw by the GERAN WG1 secretary) from all TSC sets whose final performance figure is less than 0.1 dB worse than the best TSC set. | Agreed | - | GP-140192 |
| 7 | The performance shall only be evaluated in the 900 MHz frequency band. | Agreed | - | GP-140192 |
| 8 | The different interferer/noise scenarios shall be investigated in propagation conditions TU50nFH (sensitivity and interference) and HT100nFH (sensitivity) | Agreed | - | GP-140192 |
| 9 | The performance shall be evaluated in: • Sensitivity (Auto correlation) • CCI (Cross correlation) • ACI at +200 kHz (Cross correlation) • ACI at -200 kHz (Cross correlation) | Agreed | - | GP-140192 |
| 10 | The non-ideal time synchronization model used for VAMOS UL shall apply only for the wanted signals in VAMOS UL simulations | Agreed | - | GP-140192 |
| 11 | The time shift models (separate models for CCI and ACI) as proposed in Table 1 shall be used in the performance evaluation with the delay applied independently per burst. | Agreed | - | GP-140192 |
| 12 | Wanted signal: Sensitivity: Performance is evaluated with the new TSC set assigned Interference: Performance is evaluated with the new TSC set assigned (both legacy TSC and new TSCs interfering) and with legacy TSC set assigned (only new TSCs interfering). | Agreed | - | GP-140192 |
| 13 | Interfering signal: All TSCs (CCI: All TSCs except the one assigned the wanted signal, ACI: All TSCs) are assumed to interfere each assigned wanted signal (including both legacy TSC set and new TSC sets for different modulations). Note: All legacy TSCs in this regard includes the normal burst TSCs defined in 3GPP TS 45.002 for NSR, as well as the dummy burst as defined in subclause 5.2.6. | Agreed | - | GP-140192 |
| 14 | All TSC combinations shall be evaluated at a raw BER level of 5% except for 16QAM and 32QAM where 1 % shall be used | Agreed | - | GP-140192 |
| 15 | The distance between two simulation points used for interpolation shall not be more than 2 dB | Agreed | - | GP-140192 |
| 16 | Each simulation point shall be simulated using at least 4000 bursts. | Agreed | - | GP-140192 |
| 17 | For a given TSC proposal, for each company evaluation: For each simulated carrier modulation, and, in case of interference simulations, interference type and interferer modulation, all intersection points (dB) are converted to linear values and averaged to arrive at a performance metric (dB). In case of sensitivity limited simulations the intersection points are averaged separately for HT and TU propagation profile, converted to dB, and weighted according to TU: 70%, HT: 30%, to arrive at a representative value for both propagation channels | Agreed | - | GP-140192 |
| 18 | For all TSC proposals, for each company evaluation: The dB-deviation of each proposed TSC set from the averaged performance of all TSC proposals is recorded for each carrier modulation and scenario simulated (see WA 17). | Agreed | - | GP-140192 |
| 19 | For a given TSC proposal, for each company evaluation: All carrier modulations (see WA 5) shall be evaluated in sensitivity. All carrier modulations (see WA5) excluding AQPSK, shall be evaluated in interference. | Agreed | - | GP-140192 |
| 20 | For a given TSC proposal, for each company evaluation: The carrier evaluation for VAMOS shall be simulated for - SCPIR=0 and -10 dB in case of VAMOS UL - SCPIR=0,-4 dB in case of VAMOS I MS on the DL - SCPIR=0,-4,-10 dB in case of VAMOS II or VAMOS III MS on the DL The performance need only be evaluated for one of the VAMOS sub-channels in case of SCPIR=0 dB and the weak sub-channel in case of negative SCPIR. The performance shall only be evaluated for the TU channel propagation profile. Only TU is specified for VAMOS in 3GPP TS 45.005 The performance at each SCPIR value shall be separately recorded and averaged according to WA17. The resulting 2 or 3 values (depending on the number of SCPIR) are then averaged to arrive at a single representative value for the different SCPIRs. Since different SCPIRs will provide quite different operating points, weighting all intersection points together will effectively result in higher weighting factor for low SCPIR values (high operating points). | Agreed | - | GP-140192 |
| 21 | For a given TSC proposal, for each company evaluation: AQPSK shall not be simulated as an interfering modulation. | Agreed | - | GP-140192 |
| 22 | For all TSC proposals, across different company evaluations: The derived performance figure for each carrier and interfering modulation, interference scenario (see WA 17 and WA 19) and TSC proposal from each contributing company shall be averaged (dB). | Agreed | - | GP-140192 |
| 23 | For all TSC proposals, across different company evaluations, interference simulations: The performance figures for all TSC proposals (see WA 22) shall be weighted depending on carrier modulation with: GMSK: 70%; 8PSK: 20%; 16QAM: 5%; 32QAM: 5%. | Agreed | - | GP-140192 |
| 24 | For all TSC proposals, across different company evaluations, interference simulations: The performance figures for all TSC proposals (see WA 22) for each carrier modulation shall be weighted across interfering modulations according to: GMSK: 70%; 8PSK: 20%; 16QAM: 5%; 32QAM: 5%. | Agreed | - | GP-140192 |
| 25 | For all TSC proposals, across different company evaluations, sensitivity simulations: The performance figures for all TSC proposals (see WA 22) shall be weighted depending on carrier modulation with: GMSK: 50%; VAMOS (DL: AQPSK, UL: paired GMSK): 20% 8PSK: 20%; 16QAM: 5%; 32QAM: 5%. | Agreed | - | GP-140192 |
| 26 | For all TSC proposals, across different company evaluations: The different propagation profiles and scenarios shall be weighted according to: Sensitivity: 25%; CCI: 60%; ACI-: 7.5%, ACI+: 7.5%. | Agreed | - | GP-140192 |

## Other

| # | WA | Status |  | Related discussion paper(s) | Impacted specification |
| --- | --- | --- | --- | --- | --- |
|  |  | WG1 | WG2 |  |  |
| 27 | To avoid any resource segregation, NewToN shall allow for an optional assignment of a second TSC (DL TBF) to the MS. | Agreed | Agreed | GP-140141 | 44.060 44.018 |

# References

GP-131139, “Work Item Description: New Training Sequence Codes for GERAN”, source Telefon AB LM Ericsson, Telecom Italia S.p.A., China Mobile Com. Corporation, NOKIA Corporation, Alcatel-Lucent, Vodafone Group, Com-Research GmbH. GERAN#60.

# Annex

This Annex contains an illustrative example on how three different companies contribute to the final performance evaluation of NewToN. Three companies contributing with TSC sets means that three sets shall be evaluated since each company can propose at most one TSC set (WA3).

It should be noted that not only the companies proposing TSC sets need to be the ones contributing to the final evaluation. For example, company D need not have proposed a TSC set but still contributes to the evaluation. This scenario is however not covered in the example below, but would be a straightforward extension to cater for.

The three companies are: A, B and C.

Company A evaluates all modulations while company B only evaluates GMSK and 8PSK carrier but all interfering modulation, and company C only GMSK and 8PSK as both carrier and interfering modulations (WA5).

Company B on the other evaluates two different receiver architectures, but will, according to WA4 only contribute with one set of performance figures.

## The evaluation for each company

The evaluation procedure for each company is illustrated by the co-channel interference (CCI) evaluation by company A.

First all modulation combinations (carrier and interferer) are simulated to arrive at an average performance figure for the TSC set from company A for each combination (table below).

The procedure is then repeated for all proposed TSC sets (WA2). In this case it is assumed that there are three sets in total from company A, B and C.

The table derived will contain relative performance metrics for each proposed TSC set for each modulation combination (carrier and interferer) relative to the average performance of all sets (WA18). This is the table that is provided to GERAN for the final evaluation.

The evaluation is performed also for ACI+ and ACI-, in addition to CCI (as described above. WA9).

Also sensitivity is evaluated, and in this case VAMOS is also included in the evaluation as carrier (AQPSK on DL and 2 multiplexed GMSK channels on UL, see WA19). In this case different SCPIR values are also used to derive at the VAMOS performance figure (WA20).

For company B that evaluates two different receiver architectures, only one set of tables is provided to GERAN (average of the two receivers), see WA4.

The set of tables provided to GERAN for the final performance evaluation will be as shown below.

## The evaluation of all companies

The final tables in Section 4.1 are averaged to arrive at a common performance table (WA22) for the evaluation by all companies. Since not all companies have evaluated all modulation combinations, there will be different number of samples constituting the value in each cell.

All performance figures are then weighted according to WA23, WA24, WA25 and WA26 to arrive at the final performance.

If the final performance figure of the best TSC set is less than (<) 0.1 dB better than the second best TSC set, a TSC set is randomly chosen (by blind draw by the GERAN WG1 secretary) from all TSC sets whose final performance figure is less than 0.1 dB worse than the best TSC set (WA6).