C-UNB architecture

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. One of the radio access technology candidates is the Cooperative Ultra Narrow Band (C-UNB), for which key radio features are: ultra-narrow band transmission, random access and cooperative reception.

An overall description of the C-UNB radio access technology can be found in [2], whereas main principles of the C-UNB radio resource management are depicted in [3]. This contribution details how a C-UNB radio architecture can be deployed over a GSM/GPRS legacy architecture.

Note: the present contribution assumes a Gb architecture for the legacy cellular architecture on which C-UNB is deployed. C-UNB is expected to be compatible with S1 architecture also.

# C-UNB architecture

## Requirements

In C-UNB, the de-duplication of UL messages implies a communication between base stations in the data plane, in order to forward received copies of a UL message to the BTS in charge of de-duplication (see figure 1). This feature can hardly be implemented in a legacy RAT, such as the GSM/GPRS, because of the tree shape of the GSM/GPRS architecture; no direct data plane exists between bases stations.

Therefore, the C-UNB de-duplication requires a central equipment to be deployed in the core network but with minimal impact on the legacy Gb architecture.

Figure 1: Data connection required by cooperative reception

## Architecture elements

The proposed C-UNB architecture consists in three elements: C-UNB server, C-UNB software and MS avatars (see figure 2). Together they build a C-UNB RAT compliant with the Gb architecture.

Figure 2: Elements of a C-UNB architecture

The C-UNB server is a central server that mainly performs message de-duplication in UL and micro-channel frequency evaluation for DL messages. The C-UNB server terminates the C-UNB protocol layers used by IoT devices and makes the gateway for user's IP packets towards the Internet. The C-UNB server can be on the Internet or, if connected with a Gp/Gn interface, it can be seen as a GGSN node.

The C-UNB software is deployed in each C-UNB base station. It is split into two parts. The first part, called C-UNB agent, manages the C-UNB air interface. The second part of the C-UNB software is called Agent-Avatar. It is a mobile station emulator that hides the C-UNB agent to the Gb architecture. The C-UNB agent is seen as a common MS by the Gb architecture: control plane is managed locally by the Agent Avatar, whereas data plane is re-routed to the C-UNB server, by means of a standard PDP-context established between the Agent-Avatar and the C-UNB server (see figure 3).

Figure 3: C-UNB software and C-UNB server protocol stacks and connection

The last part of the compatibility architecture consists in mobile station avatars. They act as mobile station emulators for the control plane and as gateway for the data plane.

Each IoT device, which complies with C-UNB RAT, has an MS-Avatar, which is hosted by the C-UNB server. Figure 4 depicts the corresponding protocol stack.

Figure 4: C-UNB protocol stack in C-UNB device and C-UNB server

An MS-Avatar of a given C-UNB device is attached to a base station and seen as such by the core network.

Control exchanges between the core network and the IoT device are re-routed by the Agent-Avatar corresponding to the base station, where the IoT device is supposed to be attached (figure 5a).

Data exchange between the IoT device and the Internet are forwarded by Agent-Avatars to the C-UNB server and then to the Internet (see figure 5b).

Figure 5a: MS control plane re-routing in C-UNB architecture

Figure 5b: Data plane re-routing in C-UNB architecture

## SMS service

Document [1] requires that SMS can be delivered to IoT devices. This service, which is part of the signaling layer in legacy GSM/GPRS architecture, requires specific care in the C-UNB architecture. Fore a given MS, the SMS layer starts in the corresponding SGSN, goes to the BTS where the MS is supposed to be attached and then it is tunneled by the Agent-Avatar towards the MS-Avatar hosted by the C-UNB server. The SMS is stored by the C-UNB server and transmitted in the C-UNB data plane to the corresponding IoT device. Thus two PDP-contexts are involved in the SMS delivery process: one PDP-context to carry the SMS from the base station, where the C-UNB device is supposed to be, to the C-UNB server, and one PDP-context between the C-UNB server and the selected base station for carrying the SMS over the air. This creates a tromboning as depicted in figure 6.

Figure 6: SMS delivery creating a tromboning in C-UNB architecture.

# Conclusion

This contribution explains how C-UNB RAT can be connected to the Gb architecture and how various PDP-context used to tunneling C-UNB streams can make C-UNB compatible with legacy GSM/GPRS architecture.

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia

[2]	GPC150052, "Cooperative ultra narrow band technology for Cellular IoT", Sigfox, GERAN Ad'hoc meeting, 02-05 Feb'15, Sophia Antipolis

[3]	GPC150146, "C-UNB technology for Cellular IoT - Radio Resource Management", Sigfox, GERAN Ad'hoc meeting, 20-23 Apr'15, Sophia Antipolis