# EC-GSM, MAR Periodic and Network Command system capacity evaluation 
(update of GPC150622)

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

One objective of the study is to “Scale to support a massive number of MTC Mobile Stations”. This includes providing sufficient capacity on the traffic channels.

In this contribution the system capacity and latency of EC-GSM is evaluated on system level for the MAR Periodic traffic model and Network Command traffic model.

This document is an update of [4]. The updates include:

- Non-ideal cell selection and coverage class estimation

- DL application acknowledgment latency evaluation

- All four combinations of building penetration loss scenario and inter-site correlation coefficient evaluated

- Random access delay CDFs from initiated access until contention has been resolved from the perspective of the device, for one phase access.

- System capacity according to definition in [2] is provided.

# EC-GSM basics

## Coverage classes

There are in total six coverage classes defined. These are also used by the system level simulations. The determination of coverage class is based on the received signal strength. Estimation errors of the received signal strength are modeled, see further section 4.1.3.

## Fixed allocation

The EC-GSM concept makes use of fixed allocation in the uplink. In short, this implies that the device requests a limited number of resources in the random access attempt, and the network schedules resources by the use of EC-AGCH and/or EC-PACCH.

The fixed allocation in the performed system simulations is done in accordance with the EC-GSM logical channels for EC-PACCH and EC -PDTCH. This implies:

- Users that are in normal coverage will only be allocated one TS per TTI.

- Users in extended coverage performing two blind repetitions will be allocated two TS per TTI.

- Users in extended coverage performing four or more blind repetitions will be allocated four TS per TTI.

# Traffic generation

Traffic is generated according to the MAR periodic reporting and Network Command traffic models [2]. The split between these is 80 % MAR periodic and 20 % Network command.

## MAR periodic reporting

MAR periodic reports are sent in UL. The inter-arrival times and message size distribution described in [2] has been used in the simulations. 50 % of the MAR periodic reports result in a DL Application ACK with application payload size of 0 bytes sent immediately after the base station successfully receives the UL packet (i.e., no delay assumed in CN, application, etc).

## Network Command

The Network Commands are sent in DL. They have the same inter-arrival times as the MAR periodic reporting [2] and an application payload size of 20 bytes. 50 % of the Network Commands result in an UL Application Response. The application payload size distribution for the response is the same as for the MAR periodic reporting.

# Simulations

## Simulation assumptions

The system level simulation assumptions in [2] have been followed. Other specific assumptions are shown in Table 1.

### System parameters

Table 1. Simulation assumptions, in addition to [2]

| Parameter | Value |
| --- | --- |
| General |  |
| Simulation time | 100 s |
| System size | 192 cells |
| Direction | UL and DL |
| Frequency band | 900 MHz |
| Layer | BCCH |
| Frequency re-use | 12 |
| BTS antenna diversity | MRC |
| BTS output power | 43 dBm |
| CIoT parameters |  |
| EC-PDTCH timeslots per cell | 4 shared with legacy PDTCH(note 1) |
| CIoT arrival rate per cell and second | 1, 5, 6.8(note 2), 9 |
| Fixed UL allocation | On |
| MCSs | MCS-4, MCS-1 |
| Minimum delay between subsequent transmissions on EC-PDTCH and EC-PACCH (scaled per CC) | 1 radio block |
| Chase combining | On |
| Incremental Redundancy | Off |
| Power control | Off |
| IP header compression | Off/On |
| Device output power | 23, or 33 dBm (100%) |
| BPL model(note 3) | 100 % CIoT devices subject to BPL |
| BPL scenario and inter-site correlation | Scenario 1, correlation 0.50 Scenario 1, correlation 0.75 Scenario 2, correlation 0.50 Scenario 2, correlation 0.75 |
| Device timeout | 40 seconds |
| CS parameters |  |
| CS timeslots per cell | 4 dedicated(note1, note4) |
| CS load | 4 E per cell |
| CS output power | 43 dBm in DL, max 33 dBm in UL |
| CS power control | Off in DL, on in UL |
| DTX | Off in DL, on in UL |
| BPL model | CS devices are not subject to BPL |
| NOTE 1: The allocation of the CS and IoT timeslots in the TDMA frame are randomly offset per cell to model an unsynchronized network. NOTE 2: Derived from traffic models in [2]. 6.8 reports/commands per cell and second corresponds to the targeted number of devices per sector in the study. NOTE 3: According to model in [2]. NOTE 4: CS are put on timeslots 0 – 1 to model BCCH and RACH. |  |

### Coverage classes

Table 2 gives the details of the coverage classes.

Table 2. MCS and number of repetitions per coverage class

| Coverage Class | Number of blind transmissions for PDTCH and PACCH | MCS for PDTCH |
| --- | --- | --- |
| CC1 | 1 | MCS-4 |
| CC2 | 1 | MCS-1 |
| CC3 | 2 | MCS-1 |
| CC4 | 4 | MCS-1 |
| CC5 | 8 | MCS-1 |
| CC6 | 16 | MCS-1 |

### Cell selection and coverage class estimation

Based on the findings in [7] the error in the signal strength estimation can be modeled by a normal distribution with standard deviation of 4 dB. This is modeled by applying an independent estimation error, according to N(0,4 dB), to each base station. This implies that some users will not select the optimum serving cell, and also not the most appropriate coverage class (the one that minimizes resource utilization). That is, some cells will appear stronger or weaker than they actually are. The device always selects what is believed to be the strongest cell. Effectively this increases the interference levels in the network, as well as the resource utilization.

### Control signaling

Packet uplink ACK/NACK (PUAN) is sent on EC-PACCH/D to (negatively) acknowledge data sent in the UL and assign fixed allocations to the MS. In the simulations its performance is pessimistically modeled with MCS-1 using the same number of blind transmissions as EC-PDTCH/D (but without HARQ retransmissions). If a PUAN with a fixed UL allocation assignment is unsuccessfully received the corresponding fixed UL allocation will be silent, i.e. the allocated MS will not transmit anything, but the radio block resources are consumed.

Packet downlink ACK/NACK (PDAN) is sent on EC-PACCH/U to (negatively) acknowledge data sent in the DL. In the simulations its performance is pessimistically modeled with MCS-12 using the same number of blind transmissions as EC-PDTCH/U (but without HARQ retransmissions).

### Circuit switched users

The CS users are used as interferers on the remaining 4 TS on the used UL and DL carrier. In UL, power control and DTX are used, according to common assumptions, see [3]. The maximum UL output power for CS users is 33 dBm in all scenarios. For DL, power control is not used since the BCCH layer is modelled.

No BPL is applied to the CS users.

### Simulated scenarios

Table 3 summarizes the simulated scenarios and clarifies the legends in the figures presented in section 4.2.

Table 3: Simulated scenarios

| Legend text | CIoT output power [dBm] | IP Header Compr. | BPL scenario | BPL inter-site correlation coefficient |
| --- | --- | --- | --- | --- |
| 33 dBm BPL scenario 1/0.5 | 33 | Off | 1 | 0.5 |
| 33 dBm BPL scenario 1/0.75 | 33 | Off | 1 | 0.75 |
| 33 dBm BPL scenario 2/0.5 | 33 | Off | 2 | 0.5 |
| 33 dBm BPL scenario 2/0.75 | 33 | Off | 2 | 0.75 |
| 33 dBm BPL scenario 1/0.5 IPc | 33 | On | 1 | 0.5 |
| 33 dBm BPL scenario 1/0.75 IPc | 33 | On | 1 | 0.75 |
| 33 dBm BPL scenario 2/0.5 IPc | 33 | On | 2 | 0.5 |
| 33 dBm BPL scenario 2/0.75 IPc | 33 | On | 2 | 0.75 |
| 23 dBm BPL scenario 1/0.5 | 23 | Off | 1 | 0.5 |
| 23 dBm BPL scenario 1/0.75 | 23 | Off | 1 | 0.75 |
| 23 dBm BPL scenario 2/0.5 | 23 | Off | 2 | 0.5 |
| 23 dBm BPL scenario 2/0.75 | 23 | Off | 2 | 0.75 |
| 23 dBm BPL scenario 1/0.5 IPc | 23 | On | 1 | 0.5 |
| 23 dBm BPL scenario 1/0.75 IPc | 23 | On | 1 | 0.75 |
| 23 dBm BPL scenario 2/0.5 IPc | 23 | On | 2 | 0.5 |
| 23 dBm BPL scenario 2/0.75 IPc | 23 | On | 2 | 0.75 |

### Failed attempts

A timeout function is implemented in the simulator that cancels a transfer if it last for more than 40 seconds. In this case, the attempt to transfer the report is considered to have failed.

## Results

The results presented are:

- Latency of MAR periodic reports

  - The latency includes time to synchronize to the network, time for random access and time to transfer the message.

  - Network synchronization and random access are evaluated in separate simulations [5][6]. Delays from these are added in post-processing based on latency CDFs per coverage class for network synchronization time and random access time, respectively. For the random access delay, use of access burst has been assumed.

  - The results are presented as CDFs of the delay at the target traffic load (6.8 users per cell and second), see Figure 1 and Figure 2.

  - Failed attempts are not included in the statistics (following the agreed methodology).

- Latency of DL application Ack

  - Latency is measured from the time an application layer DL ACK is received at the base station till the time when the device has successfully received the application layer DL ACK

  - The results are presented as CDFs of the delay at the target traffic load (6.8 users per cell and second), see Figure 3 and Figure 4.

- Random access delay

  - According to the definition of random access delay in [2], it is measured until the point in time where contention is resolved from the perspective of the device. In case an access burst is used on EC-RACH (i.e., one phase access), contention is resolved when the first PUAN containing a valid TLLI is received by the MS, which for EC-GSM will typically happen after the first fixed allocation has ended. In other words, the random access delay, as defined in [2], includes both time for the actual random access procedure and part of the data transfer.

  - Random access performance is evaluated in separate simulations [5]. Random access delays from these are added in post-processing based on latency CDFs per coverage class.

  - CDFs of the random access delay in case of one-phase access are presented in Figure 5 and Figure 6.

- Resource utilization

  - This represents the average amount of EC-PDTCH UL and DL resources required per cell in the system, plotted against the traffic load, see Figure 7, Figure 8, Figure 9 and Figure 10.

- Failed attempts

  - This represents the percentage of the attempts that were not successful, i.e. did not manage to get the report through during 40 seconds.

- Uplink capacity

  - Uplink capacity is defined as “spectral efficiency in number of reports/200 kHz/hour”. Results are shown in Figure 13 and Figure 14.

### Latency of MAR periodic reports

In Figure 1 and Figure 2, the CDF over the delay for the traffic load 6.8 users per cell and second are shown.

Figure 1: CDF of latency for MAR periodic reports with 6.8 users per cell-second, 33 dBm CIoT MS output power.

Figure 2: CDF of latency for MAR periodic reports with 6.8 users per cell-second, 23 dBm CIoT MS output power.

### Latency of DL Application Ack

In Figure 3 and Figure 4, latency CDFs for DL Application ACK are shown.

Figure 3: CDF of latency for DL Application Ack with 6.8 users per cell-second, 33 dBm CIoT MS output power.

Figure 4: CDF of latency for DL Application Ack with 6.8 users per cell-second, 23 dBm CIoT MS output power.

### Random access delay

Figure 5 and Figure 6 show the random access delay in case of one-phase access.

Figure 5: CDF of random access delay with 6.8 users per cell-second, 33 dBm CIoT MS output power.

Figure 6: CDF of random access delay with 6.8 users per cell-second, 23 dBm CIoT MS output power.

### Resource utilization

Figure 7, Figure 8, Figure 9 and Figure 10 show the resource utilization for the CIoT users against the CIoT traffic load.

Figure 7: Timeslot utilization in UL per cell, 33 dBm CIoT MS output power.

Figure 8: Timeslot utilization in UL per cell, 23 dBm CIoT MS output power.

Figure 9: Timeslot utilization in DL per cell, 33 dBm CIoT MS output power.

Figure 10: Timeslot utilization in DL per cell, 23 dBm CIoT MS output power.

### Failed attempts

At the traffic load 6.8 users per cell and second, the percentage of failed attempts (i.e., the report did not get delivered within 40 seconds) is found to be less than 1.5 % in all simulated scenarios.

### Uplink capacity

In the TR [2] capacity is defined as “spectral efficiency in number of reports/200 kHz/hour. This definition is made with a standalone CIoT system in mind. Since EC-GSM is embedded in a legacy GSM network, in which only a fraction of the traffic is CIoT traffic, it does not make much sense to measure the system capacity using that definition. Measuring only the delivered CIoT traffic in relation to the total amount of radio resources used by the system is misleading since most of the radio resources are used by other services. Nevertheless, the defined system capacity is shown in Figure 13 and Figure 14 for completeness.

Figure 13: Uplink capacity, 33 dBm MS output power.

Figure 14: Uplink capacity, 23 dBm MS output power.

### Summary

The simulation results are summarized in Table 4.

Table 4: Summary of results

| CIoT output power [dBm] | IP Header Compr. | BPL scenario/ inter-site correlation coefficient | 95th percentile latency MAR Periodic [s] | 95th percentile latency DL App Ack [s] | 95th percentile random access delay [s] | Average Resource Utilization UL [TS] | Average Resource Utilization DL [TS] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | Off | 1/0.5 | 1.1 | 0.37 | 0.31 | 0.7 | 0.5 |
| 33 | Off | 1/0.75 | 1.2 | 0.32 | 0.38 | 0.8 | 0.5 |
| 33 | Off | 2/0.5 | 1.3 | 0.38 | 0.47 | 0.8 | 0.5 |
| 33 | Off | 2/0.75 | 1.5 | 0.37 | 0.62 | 0.9 | 0.5 |
| 33 | On | 1/0.5 | 1.0 | 0.39 | 0.28 | 0.5 | 0.3 |
| 33 | On | 1/0.75 | 1.1 | 0.29 | 0.34 | 0.5 | 0.3 |
| 33 | On | 2/0.5 | 1.1 | 0.35 | 0.38 | 0.6 | 0.4 |
| 33 | On | 2/0.75 | 1.3 | 0.29 | 0.48 | 0.6 | 0.4 |
| 23 | Off | 1/0.5 | 1.3 | 0.36 | 0.52 | 0.9 | 0.5 |
| 23 | Off | 1/0.75 | 1.7 | 0.34 | 0.84 | 1.1 | 0.5 |
| 23 | Off | 2/0.5 | 2.0 | 0.44 | 1.1 | 1.2 | 0.5 |
| 23 | Off | 2/0.75 | 4.3 | 0.70 | 2.5 | 1.7 | 0.6 |
| 23 | On | 1/0.5 | 1.1 | 0.36 | 0.38 | 0.6 | 0.4 |
| 23 | On | 1/0.75 | 1.3 | 0.25 | 0.54 | 0.8 | 0.4 |
| 23 | On | 2/0.5 | 1.5 | 0.29 | 0.70 | 0.9 | 0.4 |
| 23 | On | 2/0.75 | 2.8 | 0.19 | 1.4 | 1.2 | 0.4 |

# Conclusions

In this contribution the system performance of EC-GSM data traffic channels has been investigated in a mixed traffic scenario (CIoT and legacy CS). Performance at the target CIoT traffic load of 6.8 users per cell and second is summarized in Table 4 above.

E.g., with an MS output power of CIoT devices of 33 dBm in BPL scenario 2 with correlation 0.75, without IP header compression, and with interfering CS users with a traffic load of 4 Erlang per cell:

- The 95th percentile latency of MAR periodic reports is 1.5 seconds

- The 95th percentile latency of DL Application Ack is 0.37 seconds

- Only 0.9 EC-PDTCH UL and 0.5 EC-PDTCH DL timeslots per cell are needed on average to support the CIoT traffic

# References

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

- TR 45.820 V1.4.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (Release 13)”

- 3GPP TR 45.914, “Circuit switched voice capacity evolution for GERAN“

- GPC150622, “EC-GSM, EC-PDTCH UL system capacity evaluation”, source Ericsson LM. GERAN Ad Hoc #3 on FS_IoT_LC.

- GP-150760, “EC-GSM, Random access system capacity evaluation (update of GPC150491)”, source Ericsson LM. GERAN#67.

- GPC150437, “EC-GSM, Network synchronization with alternative EC-SCH design (update of GPC150198)”, source Ericsson LM. GERAN Ad Hoc #3 on FS_IoT_LC.

- GP-150431, “EC-GSM, DL signal level and coverage class estimation”, source Ericsson LM. GERAN#66.

This applies only to the UL since a BCCH carrier is modelled in these simulations.

L2S for EC-PACCH has not been implemented.